3GPP TSG-RAN WG1 Meeting #114 	R1-2308687

Toulouse, France, August 21 – 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0143 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 editorial corrections for TS 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, NR_IIOT-Core, NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Correction on higher-layer parameter per the outcome of agenda 7.1 in RAN1#114 meeting. The current description of the CBGTI field refers to the Rel-15 RRC parameter codeBlockGroupTransmission, which does not include the case of 0 bit for the Rel-16 RRC parameter for the two HARQ codebooks with different PHY priorities provided in pdsch-CodeBlockGroupTransmissionList. - Correction on the field of frequency domain resource assignment in DCI format 4_2 per the outcome of agenda 7.2 in RAN1#114 meeting. The description of the size of the common frequency resource doesn't cover the case of MBS frequency resource determined by DL BWP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Generalize the description of the 0 bit CBGTI condition to PDSCH-CodeBlockGroupTransmission instead of codeBlockGroupTransmission to also cover the case of the Rel-16 pdsch-CodeBlockGroupTransmissionList. - Update the description of the size of the common frequency resource to refer to the definitions in Clause 18 of TS38.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.2.2, 7.3.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell.

< Unchanged parts are omitted >

-	SRS offset indicator – 0, 1 or 2 bits.

-	0 bit if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperiodic SRS resource set in the scheduled cell  and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214],  where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

-	CBG transmission information (CBGTI) – 0 bit if higher layer parameter PDSCH-CodeBlockGroupTransmission for PDSCH is not configured, otherwise, 2, 4, 6, or 8 bits as defined in Clause 5.1.7 of [6, TS38.214], determined by the higher layer parameters maxCodeBlockGroupsPerTransportBlock and maxNrofCodeWordsScheduledByDCI for the PDSCH.

If higher layer parameter priorityIndicatorDCI-1-1 is configured, if the bit width of the CBG transmission information in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the CBG transmission information in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller CBG transmission information until the bit width of the CBG transmission information in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

-	CBG flushing out information (CBGFI) – 1 bit if higher layer parameter codeBlockGroupFlushIndicator is configured as "TRUE", 0 bit otherwise.

If higher layer parameter priorityIndicatorDCI-1-1 is configured, if the bit width of the CBG flushing out information in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the CBG flushing out information in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller CBG flushing out information until the bit width of the CBG flushing out information in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

< Unchanged parts are omitted >

##### 7.3.1.5.3	Format 4_2

DCI format 4_2 is used for the scheduling of PDSCH for multicast in DL cell.

The following information is transmitted by means of the DCI format 4_2 with CRC scrambled by G-RNTI for multicast or G-CS-RNTI configured by MBS-RNTI-SpecificConfig:

-	Frequency domain resource assignment – number of bits determined by the following, where NRBDL,CFR is the size of the common frequency resource as defined in Clause 18 of [5, TS38.213]:

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 5.1.2.2.1 of [6, TS38.214],

-	log2(NRBDL,CFR(NRBDL,CFR+1)2) bits if only resource allocation type 1 is configured, or

-	maxlog2(NRBDL,CFR(NRBDL,CFR+1)2),NRBG+1 bits if resourceAllocation in pdsch-ConfigMulticast is configured as 'dynamicSwitch'.

-	If resourceAllocation in pdsch-ConfigMulticast is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 5.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the log2(NRBDL,CFR(NRBDL,CFR+1)2) LSBs provide the resource allocation as defined in Clause 5.1.2.2.2 of [6, TS 38.214]

< Unchanged parts are omitted >