3GPP TSG-RAN WG2 Meeting #123	R2-2309123

# Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0701 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous stage 2 corrections for sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2023-08-23 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 38.300 there are several inconsistencies for sidelink that would need to be corrected in order to avoid any misleading behaviour by the UE or the network. This CR is to correct such inconsistencies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 16.9.2.6 - Clarified that SL RRC decides measurements for sidelink. Section 16.9.3.1 - Fix identation on the last bulled of the section Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: Sidelink Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. 3. If the TX UE implements the CR and the RX UE does not, there is no inter-operability issue. 4. If the RX UE implements the CR and the TX UE does not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approve, inconsistencies about sidelink will remain in the specification and this may cause some uncertain behaviour at the UE and/or the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.9.2.6, 16.9.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 0: initial revision captured in R2-2308432 Revision 1: remove changes which were not agreed in [AT123][501][V2X/SL] SL 38.300 corrections (Ericsson) |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 16.9.2.6	RRC

The RRC sublayer provides the following services and functions over the PC5 interface:

-	Transfer of a PC5-RRC message between peer UEs;

-	Maintenance and release of a PC5-RRC connection between two UEs;

-	Detection of sidelink radio link failure for a PC5-RRC connection;

-	Measurement configuration and reporting related to sidelink.

A PC5-RRC connection is a logical connection between two UEs for a pair of Source and Destination Layer-2 IDs which is considered to be established after a corresponding PC5 unicast link is established as specified in TS 23.287 [40]. There is one-to-one correspondence between the PC5-RRC connection and the PC5 unicast link. A UE may have multiple PC5-RRC connections with one or more UEs for different pairs of Source and Destination Layer-2 IDs.

Separate PC5-RRC procedures and messages are used for a UE to transfer UE capability and sidelink configuration to the peer UE, as specified in TS 38.331 [3]. Both peer UEs can exchange their own UE capability and sidelink configuration using separate bi-directional procedures in both sidelink directions.

If it is not interested in sidelink transmission, if sidelink RLF on the PC5-RRC connection is declared, or if the Layer-2 link release procedure is completed as specified in TS 23.287 [40], UE releases the PC5-RRC connection.

END OF CHANGES

START OF CHANGES

#### 16.9.3.1	General

For NR sidelink communication, the UE can operate in two modes as specified in 5.7.2 for resource allocation in sidelink:

-	Scheduled resource allocation, characterized by:

-	The UE needs to be RRC_CONNECTED in order to transmit data;

-	NG-RAN schedules transmission resources.

-	UE autonomous resource selection, characterized by:

-	The UE can transmit data when inside NG-RAN coverage, irrespective of which RRC state the UE is in, and when outside NG-RAN coverage;

-	The UE autonomously selects transmission resources from resource pool(s).

For NR sidelink communication, the UE performs sidelink transmissions only on a single carrier.

END OF CHANGES