3GPP TSG-RAN WG2 Meeting #123	R2-2309299

# Toulouse, August 21th – 25th 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4248 | rev | 2 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the PLMN check for the reconnectCellID in the RLF report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per the current specification, the UE records the reconnectCellID if the reconnection occurs in the same PLMN or in a PLMN which is part of the stored plmn-IdentityList in the RLF report. The intention of the reconnectCellID is to capture the first cell in which the UE reconnects after the RLF/HOF was declared by the UE and failing in the subsequent reestablishment. This scenario is further depicted in Figure 1 1> if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report: 2> if reconnectCellId in VarRLF-Report is not set: 3> set timeUntilReconnection in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure; 3> set nrReconnectCellId in reconnectCellId in VarRLF-Report to the global cell identity and the tracking area code of the PCell; 1> if the UE supports RLF report for inter-RAT MRO NR as defined in TS 36.306 [62], and if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]: 2> if reconnectCellId in VarRLF-Report of TS 36.331[10] is not set: 3> set timeUntilReconnection in VarRLF-Report of TS 36.331[10] to the time that elapsed since the last radio link failure or handover failure in LTE; 3> set nrReconnectCellId in reconnectCellId in VarRLF-Report of TS 36.331[10] to the global cell identity and the tracking area code of the PCell; Figure 1: Expected scenario when the reconnectCellID inclusion is RLF report is helpful However, the reconnectCellID included in the following scenario is not as per the expectation listed in Observation 2. Consider the following scenario where the cell in which the UE reconnects to belongs to a different PLMN than the PLMN in which the UE declared RLF/HOF. Figure 2: Scenario in which the reconnectCellID included in the RLF report is not as per Observation 2. In the scenario depicted in Figure 2, the UE comes back to connected in eNB1 (or can be a gNB) which happens to belong to PLMN2. However, as per the current procedural text, the UE refrains from including the eNB1 (or a gNB) of PLMN2 as the reconnectCellID. This is the correct behaviour as the UE should not include the cell identifiers belonging to a different PLMN (at least those that are not part of plmn-IdentityList stored in the RLF report. However, the UE comes back to PLMN1 at a later point in time and at that time the UE if the UE performs the RRCSetup/RRCConnectionSetup then the UE includes the corresponding cell ID as the reconnectCellID. Such wrong information should not be included in the RLF report. There are different way to prevent this. Hence, we propose that the UE does not include reconnectCellId in the RLF report if the first cell in which the UE came back to connected after the RLF and subsequent failed reestablishment belongs to a PLMN that is not listed in plmn-IdentityList stored in the RLF report. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE does not include reconnectCellId in the RLF report if the first cell in which the UE came back to connected after the RLF and subsequent failed reestablishment belongs to a PLMN that is not listed in plmn-IdentityList stored in the RLF report. Impact analysis Impacted functionality: SCGFailureInformation Inter-operability analysis: If the change is implemented by the UE, and not by the network, there is no inter-operability issue. If the change is implemented by the NW, and not by the UE, no interoperability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedural text for the setting of including the reconnectCellID is clarified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | [MCC]: Coversheet revision; wrong clauses-affected value. |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.3.3.4	Reception of the RRCSetup by the UE

The UE shall perform the following actions upon reception of the RRCSetup:

[omitted text]

1>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	if reconnectCellId in VarRLF-Report is not set after failing to perform reestablishment and if this is the first RRCSetup received by the UE after declaring the failure:

3>	if the UE supports RLF-Report for conditional handover and if choCellId in VarRLF-Report is set:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the radio link failure or handover failure experienced in the failedPCellId stored in VarRLF-Report;

3>	else:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure;

3>	set nrReconnectCellId in reconnectCellId in VarRLF-Report to the global cell identity and the tracking area code of the PCell;

1>	if the UE supports RLF report for inter-RAT MRO NR as defined in TS 36.306 [62], and if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

2>	if reconnectCellId in VarRLF-Report of TS 36.331[10] is not set after failing to perform reestablishment and if this is the first RRCSetup received by the UE after declaring the failure:

3>	set timeUntilReconnection in VarRLF-Report of TS 36.331[10] to the time that elapsed since the last radio link failure or handover failure in LTE;

3>	set nrReconnectCellId in reconnectCellId in VarRLF-Report of TS 36.331[10] to the global cell identity and the tracking area code of the PCell;

1>	set the content of RRCSetupComplete message as follows:

END OF CHANGES