3GPP TSG RAN WG Meeting #101	RP-232577

Bangalore, India - September 11-15, 2023

Agenda Item:	8A.2.2

Source: CEWiT, IIT-M, IIT-H, IIT-K

Title:	Views on Rel-19 MIMO

Document for:	Discussion

# Introduction

In the recent Rel-19 workshop [1], the following topics were down-selected and prioritized for further discussion in the current RAN#101 meeting.

Multiple companies proposed various enhancements for MIMO. Among these proposals, Frequency selective precoding for UL and CSI-RS enhancements for supporting BS with higher Tx antennas was proposed by majority of the companies. In this contribution, we present our views on possible scope for Rel-19 MIMO enhancements focusing on frequency selective precoding for UL and CSI-RS enhancements.

- Discussion

  - Frequency selective precoding

The current UL precoding design up to Rel18 supports only wideband level precoding i.e., same precoder will be used across all the RBs that are scheduled for PUSCH transmission. The disadvantage of wideband precoding is that in high delay scenarios, this is not an optimal method of precoding as the channel variations over a wider bandwidth cannot be captured by a single precoder matrix. Hence, because of this restriction, UL throughput is taking a significant hit.

Observation 1: The current wideband precoding design in UL is not an optimal design especially with the introduction of 8Tx devices

Hence, the UL performance can be further enhanced by employing frequency selective precoding. This frequency selective precoding is already employed in downlink and it showed significant improvement in the performance compared with the wideband precoding. Even though frequency selective precoding was proposed in the previous releases as a potential UL enhancement in MIMO, many companies didn’t see a significant gain in performance compared with increase in the signaling overhead. Hence, we need to study the performance gain of frequency selective precoding while striving to minimize the signaling overhead using appropriate methods.

Proposal 1: Study multiple techniques of frequency selective precoding providing minimum additional signaling overhead by taking the Rel-18 UL codebook design as the baseline

  - CSI-RS Enhancements

Massive MIMO is one of the key elements in the current NR technology due to which we are able to generate high data rate, high coverage and high spectral efficiency. As the antenna element size is smaller in mid-bands compared to low-bands, BS is capable of accommodating more number of antenna elements especially in the mid and high bands. Currently the 5G market is shifting to 64T64R design from 32T32R design in the mid-bands. As per Rel-18 NR, the CSI-RS design supports only up to 32 CSI-RS ports. With the market shifting from 32Tx antennas to 64 or more Tx antennas, there is a need to specify the CSI-RS design for greater than 32 Tx antennas in order to exploit the benefits of the increased number of Tx antennas.

Observation 2: There is a need to enhance the current CSI-RS design in order to exploit higher number of Tx antennas at the BS

One method to exploit the increased number of Tx antennas is by using the channel reciprocity that exists in the TDD bands together with the antenna switching SRS to get the DL channel. Though this is a feasible solution in the bands where channel reciprocity stays true, we still have to specify for the bands where channel reciprocity doesn’t exist. Even in TDD bands where the channel reciprocity holds good, being dependent on SRS to get the DL channel for all the UE’s in a cell is a tedious task and requires a lot of UL resources which is not optimal in a DL dominant framework.

Observation 3: Using the current SRS design and channel reciprocity for getting the DL channel is not a feasible solution.

Hence, we propose to enhance the current CSI-RS design in order to support atleast 64 Tx antennas by considering Rel-18 CSI-RS framework as the baseline.

Proposal 2: Support CSI-RS design for higher than 32 ports and specify the impacts.

# Summary

The proposals of this contribution are summarized as follows:

Observation 1: The current wideband precoding design in UL is not an optimal design especially with the introduction of 8Tx devices

Proposal 1: Study multiple techniques of frequency selective precoding with minimum additional signaling overhead by taking the Rel-18 UL codebook design as the baseline

Observation 2: There is a need to enhance the current CSI-RS design in order to exploit higher number of Tx antennas at the BS

Observation 3: Using the current SRS design for getting the DL channel is not a feasible solution

Proposal 2: Support CSI-RS design for higher than 32 ports and specify the impacts

# References

[1] RWS-230488 – RAN Chair’s summary of Rel-19 workshop