3GPP TSG-RAN Meeting #101                                                                RP-232612

Bangalore, India, 11th – 15th September, 2023

Agenda item:			8A.2.2

Source:			RAN vice-chair (AT&T)

Title:		Moderator's summary for REL-19 RAN1 topic MIMO Evolution

Document for:	 	Discussion

# General

Discussions on Rel-19 RAN1-led MIMO evolution have been submitted under this agenda item. The relevant contributions are [1]-[28].

# Potential Justification/Scopes Based on Submitted Contributions

# Potential Justifications

Given the large number of potential scopes submitted for this meeting and the short time for discussion, the initial focus of discussion will be on deriving a set of potential scopes with sufficient company support which can realistically fit within the proposed TU budget endorsed in [29]. After the discussion on topics/objectives, the goal will be to list the corresponding set of justifications associated with the consolidated list of topics and corresponding objectives from the relevant contributions.

# Potential Scopes

The documents in [1]-[28] presented many topics and corresponding objectives with varying levels of support and categorization.

### 2.2.1	Summary of Submitted Topics and Proposed Objectives

The following list of topics and proposed objectives is a summary of all of the submissions and is not listed in any order of preference or importance. It is meant to categorize the potential scopes in order to develop a starting point for further discussion.

Topic 1: Beam management enhancements to reduce overhead/latency through UE-initiated/event-driven beam management

Supporting Companies: OPPO, vivo, Spreadtrum, NEC, MediaTek, Interdigital, Samsung, Nokia, Xiaomi, Intel, Lenovo, ZTE, Qualcomm, Huawei, HiSilicon (FR1 assisted FR2 cases), NTT DOCOMO (for both S-TRP and M-TRP. If TUs are limited, support UE initiated L1 beam reporting only), CATT, Apple

Objective 1. Signalling/mechanism to facilitate UE-initiated beam management procedure including UE-initiated beam reporting/switch

Topic 2: Enhancements to CSI framework to support > 32 (64/128) CSI-RS ports

Overall Company Support: Deutsche Telekom, Futurewei, Ericsson, vivo, NEC, SK Telecom, Samsung, Nokia, Xiaomi, Intel, CeWIT, China Unicom, ZTE, NTT DOCOMO, Lenovo, Huawei, HiSilicon, Fraunhofer IIS, Fraunhofer HHI

Objective 1: Type I codebook enhancements to support > 32 CSI-RS ports

Supporting Companies: Ericsson, SK Telecom, ZTE, NTT DOCOMO, Lenovo, Fraunhofer IIS, Fraunhofer HHI

Objective 2: Type II codebook enhancements to support > 32 CSI-RS ports

Supporting Companies: Ericsson, ZTE, Huawei, HiSilicon, Fraunhofer IIS, Fraunhofer HHI

Objective 3: Hybrid beamforming enhancements (CRI based reporting enhancements)

Supporting Companies: Ericsson, ZTE (up to 32 ports), Huawei, HiSilicon, Fraunhofer IIS, Fraunhofer HHI

Objective 4: PMI reporting for analog beamforming (combining coefficient of wide analog beams) in FR2

Supporting Companies: Huawei, HiSilicon

Topic 3: CJT/DL multi-TRP enhancements

Overall Company Support: Futurewei, NEC, Deutsche Telekom, Ericsson, Spreadtrum, Samsung, Lenovo, ZTE (TDD & FDD), Huawei, HiSilicon, NTT DOCOMO, CATT, Interdigital, Xiaomi, Nokia, CMCC

Objective 1: UE-assisted calibration for CJT with non-ideal backhaul

Supporting Companies: Deutsche Telekom, Ericsson, Spreadtrum, Samsung, Lenovo, ZTE (TDD & FDD), Huawei, HiSilicon, NTT DOCOMO, CATT, CMCC

Objective 2: Enhancement of the unified TCI framework for CJT/mTRP

Supporting Companies: Interdigital, Lenovo (up to 4 TCI states), ZTE

Objective 3: Calibration of antenna arrays among TRP's with imperfect array synthesis

Supporting Companies: Xiaomi, CATT

Objective 4: Support UE assisted scheduling, precoding alignment between TRPs for coherent joint transmission/reception with non-ideal backhaul

Supporting Companies: Huawei, HiSilicon

Objective 5: Extension of Type II CSI to up to Rank 8

Supporting Companies: Nokia

Topic 4: UL enhancements

Overall Company Support: Deutsche Telekom, OPPO, NEC, Interdigital, Nokia, Xiaomi, LG, Lenovo, ZTE, NTT DOCOMO, Intel, Spreadtrum, Samsung, Huawei, HiSilicon, CATT, CeWIT, Google, Ericsson, CMCC, Fraunhofer IIS, Fraunhofer HHI

Objective 1: STxMP enhancements (e.g. Simultaneous TX of PUCCH and PUSCH, Asymmetric panel implementations, mDCI PUCCH + PUCCH, STxMP with up to rank 8, Coherent SFN STxMP)

Supporting Companies: Deutsche Telekom, OPPO, NEC, Interdigital, Nokia, Xiaomi, LG, Lenovo, ZTE, NTT DOCOMO, Intel

Objective 2: Frequency selective precoding

Supporting Companies: Deutsche Telekom, Spreadtrum, NEC, Interdigital, Samsung, Xiaomi, Lenovo, Huawei, HiSilicon, CATT, CeWIT, Fraunhofer IIS, Fraunhofer HHI

Objective 3: More than 1 layer DFT-s-OFDM waveform based PUSCH

Supporting Companies: Google, Ericsson, Xiaomi, Lenovo

Objective 4: Enhancements for UL 3Tx including 3Tx for UL codebook and non-codebook based transmission

Supporting Companies: OPPO, Google, ZTE, CMCC

Objective 5: More than 2 UE panels

Supporting Companies: LG, Lenovo

Topic 5: Enhancement for UL-TRP only scenario

Overall Company Support: OPPO, Samsung, Intel, Google, ZTE, NTT DOCOMO, Huawei, HiSilicon

Objective 1: TA enhancements

Supporting Companies: Google, ZTE, NTT DOCOMO, Huawei, HiSilicon

Objective 2: UL power control

Supporting Companies: Google, ZTE, NTT DOCOMO, Huawei, HiSilicon

Objective 3: UL BM enhancements

Supporting Companies: Google, ZTE, NTT DOCOMO

Topic 6: 6Rx/8Rx UE enhancements with lower complexity up to 8-layer DL Tx

Supporting Companies: Ericsson, Samsung, Nokia, ZTE, Huawei, HiSilicon, NTT DOCOMO, Fraunhofer IIS, Fraunhofer HHI

Topic 7: Miscellaneous MU MIMO enhancements

Objective 1: Enhancements to facilitate more advanced UE reception

Supporting Companies: Deutsche Telekom

Objective 2: Enhancements for MIMO operation with UE coordination

Supporting Companies: Deutsche Telekom

Objective 3: DL precoding and CQI refinement for assisting NW level traffic dependent rank adaptation

Supporting Companies: Deutsche Telekom, ZTE

Objective 4: Specify beam report enhancement to identify "weak" beams for UL/DL MU-MIMO UE pairing

Supporting Companies: Google

Topic 8: Miscellaneous CSI enhancements

Objective 1: Specify sub-band L1-SINR report to facilitate the rank and MCS selection for the UL/DL reciprocity-based CSI acquisition

Supporting Companies: Google

Objective 2:Study and if beneficial, specify CQI reporting based on actual received PDSCH quality (e.g., PDSCH DMRS or PDSCH decoding based CQI).

Supporting Companies: Ericsson

Objective 3: Study gNB-side CSI prediction for TDD high-speed scenarios in Rel-19, considering the following two aspects: (1) SRS capacity enhancement and/or overhead reduction and (2) Doppler information reporting based on CSI-RS

Supporting Companies: vivo

Objective 4: Study UE-side CSI prediction enhancement for higher UE speed, e.g., up to 120km/h, in Rel-19: Enhance CSI-RS and/or CSI configurations to capture larger Doppler spread

Supporting Companies: vivo

Objective 5: Study performance monitoring for UE-side CSI prediction to ensure reliability in Rel-19: Consider UE reporting of performance metrics to enable gNB monitoring of the prediction performance

Supporting Companies: vivo

Objective 6: Enhancement for Type II PS codebooks considering joint spatial/frequency/ temporal precoding for CSI RS targeting UE with high/medium velocities

Supporting Companies: Xiaomi

Objective 7: Enhancement on CSI reporting taking into consideration of past CSI

Supporting Companies: Apple, Mediatek

Objective 8: Enhancement on CSI reporting with multiple hypothesis

- To assist NW with different scheduling needed, e.g., new transmission vs. retransmission, different rank, MU pairing, etc.

- Enhancement on CSI reporting to accommodate UE with dynamic CSI processing load/condition

Supporting Companies: Apple

Objective 9:  Enhancement to facilitate more advanced UE reception, e.g., MU detection and co-scheduling information enhancement

Supporting Companies: Apple

Objective 10: Enhancement for MIMO operation with UE coordination

- UE coordination-based beam-management, e.g., beam report, beam indication and beam failure recovery

- UE coordination-based CSI feedback

Supporting Companies: Apple

Objective 11: Enhancement on CSI reporting taking into consideration of inference and noise power reporting, and/or inference and noise covariance matrix reporting.

Supporting Companies: CMCC

Topic 9: Miscellaneous SRS enhancements

Objective 1: Specify interference reduction mechanism for SRS based on precoded SRS with interference suppression

Supporting Companies: Google

Objective 2: Support the gNB to obtain information of per SRS port imbalance to enable gNB side CSI correction

Supporting Companies: Ericsson

Objective 3: SRS phase-consistency for assisting gNB-side CSI prediction for Middle & high speed UEs in TDD MU

Supporting Companies: ZTE

Topic 10: Miscellaneous Beam Management Enhancements

Objective 1: Specify CSI feedback to facilitate the channel-based beamforming: Specify CSI-RS with antenna switching for CSI feedback for the UE to identify the raw channel for beamforming weight selection

Supporting Companies: Google, OPPO, ZTE

Objective 2: Introduce dynamic switching for QCL source of TRS

Supporting Companies: Ericsson

Objective 3: Enhance the DCI-based spatial relation/TCI state indication for SRS

Supporting Companies: Ericsson

Objective 4: To reduce latency of beam measurement, study fast beam training for extremely large antenna array

Supporting Companies: vivo

Objective 5: To reduce TCI state activation latency, study aperiodic TRS tracking is triggered together with TCI state activation

Supporting Companies: vivo

Objective 6: Support of 2TA for S-DCI mTRP & dynamic TRP switching

Supporting Companies: LG, Intel

Objective 7: Specify up to 4 PT-RS ports

Supporting Companies: LG

Objective 8: SSB correlation information indication for faster beam acquisition

Supporting Companies: Intel

Objective9: Extension of unified TCI to periodic/semi persistent (P/SP) transmission occasion in both DL/UL, targeting both FR1/FR2

Supporting Companies: ZTE

Objective 10: Support L1 SINR based beam group report

Supporting Companies: Qualcomm

Objective 11: Enhancement to provide assisting information on beam structure to facilitate hierarchical beam tracking

Supporting Companies: Apple

Objective 12: Enhancement on CSI-RS configuration to facilitate intra-symbol beam tracking

Supporting Companies: Apple

Objective 13: New antenna architecture for fast BM enhancements

Supporting Companies: Huawei, HiSilicon

Topic 11: Miscellaneous UL MIMO enhancements

Objective 1: Specify UL open-loop MIMO transmission with antenna port cycling. Study, and if needed, specify UL open loop MIMO transmission scheme with precoder cycling.

Supporting Companies: Google, vivo

Objective 2: Introduce FDM-based rate matching of PUSCH in both FR1 and FR2 to avoid collision with an SRS transmission

Supporting Companies: Ericsson

Objective 3: Study, and if beneficial, specify enhanced closed-loop antenna selection for PUSCH

Supporting Companies: Ericsson

Objective 4: UL Dynamic Transmit Antenna Selection

Supporting Companies: CATT

Objective 5: High rank UL transmission for more than 4 layers

Supporting Companies: Lenovo

Objective 6: Enhancement on 8TX UL: Support “fullPowerMode2” for 8Tx with Ng=4 and Ng=8, and “fullPowerMode1” with rank = 4~7 for 8Tx with Ng=8

Supporting Companies: NTT DOCOMO

Objective 7: Enhance UL/DL system throughput via power aggregation, BW aggregation, antenna diversity

Supporting Companies: Apple

Objective 8: UE assistance on available distributed antennas

Supporting Companies: Apple

Objective 9: Support multi-UE UL transmission, including:

- UL NCJT SDM scheme to improve throughput

- UL NCJT TDM/FDM  scheme to improve reliability

- UL SFN scheme to improve reliability

- Both S-DCI and M-DCI based scheduling could be considered

Supporting Companies: CMCC

Topic 12: Miscellaneous mTRP enhancements

Objective 1: Enhance the function of multi-TA for mTRP transmission

Supporting Companies: OPPO, Lenovo

Objective 2: Evaluate and study system performance as well as CSI overhead to support UE assisted interference management for throughput improvement. If needed, specify necessary spec enhancement.

Supporting Companies: vivo

Objective 3: Introduce necessary enhancement to support various multi TRP operation in a single framework.

- Joint CSI measurement and report across multi-carrier aggregation

Supporting Companies: vivo

Objective 4: Study, and if justified, specify CSI enhancement to support different antenna port layouts across TRPs

Supporting Companies: Spreadtrum

Objective 5: Support of single DCI based multi TA and asynchronous multi TRP based on different DL reference timings

Supporting Companies: NEC, Lenovo

Objective 6: Support up to 4 TRPs for M-DCI based as well as S DCI based multi TRP DL operation (NCJT, TDM, FDM, SFN)

Supporting Companies: Lenovo

Objective 7: CJT+NCJT for the scenario with more than 2 TRPs, e.g., 4

Supporting Companies: Lenovo

Objective 8: Enable data REs of one PDSCH to be rate matched around PTRS of another PDSCH for mDCI based mTRP at least for fully-overlapping PDSCHs

Supporting Companies: Qualcomm

Objective 9: Enhancements for interference management/coordination

Supporting Companies: Deutsche Telekom

Objective 10: CSI overhead reduction

Supporting Companies: Interdigital

Objective 11: Enhancement among TRP’s with different antenna configurations or layouts

Supporting Companies: Xiaomi

Objective 12: Enhancement of R18 CJT Type II codebook for UE’s with high/medium velocities

Supporting Companies: Xiaomi, LG

Objective 13: Downlink interference avoidance via SRS using reciprocity

Supporting Companies: Futurewei

Topic 13: Other Topics:

1) Rate-splitting multiple access study item (RSMA)		BBC

2) Link Adaptation enhancement

Supporting Companies: Qualcomm

Objective 1: Enhanced coding rate and link adaptation through HARQ combining

Objective 2: Introduce 1024-QAM for UL

3) Support DFT-s-OFDM for PDSCH to enhance the coverage of FR2

Supporting Companies: Huawei, HiSilicon

4) Support multi-UE UL/DL transmission, including:

- DL NCJR SDM scheme to improve throughput

- DL NCJR TDM/FDM  scheme to improve reliability

- Both S-DCI and M-DCI based scheduling could be considered

Supporting Companies: CMCC

5) Enhancement for HetNet scenario

Objective 1: Exchange of macro gNB’s slot pattern (reduced-power/normal-power) between macro gNB and indoor gNB

Supporting Companies: CMCC

Objective 2: CSI report enhancement for UEs under indoor gNB to report two kinds of CSI corresponding to different IMR power levels (macro gNB transmit power levels)

Supporting Companies: CMCC

Objective 3: Advanced UE receiver

Supporting Companies: CMCC

### 2.2.1	Further Refinement of Topics and Proposed Objectives

Given the large number of topics and associated objectives, it is proposed to consider the following list of topics as a starting point for further discussion. The goal should be to develop a realistic set of topics and subsequent objectives which can fit within the proposed TU budget endorsed in [29].

Topic 1: Beam management enhancements to reduce overhead/latency through UE-initiated/event event-driven beam management

Topic 2: Enhancements to CSI framework to support > 32 (64/128) CSI-RS ports

Topic 3: CJT/DL multi-TRP enhancements

Topic 4: UL enhancements

Topic 5: Enhancement for UL-TRP only scenario

Topic 6: 6Rx/8Rx UE enhancements with lower complexity up to 8-layer DL Tx

# Summary

An offline session was held on Wednesday, 13 September, to collect company comments and views on the list of topics and associated objectives. The outcome of the discussions will not be considered as an endorsed set of topics and objectives. The goal will be to have a set for further discussion, refinement, and downscoping in December in order to fit within the proposed TU budget endorsed in [29].

Comments:

Spreadtrum: Study or WI phase? We prefer WI with no study phase.

Topic 1: Beam management enhancements to reduce overhead/latency through UE-initiated/event-driven beam management

Objective 1. Signalling/mechanism to facilitate UE-initiated beam management procedure including UE-initiated beam reporting/switch

Comments:

Ericsson: Too much to take into account given the limited time. Focus on only beam reporting and remove the switch.

Samsung: Focus on uplink signalling content and medium.

Qualcomm: Where it says UE-initiated beam management, we would like to add CSI report.

Futurewei: Switch is actually needed. Exact wording can be discussed.

OPPO: Share view of E///.

Nokia: OK with comments from E///, Samsung, and Qualcomm. Switch can be usual but can be the next step.

DOCOMO: Agree with E/// and Samsung. RAN2 also discussing this. We need to consider the overlapping topics.

ZTE: Agree with E/// and Samsung. Follow legacy approach in terms of beam switching.

Apple: OK with E/// and Samsung. Clarification requested from QC on CSI report.

QC: We have in mind CQI report. Compromise to change switch to switch request.

Vivo: This topic should be more about reporting than switching.

Lenovo: Generally OK with objective.

Futurewei: Having to go through the legacy procedure is too slow.

MediaTek: We think that we have to do the reporting and the switching.

Intel: Overall supportive of the objective. There are other topics of more interest commercially.

Huawei: The switching is main target of enhancement.

Moderator: Leave Topic 1 objective as is for now. Further discussion can be handled in December.

Topic 2: Enhancements to CSI framework to support > 32 (64/128) CSI-RS ports

Objective 1: Type I codebook enhancements to support > 32 CSI-RS ports

Objective 2: Type II codebook enhancements to support > 32 CSI-RS ports

Objective 3: Hybrid beamforming enhancements (CRI based reporting enhancements)

Objective 4: PMI reporting for analog beamforming (combining coefficient of wide analog beams) in FR2

Note 1: Extension of legacy codebooks and legacy CSI-RS resources

Note 2: Objective 3 may require further study or clarification.

Comments:

Telstra: Support this item and particulary Obj 1.

Nokia: Focus on Obj 1. Maybe Obj 2. Obj 3 and 4 have much more system implications.

SK Telecom: We think that Obj 1 is essential and should be the minimum set. We are OK with Obj 2 and 3 and could be beneficial.

LG: Generally supportive of enhancement. Obj 3 and 4 are more about FR2 and totaling different. Prefer to be separate topic.

Samsung: Supportive of Obj 1, 2, and 3. Qualification that it is an extension of legacy codebooks and legacy CSI-RS resources.

Vivo: Supportive of Obj 1, 2, and 3. Not necessary to do CSI-RS re-design.

Interdigital: Same view as Nokia and LG.

CEWIT: Supportive of Obj 1, 2. Fine with Samsung suggestion.

ZTE: Supportive of Obj 1, 2, and 3. Need clarification on legacy CSI-RS. Some extension may be necessary due to OBj 3.

Lenovo: Supportive of Obj 1, 2.

OPPO: Obj 1 and 2 are high priority.

CATT: Obj 1 and 3. We think that Obj 3 does not have to be limited to hybrid beamforming. Can be applied to any type of array.

E///: Supports all 4 objectives. With Samsung restriction, the scope should be manageable.

MediaTek: We would prefer to do just 64 ports if we do anything. We want it agreed that it is not applicable for below 6.425 GHz.

Qualcomm: One of the objectives should be chosen.

DOCOMO: We support Obj 1, 2, and 3 and support extension of legacy codebook and CSI-RS.

Huawei: We support all 4 but OK to limit to Obj 1, 2, and 3 with same restriction as proposed by Samsung.

Intel: We support Obj 1, 2, and 3. Important to support up to 128 ports. Can enable up to 128 ports even with legacy codebook design.

Apple: Keep CSI per resource design.

AT&T: Obj 1 is highest priority.

E///: Obj 3 is still limited to 32 ports. Can talk until Dec.

ZTE: Change note 2 to “may require”.

Moderator: Adjust Topic 2 objectives including the notes as shown above. Further discussion can be handled in December.

Topic 3: CJT/DL multi-TRP enhancements

Objective 1: UE-assisted calibration reporting of delay and frequency/phase offsets for CJT with non-ideal synchronization and backhaul

Objective 2: Enhancement of the unified TCI framework for CJT/mTRP

Objective 3: Calibration of antenna arrays among TRP's with imperfect array synthesis

Objective 4: Support UE assisted scheduling, precoding alignment between TRPs for coherent joint transmission/reception with non-ideal backhaul

Objective 5: Extension of Type II CSI to up to Rank 8

Note: Assume legacy CSI-RS design and standalone aperiodic reporting on PUSCH.

Comments:

Samsung: Text on Obj 1 modified. Obj 3 and 4 are similar. Can be combined with Obj 1.

Vivo: All of these objectives are meant for non-ideal backhaul. How does the framework work for non-ideal backhaul scenario?

Huawei: Prefer to keep non-ideal backhaul. Fine with modified version of Obj 1. Obj 2 and 5 do not seem to belong.

AT&T: Prefer to keep non-ideal backhaul. Support Samsung proposal for Obj 1 and Obj 3 and 4 are covered in Obj 1 now.

Nokia: On Obj 1, the intension is to extend the scope to different scenarios beyond Rel-18. We should do an evaluation stage first before any standardization.

CATT: Support Samsung Obj 1. Both ideal and non-ideal backhaul should be considered.

Qualcomm: Keep Obj 1 only.

MTK: We don’t see the value with any of the objectives but would not object to Obj 1.

CMCC: Focus on non-ideal backhaul only.

DOCOMO: Support Samsung’s Obj 1.

Lenovo: Support Samsung’s Obj 1.

E///: Support Samsung’s Obj 1.

Moderator: Adjust Topic 3 objectives including the note as shown above. Further discussion can be handled in December.

Topic 4: UL enhancements

Objective 1: STxMP enhancements (e.g. Simultaneous TX of PUCCH and PUSCH, Asymmetric panel implementations, mDCI PUCCH + PUCCH, STxMP with up to rank 8, Coherent SFN STxMP)

Objective 2: Frequency selective precoding

Objective 3: More than 1 layer DFT-s-OFDM waveform based PUSCH

Objective 4: Enhancements for UL 3Tx including 3Tx for UL codebook and non-codebook based transmission

Objective 5: More than 2 UE panels

Comments:

Vivo: Obj 1 under Topic 11 is also part of UL enhancements. Subset of Obj 2 here.

OPPO: Obj 4 is the most important for near future deployment.

Apple: Too many objectives. Concern with Obj 1 and 5.

LG: Strongly support this topic and we certainly need to do downscoping but need to include as many of Topic 4 as possible. Most important is Obj 1.

TIM, T-Mobile USA, Telus, Bell Mobility, Vodafone, Google, China Unicom, China Telecom, ZTE, OPPO, Orange: Support Obj 4.

Moderator: Adjust Topic 4 objectives as shown above. Further discussion can be handled in December.

Topic 5: Enhancement for UL-TRP only and asymmetric downlink S-TRP/UL M-TRP scenario assuming intra-band intra-cell non-co-located M-TRP scenarios without changing existing cell definition or defining a new cell

Objective 1: Extension of Rel-18 2TA mDCI to sDCI assuming legacy PRACH resources

Objective 2: Separate UL power control for SRS only to downlink S-TRP from SRS to UL M-TRP and introduce path loss measurement to uplink M-TRP

Objective 3: UL BM enhancements

Comments:

Samsung: We can update the description as above.

LG: Intention is not to consider UL S-TRP operation. This feature is not workable. Add “at least”.

MediaTek: The whole point was to downscope. Do not agree to “at least” addition.

Intel: Similar comment as LG. Original intention was UL-TRP only. Add a note “Objective is aiming to only enable UL TRP”

DOCOMO: Fine with the current formulation.

OPPO: The note is not needed. It is up to implementation.

CMCC: Share same view as OPPO. Asymmetric downlink S-TRP is from a UE perspective only.

Qualcomm: Agree with Samsung but the wording is unclear. Suggested revisions above.

ZTE: We also support this Topic.

Nokia: In principle, downlink and uplink could be on different cells but to keep the scope limited we prefer to keep intra-cell. We also support.

China Unicom/China Telcom/Ericsson also support.

AT&T: OK with the formulation but need to understand the rationale for the work to know what the gains are.

DOCOMO: Two scenarios to improve the UL.

CMCC: No need to limit to “intra-cell”.

Vivo: Is it power control for SRS only?

Moderator: Adjust Topic 5 objectives as shown above. Further discussion can be handled in December.

Topic 6: 6Rx/8Rx UE enhancements with lower complexity utilizing two segments of 3/4 Rx antenna units up to 8-layer DL Tx based on legacy codebook and legacy codeword to layer mapping

Objective 1: SRS antenna port grouping, CSI and codeword association to the segments of receive antenna.

Comments:

Huawei: Add “utilizing two segments of 3/4 Rx antenna units” to topic 6 as above.

Samsung: Further adjust the text as above.

Vivo: From a receiver perspective, this proposal is for rank higher than 4. Inter-layer interference should be studied first.

Spreadtrum: This is a new antenna structure. How about four segments or 2 Rx antenna units. CSI part: does it include BM and reporting?

Nokia: I am sympathetic with Spreadtrum as to the need to work on a specific receiver architecture in RAN1. We need to study the possible receivers and understand the gains before we can do any work in RAN1.

Samsung: The scope is clear and why we need Obj 1.

Moderator: Adjust Topic 6 objectives as shown above. Further discussion can be handled in December.

Outcome of Offline Discussions:

The list of topics and objectives below is not considered as an endorsed set of topics and objectives. The goal will be to use the list of topics and objectives for further discussion, refinement, and downscoping in December in order to fit within the proposed TU budget endorsed in [11].

Topic 1: Beam management enhancements to reduce overhead/latency through UE-initiated/event-driven beam management

Objective 1. Signalling/mechanism to facilitate UE-initiated beam management procedure including UE-initiated beam reporting/switch

Topic 2: Enhancements to CSI framework to support > 32 (64/128) CSI-RS ports

Objective 1: Type I codebook enhancements to support > 32 CSI-RS ports

Objective 2: Type II codebook enhancements to support > 32 CSI-RS ports

Objective 3: Hybrid beamforming enhancements (CRI based reporting enhancements)

Note 1: Extension of legacy codebooks and legacy CSI-RS resources

Note 2: Objective 3 may require further study or clarification.

Topic 3: CJT/DL multi-TRP enhancements

Objective 1: UE-assisted calibration reporting of delay and frequency/phase offsets for CJT with non-ideal synchronization and backhaul

Note: Assume legacy CSI-RS design and standalone aperiodic reporting on PUSCH.

Topic 4: UL enhancements

Objective 1: STxMP enhancements (e.g. Simultaneous TX of PUCCH and PUSCH, Asymmetric panel implementations, mDCI PUCCH + PUCCH, STxMP with up to rank 8, Coherent SFN STxMP)

Objective 2: Enhancements for UL 3Tx including 3Tx for UL codebook and non-codebook based transmission

Topic 5: Enhancement for asymmetric downlink S-TRP/UL M-TRP scenario assuming intra-band intra-cell non-co-located M-TRP scenarios without changing existing cell definition or defining a new cell

Objective 1: Extension of Rel-18 2TA mDCI to sDCI assuming legacy PRACH resources

Objective 2: Separate UL power control for SRS only to downlink S-TRP from SRS to UL M-TRP and introduce path loss measurement to uplink M-TRP

Topic 6: 6Rx/8Rx UE enhancements with lower complexity utilizing two segments of 3/4 Rx antenna units up to 8-layer DL Tx based on legacy codebook and legacy codeword to layer mapping

Objective 1: SRS antenna port grouping, CSI and codeword association to the segments of receive antenna.

# References

- RP-231551: Views on scope for NR MIMO evolution in Rel-19, Qualcomm Incorporated

- RP-231586: Deutsche Telekom’s MIMO Evolution objectives, Deutsche Telekom AG

- RP-231601: Discussion of R19 work on MIMO Evolution, Futurewei

- RP-231621:	Views on R19 MIMO Enhancement, Google

- RP-231748:	Discussion on Release 19 MIMO enhancements, Ericsson

- RP-231772:	Discussion on MIMO enhancement for Rel-19, OPPO

- RP-231790:	Rel-19 MIMO evolution (RAN1-led), vivo

- RP-231852:	Views on Rel-19 MIMO enhancement, Spreadtrum Communications

- RP-231867:	Discussion on further NR MIMO evolution, NEC

- RP-231890:	MIMO Evolution in Rel-19, Nokia, Nokia Shanghai Bell

- RP-231928:	MIMO evolution in Rel-19, Huawei, HiSilicon, HiSilicon

- RP-231938:	Rate Splitting Multiple Access (RSMA) for Multi-User MIMO - Enhancement of Unicast and Joint Unicast/Multicast Delivery, BBC

- RP-231983:	On Rel-19 MIMO Evolution, InterDigital, Inc.

- RP-232011:	On CSI-RS extension beyond 32 ports in R19 MIMO evolution, SK Telecom, Deutsche Telekom

- RP-232021:	Scope of MIMO enhancements in Rel-19, Samsung

- RP-232057: 	On Rel-19 MIMO enhancements, CATT

- RP-232070:	Views on MIMO enhancements in Rel-19, Xiaomi Communications

- RP-232073:	Views on Rel-19 MIMO/UL enhancements, NTT DOCOMO, INC.

- RP-232093:	Views On MIMO Evolution in R19, Apple Inc.

- RP-232146:	Views on Rel-19 MIMO, Lenovo

- RP-232152:	Views on Rel19 MIMO evolution, ZTE, Sanechips

- RP-232193:	Proposal on MIMO enhancements in Rel-19, LG Electronics

- RP-232201:	MIMO evolution - Views from Reliance Jio, Reliance Jio

- RP-232204:	Discussion on Rel-19 MIMO Evolution, China Mobile International Ltd

- RP-232331:	[RAN1-led] MIMO Enhancements, MediaTek Inc.

- RP-232399:	Millimeter wave enhancements for FR2 in Rel-19, China Unicom

- RP-232561:	Discussion on Rel-19 MIMO enhancements, Intel Corporation

- RP-232577:	Views on Rel-19 MIMO, CEWiT

- RP-231540: RAN Chair’s Guidance for RAN Release 19, RAN Chair