3GPP TSG-RAN Meeting #101                                                                RP-23xxxx

Bangalore, India, 11th – 15th September, 2023

Agenda item:			8A.2.11

Source:			Nokia, Nokia Shanghai Bell

Title:		[DRAFT] Moderator's summary for REL-19 RAN1 topic ISAC & Exploring Study in New Spectrum (7-24GHz)

Document for:	 	Information

# General

In line with the endorsed guidance in [1] from the 3GPP RAN Chair, this document covers the following topics submitted under this agenda item, in decreasing order of the level of interest:

- Integrated Sensing and Communications (ISAC)

- 7-24 GHz

The relevant contributions are [2]-[33].

# Potential Justification/Scopes Based on Submitted Contributions

# ISAC

The main justification cited by many companies for studying ISAC in the extensive study already carried out in SA1 and the many use cases documented by SA1 in TR22.837, namely:

- intruder detection in smart home

- pedestrian/animal intrusion detection on a highway

- rainfall monitoring

- transparent Sensing Use Case

- sensing for flooding in smart cities

- intruder detection in surroundings of smart home

- sensing for railway intrusion detection

- Sensing Assisted Automotive Manoeuvring and Navigation

- AGV detection and tracking in factories

- UAV flight trajectory tracing

- sensing at crossroads with/without obstacle

- Network assisted sensing to avoid UAV collision

- sensing for UAV intrusion detection

- sensing for tourist spot traffic management

- contactless sleep monitoring service

- Protection of Sensing Information

- health monitoring at home

- service continuity of unobtrusive health monitoring.

- Sensor Groups

- Use case of Sensing for Parking Space Determination

- Use case of Seamless XR streaming

- Use case of UAVs/vehicles/pedestrians detection near Smart Grid equipment

- AMR collision avoidance in smart factories

- roaming for sensing service of sports monitoring

- immersive experience based on sensing

- accurate sensing for automotive manoeuvring and navigation service

- Vehicles Sensing for ADAS

- Gesture Recognition for Application Navigation and Immersive Interaction

- sensing for automotive manoeuvring and navigation service when not served by RAN

- blind spot detection

- integrated sensing and positioning in factory hall

Many companies highlight the importance of having a clear view of a limited set of priority use cases in order to guide the next steps. It is also widely suggested that use case identification, by downselection from the SA1 list, would be a suitable initial task for a RAN plenary-level study. Without prejudice to the outcome of such a selection process, it is worth noting that UAV tracking/detection and intruder detection are two use cases that have come to the attention of multiple companies. Given the comprehensive nature of SA1’s list of use cases, it also seems wise for RAN to downselect from these, rather than inventing additional use cases.

Other aspects that are highlighted as being prerequisites for further work are identification of target frequency bands, and identification of intended sensing modes, the latter especially being related to the choice of use cases.

Some companies also mention identification of KPIs as being a potential component of a RAN-level study that is a prerequisite for further work, but the consensus that this is necessary at an early stage of Rel-19 seems less clear.

Nearly all companies who highlight ISAC point out that channel modelling work is needed, as the models in TR38.901 are not designed for sensing. In particular, it is identified that the following aspects would be needed for future work and evaluations:

- modelling of sensing targets, including radar cross-section (RCS) and mobility

- spatial consistency.

Several companies have identified that the pros and cons of deterministic and stochastic channel modelling for sensing need to be considered, as well as the possibility of hybrid modelling.

There was, however, a suggestion from a few companies that channel modelling work might not be necessary if only gNB-based sensing modes are addressed, and that in this case only RAN3 consideration of RAN interface signalling would be needed.

It generally seems to be assumed that aspects such as details of architecture, signalling, reference signals and frame structure would be subsequent to completion of channel modelling.

# 7-24 GHz

It is widely recognised that the 7-24 GHz frequency range is expected to be relevant for 6G. With discussions taking place at WRC, there is a desire by several companies to make visible 3GPP’s readiness in this area.

There is also an awareness that the TR38.901 was not extensively validated by measurements at these frequencies. Further, the expected use of higher-order massive MIMO at these frequencies raises the importance of ensuring the applicability of the model.

The areas of the model where it is most widely recognised that adaptations/extensions may be needed are:

- Near-field propagation, where plane wave propagation and uniform array behaviour cannot be assumed;

- Spatial non-stationarity;

- Outdoor-to-indoor penetration modelling.

The timing of a study for 7-24 GHz may need discussion. Such a study would be independent of any RAN-level study on ISAC, and could, in theory, therefore start in RAN WG1 in parallel. However, some companies indicate a need of sufficient time for companies to gather measurement data first.

# Summary

From the offline session on ISAC & Exploring Study in New Spectrum (7-24GHz) on 13th September at RAN#101:

Regarding ISAC, the following way forward seems agreeable:

- If not already done in SID, a RAN-led study objective to select a small number of ISAC use cases from TR22.837, and to identify appropriate corresponding information needed for point 2 below, i.e. scenarios, frequency ranges and sensing modes.

- A RAN1-led study objective to define channel modelling details for sensing using 38.901 (if applicable to the selected use cases) as a starting point, and taking into account relevant measurements, including:

  - modelling of sensing targets, including, for example (if needed by the selected use cases), radar cross-section (RCS), mobility and clutter/scattering patterns;

  - spatial consistency.

This objective to start after completion of the first (RAN-led) objective if not already done in SID.

Regarding the other channel modelling aspects, the following way forward seems close to being agreeable, possibly after some minor further refinement:

- RAN1-led study objectives to:

  - validate using measurements the channel model of TR38.901 for 7-24 GHz, and

  - to adapt/extend as necessary the channel model of 38.901 for at least 7-24 GHz, including at least the following aspects for applicable scenarios:

    - Near-field propagation;

    - Spatial non-stationarity.

Note that continuity of the channel model in the frequency domain shall be ensured.

Note that mathematical and/or theoretical aspects (if any) may be studied before results of measurement campaigns are available. Study of measurement results is likely to be able to start in Q3 2024.

# References

- RP-231540 RAN Chair’s Guidance for RAN Release 19	3GPP RAN Chair

- RP-231569 Channel Models 	Spark NZ Ltd

- RP-231575 Consideration on Rel-19 Channel Modeling of ISAC 	New H3C Technologies Co., Ltd.

- RP-231595 Deutsche Telekom’s Channel Modeling objectives 	Deutsche Telekom AG

- RP-231737 Study on Channel Model for ISAC Techniques 	Lekha Wireless Solutions

- RP-231751 Apple Views on Channel Modeling for Rel-19 	Apple

- RP-231760 Views on Channel modeling and ISAC studies in Rel-19 	Qualcomm India Pvt Ltd

- RP-231779 Views on channel modelling in Rel-19 	OPPO

- RP-231798 Rel-19 Channel Modeling for ISAC and new spectrum (7~24GHz) (RAN1-led) 	vivo

- RP-231876 Channel modeling and additional techniques for ISAC 	NEC

- RP-231892 Channel modelling for 7-24 GHz and sensing 	Nokia, Nokia Shanghai Bell

- RP-231893 Rel-19 channel modeling for ISAC 	Xiaomi, CAICT, China Telecom, China Unicom, Spark NZ Ltd, CEWiT, ITRI, CATT, Inspur, Spreadtrum, TCL, Nvidia, ETRI, Lenovo, Transsion Holdings, Continental Automotive, Google

- RP-231910 Channel modeling for ISAC in Rel-19 	Spreadtrum Communications

- RP-231941 On channel Modelling for Rel-19 	Ericsson

- RP-231962 Study on channel models with ray tracing 	NVIDIA

- RP-231990 Views on Rel-19 Channel Modelling 	InterDigital, Inc.

- RP-232025 Samsung's views on scope of Channel Modelling 	Samsung

- RP-232052 Consideration on channel modeling 	CATT

- RP-232081 Views on channel modeling related topics in Rel-19 	NTT DOCOMO, INC.

- RP-232111 Feasibility study on enabling Integrated Communication and Sensing (ISAC) for BS 	BUPT, China Telecom

- RP-232124 Scope of Integrated Sensing and Communications in Rel-19 	Huawei, HiSilicon, CAICT

- RP-232153 Views on channel modeling for Rel-19 	ZTE, Sanechips

- RP-232197 Proposals on channel modeling in Rel-19 	LG Electronics

- RP-232200 Views on ISAC to enable Base-station based sensing 	Reliance Jio

- RP-232217 Discussion on Rel-19 Channel Modeling 	China Mobile International Ltd

- RP-232275 Study on gNB-based Integrating Sensing and Communication in Rel-19 	China Telecom,CATT,H3C,Lenovo,BUPT

- RP-232332 [RAN1-led] Study on Channel Modeling for Integrated Sensing and Communication 	MediaTek Inc.

- RP-232541 Views on Rel. 19 ISAC and Channel Modeling for 7-24GHz 	AT&T

- RP-232559 Discussion on Rel-19 Channel Modeling Studies 	Intel Corporation

- RP-232583 Channel Modeling 	CEWiT

- RP-231916 Motivation of study on Reconfigurable Intelligent Surface 	Kyocera

- RP-232050 Discussion on Channel Modeling of RIS in Rel.19 	KDDI Corporation

- RP-232402 Views on RIS in Rel-19 	China Unicom