3GPP TSG RAN #101			RP-232626

Bangalore, India, September 11th – 15th, 2023

Title:	Moderator’s summary for REL-19 RAN1 additional topics

Source:	RAN1 Chair (Samsung)

# Introduction

This document summarizes the discussion on five additional candidate Rel-19 areas which are to be led by RAN1.

- Multi-Carrier enhancements (8A.2.12.2)

- Coverage enhancements (8A.2.12.3)

- Positioning enhancements (8A.2.12.4)

- Sidelink evolution (8A.2.12.5)

- Other RAN1-led proposals (8A.2.15)

As noted by the RAN chair, the room for RAN-led WI/SIs in addition to those listed in slide 9 of RP-231540 is very limited. Given the situation, if any of the potential WIs in the above list are to be included as part of Rel-19, the WI scope will have to be focused and require small TU assignment. Offline discussions will focus on assessing the level of necessity/urgency of the proposed topics along with potential scope that is acceptable/reasonable. The outcome of the offline discussions will be presented in the online session for consideration towards finalizing the scope of RAN1-led Rel-19 WI/SIs.

# Multi-Carrier Enhancements (agenda item 8A.2.12.2)

RP-231556	Views on scope for NR Multi-carrier enhancements in Rel-19	Qualcomm Incorporated

RP-231754	Multi-carrier enhancements	Nokia, Nokia Shanghai Bell

RP-231781	Discussion on multi-carrier enhancement in Rel19	OPPO

RP-231800	Rel-19 Multi-carrier Enhancements (RAN1-led)	vivo

RP-231932	Multicarrier operation enhancements in Rel-19	Huawei, HiSilicon

RP-231949	On Multi-carrier enhancements	Ericsson

RP-232061	Further enhancements on multi-carrier in Rel-19	CATT

RP-232083	Rel-19 Multi-carrier enhancements for NR	NTT DOCOMO, INC.

RP-232127	Multi-carrier Enhancements in Rel-19	Fujitsu Limited

RP-232180	Views on Multi-carrier Enhancements in Rel-19	ZTE, Sanechips

RP-232219	Multiple carriers enhancement for R19	China Mobile International Ltd

RP-232246	Views on Multi-carrier enhancements in Rel-19	Beijing Xiaomi Mobile Software

RP-232264	Multi-carrier enhancements for Rel-19	China Telecom

RP-232326	[RAN1-led] Multi-Carrier Enhancements	MediaTek Inc.

RP-232432	Multi-carrier enhancements in Rel-19	Lenovo, CATT, China Unicom, New H3C, TCL Communication, xiaomi

RP-232575	Fragmented carriers in the DL 	TELUS

| Potential areas for Rel-19 | Supportive companies |
| --- | --- |
| Enhancements on multi-cell scheduling via single DCI | Nokia, OPPO, vivo, Huawei, CATT, Xiaomi, China Telecom, Lenovo, China Unicom, New H3C, TCL – 11 sources Multi-slot scheduling: Ericsson, DOCOMO Multi-UE scheduling: CMCC |
| Single cell with multiple carriers | CATT, CMCC, Xiaomi, China Telecom, MTK, TELUS, Lenovo, China Unicom, New H3C, TCL – 10 sources |
| Fast downlink carrier switching | Huawei, CATT, MTK (DL/UL), Lenovo, China Unicom, New H3C, TCL, Xiaomi – 8 sources |
| Cross cell HARQ retransmission | Huawei, CATT, MTK, Lenovo, China Unicom, New H3C, TCL, Xiaomi – 8 sources |
| Two-stage DCI | Xiaomi, China Telecom, MTK – 3 sources |
| SUL enhancement | Huawei, CMCC, Qualcomm – 3 sources |
| Other multi-carrier enhancements | Simplified CA operation and/or UE architecture: Fujitsu, CMCC Complementary UL-DL config: China Telecom Flexible DL-UL association: ZTE UL only SCell: Nokia, ZTE Improve MPR for intra-band CA in FR2: DOMOMO Joint CSI measurement across multiple carriers: Huawei Simultaneous scheduling via multi-cell DCI and legacy DCIs: Huawei/CATT UE capability sharing: ZTE Flexible cross carrier scheduling: ZTE |

Moderator’s observation:

- Some sources have indicated that they do not see urgent/critical need to introduce certain enhancements (even the entire WI).

- Strong level of support for the following proposed enhancements:

  - Enhancements on multi-cell scheduling via single DCI (11 sources out of 19 sources)

    - However, at least two companies have indicated that they do not see strong need

  - Single cell with multiple carriers (10 sources out of 18 sources).

    - However, at least two companies have indicated that they do not see strong need

## Offline discussion on multi-carrier enhancement (Wednesday)

The discussion during the offline session on Wednesday will focus on the following aspects:

Check if there is consensus (or very high level of support) on the necessity/urgency of having multi-carrier enhancement in Rel-19.

Outcome of offline session:

Companies who are supportive of multi-carrier enhancement in Rel-19: Futurewei, Lenovo/Motorola, Huawei/HiSi, DCM, TELUS, Bell Mobility, ZTE, MTK, Telstra, Nokia (if it’s a new proposal with sufficient operator demand), vivo, Xiaomi, Samsung (if scope is small) - Some of the companies indicated that it depends on the scope Companies who disagree (i.e., multi-carrier enhancement should not be part of Rel-19): - Some of the companies indicated that it depends on the scope

If there is consensus on the necessity/urgency, what would be the most critical aspects that needs to be treated under Rel-19 multi-carrier enhancement considering the limit TUs in RAN1?

Outcome of offline session:

From moderator point of view, the following topics seem to be a good starting point for further discussion on potential multi-carrier enhancement in Rel-19: - Enhancements on multi-cell/user scheduling via single DCI - Spec impact could depend on scope (ZTE), need to separate different proposals under this area (MTK/Apple) - Single cell with multiple carriers: - Concern on large spec impact (Ericsson/ZTE/Qualcomm/LGE/Lenovo/Motorola), need to involve RAN1/2/4 and should start with a study (Nokia) Note: Other companies indicated their preference on other proposals not included above

# Coverage Enhancements (agenda item 8A.2.12.3)

RP-231557	Views on scope for NR Coverage enhancements in Rel-19	Qualcomm Incorporated

RP-231708	Coverage enhancements in Rel-19	Nokia, Nokia Shanghai Bell

RP-231740	Study on Coverage Enhancements using RIS	Lekha Wireless Solutions

RP-231782	Discussion on coverage enhancement in Rel19	OPPO

Tdoc not available

RP-231801	Rel-19 NR coverage enhancements (RAN1-led)	vivo

RP-231912	UL coverage enhancements for Rel-19	Spreadtrum Communications

RP-231950	On Coverage enhancements	Ericsson

RP-231992	Views on Rel-19 Coverage Enhancements	InterDigital, Inc.

RP-232069	Rel-19 coverage enhancement	CATT

RP-232084	Views on Rel-19 UL Coverage Enhancement	NTT DOCOMO, INC.

RP-232147	Views on Rel-19 Coverage enhancements	Lenovo

RP-232181	Views on UL coverage enhancements in Rel-19	ZTE, Sanechips

RP-232213	Motivation for SI on IRS/RIS for coverage enhancements	KT Corp.

RP-232221	Discussion on Rel-19 Coverage Enhancement	China Mobile International Ltd

RP-232265	NR coverage enhancements for Rel-19	China Telecom

| Potential areas for Rel-19 | Supportive companies |
| --- | --- |
| Multiple PRACH TX over different beams (including beam selection for Msg3) | Qualcomm, Nokia, vivo, Spreadtrum, CATT, Ericsson, DOCOMO, Lenovo, ZTE, CMCC, China Telecom, Orange, Xiaomi, New H3C, TCL – 15 sources |
| PUSCH repetition scheduled by DCI 0_0 with C-RNTI | DOCOMO, Lenovo, ZTE, Nokia, Spreadtrum, CATT, China Telecom, Orange, Xiaomi, New H3C, TCL – 11 sources For CFRA: vivo |
| Waveform shaping using spectrum extension and/or tone reservation for PAPR reduction | IDC, Lenovo, China Telecom, Orange, Xiaomi, New H3C, TCL – 7 sources |
| DMRS bundling enhancement | Nokia, China Telecom, Orange, Xiaomi, New H3C, TCL – 6 sources |
| MPR related enhancement | (RAN4) MPR reduction for UL CA: Nokia (RAN4) Enhancement on P-MPR reporting: DOCOMO China Telecom, Orange, Xiaomi, New H3C, TCL – 6 sources |
| Other coverage enhancements | UL imbalance reduction (MMIMO): Qualcomm Reconfigurable Intelligent Surface (RIS): Lekha, KT Inter-slot frequency hopping: Ericsson Enhancement on PRACH repetition with same beam: Lenovo Dynamic waveform switching enhancement: Nokia, Lenovo (RAN2) Protocol enhancement for low-rate voice: Qualcomm |

Moderator’s observation:

- Strong level of support for the following proposed enhancements:

  - Multiple PRACH TX over different beams including beam selection for Msg3 (15 sources out of 18 sources)

  - PUSCH repetition scheduled by DCI 0_0 with C-RNTI (11 sources out of 18 sources)

## Offline discussion on coverage enhancement (Wednesday)

The discussion during the offline session on Wednesday will focus on the following aspects:

Check if there is consensus (or very high level of support) on the necessity/urgency of having coverage enhancement in Rel-19.

Outcome of offline session:

Companies who are supportive of coverage enhancement in Rel-19: ZTE, DCM, Verizon, Nokia, Xiaomi, OPPO, vivo, Orange, Lekha, NEC - QC/Ericsson/Samsung/Fujitsu/CATT (only okay if scope is limited i.e. only multiple PRACH TX over different beams and PUSCH repetition scheduled by DCI 0_0 with C-RNTI in Rel-19) Companies who disagree (i.e., coverage enhancement should not be part of Rel-19): Futurewei, IDC, MTK (not okay with multiple PRACH TX over different beams)

If there is consensus on the necessity/urgency, what would be the most critical aspects that needs to be treated under Rel-19 coverage enhancement considering the limit TUs in RAN1? From moderator point of view, the following potential enhancements seem to be a reasonable starting point considering the submitted contributions and the limited TU availability in RAN1.

- Multiple PRACH TX over different beams including beam selection for Msg3

- PUSCH repetition scheduled by DCI 0_0 with C-RNTI

Outcome of offline session:

From moderator point of view, the following topics seem to be a good starting point for further discussion on potential coverage enhancement in Rel-19: - Multiple PRACH TX over different beams including beam selection for Msg3 - MTK has concern on this proposal - CATT: In order to ensure RAN1 work can be contained, need to limit to beams with same SSB - PUSCH repetition scheduled by DCI 0_0 with C-RNTI

# Positioning Enhancements (agenda item 8A.2.12.4)

RP-231564	Introduction of A-GNSS support for NavIC L1 in NR	Reliance Jio

RP-231709	Positioning enhancements in Rel-19	Nokia, Nokia Shanghai Bell

RP-231783	Views on positioning enhancement in Rel-19	OPPO

RP-231802	Rel-19 Positioning Enhancements (RAN1-led)	vivo

RP-231878	View on Rel-19 Positioning	NEC

RP-231933	Positioning enhancements in Rel-19	Huawei, HiSilicon

RP-231943	On positioning enhancements in Rel-19	Ericsson

RP-231976	A-GNSS support for NavIC L1 band SPS in Release19	Reliance Jio, ISRO, CEWiT

RP-231993	Views on Rel-19 Positioning Enhancements	InterDigital, Inc.

RP-232010	Views on Rel-19 Positioning	Beijing Xiaomi Electronics

RP-232085	Rel-19 positioning enhancements for NR	NTT DOCOMO, INC.

RP-232151	Views on Rel-19 NR positioning	ZTE, Sanechips

RP-232198	Proposal on positioning enhancements in Rel-19	LG Electronics

RP-232295	Considerations for R19 work on Positioning	Philips International B.V.

RP-232343	Views on Positioning Enhancements in Rel-19	CATT, CEWiT, China Telecom, Fraunhofer HHI, Fraunhofer IIS, IIT Kanpur, InterDigital, KT Corp., Kyocera, LG E, NEC, Nokia, Robert Bosch GmbH, Sharp, Xiaomi

RP-232385	Sidelink Positioning Enhancements for Automotive Requirements	ROBERT BOSCH GmbH

RP-232400	Positioning enhancements for FR2 in Rel-19	China Unicom

RP-232466	Views on sidelink positioning enhancements for Rel-19	TOYOTA InfoTechnology Center, Continental Automotive, Hyundai Motor Company, DENSO CORPORATION

RP-232526	Vertical Requirements on RAN Rel-19 for Industrial 5G - Positioning	Siemens AG

RP-232544	Views on Rel-19 NR Positioning Enhancements	AT&T, FirstNet

RP-232557	Views on Release 19 Positioning	"IIT Kanpur, CEWiT, Locaila,Inc,  Indian Institute of Tech (M)	"

RP-232560	Discussion on Rel-19 Positioning enhancements	Intel Corporation

RP-232584	Positioning enhancement prioritization for Rel 19	CEWiT

Tdoc not available

| Potential areas for Rel-19 | Supportive companies |
| --- | --- |
| (Uu) CPP performance enhancement | Nokia, OPPO, NEC, Huawei, LGE, CATT, CEWiT, China Telecom, Fraunhofer, IIT Kanpur, IDC, KT, Kyocera, Bosch, Sharp, Xiaomi, Locaila, Indian Institute of Tech (M) – 18 sources |
| (Uu) Support of NavIC for L1 navigation band | Reliance Jio, Nokia, ISRO, CEWiT – 4 sources |
| (Uu) Enhanced LPHAP | NEC, ZTE, Philips, Intel – 4 sources |
| (Uu) Positioning on unlicensed spectrum | AT&T, FirstNet, Intel – 3 sources |
| (Uu) Enhancements for FR2 positioning | Huawei, China Unicom – 2 sources |
| Other positioning enhancements for Uu link | (Uu) UWB based positioning: ZTE (Uu) Enhanced NW resource efficiency of positioning: DOCOMO (Uu) CPP application extension (RedCap, mobility): Nokia |
| (SL) Sidelink positioning on unlicensed spectrum | OPPO, NEC, IDC, Xiaomi, ZTE, LGE, Philips, Toyota, Continental, Hyundai, DENSO, AT&T, FirstNet, Intel – 13 sources (multi-band: licensed+unlicensed) Bosch |
| (SL) Power saving | OPPO, Xiaomi, ZTE, Toyota, Continental, Hyundai, DENSO, AT&T, FirstNet – 9 sources |
| (SL) Sidelink CPP | Nokia, OPPO, Xiaomi, Bosch, Toyota, Continental, Hyundai, DENSO – 8 sources |
| (SL) Enhancements for FR2 positioning | OPPO, NEC, Xiaomi, Toyota, Continental, Hyundai, DENSO – 7 sources (multi-band: FR1+FR2) Bosch |
| (SL) Positioning integrity | OPPO, IDC, Bosch, AT&T, FirstNet – 5 sources |
| Other enhancements for SL positioning | (SL) Enhanced assistance information reporting: Bosch (SL) XR positioning: vivo (SL) Enhanced assistance information reporting: Bosch |
| (Uu+SL) Extension of positioning to other verticals (e.g. industrial use cases) | Siemens, Bosch |

Note 1: AI/ML based positioning will not be covered in this document.

Note 2: RP-232343 and RP-232557 which indicated support for enhancements on CPP and SL positioning were submitted by 14 sources and 3 sources, respectively. For enhancements on CPP, the supporting companies were added in the above table. However, for enhancements on SL positioning, there were not enough details for the moderator to include as part of the above table (moderator could not tell which of the SL positioning sub-topics the companies were supporting).

Moderator observations:

- Some sources have indicated that they do not see urgent/critical need to introduce certain enhancements (even the entire WI).

- Most of the contributions supported

  - (Uu) CPP performance enhancement

  - Some form of sidelink positioning enhancement

## Offline discussion on positioning enhancement (Wednesday)

The discussion during the offline session on Wednesday will focus on the following aspects:

Check if there is consensus (or very high level of support) on the necessity/urgency of having positioning enhancement in Rel-19.

Outcome of offline session:

Companies who are supportive of positioning enhancement in Rel-19: IDC, Bosch, Intel, LGE, CATT, IIT Kanpur, Lekha, Reliance Jio, IIT Jodhpur, Xiaomi, Philips, AT&T, NEC, IIT Delhi, Huawei/HiSi (if FR2 positioning is included), CeWiT, Sharp, Tejas Network, Fraunhofer, vivo (only SL positioning), Vodafone (only CPP) Companies who disagree (i.e., positioning enhancement should not be part of Rel-19): Samsung, Qualcomm, DOCOMO, Nokia, Ericsson, MTK, Fujitsu, Vodafone (any proposal on SL positioning) - Above does not apply for the NavIC support Futurewei/CMCC: If anything is to be done in Rel-19, scope has to be really small Intel/Bosch/Huawei/HiSi/CATT/Philips: Outcome of Rel-18 positioning should be considered (e.g. Rel-18 downscoping) Reliance Jio/Bharti Airtel/QC/Nokia/Ericsson/Bosch/CeWiT/IIT Kanpur/IIT Delhi/IIT Jodhpur/DOCOMO/Lekha: Support of NavIC for L1 navigation band should be supported. Support of NavIC is important for the Indian market. Impact is mainly on RAN2 and there is no impact on RAN1. This enhancement should be considered separately from other positioning enhancements. ZTE/AT&T: If there is anything to be done in Rel-19, enhancements in RAN2 should be the focus.

# SL Evolution (agenda item 8A.2.12.5)

RP-231630	Rel-19 Sidelink Evolution	Nokia, Nokia Shanghai Bell

RP-231762	Views on scope for NR Sidelink evolution in Rel-19	Qualcomm India Pvt Ltd

RP-231784	Discussion on sidelink further evolution project scope for Rel-19	OPPO

RP-231803	Rel-19 Sidelink Evolution (RAN1-led)	vivo

RP-231879	Continual Sidelink evolution	NEC

RP-231994	Views on Sidelink Enhancements for Rel-19	InterDigital, Inc.

RP-232066	Views on Rel-19 Sidelink	CATT, GOHIGH

RP-232098	Views On Sidelink Evolution in R19	Apple Inc.

RP-232110	Discussion on NR sidelink evolution in Rel-19	LG Electronics UK

RP-232112	Views on sidelink evolution in Rel-19	xiaomi

RP-232172	Views on Rel-19 sidelink evolution	ZTE, Sanechips

RP-232287	Views on 5G Advanced Rel-19 Sidelink Evolution	Fraunhofer IIS, Fraunhofer HHI

RP-232333	[RAN1-led] Sidelink Enhancements	MediaTek Inc.

RP-232392	Sidelink Enhancements for Automotive Requirements	ROBERT BOSCH GmbH

RP-232417	Sidelink enhancements in Rel-19	Huawei, HiSilicon

RP-232465	Views on sidelink enhancements for Rel-19	TOYOTA InfoTechnology Center, Continental Automotive, Hyundai Motor Company, DENSO CORPORATION

RP-232528	Vertical Requirements on RAN Rel-19 for Industrial 5G- Sidelink Enhancements	Siemens AG

RP-232545	Views on Rel-19 Sidelink and SL Relay Enhancements	AT&T, FirstNet

RP-232564	Views on sidelink evolution in Rel-19?	Volkswagen AG

RP-232585	Sidelink enhancement prioritization for Rel 19	CEWiT

| Potential areas for Rel-19 | Supportive companies |
| --- | --- |
| Carrier aggregation enhancement | Nokia, OPPO, NEC, IDC, CATT, Apple, LGE, xiaomi, ZTE, Fraunhofer, MTK, Bosch, Huawei, Toyota/Continental/Hyundai/Denso, AT&T/FirstNet, Volkswagen – 20 sources |
| FR2 enhancement (multi-beam operation) | Qualcomm, OPPO, vivo, NEC, IDC, CATT, Apple, LGE, xiaomi, ZTE, Fraunhofer, MTK, Bosch, Huawei, Toyota/Continental/Hyundai/Denso, CEWiT – 19 sources |
| Power saving enhancement | OPPO, IDC, xiaomi, ZTE, MTK, Toyota/Continental/Hyundai/Denso, AT&T/FirstNet – 11 sources |
| Extending SL to other verticals (RedCap, IIoT/URLLC) | RedCap: Qualcomm, vivo IIoT/URLLC: Siemens |
| Other SL enhancements | (SL underlay transmission over licensed spectrum) vivo (Coverage enhancement) Apple (Coexistence enhancement) Bosch, Volkswagen (TSC enhancement for SL) Bosch (SL-unlicensed enhancement) IDC |

Note: SL relay will not be covered in this document.

Moderator’s observation:

- Proposed enhancements for Rel-19 SL evolution are relatively more converged compared to the other areas. However, the proposed enhancements could potentially include wide range of sub-topics. A more focused scope would be beneficial in order to move forward.

- Strong level of support for the following proposed enhancements:

  - SL carrier aggregation enhancement (20 sources out of 24 sources)

  - SL FR2 enhancement (19 sources out of 24 sources).

## Offline discussion on sidelink evolution (Wednesday)

The discussion during the offline session on Wednesday will focus on the following aspects:

Check if there is consensus (or very high level of support) on the necessity/urgency of having sidelink evolution in Rel-19.

Outcome of offline session:

Companies who are supportive of sidelink evolution in Rel-19: NEC, IDC, Bosch, Huawei/HiSi, Apple, OPPO, LGE, WILUS, ETRI, Xiaomi, AT&T, ZTE, Sharp, CeWiT, Fraunhofer, Futurewei, Sony Companies who disagree (i.e., sidelink evolution should not be part of Rel-19): Ericsson, VF, Samsung, Nokia, DOCOMO, Fujitsu, Telstra, TMobile, TIM, MTK Should prioritize SL FR2 in Rel-19: Huawei/HiSi, CMCC (not happy but acceptable), QC (not happy but FR2 and RedCap are acceptable), Verizon, CeWiT, Futurewei, OPPO, vivo (not happy but FR2 and RedCap are acceptable), Fraunhofer (needed for industrial application) - Intel: There was no study/consideration on FR2 SL in RAN4. Study is needed in RAN4 before normative work in RAN1 SL CA enhancement is needed: LGE, Bosch (needed for automotive industry), Fraunhofer (needed for automotive application). Current SL CA is worse than that of LTE Rel-15 SL CA.

# Other Proposals (agenda item 8A.2.15)

Only the following tdocs have proposals on RAN1-led enhancements.

RP-232006	Motivation for Release-19 Network Energy Savings for LTE eNB	KT Corp.

RP-232129	Study on the scenarios and requirements for high density access in power system	CEPRI

RP-232164	Views on high reliability and low complexity IoT in Rel-19	ZTE, Sanechips

RP-232276	Demonstration of RIS and ISAC techniques in Hangzhou Asian Games	China Telecom, ZTE

RP-232277	Consideration on Rel-19 ISAC	China Telecom

RP-232279	Views on RIS for Rel-19	China Telecom

RP-232396	Discussion of Integrated Sensing and Communication in Rel-19	China Unicom

RP-232425	A Simple PHY-Aware, MAC-Layer Outer-Coding Scheme for Reliable, Single-TTI Communication	IISc, Bangalore

RP-232529	Vertical Requirements on RAN Rel-19 for Industrial 5G - Time Synchronization	Siemens AG

RP-232278	Views on NR Rel-19 AIML for air interface	China Telecom

Should be considered as part of discussions in 8A.2.1

## Offline discussion on other proposals (Wednesday)

During the offline session, moderator intends to check the level of support for each of the following potential enhancements:

- Network energy saving for LTE (KT)

- High density access in power system (CEPRI)

- High reliability and low complexity IoT (ZTE)

- RIS and ISAC (China Telecom, China Unicom, ZTE)

- Outer-coding scheme for single TTI (IISc, Bangalore)

- Time synchronization for verticals (Siemens)

Outcome of offline session:

During the offline session, the moderator checked the support for the proposals listed below. Due to the lack of support, there was no additional discussions. - Network energy saving for LTE (KT) - High density access in power system (CEPRI) - High reliability and low complexity IoT (ZTE) - RIS and ISAC (China Telecom, China Unicom, ZTE) - Outer-coding scheme for single TTI (IISc, Bangalore) - Time synchronization for verticals (Siemens)

# Conclusions

The following subsections summarizes the outcome of the offline discussions.

## Multi-carrier enhancements

Outcome of offline session:

Companies who are supportive of multi-carrier enhancement in Rel-19: Futurewei, Lenovo/Motorola, Huawei/HiSi, DCM, TELUS, Bell Mobility, ZTE, MTK, Telstra, Nokia (if it’s a new proposal with sufficient operator demand), vivo, Xiaomi, Samsung (if scope is small) - Some of the companies indicated that it depends on the scope Companies who disagree (i.e., multi-carrier enhancement should not be part of Rel-19): - Some of the companies indicated that it depends on the scope

Outcome of offline session:

From moderator point of view, the following topics seem to be a good starting point for further discussion on potential multi-carrier enhancement in Rel-19: - Enhancements on multi-cell/user scheduling via single DCI - Spec impact could depend on scope (ZTE), need to separate different proposals under this area (MTK/Apple) - Single cell with multiple carriers: - Concern on large spec impact (Ericsson/ZTE/Qualcomm/LGE/Lenovo/Motorola), need to involve RAN1/2/4 and should start with a study (Nokia) Note: Other companies indicated their preference on other proposals not included above

## Coverage enhancements

Outcome of offline session:

Companies who are supportive of coverage enhancement in Rel-19: ZTE, DCM, Verizon, Nokia, Xiaomi, OPPO, vivo, Orange, Lekha, NEC - QC/Ericsson/Samsung/Fujitsu/CATT (only okay if scope is limited i.e. only multiple PRACH TX over different beams and PUSCH repetition scheduled by DCI 0_0 with C-RNTI in Rel-19) Companies who disagree (i.e., coverage enhancement should not be part of Rel-19): Futurewei, IDC, MTK (not okay with multiple PRACH TX over different beams)

Outcome of offline session:

From moderator point of view, the following topics seem to be a good starting point for further discussion on potential coverage enhancement in Rel-19: - Multiple PRACH TX over different beams including beam selection for Msg3 - MTK has concern on this proposal - CATT: In order to ensure RAN1 work can be contained, need to limit to beams with same SSB - PUSCH repetition scheduled by DCI 0_0 with C-RNTI

## Positioning enhancements

Outcome of offline session:

Companies who are supportive of positioning enhancement in Rel-19: IDC, Bosch, Intel, LGE, CATT, IIT Kanpur, Lekha, Reliance Jio, IIT Jodhpur, Xiaomi, Philips, AT&T, NEC, IIT Delhi, Huawei/HiSi (if FR2 positioning is included), CeWiT, Sharp, Tejas Network, Fraunhofer, vivo (only SL positioning), Vodafone (only CPP) Companies who disagree (i.e., positioning enhancement should not be part of Rel-19): Samsung, Qualcomm, DOCOMO, Nokia, Ericsson, MTK, Fujitsu, Vodafone (any proposal on SL positioning) - Above does not apply for the NavIC support Futurewei/CMCC: If anything is to be done in Rel-19, scope has to be really small Intel/Bosch/Huawei/HiSi/CATT/Philips: Outcome of Rel-18 positioning should be considered (e.g. Rel-18 downscoping) Reliance Jio/Bharti Airtel/QC/Nokia/Ericsson/Bosch/CeWiT/IIT Kanpur/IIT Delhi/IIT Jodhpur/DOCOMO/Lekha: Support of NavIC for L1 navigation band should be supported. Support of NavIC is important for the Indian market. Impact is mainly on RAN2 and there is no impact on RAN1. This enhancement should be considered separately from other positioning enhancements. ZTE/AT&T: If there is anything to be done in Rel-19, enhancements in RAN2 should be the focus.

## Sidelink evolution

Outcome of offline session:

Companies who are supportive of sidelink evolution in Rel-19: NEC, IDC, Bosch, Huawei/HiSi, Apple, OPPO, LGE, WILUS, ETRI, Xiaomi, AT&T, ZTE, Sharp, CeWiT, Fraunhofer, Futurewei, Sony Companies who disagree (i.e., sidelink evolution should not be part of Rel-19): Ericsson, VF, Samsung, Nokia, DOCOMO, Fujitsu, Telstra, TMobile, TIM, MTK Should prioritize SL FR2 in Rel-19: Huawei/HiSi, CMCC (not happy but acceptable), QC (not happy but FR2 and RedCap are acceptable), Verizon, CeWiT, Futurewei, OPPO, vivo (not happy but FR2 and RedCap are acceptable), Fraunhofer (needed for industrial application) - Intel: There was no study/consideration on FR2 SL in RAN4. Study is needed in RAN4 before normative work in RAN1. SL CA enhancement is needed: LGE, Bosch (needed for automotive industry), Fraunhofer (needed for automotive application). Current SL CA is worse than that of LTE Rel-15 SL CA.

## Other proposals

Outcome of offline session:

During the offline session, the moderator checked the support for the proposals listed below. Due to the lack of support, there was no additional discussion. - Network energy saving for LTE (KT) - High density access in power system (CEPRI) - High reliability and low complexity IoT (ZTE) - RIS and ISAC (China Telecom, China Unicom, ZTE) - Outer-coding scheme for single TTI (IISc, Bangalore) - Time synchronization for verticals (Siemens)