3GPP TSG-RAN Meeting #101	RP-232629

# Bangalore, India, 11– 15 September 2023													R2-2309009

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4939 | rev | 2 | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NS-value utilization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | - |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2023-09-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | IAB support was included in Rel-16 for NR and EN-DC, with the LTE CR being agreed in CR4233 in R2-2002358. The CR included handling of cell barring for IAB-MT if the iab-Support is not broadcast in SIB1. However, the procedural text change has been inserted in wrong place (i.ew. in the middle of p-Max and additionalSpectrumEmission selection for UE), which means the “else” branch using the legacy NS-value and P-Max selection is only entered if the cell is not barred for IAB-MT, which breaks the logic of multiple NS-values for a given (m)FBI. This can also be seen easily from the SIB1-NB procedural text, where the IAB-MT cell barring is not used so the original procedural text is visible. This means the LTE specification is broken for the NS-value and P-max selection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Move the procedural text of cell barring for IAB-MT node outside of the existing procedural text of selecting the p-Max and additionalSpectrumEmission in case of multiple values broadcast in SIB1. Impacted architecture options: EN-DC, NE-DC Impact analysis Impacted functionality: UL power control. Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE may use the incorrect A-MPR and P-Max for the serving carrier. - If the UE is implemented according to the CR and the network is not, there are no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE selection of NS-value and P-Max is incorrect in LTE RRC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is mandatory for all UEs implementing LTE Rel-16. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.2: Corrects indenting changes omitted from Rev.1 (but which were taken into account in Rel-17 shadow CR in R2-2309010) |  |  |  |  |  |  |  |  |

First Modified Subclause

#### 5.2.2.7	Actions upon reception of the SystemInformationBlockType1 message

Upon receiving the SystemInformationBlockType1 or SystemInformationBlockType1-BR either via broadcast or via dedicated signalling, the UE shall:

1>	if the upper layers indicate the selected core network type as 5GC:

2>	if the cellAccessRelatedInfoList-5GC contains an entry with the plmn-Identity or plmn-Index of the selected PLMN:

3>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding cellAccessRelatedInfoList-5GC containing the selected PLMN;

1>	else if the cellAccessRelatedInfoList contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding cellAccessRelatedInfoList containing the selected PLMN;

1>	if in RRC_IDLE or in RRC_CONNECTED while T311 is running; and

1>	if the UE is a category 0 UE according to TS 36.306 [5]; and

1>	if category0Allowed is not included in SystemInformationBlockType1:

2>	consider the cell as barred in accordance with TS 36.304 [4];

1>	if in RRC_CONNECTED while T311 is not running, and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators:

2>	disregard the freqBandIndicator and multiBandInfoList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

1>	else:

2>	if UE is IAB-MT and if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

3>	consider the cell as barred for IAB-MT in accordance with TS 36.304 [4];

3>	perform barring as if intraFreqReselection is set to allowed, and as if the csg-Indication is set to FALSE;

2>	else

3>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE and it is not a downlink only band; or

3>	if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE and they are not downlink only bands:

4>	forward the cellIdentity to upper layers;

4>	forward the trackingAreaCode to upper layers;

4>	forward the PLMN identity to upper layers;

4>	if in RRC_INACTIVE and the forwarded information does not trigger message transmission by upper layers:

5>	if the serving cell does not belong to the configured ran-NotificationAreaInfo:

6>	initiate an RNA update as specified in 5.3.17.2;

4>	forward the ims-EmergencySupport to upper layers, if present;

4>	forward the eCallOverIMS-Support to upper layers, if present;

4>	if the UE is capable of 5G NAS:

5>	forward the ims-EmergencySupport5GC to upper layers, if present;

5>	forward the eCallOverIMS-Support5GC to upper layers, if present;

5>	forward cp-CIoT-5GS-Optimisation to upper layers, if present for the selected PLMN;

5>	forward up-CIoT-5GS-Optimisation to upper layers, if present for the selected PLMN;

4>	if, for the frequency band selected by the UE (from freqBandIndicator or multiBandInfoList), the freqBandInfo or the multiBandInfoList-v10j0 is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within the freqBandInfo or multiBandInfoList-v10j0:

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo or multiBandInfolist-v10j0;

5>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList:

6>	apply the additionalPmax;

5>	else:

6>	apply the p-Max;

3>	if UE is IAB-MT and if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

4>	consider the cell as barred for IAB-MT in accordance with TS 36.304 [4];

4>	perform barring as if intraFreqReselection is set to allowed, and as if the csg-Indication is set to FALSE;

4>	else:

5>	apply the additionalSpectrumEmission in SystemInformationBlockType2 and the p-Max;

3>	else:

4>	consider the cell as barred in accordance with TS 36.304 [4]; and

4>	perform barring as if intraFreqReselection is set to notAllowed, and as if the csg-Indication is set to FALSE;

Upon receiving the SystemInformationBlockType1-NB, the UE shall:

1>	if the upper layers indicate the selected core network type as 5GC:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the cellAccessRelatedInfo-5GC;

1>	else:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the cellAccessRelatedInfo;

1>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE; or

1>	if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE:

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

2>	if attachWithoutPDN-Connectivity is received for the selected PLMN:

3>	forward the attachWithoutPDN-Connectivity to upper layers;

2>	else

3>	indicate to upper layers that attachWithoutPDN-Connectivity is not present;

2>	if the UE is capable of 5G NAS:

3>	forward ng-U-DataTransfer to upper layers, if present for the selected PLMN;

3>	forward up-CIoT-5GS-Optimisation to upper layers, if present for the selected PLMN;

2>	if, for the frequency band selected by the UE (from freqBandIndicator or multiBandInfoList), the freqBandInfo is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within the freqBandInfo:

3>	apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo;

3>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList:

4>	apply the additionalPmax;

3>	else:

4>	apply the p-Max;

2>	else:

3>	apply the additionalSpectrumEmission in SystemInformationBlockType2-NB and the p-Max;

1>	else:

2>	consider the cell as barred in accordance with TS 36.304 [4]; and

2>	perform barring as if intraFreqReselection is set to notAllowed.

No UE requirements related to the contents of SystemInformationBlockType1-MBMS apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

End of Changes