3GPP TSG RAN Meeting #101	RP-232657

Bengaluru, India 11th – 15th September, 2023

Agenda item:	9.3.2.9

Source:		Offline session moderator (Nokia)

Title:		Summary of offline session on 2Rx for XR

Document for:	Discussion and Decision

# Introduction

Offline sessions were held at Monday lunchtime and Tuesday afternoon coffee break to progress matters on 2Rx XR devices, based on the contributions submitted to Agenda Item 9.3.2.9.

# Way forward and next steps

RAN#99 had outlined 2 steps towards progressing discussions on 2Rx XR devices, cf. RP-230740:

“Step-1: Recognize that form factor limitations of a subset of XR devices can make it impossible for these devices to support 4 antenna ports.

- Exact characterization and form factor limitations of these devices are to be studied in RAN plenary. Contributions are invited to TSG-RAN#100 to address this.”

Proposed resolution for Step-1 for the purpose of continuing discussions to Step-2:

- Handheld smartphone UEs are excluded from any 2Rx relaxation for XR wearables.

The default for non-RedCap XR-wearable UEs is 4Rx (for frequency bands where 4Rx is mandated).

A non-RedCap XR-wearable UE can be considered for 2Rx relaxation (for frequency bands where 4Rx is mandated) if and only if:

- Intended to be worn on the human head;

- When in use, is intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains;

-	No other relaxation is being considered apart from number of Rx antennas.

________________________________________________________________________________________

Step-2 is outlined as follows (cf. RP-230740):

“Step-2: Recognize the importance of identifying these devices in the network, and potential network impact (e.g. performance, coverage, capacity) and UE impact

- Details of Step-2 will be further defined following the maturity of Step-1”

Proposed scope for Step-2 for RAN#102:

Address the impacts to RF and OTA requirements for non-RedCap 2Rx XR devices, and their potential network impact (performance, coverage, capacity, etc…) and the mitigation techniques.

It is assumed that there is a need for the network to clearly identify non-RedCap XR devices with 2Rx restriction.