3GPP TSG-RAN WG Meeting #101	RP‑232678

Bangalore, India, September 11-15, 2023

Title:	LS on signaling support for intra-band non-collocated NR-CA,EN-DC

Response to:

Release:	Rel-18 (NR-CA), Rel-18 (EN-DC)

Work Item:	NonCol_intraB_ENDC_NR_CA-Core

Source:	RAN

To:	RAN2

Cc:	RAN4

Contact person:

Name:	Yasuki Suzuki

Email Address:	ui-suzuki@kddi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

1) New UE capability

The UE capability “interBandMRDC-WithOverlapDL-Bands-r16” was introduced since Rel-16 for UE to indicate the support of non-collocated deployment for FDD-FDD or TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping bands, while the UE capability to support non-collocated deployment for TDD-TDD intra-band NR-CA is being discussed in Rel-18.

[For TDD-TDD intra-band NR-CA]

During RAN4#107 meeting, RAN4 concluded a separate new capability signalling is needed, which is to indicate the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1 (Type 2 capability requirement).

If the new UE capability is absent, the UE supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1 (Type 1 capability requirement).

RAN4 also concluded the UE indicating the [New UE capability] can also support Type 1 capability requirement.

This new UE optional capability is expected to be per BC, and applicable to only band n77 and n78.

RAN would like to request RAN2 to specify [New UE capability] based on the RAN4 agreements.

[For TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands]

In this case, the existing UE capability “interBandMRDC-WithOverlapDL-Bands-r16” and existing RAN4 requirements for Type 1 capability and Type 2 capability shall be used. See “interBandMRDC-WithOverlapDL-Bands-r16” in 38.306.

2) New BS signalling

During RAN#101 meeting, RAN agreed to introduce new BS signalling by RRC to indicate whether Type 1 or Type 2 capability requirements will be applied by a UE configured by the network with the value of IE maxMIMO-Layers no more than 2 and task RAN2 to develop relevant signalling.

This new BS signalling is applied to the below cases.

[For TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands]

This new BS signalling is expected to be applicable for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands provided the UE has indicated “interBandMRDC-WithOverlapDL-Bands-r16”.

[For TDD-TDD intra-band NR-CA]

This new BS signalling is expected to be applicable for TDD-TDD intra-band NR-CA provided the UE has indicated [New UE capability].

2	Actions

To RAN2:

ACTION: 	RAN respectfully requests RAN2 to specify the above UE capability reporting for intra-band non-collocated NR-CA in Rel-18, and the above new BS signalling by RRC to indicate whether Type 1 or Type 2 capability requirements will be applied by a UE configured by the network with the value of IE maxMIMO-Layers no more than 2 and task RAN2 to develop relevant signalling.

3	Dates of next TSG-RAN WG meetings

TSG-RAN WG2 Meeting #123bis              09th – 13th Oct 2023             	 Xiamen, China

TSG-RAN WG2 Meeting #124                  13th – 17th Nov 2023             	 Chicago, US