3GPP TSG RAN Meeting #101	RP-232690

Bengaluru, India, September 11-15, 2023

Source:	Huawei, HiSilicon

Title:	pCR for TR 38.848 on Deployment scenarios and their characteristics

Agenda item:	9.2.2

Document for:	Agreement

- Introduction

This pCR proposes text to add to characteristics of deployment scenarios of Ambient IoT into TR 38.848 v0.2.0.

- Proposed changes

# <Unchanged text is omitted>

## 4.2	Deployment scenarios and connectivity topologies

Deployment scenarios for Ambient IoT have been studied on the basis of a list of characteristics, and the representative use case(s) applicable to a scenario. The possible descriptions of the characteristics are as follows:

Table 4.2-1: Characteristics of deployment scenarios

| Characteristic | Possible description entries |
| --- | --- |
| Environment (of the device) | Indoor Outdoor Indoor or outdoor |
| Basestation characteristic (if any) | Macro-cell-based deployment Micro-cell-based deployment Pico-cell-based deployment None |
| Connectivity topology | See section 4.2.1 |
| Spectrum | Licensed FDD Licensed TDD Unlicensed Note: In each connectivity topology of the study, if a BS is present, it is assumed that the BS uses licensed spectrum |
| Coexistence with existing 3GPP technologies | Deployed on the same sites as an existing 3GPP deployment corresponding to the basestation type. Deployed on new sites without an assumption of an existing 3GPP deployment. |
| Traffic assumption | Device-terminated (DT) Device-originated (DO) DO traffic includes DO autonomous (DO-A), and DO device-terminated triggered (DO-DTT) |
| Device characteristic | See Section 4.3: Device A Device B Device C |

The study has considered Ambient IoT deployments in-band to NR, in guard-band of NR, and in standalone band from NR.

<Unchanged text is omitted>