3GPP TSG RAN Meeting #102																	RP-233396

Edinburgh, Scotland, December 11-15, 2023

Agenda Item:		9.1.1.2

Source:				InterDigital, Inc.

Title:						On the scope of Rel-19 MIMO Evolution

Document for:		Discussion and Decision

# Introduction

In the last plenary meeting, based on the collective interest of companies, an initial list of potential topics for Rel-19 MIMO Evolution was compiled. Table 1 captures the outcome of the offline discussion where 6 main topics were identified for further discussion [1]. Per Chairman guidance, there will be only about 2 TUs allocated for Rel-19 MIMO, hence further discussion for down-selection from the list of identified 6 topics is needed [2]. In this contribution, we share our views on the scope and priorities for Rel-19 MIMO Evolution.

Table 1 – Outcome of Offline Discussions (RANP #101) [1]

Topic 1: Beam management enhancements to reduce overhead/latency through UE-initiated/event-driven beam management Objective 1. Signalling/mechanism to facilitate UE-initiated beam management procedure including UE-initiated beam reporting/switch

Topic 2: Enhancements to CSI framework to support > 32 (64/128) CSI-RS ports Objective 1: Type I codebook enhancements to support > 32 CSI-RS ports Objective 2: Type II codebook enhancements to support > 32 CSI-RS ports Objective 3: Hybrid beamforming enhancements (CRI based reporting enhancements) Note 1: Extension of legacy codebooks and legacy CSI-RS resources Note 2: Objective 3 may require further study or clarification.

Topic 3: CJT/DL multi-TRP enhancements Objective 1: UE-assisted calibration reporting of delay and frequency/phase offsets for CJT with non-ideal synchronization and backhaul Note: Assume legacy CSI-RS design and standalone aperiodic reporting on PUSCH.

Topic 4: UL enhancements Objective 1: STxMP enhancements (e.g. Simultaneous TX of PUCCH and PUSCH, Asymmetric panel implementations, mDCI PUCCH + PUCCH, STxMP with up to rank 8, Coherent SFN STxMP) Objective 2: Enhancements for UL 3Tx including 3Tx for UL codebook and non-codebook based transmission

Topic 5: Enhancement for asymmetric downlink S-TRP/UL M-TRP scenario assuming intra-band intra-cell non-co-located M-TRP scenarios without changing existing cell definition or defining a new cell Objective 1: Extension of Rel-18 2TA mDCI to sDCI assuming legacy PRACH resources Objective 2: Separate UL power control for SRS only to downlink S-TRP from SRS to UL M-TRP and introduce path loss measurement to uplink M-TRP

Topic 6: 6Rx/8Rx UE enhancements with lower complexity utilizing two segments of 3/4 Rx antenna units up to 8-layer DL Tx based on legacy codebook and legacy codeword to layer mapping Objective 1: SRS antenna port grouping, CSI and codeword association to the segments of receive antenna.

# Discussions

Progression of NR MIMO Enhancements

The scope of NR MIMO has been always a mix of enhancements for downlink and uplink features; however, their corresponding proportions have varied over time. In Rel-15 and -16, the focus of the NR MIMO was on developing a robust and versatile framework for downlink transmission and beam management, such as: Codebook Types I & II, CJT, BFR, beam indication/selection procedures, etc. However, from Rel-17 forward, uplink-oriented enhancements have started to get more traction, by inclusion of features such as PUCCH/PUSCH repetition, SRS capacity/interference enhancements, 8TX UEs, etc.

In our view, the short list of potential topics summarized in Table 1 seems a reasonable starting point. However, given the limited time units considered for Rel-19 MIMO, further down-selection is required. In the following, we share our view for each of the identified topic.

Topic 1:

Beam management enhancements to reduce overhead/latency through UE-initiated/event-driven beam management - Objective 1. Signalling/mechanism to facilitate UE-initiated beam management procedure including UE-initiated beam reporting/switch

We are supportive of this topic. In general, BFR latency is considered one of critical factors in operation of NR. In some use-cases, such as operation in FR2, high-mobility UEs, etc., transmission/reception beams are more exposed to fast changes in the channel and transmission environment, and hence they may be more susceptible to frequent beam failure occurrences. Therefore, an event-driven beam management approach can take certain actions, such as reporting of the state of the beams prior to a failure, autonomous beam switching, etc. can be very beneficial in overall robustness and performance of the system.

We appreciate that some companies may have concern with consideration of autonomous beam switching. However, such concern may be alleviated by limiting such autonomous beam switching to very limited scenarios, such as entering a specific time window or geolocation. For example, a high-speed UE can be configured with a series of beam information, then UE would assume a new beam configuration as every time it enters a new time-reference or geo-location.

Observation 1: By allowing a UE to initiate a beam switch and/or report procedure, a highly robust beam operation with a low latency and overhead can be achieved.

.

Proposal 1: Support Topic 1 to enhance robustness and latency of beam operation through event-driven beam management, where an event may be defined as a measurement condition at the UE or entering a new time-reference or geo-location.

Topic 2:

Enhancements to CSI framework to support > 32 (64/128) CSI-RS ports - Objective 1: Type I codebook enhancements to support > 32 CSI-RS ports - Objective 2: Type II codebook enhancements to support > 32 CSI-RS ports - Objective 3: Hybrid beamforming enhancements (CRI based reporting enhancements) Note 1: Extension of legacy codebooks and legacy CSI-RS resources Note 2: Objective 3 may require further study or clarification.

In general, we are supportive of this topic mainly for two reasons. Firstly, in our view, this topic reflects the natural evolution of the MIMO operation in a cellular system, and it is another step towards support and implementation of massive MIMO which will be the baseline transmission scheme in 6G. Hence, further enhancements in capacity, coverage and reliability can be expected through the support of this topic.  Secondly, based on operators’ feedback, there is a gap between the state-of-the-art in gNB implementation and what current specifications can support. Therefore, it is important to adapt the specifications to support implementation and deployment of more capable gNBs to enhance overall system performance and efficiency.

Having said that we have some concern about the workload under Topic 2. In our view, we should prioritize and restrict the scope of Topic 2 to only Objectives 1 and 2 which are focused on codebook enhancements to address an obvious limitation in the current specifications. While we see some benefit in support of Objective 3, we do not believe that there is an urgency for this topic.

Observation 2: Objectives 1 and 2 which are focused on codebook enhancements to address an obvious limitation in the current specifications.

.

Proposal 2: Support Topic 2 to enhance CSI framework to include > 32 (64/128) CSI-RS ports, and restrict the scope to only Objectives 1 and 2.

Topic 3:

CJT/DL multi-TRP enhancements - Objective 1: UE-assisted calibration reporting of delay and frequency/phase offsets for CJT with non-ideal synchronization and backhaul Note: Assume legacy CSI-RS design and standalone aperiodic reporting on PUSCH.

We have a favorable opinion about Topic 3. CJT-based m-TRP transmission has been the framework for several enhancements, such as repetition-based PDSCH and PDCCH in Rel-16 and rel-17 respectively. Further, in Rel-18, the Type-II codebook was enhanced to support CJT transmission with up to 4 TRPs. Throughout all these enhancements, it has been always assumed that TRPs participating in a CJT transmission are perfectly synchronized in time and frequency. However, in realistic deployments, such assumptions do not hold true, and the resulting time, phase or frequency misalignments could degrade the performance by a degree that the expected gain of CJT may never be realized. Therefore, an efficient UE-based mechanism is required to assist gNB to calibrate its transmission per UE or per group of UEs. By relying on UE assistance, a gNB can perform the required calibration in several different ways, such as direct frequency or timing corrections of TRP(s), or compensation of the incurred impairment by applying a correction on the downlink precoders.

Observation 3: In realistic scenarios, assumptions of perfect synchronization in frequency and time do not hold true, and certain UE-based mechanisms are required to assist gNB to calibrate its transmission per UE or per group of UEs.

Proposal 3: Support Topic 3 to enable a UE-based mechanism to assist with a time/frequency-synchronized transmission in a CJT deployment.

Topic 4:

UL enhancements - Objective 1: STxMP enhancements (e.g. Simultaneous TX of PUCCH and PUSCH, Asymmetric panel implementations, mDCI PUCCH + PUCCH, STxMP with up to rank 8, Coherent SFN STxMP) - Objective 2: Enhancements for UL 3Tx including 3Tx for UL codebook and non-codebook based transmission

For Topic 4, we have a neutral position and open for further discussion. Regarding Objective 1, we understand that STxMP have been discussed as recently as in Rel-18. Nonetheless, as listed in Objective 4, we believe that there still could be further potential enhancements that an STxMP UE can benefit from.  For example, an SFN-based PUCCH+PUCCH transmission mode, can be very effective in further enhancement in reliability and coverage of the uplink control channel. On the other hand, PUSCH+PUCCH modes can significantly enhanced the flexibility and spectrum efficiency of the overall system. That said, during the discussion in Rel-18, some companies expressed their strong concern about market support, cost and the need to STxMP UE, and at times not much progress was made. Therefore, we think it may be better for Rel-19, if we concentrate the work on topics that can receive a broader support. As for Objective 3, we are doubtful. Given the existing support of 2 and 4TX UEs, we don’t believe that there is an urgency to provide specifications support for 3TX UEs. Nevertheless, we would be open to support this topic if sufficient evidence indicating performance benefits and market needs are provided.

Observation 4: Neither Objectives 1 or 2 may have sufficient justification in terms of urgency and market support.

Proposal 4: Given the limited TUs, discard Topic 4 or at most support only one objective; preferably Objective 1.

Topic 5:

Enhancement for asymmetric downlink S-TRP/UL M-TRP scenario assuming intra-band intra-cell non-co-located M-TRP scenarios without changing existing cell definition or defining a new cell - Objective 1: Extension of Rel-18 2TA mDCI to sDCI assuming legacy PRACH resources - Objective 2: Separate UL power control for SRS only to downlink S-TRP from SRS to UL M-TRP and introduce path loss measurement to uplink M-TRP

We have a positive view for both objectives under Topic 5. Regarding Objective 1, the 2 TA operation was discussed for m-DCI uplink in Rel-18, where each TA was associated to a different TAG. Support of 2 TA operation is also beneficial in the case of an intra-cell UL m-TRP deployment with a large cell size. As such, a UE can be scheduled for an uplink transmission by an s-DCI, where the transmission can be directed to any of the TRPs. Given the possibility of different distances between the UE and each of the TRPs, it makes sense to support possibility of TA adjustment of uplink transmission according to the target TRP or a subset of TRPs in an UL m-TRP deployment. As for Objective 2, it is geared to enable deployments where UL-only TRP or more accurately named, received only points (ROP) are employed. Given the strong interest of some of operators and its limited workload, we support Objective 2 as well.

Observation 5: By considering relatively small enhancements, both Objectives 2 and 3 can meaningfully enhance reliability of UL m-TRP operation in an intra-cell deployment.

Proposal 5: Support both objectives under Topic 5 to enhance reliability of UL m-TRP operation in an intra-cell deployment.

Topic 6:

6Rx/8Rx UE enhancements with lower complexity utilizing two segments of 3/4 Rx antenna units up to 8-layer DL Tx based on legacy codebook and legacy codeword to layer mapping - Objective 1: SRS antenna port grouping, CSI and codeword association to the segments of receive antenna.

For Topic 6, our recommendation is to discard this topic, however we remain open to further discuss the objective. Our concern is multi-fold, as we briefly discuss them here. First, it is not clear to us whether there is a strong market demand for such UE. Second, if such demand is real and significant, then there may be ways to achieve cost reduction through implementation techniques and proprietary solutions, and a specifications support may not be needed. Third, since the exact type and application of such UE is not clear, it may not be possible to accurately define the scope of the objective and outline the required enhancements.

Observation 6: For the described objective, there is not a clear picture of market urgency, need to specifications support and the type of the targeted UE.

Proposal 6: Given the limited TUs, and not having a clear picture of its urgency and the scope of the enhancements discard Topic 6.

# Conclusions

In this contribution, we shared and discussed our views on potential topics for Rel-19 MIMO. Based on presented discussions, following observations and proposals are made,

Observation 1: By allowing a UE to initiate a beam switch and/or report procedure, a highly robust beam operation with a low latency and overhead can be achieved.

Observation 2: Objectives 1 and 2 which are focused on codebook enhancements to address an obvious limitation in the current specifications.

Observation 3: In realistic scenarios, assumptions of perfect synchronization in frequency and time do not hold true, and certain UE-based mechanisms are required to assist gNB to calibrate its transmission per UE or per group of UEs.

Observation 4: Neither Objectives 1 or 2 may have sufficient justification in terms of urgency and market support.

Observation 5: By considering relatively small enhancements, both Objectives 2 and 3 can meaningfully enhance reliability of UL m-TRP operation in an intra-cell deployment.

Observation 6: For the described objective, there is not a clear picture of market urgency, need to specifications support and the type of the targeted UE.

Proposal 1: Support Topic 1 to enhance robustness and latency of beam operation through event-driven beam management, where an event may be defined as a measurement condition at the UE or entering a new time-reference or geo-location.

Proposal 2: Support Topic 2 to enhance CSI framework to include > 32 (64/128) CSI-RS ports and restrict the scope to only Objectives 1 and 2.

Proposal 3: Support Topic 3 to enable a UE-based mechanism to assist with a time/frequency-synchronized transmission in a CJT deployment.

Proposal 4: Given the limited TUs, discard Topic 4 or at most support only one objective; preferably Objective 1.

Proposal 5: Support both objectives under Topic 5 to enhance reliability of UL m-TRP operation in an intra-cell deployment.

Proposal 6: Given the limited TUs, and not having a clear picture of its urgency and the scope of the enhancements discard Topic 6.

# References

- RP-232612, Moderator's summary for REL-19 RAN1 topic MIMO Evolution, RAN#101, September 2023

- RP-231540, RAN Chair’s Guidance for RAN Release 19, RAN#101, September 2023