3GPP TSG-RAN WG1 Meeting #115 	R1-2312745

Chicago, USA, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0171 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Rel-18 XR enhancements for NR in 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In TS 38.212, there are two “given by clause x.x of [5, TS 38.213]” in Clause 6.3.2.1.3A and Clause 6.3.2.1.5. As the corresponding clause has been updated in TS 38.213, TS 38.212 needs to be updated accordingly. - The procedures in clause 6.3.2.1.4 for CG-UCI can be reused for UTO-UCI. However, the current specification does not clarify that the procedure in this clause is applicable when UTO-UCI and HARQ-ACK have the same priority and are jointly encoded. - The maximum length of UTO-UCI bit sequence is 8, which is not larger than 11. Hence, polar code is not applicable to UTO-UCI when it is not jointly encoded with HARQ-ACK. However, when UTO-UCI is jointly encoded with HARQ-ACK, depending on the size of HARQ-ACK code book, the UTO-UCI and HARQ-ACK sequences together may result in a code book with a size larger than 11 bits. In this case Polar codes should be applied for encoding. Currently, joint encoding of UTO-UCI and HARQ-ACK with Polar code is missing from the specification. - The procedures in clause 6.3.2.7 for CG-UCI can be reused for UTO-UCI. However, the following highlighted case described in this clause is not applicable to UTO-UCI since UTO-UCI has the same priority as the CG-PUSCH that is multiplexed in: If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1 and/or CG-UCI associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH, - if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 1, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 as HARQ-ACK, and taking HARQ-ACK with priority index 0 as CSI part 2; - otherwise, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 if any as HARQ-ACK, taking CG-UCI associated with priority index 1 if any as CG-UCI, taking HARQ-ACK with priority index 0 as CSI part 1, and taking CSI part 1 as CSI part 2 if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0.. The inconsistency can be resolved by considering applicable cases for UTO-UCI when the corresponding CG-UCI procedures can be reused. |
| Summary of change: |  | - Update the clause references of TS 38.213 in clause 6.3.2.1.3A and clause 6.3.2.1.5. - Clarify that the procedures in clause 6.3.2.1.4 are applicable when UTO-UCI and HARQ-ACK have the same priority and are jointly encoded. - Update clause 6.3.2.4.1 to include joint encoding of UTO-UCI and HARQ-ACK with Polar code when applicable. - Add “when applicable” to the condition to resue the CG-UCI procedures for UTO-UCI in clause 6.3.2.7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for Rel-18 XR enhancements is incomplete or incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.3A, 6.3.2.1.4, 6.3.2.1.5, 6.3.2.4.1, 6.3.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.213, TS 38. 214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.3.2.1	UCI bit sequence generation

< Unchanged parts are omitted >

##### 6.3.2.1.3A	UTO-UCI

For UTO-UCI bits transmitted on a CG PUSCH when the higher layer parameter nrof_UTO_UCI is configured, the UTO-UCI bit sequence a0, a1, a2, a3, …,aA-1  is determined as follows:

-	set ai=oiUTO-UCI  for i=0,1, …, OUTO-UCI-1 and A=OUTO-UCI, where OUTO-UCI is provided by nrof_UTO_UCI, and the UTO-UCI bit sequence o0UTO-UCI, o1UTO-UCI, …, oOUTO-UCI-1UTO-UCI is given by clause 9.3.1 of [5, TS 38.213].

##### 6.3.2.1.4	HARQ-ACK and CG-UCI/UTO-UCI

If the higher layer parameter nrof_UTO_UCI is configured, the procedure in this clause 6.3.2.1.4 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, and replacing "When higher layer parameter cg-UCI-Multiplexing is configured" with "When UTO-UCI and HARQ-ACK have the same priority index and are jointly encoded and transmitted on a PUSCH".

When higher layer parameter cg-UCI-Multiplexing is configured, the UCI bit sequence a0, a1, a2, a3, …,aA-1  is determined as follows, where A=OCG-UCI+OACK.

-	The CG-UCI bits are mapped to the UCI bit sequencea0, a1, a2, a3, …,aOCG-UCI-1 , where ai=oiCG-UCI for i=0,1, …, OCG-UCI-1. The CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part, and OCG-UCI is number of CG-UCI bits;

-	The HARQ-ACK bits are mapped to the UCI bit sequence aOCG-UCI, aOCG-UCI+1,  …,aOCG-UCI+OACK-1, where ai+OCG-UCI=oiACK for i=0,1, …, OACK-1. The HARQ-ACK bit sequence o0ACK, o1ACK, …, oOACK-1ACK is given by Clause 9.1 of [5, TS38.213], and OACK is number of HARQ-ACK bits.

##### 6.3.2.1.5	UCI with different priority indexes

If the higher layer parameter nrof_UTO_UCI is configured, the procedure in this clause 6.3.2.1.5 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, and replacing "is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part" with "is given by clause 9.3.1 of [5, TS 38.213]".

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1, the following UCI bit sequences are generated, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12  if any, according to the following:

-	If CSI part 1 is also transmitted on the PUSCH,

-	Set ai(1) for i=0,1,…,A(1)-1 as the bit sequence of CSI part 1, where the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 starting with a0(1).

-	Set  ai(2)=oiACK-LP  for i=0,1,…,OACK-LP-1 and A(2)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213].

-	Otherwise, set  ai(1)=oiACK-LP  for i=0,1,…,OACK-LP-1 and A(1)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213].

< Unchanged parts are omitted >

#### 6.3.2.4	Rate matching

In case where there are more than one UL-SCH transport blocks for the PUSCH transmission, the UCI information is multiplexed only on the UL-SCH transport block with highest IMCS value for the initial PUSCH, where IMCS is as defined in Clause 6.1.4.1 in [6, TS 38.214]. In case the two transport blocks have the same IMCS value for the initial PUSCH, the UCI information is multiplexed with data only on the first transport block. The PUSCH for UCI multiplexing in this Clause refers to the UL-SCH transport block for UCI multiplexing.

##### 6.3.2.4.1	UCI encoded by Polar code

If the higher layer parameter nrof_UTO_UCI is configured, the procedures in this clause and the clauses it refers to apply by replacing CG-UCI with UTO-UCI in all the notations and texts, when applicable.

6.3.2.4.1.1	HARQ-ACK

For HARQ-ACK transmission on PUSCH not using repetition type B with UL-SCH and if numberOfSlotsTBoMS is not present in the resource allocation table, or if numberOfSlotsTBoMS is present in the resource allocation table and the value of numberOfSlotsTBoMS in the row indicated by the Time domain resource assignment field in DCI is equal to 1, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as , is determined as follows:

where

-	 is the number of HARQ-ACK bits;

-	if , ; otherwise  is the number of CRC bits for HARQ-ACK determined according to Clause 6.3.1.2.1;

-	;

-	 is the number of code blocks for UL-SCH of the PUSCH transmission;

-	if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; otherwise,  is the -th code block size for UL-SCH of the PUSCH transmission;

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, ;

-	 is configured by higher layer parameter scaling;

-	 is the symbol index of the first OFDM symbol that does not carry DMRS of the PUSCH, after the first DMRS symbol(s), in the PUSCH transmission.

< Unchanged parts are omitted >

#### 6.3.2.7	Multiplexing of coded UCI bits with different priority indexes to PUSCH

If the higher layer parameter nrof_UTO_UCI is configured, the procedure in this clause 6.3.2.7 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, when applicable.

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1,

-	If CSI part 1 is also transmitted on the PUSCH, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 0 as CSI part 2, and assuming the number of HARQ-ACK information in Clause 6.2.7 is 0 bit;

-	Otherwise, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 0 as CSI-part 1, and assuming the number of HARQ-ACK information in Clause 6.2.7 is 0 bit.

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 1, and CSI if any are transmitted on a PUSCH associated with priority index 0, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 as HARQ-ACK.

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1 and/or CG-UCI associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH,

-	if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 1, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 as HARQ-ACK, and taking HARQ-ACK with priority index 0 as CSI part 2;

-	otherwise, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 if any as HARQ-ACK, taking CG-UCI associated with priority index 1 if any as CG-UCI, taking HARQ-ACK with priority index 0 as CSI part 1, and taking CSI part 1 as CSI part 2 if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0.

If uci-MuxWithDiffPrio is configured, and CG-UCI associated with priority index 0 and HARQ-ACK bits associated with priority index 0 if any, HARQ-ACK bits associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 0, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 as HARQ-ACK, taking CG-UCI associated with priority index 0 and HARQ-ACK bits associated with priority index 0 if any as CSI part 1, and taking CSI part 1 as CSI part 2 if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0.