3GPP TSG RAN WG1 #115	        		   			R1-2312733

# Chicago, USA, November 13th – 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0582 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance of XR enhancements for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections on XR enhancements for NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the procedures in Clause 11.1 that result in an invalid CG PUSCH TO in Clause 9.3.1. - Capture resumption of PDCCH monitoring when a UE reports NACK during a PDCCH skipping period in Clause 10.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support of XR enhancements for NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1, 10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.321 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged text is omitted ***

### 9.3.1	UE procedure for reporting UTO-UCI

If the UE is provided nrof_UTO_UCI with value equal to OUTO-UCI in configuredGrantConfig of a CG-PUSCH configuration, the UE multiplexes UTO-UCI represented by a bitmap of OUTO-UCI bits in each CG-PUSCH transmission for the CG-PUSCH configuration.

The OUTO-UCI bits of UTO-UCI, o0UTO-UCI, o1UTO-UCI, …, oOUTO-UCI-1UTO-UCI, have a one-to-one mapping to OUTO-UCI subsequent CG-PUSCH TOs in ascending order of start time. For unpaired spectrum operation, the OUTO-UCI subsequent CG-PUSCH TOs exclude invalid ones where a UE does not transmit a PUSCH due to collision of the PUSCH with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or with symbol(s) of an SS/PBCH block with index provided by ssb-PositionsInBurst, based on the procedures in Clause 11.1. A bit value of ‘0’ indicates that the UE may transmit CG-PUSCH, and a bit value of ‘1’ indicates that the UE will not transmit CG-PUSCH, in a corresponding CG-PUSCH TO. When the UE indicates by UTO-UCI a value of ‘1’ for a CG-PUSCH TO, the UE continues to indicate the value of ‘1’ for the CG-PUSCH TO by UTO-UCI multiplexed in subsequent CG-PUSCH transmissions, and the UE does not transmit CG-PUSCH in the CG-PUSCH TO.

## *** Unchanged text is omitted ***

## 10.4	Search space set group switching and skipping of PDCCH monitoring

## *** Unchanged text is omitted ***

When the PDCCH monitoring adaptation field indicates to a UE to start PDCCH monitoring according to search space sets with a first group index and stop PDCCH monitoring according to search space sets with a second group index, the UE applies the indication

-	at the beginning of a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈0, 1, 2, 3,

-	at the beginning of a first slot, of a slot group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈5, 6

When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field.

-	If the UE transmits a PUCCH providing a positive SR before the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE shall monitor PDCCH regardless of PDCCH skipping indication on all serving cells of the corresponding Cell Group when the SR is pending [11, TS 38.321].

-	If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group.

-	When the UE is provided pdcchMornitoringResumptionAfterNack, after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, if the UE transmits a PUCCH or a PUSCH providing a NACK value associated with a PDSCH reception that is scheduled by a DCI format in a PDCCH reception on the serving cell, the UE terminates PDCCH skipping, starting from the beginning of a first slot that is after a last symbol of the PUCCH or PUSCH transmission on the serving cell.

-	During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell.

-	After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a SpCell, when contention resolution is successful [11, TS 38.321], the UE resumes PDCCH monitoring on the SpCell.

-	After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a serving cell, when a pending SR is cancelled [11, TS 38.321], the UE resumes PDCCH monitoring in all serving cells of the corresponding Cell Group.

-	If UE transmits a RACH due to positive SR, the UE shall not skip PDCCH monitoring on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell.

## *** Unchanged text is omitted ***