3GPP TSG RAN Meeting #105	RP-241750

Melbourne, Australia, 09 – 12 September, 2024

Status Report to TSG

Agenda item:			9.3.2.4

| WI / SI Name | NR sidelink multi-hop relay |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No | Testing part: No |
| Acronym | NR_SL_relay_multihop |  |  |  |
| Unique ID | 1040114 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241609 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 09/2025 | Performance part: mm/yyyy | Testing part: mm/yyyy |
| Overall Completion level | Study Item: xx % | Core part: 15% | Performance Part: x% | Testing part: xx% |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Youngdae Lee |
|  | Company | LG Electronics Inc. |
|  | Email | youngdae.lee@lge.com |
| Rapporteur | Name | Martino Freda |
|  | Company | InterDigital Communications |
|  | Email | martino.freda@interdigital.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#127 (August 2024)

48 contributions ([1] ~ [48]) were submitted in this meeting. The following agreements were made:

Organizational

- n-hop relaying involves n relay UEs by definition.

- Distinguish the “last” relay UE as the U2N relay UE directly connected to the gNB, for RAN2 discussion

- Distinguish the “intermediate” relay UEs as the other relay UEs, for RAN2 discussion

- Distinguish the “first” relay UE as the intermediate relay UE directly connected to the remote UE, for RAN2 discussion

- FFS if we will capture the requirements according to these terms when implementing CRs.

- FFS link terminology.

- RAN2 intend to align with SA2 terminology when it is stable

- A candidate last relay UE for discovery or (re)selection can be in any RRC state, but not outside network coverage.  FFS if acting as a relay requires it to transition to RRC_CONNECTED.

- Some or all of the intermediate relay UEs on a multi-hop indirect path can be inside or outside the network coverage.

- RAN2 intend to minimize the impact of hop count on the multi-hop relay mechanisms.

- RAN2 intend to design mechanisms from the start of the WI that are flexible enough to be adapted to at least two additional hops in Rel-19.

Relay discovery and (re)selection for multi-hop U2N relay

- From RAN2 perspective, models A and B can both be supported.

- Reuse existing AS discovery protocol to transmit discovery message for multi-hop U2N relaying.

- Reuse SL-SRB4 to transmit discovery message for multi-hop U2N relaying.

- Reuse existing resource pools defined for discovery message transmission and reception

- Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE.

- Configuration can be provided by SIB/dedicated message or pre-configured as in Rel-17/18.

- If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18.

- If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds).

- FFS discovery conditions for the intermediate relay UEs.

Control plane procedures for multi-hop U2N relay

- The following connections are assumed as a baseline to be needed:

  - From last Relay UE perspective:

    - A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and

    - A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED.

  - From intermediate relay UE perspective (including first relay UE):

    - A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and

    - An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED.

    - FFS what RRC states are supported for the intermediate relay UE.

  - From U2N Remote UE perspective:

    - A direct (non-relayed) PC5 connection with Intermediate Relay UE, and

    - An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED.

- The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

- The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

Service continuity for multi-hop U2N relay

- No progress

2.2.2	Remaining Open issues

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay.

  - Relay discovery and (re)selection

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed

  - Impact on SRAP and QoS handling for multi-hop

  - Control plane procedures

- Specify intra-gNB service continuity for multi-hop U2N relay

  - Intra-gNB multi-hop indirect to direct path switching using existing framework

  - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

  - Intra-gNB direct to multi-hop indirect path switching

  - Intra-gNB single-hop indirect to multi-hop indirect path switching

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#127

- R2-2406365	Discovery and relay (re)selection for multi-hop U2N relay	OPPO

- R2-2406366	Control plane procedures of multi-hop U2N relay	OPPO

- R2-2406494	Discussion on control plane procedures for multi-hop relays	MediaTek Inc.

- R2-2406506	Considerations on Control Plane of Multi-hop Relay	NEC

- R2-2406528	Discussions on relay discovery for multi-hop U2N Relay	ASUSTeK

- R2-2406529	Discussions on the L2 Intermediate U2N Relay in multi-hop L2 U2N Relay	ASUSTeK

- R2-2406553	Discussion on multi-hop U2N relay discovery and relay selection	NEC

- R2-2406562	Discussion on multi-hop discovery and (re)selection	CATT

- R2-2406563	E2E Connection Setup and QoS Split for Multi-hop Relay	CATT

- R2-2406611	Initial considerations on relay discovery and (re)selection	Samsung

- R2-2406612	Initial considerations on CP and UP aspects for R19 multi-hop relay	Samsung

- R2-2406632	Multi-hop relay selection/re-selection	Sony

- R2-2406633	Control plane procedure for multi-hop U2N relay	Sony

- R2-2406683	Relay discovery and selection for Multi-hop UE-to-NW Relay	Apple

- R2-2406684	Control Plane Design for Multi-hop UE-to-NW Relay	Apple

- R2-2406695	Discussion on discovery and (re)selection for support of multi-hop SL Relay	ZTE Corporation, Sanechips

- R2-2406696	Discussion on architecture and control plane procedures for support of multi-hop SL relay	ZTE Corporation, Sanechips

- R2-2406713	Scenarios, QoS Handling, and Control Plane Procedures for Multi-hop	InterDigital France R&D, SAS

- R2-2406714	Discovery and relay (re)selection for multi-hop relay 	InterDigital France R&D, SAS

- R2-2406735	Discussion on multi-hop relay discovery and (re)selection	vivo

- R2-2406736	Discussion on Multi-hop Control Plane Procedures	vivo

- R2-2406755	Discussion on QoS handling for NR sidelink multi-hop relay	Spreadtrum Communications

- R2-2406887	Relay discovery and (re)selection in Multi-hop relay	Lenovo

- R2-2406888	Control plane in Multi-hop relay	Lenovo

- R2-2406898	Discussion on multi-hop relay discovery and reselection	China Telecom

- R2-2407007	Discussion on multi-hop U2N Relay discovery and (re)selection	vivo

- R2-2407008	Discussion on multi-hop U2N Relay Control Plane Procedures	vivo

- R2-2407034	discussion on control plane procedure	Ericsson, FirstNet, AT&T

- R2-2407035	discussion on discovery and relay (re)selection	Ericsson, FirstNet, AT&T

- R2-2407057	Discussion on Relay discovery and selection	LG Electronics France

- R2-2407058	Discussion on Control Plane Procedure	LG Electronics France

- R2-2407101	Discovery and Relay (re)selection for multi-hop U2N relay	Qualcomm Incorporated

- R2-2407102	Control procedure for multi-hop L2 based U2N relay	Qualcomm Incorporated

- R2-2407111	Discussion on control plane procedures for NR sidelink multi-hop relay	China Telecom

- R2-2407145	Work plan for NR sidelink multi-hop relay	LG Electronics, InterDigital

- R2-2407147	Terminologies and Scenarios for SL multihop relay	LG Electronics Inc.

- R2-2407204	Discussion on relay discovery and (re)selection for NR sidelink multi-hop relay	TOYOTA InfoTechnology Center

- R2-2407205	Discovery and (re)selection under multihop relay 	Kyocera

- R2-2407206	Control Plane under multihop L2 U2N relaying 	Kyocera

- R2-2407224	Discussion on Working Assumptions for Multi-hop Relay Mechanisms	FirstNet, Ericsson, AT&T, LG Electronics, InterDigital, Qualcomm

- R2-2407294	Relay discovery and (re)selection for multi-hop Relay	Huawei, HiSilicon

- R2-2407295	Control plane procedures for multi-hop relay	Huawei, HiSilicon

- R2-2407316	Relay discovery and reselection for multi-hop relay	Nokia

- R2-2407318	Control plane procedure for multi-hop relay	Nokia

- R2-2407378	Discussion on Working Assumptions for Multi-hop Relay Mechanisms	InterDigital France R&D, SAS, LG Electronics, FirstNet, Ericsson, AT&T, Qualcomm, Samsung

- R2-2407390	Discussion on Working Assumptions for Multi-hop Relay Mechanisms	InterDigital France R&D, SAS, LG Electronics, FirstNet, Ericsson, AT&T, Qualcomm, Samsung

- R2-2407402	discussion on Relay discovery and (re)selection for multi-hop relay	Sharp

- R2-2407403	discussion on C-plane procedure for multi-hop relay	Sharp

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template