3GPP TSG RAN Meeting #105	RP-241761

Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.2.3

Source:	Futurewei

Title:	R19 XR WI scope adjustment

Document for:	Discussion and Decision

# Introduction

A couple of check points for RAN#105 remain in the latest R19 XR WID [1]. In addition, during the RAN#104 the following note in the R19 XR WID was discussed:

NOTE: Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.

As a result of the discussion, a way forward [2] was endorsed with the aim in RAN#105 to replace the current note with the objective to address uplink rate control from RAN which would be complementary to upper layer solutions (e.g. L4S).

In this contribution, we discuss various adjustments to the R19 XR WI scope that are needed in accordance with the progress made in the WGs.

# Discussion

## On objective related to support of multi-modality services

In RAN2#127, RAN2 made it a working assumption that regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR by having the UE report existence of multi-modality application and association information among QFIs to gNB [3]. RAN2 considers that based on multi-modal information, the gNB may perform joint admission control and the gNB may consider this information during QoS flow to DRB mapping.

RAN2 also identified the following dependency on other WGs’ work [3]:

- “For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN.”

- “RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.”

Proposal 1. Add the following sub-bullet and two notes under the first objective (multi-modality) in the WID:

- Extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB. Based on multi-modal information, the gNB may perform joint admission control and the gNB may consider this information during QoS flow to DRB mapping.

Note 1: For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN.

Note 2: PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

## Progress made in RAN2 resulting in less workload for some of the existing objectives

In RAN2#127, it was agreed that “RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR” [3], potentially reducing the workload for the objective on scheduling enhancements.

RAN2 also agreed on the following w.r.t. avoiding unnecessary RLC retransmission for the objective on UP enhancements [3]:

In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where

    - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach

    - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

Each of the original Tx-based and Rx-based approaches would require new over-the-air (OTA) signaling or enhancements to the existing OTA signaling. However, the combined Rx and Tx approach doesn’t use the OTA signaling, hence significantly reducing the workload for this aspect.

Observation: Progress made in RAN2 has resulted in less workload for some of the existing objectives.

## On the Note about UL application rate control based on RAN awareness

Given the way forward that had previously been endorsed in RAN#104 and the potential reduction of the workload on some of the existing objectives, as described above, we suggest that it is the time to add the new objective on RAN awareness-based UL application rate control.

As to which layer the gNB provides the congestion information in the downlink direction, RRC, MAC (via MAC CE), and PHY (via DCI) are the candidate layers. Among the three, the MAC layer based approach would have the least impact to the standards because there is already a “Recommended bit rate” MAC CE defined today. A simple extension to it may be sufficient, e.g., by extending the range of the bit rate to cover the high bit rates, which are typical for XR video traffic. Therefore, we propose that the gNB provides the congestion information in the DL direction using MAC layer signaling, e.g., an enhanced “Recommended bit rate” MAC CE.

Proposal 2. Add the following objective to the WID:

- Specify the mechanism and MAC signaling required to support UL application rate control based on RAN congestion awareness [RAN2]

In the Annex, we provide a TP for revising the R19 XR WID.

Proposal 3. RAN Plenary consider the TP in the Annex in revising Rel-19 XR WID.

# Conclusion

We propose the following:

Proposal 1. Add the following sub-bullet and two notes under the first objective (multi-modality) in the WID:

- Extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB. Based on multi-modal information, the gNB may perform joint admission control and the gNB may consider this information during QoS flow to DRB mapping.

Note 1: For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN.

Note 2: PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

Proposal 2. Add the following objective to the WID:

- Specify the mechanism and MAC signaling required to support UL application rate control based on RAN congestion awareness [RAN2]

Proposal 3. RAN Plenary consider the TP in the Annex in revising Rel-19 XR WID.

# References

RP-240791, WID XR Revised, Nokia (Rapporteur).

RP-241651, Outcome from the XR discussion for NR Phase 3 WI scope, Nokia (Moderator).

R2-2407575, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei (Session Chair).

# Annex A. TP for revising Rel-19 XR WID [1]

<unchanged text omitted>

# Objective

## Objective of SI or Core part WI or Testing part WI

The Rel-19 XR Phase 3 objectives are as follows:

-	Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2].

- Extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB. Based on multi-modal information, the gNB may perform joint admission control and the gNB may consider this information during QoS flow to DRB mapping.

Note 1: For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN.

Note 2: PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

-	Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

-	Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

-	Specify Enhancements for Scheduling, as follows:

-	For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2].

NOTE:	LCP implementation complexity need to be taken into account when evaluating solutions.

-	Specify the following user plane enhancements [RAN2]

-	RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

-	Specify Core requirements related to the above objectives as necessary [RAN4]

- Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3]

  - PDU set based handling

  - ECN marking

  - Burst Arrival Time reporting, if needed

  - PSI Discard coordination, if needed

  - Note: No RAN2 impact from above items

- Specify the mechanism and MAC signaling required to support UL application rate control based on RAN congestion awareness [RAN2]

<unchanged text omitted>