3GPP TSG RAN #105	RP-241762

Melbourne, Australia, September 9 - 12, 2024

Agenda Item:	9.3.1.1

Source:	Futurewei

Title:	R19 MIMO WI scope adjustment

Document for:	Discussion and decision

# Introduction

During the last RAN plenary meeting, several issues were discussed, and the following proposal (in [1]) is endorsed:

Postpone discussions on the following topics until RAN#105. These topics will not be discussed in RAN1 before the decision in RAN#105.

- Support of two TAs as part of enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

- SRS port grouping as part of CSI enhancement

- Partial-coherent UL codebook for 3Tx multi-panel

- Open loop-based UL precoder cycling

Another issue related to a RAN4 work item (on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4) on 3T6R and 4T6R SRS antenna switching was intensely discussed during the meeting. The issue is that 3T6R and 4T6R SRS antenna switching is included in this R19 RAN4 WI without RAN1 involvement even though this feature is not yet supported in RAN1. RAN1 had concluded not supporting this feature before the RAN4 WI is agreed in March plenary. After multiple rounds of discussions, the following proposal (in [2]) is endorsed:

RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

Note 1: SRS antenna switching to support 4T6R is precluded.

Note 2: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

In this contribution, we discuss the status of R19 MIMO WI in RAN1 and the potential scope adjustments accordingly.

# Discussion

The overall progress of R19 MIMO WI is very good, all objectives are on track, and some are even ahead of the schedule to finish. As shown in the RAN1 status reports from RAN1 chair [3] for the past few meetings, R19 MIMO WI has used relatively less time compared to other items with respect to assigned TU. Therefore, within the currently assigned TU, some limited up-scoping for R19 MIMO WI is feasible.

On the other hand, the added topic(s) should not take priority over the clearly defined current objectives [4]. For example, for the objective of “Enhancements for UE-initiated/event-driven beam management”, although it is generally on track, a lot of issues are yet to be discussed and make needed progress. Specifically, the needed procedure related discussion as explicitly included in the WI objective is yet to be started. Some companies even proposed in RAN1 to deprioritize this topic, see Section 4 Issue 4 in [5]. Therefore, before adding new topic(s) to the WI, sufficient discussion time in RAN1 should be ensured to cover the topics already in the current objectives.

- Proposal 1: Limited up-scoping for R19 MIMO WI can be considered under the condition that sufficient discussion time in RAN1 is ensured to cover the topics already in the current objectives.

Based on the discussions during last plenary, among the topics proposed for up-scoping or explicitly including in the R19 MIMO WI, we support to consider the following:

- Support of two TAs as part of enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

- SRS port grouping as part of CSI enhancement

- SRS antenna switching to support 3T6R depending on UE capability

# Conclusions

In this contribution, we present our views on R19 MIMO scoping adjustments.

- Proposal 1: Limited up-scoping for R19 MIMO WI can be considered under the condition that sufficient discussion time in RAN1 is ensured to cover the topics already in the current objectives.

# References

RP-241633, Moderator's summary for offline discussion on REL-19 MIMO, RAN1 Chair.

RP-241621, Moderator's summary for offline discussion on SRS antenna switching 3T6R and 4T6R, CMCC.

RP-240863, RAN1 status report to RAN#104, RAN1 Chair.

RP-234007, New WID: NR MIMO Phase 5, Samsung.

R1-2406046, Moderator Summary #3 on UE-initiated/event-driven beam management, Moderator.