3GPP TSG RAN Meeting #105										RP-241775

Melbourne, Australia, September 9-12, 2024

## Status Report to TSG

Agenda item:			9.3.1.1

| WI / SI Name | NR MIMO Phase 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_Ph5 |  |  |  |  |
| Unique ID | 1020089 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-240087 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 55% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |
| Rapporteur | Name | Darcy Tsai |
|  | Company | MediaTek |
|  | Email | darcy.tsai@mediatek.com |
| Rapporteur | Name | Chai Li |
|  | Company | China Mobile |
|  | Email | chaili@chinamobile.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#118, the following agreements were made.

UE-initiated/event-driven beam management

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least type 1 CG-PUSCH.

- FFS: PUCCH as the second channel

- FFS: Whether the PUSCH can be with UL data

Agreement

Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported

  - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration

  - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set.

  - FFS: The RS in the RS resource set can be updated by MAC-CE

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported.

CRI or SSBRI #1

CRI or SSBRI #2

…

CRI or SSBRI #N

L1-RSRP #1

Differential L1-RSRP #2

…

Differential L1-RSRP #N

Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC

Note: Other contents are not precluded

.

    - Differential L1-RSRP #2~#N/current beam is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #2~#N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1

      - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP.

      - FFS: range and step size of differential L1-RSRP

    - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2.

    - FFS: Additional report content(s) (e.g., reporting configuration ID, indication for synchronization state, event ID, or cell ID).

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following

- Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

- Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration.

  - Note: The RRC parameter is NOT associated with SchedulingRequestId.

  - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following

- Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

- Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration.

  - Note: The RRC parameter is NOT associated with SchedulingRequestId.

  - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above is at least applied to a single CC case.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, select one of the following options

- Option-1: Introduce a new 1-bit field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - FFS: DCI format 0_3

- Option-2: Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - FFS: DCI format 0_3

- FFS: Whether/how to handle the case that multiple CSI report configuration(s) for the UE-initiated/event-driven beam report are associated with the same first PUCCH resource and/or the same scheduled PUSCH

Conclusion

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, there is no RAN1 consensus on additionally supporting that the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ at least comprises {1, 2, 3, 4}

- FFS: additional candidate value(s) of {5, 6, 7, 8}

- FFS: If ‘N’ is not RRC configured, only one L1-RSRP and CRI/SSBRI are reported by default.

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further down-select one of the following for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

- Option-4: No further enhancement

Note: If there is no consensus in RAN1 on one of Options 1/2/3, Option 4 will be taken.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least one or both of the following should be supported:

- Option-1: The same Type-1 CG PUSCH can carry UL-SCH and the beam report.

- Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report

  - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, the following is provided for down-selection.

- Option-1 (one-to-one): Only one first PUCCH resource and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting.

  - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel.

  - Option-1B: No restriction in terms of periodicity.

- Option-2 (one-to-M): Only one first PUCCH resource and one or more pre-configured resource(s) for second UL channel can be associated with CSI report configuration for UE-initiated/event-driven beam reporting.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured.

- Above applies to both cross-CC and same-CC beam report.

- Note: Above is applied to the case that second UL channel is PUSCH.

- FFS: Whether the first and second channels can be from the same/different CC.

Working Assumption

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality.

  - M is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A and/or Mode-B, further study the following for first PUCCH transmission

- UCI multiplexing/dropping/prioritization rule

- Conditions for the transmission of the first PUCCH.

- Whether the PUCCH resource in the first PUCCH channel can be associated with multiple CSI report configurations for UE-initiated/event-driven beam reporting from one or multiple CC(s).

- Whether/how to re-transmit the first PUCCH channel.

Whether/how to apply prohibit-timer or maximum number of (re)transmission(s) for first PUCCH channel.

CSI

Conclusion

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the number of configured associated SRS resource(s) (=Q) for antenna switching xTyR and the number of SRS ports (=PSRS) corresponding to the ‘reference UE antenna port’, there is no consensus to support the following:

- Q>1

- PSRS >1

Note: This implies no support for configuring >1 SRS resource sets for associated SRS when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset).

Note: There is a source R1-2407184 showing performance benefit of using multiple UE antenna ports non-coherent with each other

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset),

- The supported value(s) of x follows the supported configuration(s) of SRS resource for antenna switching xTyR used for reciprocity-based DL CSI acquisition

  - Note: This doesn’t have any spec impact (no new RRC parameter is needed)

- For the selection of PSRS=1 SRS port corresponding to the ‘reference UE antenna port’ (out of the available port(s)), decide, by RAN1#118, from the following schemes:

  - Scheme1: NW-configured via higher-layer (RRC) signalling

  - Scheme1B: NW dynamic selection via MAC CE or A-CSI triggering (DCI)

  - Scheme2: UE-selected, the selection included in the phase offset report

  - Scheme3: Fixed rule, e.g. the first out of the available port(s)

FFS:

- Whether/how to identify the transmission occasion of the Q=1 associated SRS resource to determine the reference UE antenna port in relation to the CSI request and/or SRS triggering

- The supported time-domain behaviour(s) for the associated SRS resource (periodic, semi-persistent, aperiodic)

  - In case the NW configures the Q=1 associated SRS resource from an existing SP and/or AP SRS antenna switching resource configuration for DL CSI acquisition (which utilizes dynamic signalling), whether/how to identify the Q=1 associated SRS resource

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific delay offset pre-compensation on PDSCH by the NW, decide, by RAN1#118, whether to support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset (when ReportQuantity is ‘cjtc-Dd’). And if supported, whether any of the following is additionally supported or not:

- NW indicates the delay offset value to be compensated for the Rel-18 eType-II CJT CSI report, and/or

- The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC delay offset report) are always jointly triggered and carried on a same PUSCH (hence on a same slot)

- The delay offset value to be compensated is the latest reported DO before the DCI triggering the CJT CSI reporting

FFS: AP-CSI-RS can be configured for the Rel-18 eType-II CJT report

The above only applies when the CMRs do not share common QCL source for average delay indication

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific frequency offset pre-compensation on PDSCH by the NW, decide, by RAN1#118, whether to support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported frequency offset (when ReportQuantity is ‘cjtc-F’). And if supported, whether any of the following is additionally supported or not:

- NW indicates the frequency offset value to be compensated for the Rel-18 eType-II CJT CSI report, and/or

- The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC frequency offset report) are always jointly triggered and carried on a same PUSCH (hence on a same slot)

- The frequency offset value to be compensated is the latest reported fO before the DCI triggering the CJT CSI reporting

FFS: AP-CSI-RS can be configured for the Rel-18 eType-II CJT report

The above only applies when the CMRs do not share common QCL source for Doppler shift indication

Conclusion

For the Rel-19 aperiodic standalone CJT calibration reporting, there is no consensus to support the following reporting formats:

- Joint FO + wideband PO reporting

- Joint Dd + FO + wideband PO reporting

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the resolution for frequency offset reporting FOn, additionally support MFO = 256

Conclusion

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset and frequency offset reporting FOn, there is no consensus on supporting additional values of dynamic range

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, the UCI mapping order is as follows:

- When ReportQuantity is ‘cjtc-Dd’ (Doffset+d):

  - nref,

  - {Dn,offset, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID,

  - {dn, n=0, 1, …, N TRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID

- When ReportQuantity is ‘cjtc-F’ (FO):

  - nref,

  - {FOn, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID,

- When ReportQuantity is ‘cjtc-Dd-F’ (joint Doffset+d and FO):

  - nref1,

  - nref2,

  - {Dn,offset, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID,

  - {dn, n=0, 1, …, N TRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID

  - {FOn, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID,

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, regarding Scheme-B, reuse the legacy Rel-15 Type-I layer pairing scheme, and fixed mapping between SD basis vectors and layers:

- The k-th SD basis vector is associated with the k-th layer-group.

- Note: This should be the baseline based on joint SD basis selection/indication using the agreed combinatorial indication (logCLGN1N2 bits).

LG denotes the number of layer groups per previous agreement

- Note: For RI=5-6, LG=3 was agreed, and the additional support for LG=4 is still FFS

Agreement

For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports,

- For Capability 1 timeline:

  - OCPU is the legacy Rel-18 Type-II Doppler OCPU multiplied by ceil(P/32) and replace K with KDOPP for AP-CSI-RS, while still not exceeding 8

  - Active resource counting follows the legacy Rel-18 Type-II Doppler

- For Capability 2 timeline:

  - OCPU is the legacy Rel-18 Type-II Doppler OCPU and replace K with KDOPP for AP-CSI-RS

  - Active resource counting follows the legacy Rel-18 Type-II Doppler

Agreement

For the Rel-19 Type-I MP codebook refinements for 48, 64, and 128 CSI-RS ports, regarding OCPU, timeline, active resource counting, and CMR restriction (FDM/TDM, EPRE offset), fully reuse those for the Rel-19 Type-I SP codebook

Agreement

For the Rel-19 Type-I MP codebook refinement for 48, 64, and 128 CSI-RS ports, the UCI parameters are captured in the tables below:

- Note: The second column includes the location of the parameters when reported with two-part UCI

| Parameter | UCI | Details/description |
| --- | --- | --- |
| RI | Part 1 | Same as Rel-15 Type-I SP: RI=v={1,2,3,4} |
| Wideband CQI for the first TB | Part 1 | Same as Rel-15 Type-I SP |
| Subband differential CQI for the first TB (*) | Part 1 | Same as Rel-15 Type-I SP |
| First SD basis vector selection indicator | Part 2 Wideband | One indicator per each of the Ng resources: - Each of the Ng indicators is the same as Rel-15 Type-I SP with the scheme following < 16-port design of Rel-15 Type-I SP codebookMode=1 |
| Second SD basis vector selection indicator | Part 2 Wideband | One indicator per each of the Ng resources: - Each of the Ng indicators is the same as Rel-15 Type-I SP with the scheme following < 16-port design of R15 Type-I SP codebookMode=1 |
| Inter-pol co-phase selection indicator | Part 2 Wideband or Subband (**) | One (set of) indicator per each of the Ng resources: - Same as Rel-15 Type-I SP with the scheme following < 16-port design of R15 Type-I codebookMode=1 |
| Inter-resource co-phasing | Part 2 Wideband | QPSK (2-bit) co-phasing on resource k=2,…,K relative to the first resource, layer-common |

(*): Not included when CQI reporting granularity is set to ‘wideband’

(**): Wideband when PMI reporting is set to ‘wideband’, Subband when PMI reporting granularity is set to ‘subband’

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, the UCI parameters are captured in the tables below for Scheme-A for RI=5-8:

- Note: The second column includes the location of the parameters when reported with two-part UCI

Scheme-A

| Parameter | UCI | Details/description |
| --- | --- | --- |
| First SD basis vector selection indicator | Part 2 Wideband | v=5-8: Decide (by RAN1#118) Alt1 vs Alt2 Alt1. Same as legacy Rel-15 Type-I SP, separate (i1, i2) indicators of log2N1O1 bits and log2N2O2 bits, respectively Alt2: Separate (i1, i2) indicators of log2N1 bits and log2N2 bits, respectively; and separate (q1, q2) indicators of log2O1 bits and log2O2 bits, respectively. |
| Second SD basis vector selection indicator | Part 2 Wideband | v=5-8: Freely select the other nSDBV=2 (v=5-6) or 3 (v=7-8) SD basis vectors such that they are orthogonal in at least one dimension (horizontal or vertical). Decide (by RAN1#118) the indication scheme. |
| Inter-pol co-phase selection indicator | Part 2 Wideband or Subband (**) | v=5-8: Same as legacy Rel-15 Type-I SP |

(*): Not included when CQI reporting granularity is set to ‘wideband’

(**): Wideband when PMI reporting is set to ‘wideband’, Subband when PMI reporting granularity is set to ‘subband’

Agreement

For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding NZP CSI-RS resource aggregation to attain 32 < P (or PCSI-RS) ≤ 128, for AP-CSI-RS where the K NZP CSI-RS resources are located in two consecutive slots,

- Except for codebook refinement based on Rel-18 Type-II Doppler, introduce per-resource higher-layer (RRC) configuration to indicate (via 1-bit per resource) whether 1-slot offset relative to the legacy resource-set-level slot offset configuration should be assumed or not

- For codebook refinement based on Rel-18 Type-II Doppler, introduce per-resource higher-layer (RRC) configuration to indicate (via 1-bit per resource) whether 1-slot offset relative to the resource group slot offset should be assumed or not

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the port subset indication (from the Rel-18 SD NES Type-1), extend the bitmap (i.e., port-subsetIndicator) by adding p48, p64, and p128 along with their appropriate lengths

- Note: Per previous agreements, K is the number of aggregated NZP CSI-RS resources to attain 32 < P (or PCSI-RS) ≤ 128

Agreement

For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the W2 structure, clarify that:

- Inter-polarization co-phasing is based on legacy Rel-15 Type-I SP codebookMode=1 for PCSI-RS<16

- QPSK inter-resource co-phasing alphabet is {+1,+j,-1,-j}

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI parameters when two-part UCI/CSI is used:

- Part 1: M CRI(s), M RI(s), M sets of CQI values for 1st CW

  - When MR is configured, the CRI(s) associated with the MR resource(s) are not reported

- Part 2: M sets of {PMI, LI (if applicable), CQI values for 2nd CW (if applicable)}

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, if MR is configured, the supported value(s) of MR are as follows:

- Rel-15 Type-I SP: {1, 2}

- Rel-16 eType-II: {1}

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding CBSR and RI restriction, support resource-specific specific CBSR

- FFS (by RAN1#118): Whether RI restriction is resource-common or resource-specific

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding first SD basis vector selection indicator for Scheme-A for RI=5-8, reuse the legacy Rel-15 Type-I SP scheme, i.e. separate (i1, i2) indicators of log2N1O1 bits and log2N2O2 bits, respectively

Agreement

At least for type 1 single panel, for the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI omission for the UCI reported in CSI part-2, other than priority 0 (G0), the UCI packing order and the 2M consecutive priority levels are defined w.r.t the M CRIs (including the non-reported MR CRIs) as follows:

- Consecutive priority levels (higher to lower):

  - Even sub-bands (G1) associated with the first configured CMR among the non-reported MR CRIs,

  - Odd sub-bands (G2) associated with the first configured CMR among the non-reported MR CRIs,

  - …,

  - Even sub-bands (G1) associated with the last configured CMR among the non-reported MR CRIs,

  - Odd sub-bands (G2) associated with the last configured CMR among the non-reported MR CRIs,

  - Even sub-bands (G1) of the 1st reported CRI,

  - Odd sub-bands (G2) of the 1st reported CRI,

  - …,

  - Even sub-bands (G1) of the (M-MR)th reported CRI,

  - Odd sub-bands (G2) of the (M-MR)th reported CRI.

Note: For UCI fields in wideband (G0), even sub-bands (G1), and odd sub-bands (G2), legacy design is fully reused.

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, the UCI parameters are captured in the tables below for Scheme-B for RI=5-8:

- Note: The second column includes the location of the parameters when reported with two-part UCI

Scheme-B

| Parameter | UCI | Details/description |
| --- | --- | --- |
| SD basis oversampling (rotation) factor q1, q2 | Part 2 Wideband | v=5-8: Values of q1, q2 follow Rel-16 eType-II, log2O1O2 bit indicator |
| SD basis vector selection indicator for each layer | Part 2 Wideband | v=5-8: log2N1N2LG bit indicator, where LG=3 for v=5-6, and 4 for v=7-8 |
| Inter-pol co-phase selection indicator for each layer | Part 2 Wideband or Subband (**) | v=5-8: QPSK: 2-bit indicator per layer group l=1, …, LG |

(*): Not included when CQI reporting granularity is set to ‘wideband’

(**): Wideband when PMI reporting is set to ‘wideband’, Subband when PMI reporting granularity is set to ‘subband’

Conclusion

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, conditioned in UE capabilities, a UE can be configured with either the group-based hard CBSR, or the 3-bit SD basis group-based scaling factor, or both, or none of the two

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the selection of PSRS=1 SRS port corresponding to the ‘reference UE antenna port’ (out of available port(s)) is NW-configured via higher-layer (RRC) signalling

Working Assumption

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the resolution for delay offset reporting Dn,offset, additionally support MD = {128, 256}

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1):

- Denoting the number of reported sub-band phase-offset values as NSB-P, and the sub-bands are indexed as {0, 1, …, NSB-P –1}, support, as a separate UE capability from wideband (=1) phase offset reporting, reporting, in one CSI reporting instance, {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref}

  - The alphabet for n, follows the previously agreed alphabet for =1, including the ‘invalid’ state

- FFS:

  - Supported sub-band size(s), e.g. following the legacy CSI sub-band definition, vs {1, 2, 4, 8, 16 PRBs}

  - Whether the UE performs measurement over the entire configured CSI reporting band WCSI

  - If needed, mechanism to limit CSI reporting overhead (e.g. maximum NSB-P)

  - If needed, configuration of whether the CSI-RSs are MRT-beamformed in frequency-domain or not

Note: UE is not required to perform DO/FO compensation for sub-band phase offset calculation

Note: There is a source R1-2407184 showing for frequency-domain-MRT-precoded CSI-RSs, inter-TRP phase has a linear rotation in frequency-domain, and is associated with a single delay component.

Agreement

For the 3-bit scaling scheme for Type-I SP codebook, support, in addition, (X1, X2)=(8,1)

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, support resource-specific RI restriction

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, OCPU = KS

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, for Scheme-A RI=5-8, the UCI parameters for the selection scheme for the other nSDBV SD basis vectors (in Part 2 UCI, wideband) are as follows:

- A 1-bit beam group indicator i3∈{0,1}

- For each of the nSDBV other selected SD basis vectors,

  - If i3=0:

    - log2N1-bit, log2N2-bit, and log2O2-bit (q2) indicators

      - In this case, q1 = mod(i1,O1) of the first SD basis vector

  - If i3=1:

    - log2N1-bit, log2N2-bit, and log2O1-bit (q1) indicators

      - In this case, q2 = mod(i2,O2) of the first SD basis vector

Note: It is up to the editor how this is captured in the specification

Note: (q1,q2) is analogous to (q1,q2) for Type-II CSI

Note (from previous agreement): nSDBV=2 (v=5-6) or 3 (v=7-8)

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI omission for the UCI reported in CSI part-1, CSI part-1 is either reported entirely or dropped entirely, with the following UCI packing order:

- CSI part-1 associated with the first configured CMR among the non-reported MR CRIs

- …

- CSI part-1 associated with the last configured CMR among the non-reported MR CRIs

- CSI part-1 of the 1st reported CRI,

- …

- CSI part-1 of the (M-MR)th reported CRI,

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1) {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref}:

- Supported sub-band size(s) {1, 2, 4, 8, 16} PRB

- The NW configures, via higher-layer (RRC) signalling, which NSB-P sub-band(s) the UE reports

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports based on Rel-16 eType-II, regarding UCI omission for the UCI reported in CSI part-2, other than priority 0 (G0), the UCI packing order and the 2M consecutive priority levels are defined w.r.t the M CRIs (including the non-reported MR CRIs) as follows:

- Consecutive priority levels (higher to lower):

  - G1 PMI components associated with the first configured CMR among the non-reported MR CRIs,

  - G2 PMI components associated with the first configured CMR among the non-reported MR CRIs,

  - …,

  - G1 PMI components associated with the last configured CMR among the non-reported MR CRIs,

  - G2 PMI components associated with the last configured CMR among the non-reported MR CRIs,

  - G1 PMI components of the 1st reported CRI,

  - G2 PMI components of the 1st reported CRI,

  - …,

  - G1 PMI components of the (M-MR)th reported CRI,

  - G2 PMI components of the (M-MR)th reported CRI.

Note: For UCI fields in wideband (G0), G1 PMI components (based on Rel-16 eType-II CB in Section 6.3.2.1.2, TS 38.212), and G2 PMI components, legacy design is fully reused.

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when ReportQuantity is ‘cjtc-P’ (PO) and =1:

- The UCI parameters are captured in the tables below:

| Parameter | Details/description |
| --- | --- |
| nref | Reference CSI-RS resource index, based on the ordering from RRC configuration: logNTRP bits |
| {n, n=0, 1, …, NTRP –1, n≠nref} | DL/UL phase offset for CSI-RS resource n: NTRP-1logMΦ bits |

- The UCI mapping order is as follows:

  - nref,

  - {n, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource ID

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding timeline:

- Multiply legacy Z’ by a factor of 2

- Multiply legacy Z by a factor of 2

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific delay offset pre-compensation on PDSCH by the NW, support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset (when ReportQuantity is ‘cjtc-Dd’).

- The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC delay offset report) can be separately or jointly triggered [and carried on a same PUSCH (hence on a same slot)] following legacy joint triggering mechanism

  - (Working Assumption) When separately triggered, the delay offset value to be compensated is the latest reported delay offset (DO) whose reporting instance’s last symbol is before the first symbol of DCI triggering of the CJT CSI reporting

    - FFS: whether some expiration time interval is needed

  - (Working Assumption) When jointly triggered, the delay offset value to be compensated is the reported delay offset (DO) in the same reporting instance

FFS: NW indicates the delay offset value to be compensated for the Rel-18 eType-II CJT CSI report

FFS: Whether only AP-CSI-RS, or any type of CSI-RS (P, SP, or AP) can be configured as the CMR for the Rel-18 eType-II CJT reporting

FFS: Support for SP-CSI for CJTC report

FFS: Whether this is also applicable for Rel-19 Type-I MP codebook

The above only applies when the CMRs do not share common QCL source for average delay indication

The above is UE optional feature.

Agreement

For the Rel-19 Type-I codebook refinement for 48, 64, and 128 CSI-RS ports, study, for RI= ϑ >1, applying the 3-bit scaling factor(s) as agreed in RAN1#117, where a per-layer scaling factor applied to the ith selected SD basis vector is given by e.g. min1,ϑsi2ri, where unit scaling factor “1” is associated with the PDSCH-to-CSIRS EPRE offset “portion” contributed by the ith selected SD basis vector without the 3-bit scaling factor si configured, e.g. si∈1, 12, 13 ,14,16,18,112,116  is the scaling factor associated with the ith SD basis vector, and ri∈1,2 is the number of layers transmitted using the ith SD basis vector.

- Note: This feature is a separate UE capability

- Study whether per-SD-basis-vector/layer power adjustment (including boosting) needs to be supported in addition

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding resource-specific CBSR,

- For Rel-16 eType-II-based: reuse Rel-16 eType-II CBSR design for each of the KS resources:

  - Just as Rel-18 Type-II CJT CBSR, decouple (N1,N2) from each of the KS CBSR IEs

  - This implies that it is possible not to configure CBSR for any of the KS resources

  - Only 1-bit hard CBSR is supported

- For Rel-15 Type-I-SP-based: reuse Rel-15 Type-I SP CBSR design for each of the KS resources:

  - Just as Rel-18 Type-II CJT CBSR, decouple (N1,N2) from each of the Ks CBSR IEs

  - This implies that it is possible not to configure CBSR for any of the KS resources

3Tx uplink

Conclusion

Support to reuse SRS features (i.e., cyclic shift hopping, and comb offset hopping) in Rel-18 for SRS resource set with usage of codebook based 3TX PUSCH transmission, without introducing new UE capabilities (i.e., reusing Rel-18 UE capabilities).

- Note: The conclusion does not include the Rel-18 8-port-specific SRS features.

Agreement

For a UE reporting UE capability of 3TX operation, support introduction of a new RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled.

Asymmetric DL-sTRP/UL-mTRP

Agreement

For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3:

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission

  - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not.

Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB

- FFS: Extending the range to lower than -10dB

- Step size is 4dB

Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH

Agreement

Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states.

- Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

Agreement

About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, down-select one from the following Alts in RAN1#118bis:

- Alt1: X = 45 (to be captured in RAN2 spec)

  - This feature is a separate UE capability and is appliable to any rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not.

  - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise

- Alt2: X = 44 (to be captured in RAN2 spec)

  - This feature is only applicable to UE who is configured with two separate SRS CLPC adjustment states.

  - Note: X=44 can be used for operations in FR1 in shared spectrum for FR2-2 and X = 42 otherwise

- Above extended value range is applied to startingBitOfFormat2-3.

Agreement

- Study whether/how to apply PL offset for SRS resource set when the SRS resource set is not configured with TCI state

- Study whether/how to apply one of the two separate SRS CLPC adjustment states on the SRS resource set when the SRS resource set is not configured with TCI state

  - E.g., defining i0 as the default CLPC for SRS resource set in this case. E.g,, configure one of the separate SRS CLPC adjustment states to the SRS resource set.

#### 2.1.2	Remaining Open issues

UE-initiated/event-driven beam management:

- Remaining details on enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - Remaining details on UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - Remaining details on UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

CSI:

- Remaining details on CSI support for up to 128 CSI-RS ports, targeting FR1

  - Remaining details on Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Remaining details on Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Remaining details on extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

- Remaining details on UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Remaining details on inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

3Tx uplink:

- Remaining details on non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Asymmetric DL-sTRP/UL-mTRP:

- Remaining details on enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Remaining details on two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

## 2.2	RAN2

#### 2.2.1	Agreements

- Not yet started

#### 2.2.2	Remaining Open issues

- Not yet started

.

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

In RAN4#112, the following agreements were made.

RF

Agreement:

- Rel-19 RAN4 MIMO workplan

  - Chair: encourage companies to follow the work plan in R4-2412134.

- UE RF impact of 3Tx

  - RAN4 will analyse the impact of 3Tx on UE RF requirements.

  - Approved TPs capture the agreements to TR

<3Tx uplink>

- Applicable UE types

  - Both handheld and non-handheld UEs will be considered for UE RF requirements

  - FFS on whether a single set or two sets of UE RF requirements, e.g., MPR, will be specified.

- Applicable Frequency Ranges

  - Both FR1 and FR2 will be considered for potential UL mTRP requirements

  - Only FR1 is considered for 3Tx objective

- Non ULFPTx mode

  - 3-layer with precoding matrix of W=13100010001.

RRM

Issue 1: RRM core impact analysis

Agreement: RRM core impact analysis:

| Topic | Description | RRM impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | YES |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | NO |
|  | Aperiodic standalone CJT calibration reporting | FFS |
| 3TX | 3-antenna-port codebook-based UL transmission | FFS |
| Asymmetric DL sTRP/UL mTRP |  | FFS |

Enhancement for UE-initiated/event-driven beam management

Issue 2-1-2: Scope of the scenario of event-triggered L1 beam reporting

Agreement:

- Intra-cell and inter-cell for FR2.

- Note: If specific requirements cannot be applicable, discuss it case by case.

Issue 2-1-3: Whether/how to define new delay requirements such as measurement reporting delay/measurement period?

Agreement:

- RAN4 will define Event triggered measurement reporting delay requirement

- Further discuss:

  - Whether to support both mode A and mode B

  - Whether to differentiate mode A and mode B

  - Other related aspects

Issue 2-1-5: measurement metrics for beam report?

Agreement:

- Based on L1-RSRP

- Note: others can be revisited if RAN1 have further agreement

Issue 2-1-6: L1-RSRP for CSI-RS measurement?

Enhancement for UE-initiated/event-driven beam management

Agreement: Define CSI-RS based L1-RSRP measurement requirements. CSI-RS is periodic CSI-RS.

CSI enhancement to support up to 128 ports

Issue 2-2-1: Whether RRM impacts exist by CSI enhancement to support up to 128 ports?

Agreement:

No RRM impact by CSI enhancement to support up to 128 ports

Enhancement for asymmetric DL sTRP/UL mTRP scenarios

Issue 2-5-5: Whether to define RRM core requirements of pathloss offset update?

Agreement:

Whether to define RRM core requirements of pathloss offset update?

- Not define RRM requirement for MAC CE decoding delay.

- The agreement can be revisited depending on further RAN1 progress.

#### 2.4.2	Remaining Open issues

RF

<3Tx uplink>

- Power classes and PA configuration for 3Tx UL MIMO

  - Option 1: PC3 and PC2

  - Option 2: PC1.5 only

  - Option 3: Others

- ULFPTx mode 0

  - Take 1-layer as baseline

  - Further discuss whether 2-layer is needed

<Others>

- UE RF impact of asymmetric DL sTRP/UL mTRP

  - Option 1: Identified (e.g., spherical coverage with a different TCI state for UL compared to DL)

  - Option 2: Not yet. Wait for further progress in RAN1

- FR2 UL enhancement scenario

  - UL TCI-state differs from DL TCI-state

  - Other options not precluded

RRM

FFS on whether to define new TCI state switching delay or existing requirements are sufficient

FFS: RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured.

FFS on impact of beam on known/unknown conditions for TCI states

FFS on whether to enhance beam sweeping in event triggered L1 beam reporting

FFS on whether to define new accuracy requirement by enhancement for E-initiated/event-driven beam management

FFS on whether to have RRM requirement impact.

FFS on whether to have RRM impacts by 2TA enhancement

FFS on whether to have RRM core impacts of active uplink TCI state switching delay for unified TCI

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template