3GPP TSG RAN Meeting #105									RP-241828

Melbourne, Australia, September 9-12, 2024

Status Report to TSG

Agenda item:		9.3.4.8

| WI / SI Name | WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | TRP_TRS_MIMO_OTA_Ph3 |  |  |  |  |
| Unique ID | 1030085 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241623 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Sep/2025 | Performance part: Mar/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 20% | Performance Part: 5% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Wang, Ruixin |
|  | Company | vivo |
|  | Email | ruixin.wang@vivo.com |
| Rapporteur | Name | Zhu, Siting |
|  | Company | CAICT |
|  | Email | zhusiting@caict.ac.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.4	RAN4

2.4.1	Agreements

RAN4 #112

- General

- Updated WI workplan is approved in [19]

- Topic summary for FR1 TRP TRS WI in [31]

- Reply LS on 3GPP NR TRP TRS OTA requirements was approved in [35]

- TP on Dynamic MIMO Aspects was agreed in [34]

- TR 38.762 skeleton on MIMO OTA dynamic test methodology for FR1 UEs was agreed in [12]

- WF for Rel-19 TRP_TRS_MIMO_OTA was approved in [36]

Agreements:

Sub-topic 1-1 General for WI

Issue 1-1-1: Reply LS to GCF CAG

Agreements:

  - Reply LS to GCF CAG in R4-2413537, is agreed.

Issue 1-1-2: Updated WP for Rel-19 OTA WI

Agreements:

  - Updated Workplan of Rel-19 OTA WI in R4-2412056 is agreed.

Topic #2: XR OTA

Sub-topic 2-1 XR test scenarios and configurations

Issue 2-1-1: Discussions on Reply LS from CTIA on XR OTA test phantom

Agreements:

  - Two types of XR, i.e., glasses and head mounted display are prioritized in this WI.

  - Need to send this two XR types to CTIA for phantom positioning and MU study.

Issue 2-1-4: Whether WI should focus on the XR devices those connected to gNB directly

Agreements:

  - Only test XR devices with 5G link to gNB in networks, not those with side link or wired connections.

Issue 2-1-6: Prioritize 1Tx XR devices?

Agreements:

  - Prioritize 1Tx XR

Issue 2-1-7: How to identify 2Rx XR devices?

Agreements:

  - Consider at least the following approach to identify 2Rx XR device

    - Use UE capability supportOf2RxXR-r18 to identify XR devices with 2Rx.

      - FFS device can set IE correctly

    - UE declaration as a backup approach

Issue 2-1-8: TRP TRS requirements for XR

Agreements:

  - Performance metric for XR OTA can be discussed, but Requirements work discussion can be postponed until device and phantom are available.

Sub-topic 2-2 Testing time reduction for XR OTA

Issue 2-2-1: Coarse measurement grids

Agreements:

The following Options can be further discussed next meeting:

Option 1: Stop the measurement in the middle and charge the battery, carefully re-place the XR device into the Chamber to continue the test.

Option 2: Test TRP at [X] dB below the maximum transmit power and scale up [X] dB to obtain TRP at maximum transmit power.

Options 3:

- Measure TRP: Averaging the measurement results from 4 of interpolated 62-points measurement grids by

TRPave=10log⁡(10TRP1/10+10TRP2/10+10TRP3/10+10TRP4/104)

TRSave=10log4/(110TRS110+110TRS210+110TRS310+110TRS410

to achieve the same accuracy level of using 266-points measurement grids during OTA tests.

(Define 4 of 62-points interpolated measurement grids based on 266-points measurement grids at the same time, the locations of each four of 62-points interpolated measurement grids is different from each other on the sphere, they can be considered orthogonal to each other.)

- Measure Peak EIPR: Full sphere coarse girds measurement + partial fine grids measurement

(For example, using 62-points measurement grids in OTA test to measure peak EIPR first, then based on the direction of peak EIPR, apply corresponding sectors (pre-defined 8 or 16 sectors) of 266-points measurement grids to measure the peak EIPR again to increase the accuracy of measured peak EIRP)

Topic #3: NTN OTA

Sub-topic 3-1 UE type and usage scenarios for NTN (NR-NTN and IoT-NTN)

Issue 3-1-1: Usage scenarios for NR-NTN mobile handheld UE

Agreements:

  - Test method development consider hand only and head+hand, RAN4 will discuss requirement applicability rules based on device type, NR-NTN/IoT-NTN support, or other criteria Prioritize hand only and head+hand

    - FFS down-selection of usage scenarios for requirements

  - Other usage scenarios are not precluded

Issue 3-1-4: UE type for IoT-NTN

Agreements:

  - For IoT-NTN test method development, RAN4 can consider handheld UE type as 1st priority.

Issue 3-1-5: Usage scenarios for IoT-NTN handheld UE

Agreements:

  - Same prioritized usage scenarios as NR-NTN handheld UE

Sub-topic 3-3 NTN OTA test methodologies

Issue 3-3-1: CBW for NR-NTN bands

Agreements:

  - Select 5MHz CBW for NR FR1 NTN OTA testing.

Issue 3-3-2: detailed test parameters for NR-NTN bands

Agreements:

  - Adopt the Table 1 and Table 2 test parameters for NR-NTN OTA testing.

- Table 1: NR FR1 NR-NTN TRP measurement parameters

| NR Band | CBW [MHz] | SCS (kHz) | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | UL RB Allocation (LCRB @ RBstart) | DL configuration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 5 | 15 | DFT-s-OFDM QPSK | Low | 396500 | 1982.5 | 434500 | 2172.5 | 12@6 | N/A |
|  |  |  |  | Mid | 399000 | 1995 | 437000 | 2185 |  |  |
|  |  |  |  | High | 401500 | 2007.5 | 439500 | 2197.5 |  |  |
| n255 | 5 | 15 | DFT-s-OFDM QPSK | Low | 325800 | 1629 | 305500 | 1527.5 | 12@6 | N/A |
|  |  |  |  | Mid | 328700 | 1643.5 | 308400 | 1542 |  |  |
|  |  |  |  | High | 331600 | 1658 | 311300 | 1556.5 |  |  |

- Table 2: NR FR1 NR-NTN TRS measurement parameters

| NR Band | CBW (MHz) | SCS (kHz) | DL modulation | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | UL RB Allocation (LCRB @ RBstart) | DL Configuration (FULL RB, LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 5 | 15 | CP-OFDM QPSK | DFT-s-OFDM QPSK | Low | 396500 | 1982.5 | 434500 | 2172.5 | 25@0 | 25@0 |
|  |  |  |  |  | Mid | 399000 | 1995 | 437000 | 2185 |  |  |
|  |  |  |  |  | High | 401500 | 2007.5 | 439500 | 2197.5 |  |  |
| n255 | 5 | 15 | CP-OFDM QPSK | DFT-s-OFDM QPSK | Low | 325800 | 1629 | 305500 | 1527.5 | 25@0 | 25@0 |
|  |  |  |  |  | Mid | 328700 | 1643.5 | 308400 | 1542 |  |  |
|  |  |  |  |  | High | 331600 | 1658 | 311300 | 1556.5 |  |  |

Issue 3-3-3: test parameters for IoT-NTN

Agreements:

  - RAN4 should further discuss test parameters for IoT-NTN OTA.

Issue 3-3-4: How to resolve UE antenna circular polarization issue

Agreements:

  - FFS RAN4 how to accommodate different UE polarization implementations, i.e., either circular polarization or cross polarization.

Topic #4: FR1 dynamic MIMO OTA

Sub-topic 4-1 Dynamic channel model generation and validation

Issue 4-1-1: TR skeleton

Agreements:

  - Approve the TR skeleton in R4-2411573.

Sub-topic 4-2 Test system for FR1 dynamic MIMO OTA

Issue 4-2-1: Link adaption configuration for Dynamic FR1 MIMO OTA

Agreements:

  - Adopt the mapping table between CQI and MCS used for demodulation in Table A.4-2 and Table A.4-3 in the annex of TS 38.101-4 for dynamic MCS adaption.

Issue 4-2-3: Test parameters for FR1 dynamic MIMO OTA

Agreements:

  - Set the maximum number of HARQ transmissions to [x] for FR1 dynamic MIMO OTA with x>1.

Sub-topic 4-3 UE Performance metric

Issue 4-3-1: UE measurement results collection

Agreements:

  - Collect measurement throughput data from different UEs during the measurement campaign to study FoM. Benchmarking Throughput vs. Noise-limited and Throughput vs. Interference-limited environmental conditions.

  - Presentation of channel model validation is encouraged.

Issue 4-3-2: TR work splitting

Agreements:

Consider the following work splitting to draft TR 38.762.

| Clause in TR 38.762 | TP owners |
| --- | --- |
| Draft TR structure, editorial correction, general information of each Clause, etc | CAICT, MVG |
| 3. Definitions of terms, symbols and abbreviations | CAICT |
| 4. General | CAICT, Apple, MVG |
| 5. Performance metrics | Keysight, Spirent, Apple, Qualcomm, Samsung, OPPO, CAICT |
| 6. Measurement methodology | Keysight, Spirent, Apple, Qualcomm, Xiaomi |
| 7. Channel models | Keysight, Spirent, Apple |
| 8. Channel model validation | Keysight, Spirent, Apple, Xiaomi |
| 9. gNodeB configurations | Keysight, Spirent, Samsung |
| Annex A: Channel model parameters | Keysight, Spirent, Apple |
| Annex B: UE coordinate system | Keysight, Spirent, Apple, OPPO |
| Annex C: Measurement uncertainty | Keysight, Apple, MVG, ETS-Lindgren |
| Annex D: Environmental requirements | Keysight, Spirent, Apple, OPPO |

Topic #5: Rel-19 FR1 OTA requirements

Sub-topic 5-1 FR1 TRP TRS requirements

Issue 5-1-1: 1Tx requirements for TRP TRS

Agreements:

  - No need to perform further lab alignment testing in Rel-19.

Issue 5-1-2: 2Tx requirements for TRP TRS

Agreements:

  - AC lab alignment for non-coherent UL MIMO at n41/n78 should be done in Rel-19, the framework can be similar to Rel-18, encourage test labs to join this activity.

    - Detailed framework will be provided next RAN4 meeting, also collect volunteer test labs feedback.

Issue 5-1-3: Size 2 TRP TRS requirements (PDA hand)

Agreements:

  - RAN4 further discuss how to specify size 2 UE TRP TRS requirements.

Sub-topic 5-2 FR1 MIMO OTA requirements

Issue 5-2-1: Further FR1 MIMO OTA lab alignment activity

Agreements:

  - No need to perform static MPAC lab alignment activities in Rel-19.

2.4.2	Remaining Open issues

- Core part

    - Further discuss XR OTA test method and UE performance metric

    - Further discuss NTN OTA test method and UE performance metric

    - Further discuss FR1 dynamic MIMO OTA test method and UE performance metric

- Performance part

    - Requirement framework and 2Tx lab alignment activity for TRP TRS

    - Requirement framework for static MIMO OTA

    - Requirements and recommended TT for TRP TRS and static MIMO OTA

2.5	RAN5

2.5.1	Agreements

RAN5 #104

- No discussion this meeting

2.4.2	Remaining Open issues

- Core part

  - RAN5 Workplan for Rel-19 OTA MU assessment

  - XR OTA test methods related MU assessment

  - FR1 NTN OTA (NR-NTN and IoT-NTN) test methods related MU assessment

  - FR1 dynamic MIMO OTA related MU assessment

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

4.	References

RAN4 #112

- R4-2411023	On MIMO dynamic OTA tests	Huawei, HiSilicon

- R4-2411024	on NTN tests	Huawei, HiSilicon

- R4-2411025	On XR devices test	Huawei, HiSilicon

- R4-2411149	Headworn XR test scenarios and configurations	Apple

- R4-2411252	TP on Dynamic MIMO Aspects	Keysight Technologies UK Ltd, Spirent Communications

- R4-2411253	NTN OTA Test Metrics	Keysight Technologies UK Ltd

- R4-2411264	On FR1 MIMO OTA Dynamic Channel model alignment with other SDO	Apple

- R4-2411265	Discussion on FR1 MIMO OTA FoM definitions	Apple

- R4-2411545	Examples of FR1 NTN high-gain and VSAT-like terminals OTA characteristics	Inmarsat, Viasat

- R4-2411546	Views on usage modes and framework for FR1 NTN UE OTA	Inmarsat, Viasat

- R4-2411572	On FR1 dynamic MIMO OTA	CAICT

- R4-2411573	TR 38.762 skeleton on MIMO OTA dynamic test methodology for FR1 UEs	CAICT

- R4-2411598	Discussion on OTA test for FR1 NTN devices	Xiaomi

- R4-2411695	On XR UE OTA test	Samsung

- R4-2411696	On NTN UE OTA test	Samsung

- R4-2411697	On FR1 dynamic MIMO OTA	Samsung

- R4-2412050	Reply LS on 3GPP NR TRP TRS OTA requirements	vivo

- R4-2412055	Discussions on NTN OTA test method	vivo

- R4-2412056	Updated Workplan of Rel-19 OTA WI 	vivo, CAICT

- R4-2412060	Discussions on Rel-19 FR1 OTA requirement work	vivo

- R4-2412263	Discussion on OTA Test for XR Devices	Meta

- R4-2412496	On the prioritization of XR device type for OTA test methodology	OPPO

- R4-2412497	On NTN OTA	OPPO

- R4-2412677	Discusion on XR OTA test method	CAICT, SAICT

- R4-2412678	Discussion on NTN OTA test method	CAICT, SAICT

- R4-2412920	On FR1 NTN devices OTA	Qualcomm Incorporated

- R4-2412921	Discussion on FR1 dynamic MIMO OTA test methodology	Qualcomm Incorporated

- R4-2413195	Test methodology for FR1 non-RedCap headworn XR devices	Nokia

- R4-2413196	Test methodology and radiated performance metric for FR1 NTN devices	Nokia

- R4-2413197	FR1 dynamic MIMO OTA test methodology	Nokia

- R4-2413433	Topic summary for [112][333] TRP_TRS_MIMO_OTA	Moderator (vivo)

- R4-2413451	Examples of FR1 NTN high-gain and VSAT-like terminals OTA characteristics	Inmarsat, Viasat

- R4-2413535	Way Forward for [112][333] TRP_TRS_MIMO_OTA	vivo

- R4-2413536	TP on Dynamic MIMO Aspects	Keysight Technologies UK Ltd, Spirent Communications

- R4-2413537	Reply LS on 3GPP NR TRP TRS OTA requirements	vivo

- R4-2413603	Way Forward for [112][333] TRP_TRS_MIMO_OTA	vivo

RAN5 #104

N/A

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template