3GPP TSG RAN Meeting #105                                                       RP-241831

Melbourne, Australia, September 9 – 12, 2024

Source:	vivo

Title:	Discussion on IMT-2030 technical performance requirements (TPR)

Agenda Item:	15

Document for:	Discussion and Decision

Introduction

In RAN#103 [1], the following was agreed regarding RAN plenary work toward 6G:

RAN plenary work is split into an ITU focused SI (study item) and General RAN SI

- IMT-2030 discussion is expected in RAN from 09/24 to 12/24

- RP SI Rel-20 focusing on ITU– IMT-2030:  approval 12/24 until 06/25

- RP SI Rel-20 focusing on 6G General: approval 03/25 (after WS), until 06/26

In this contribution, we summarize the status and progress of technical performance requirements (TPR) discussion in ITU-R WP5D and express our views on the TPR.

Current status of IMT-2030 TPRs in ITU-R WP5D

In ITU-R M.2160 “Framework and overall objectives of the future development of IMT for 2030 and beyond”, six usage scenarios are envisaged, as shown in Figure 1 (a). Among them three usage scenarios, including Immersive Communication, Hyper Reliable & Low-latency Communication and Massive Communication, are envisaged to expand from IMT-2020 (i.e., eMBB, URLLC, and mMTC) to higher performance to better meet the service requirements. Other three new usage scenarios, including Ubiquitous Connectivity, Artificial Intelligence & Communication and Integrated Sensing & Communication, are envisaged to support new services beyond IMT-2020. Furthermore, fifteen capabilities were proposed, as shown in Figure 1 (b), including nine enhanced capabilities compared to those for IMT-2020 and six new capabilities to support the usage scenarios and new services. It’s observed that more capabilities for IMT-2030 are needed to be studied compared to IMT-2020.

- Usage scenarios                                                           (b) Capabilities

Figure 1 Usage scenarios and Capabilities of IMT-2030

The technical performance requirements (TPR) for IMT-2030 are currently under discussion in ITU-R WP5D SWG radio aspect. According to the ITU-R WP5D timeline, as shown in Figure 2, TPRs have been discussed in the past two meetings, 28 contributions from different countries, alliances and companies are submitted for discussion and a working document towards a preliminary draft new report ITU-R M.[IMT-2030. TECH PERF REQ] [2] is developed. For each TPR, the definition, the requirement values and the applicable usage scenarios are to be studied and determined. Based on the input contributions, the working document and the discussion in the ITU-R WP5D meetings, the following observations on the progress of each TPR are given:

1) For some traditional TPRs, such as peak data rate, peak spectrum efficiency, the definition and applicable usage scenarios are clear and close to agreement, but the requirement values need more study.

2) For some other traditional TPRs, such as user experienced data rate, connection density, average spectrum efficiency etc., their definitions are clear and close to agreement, but their applicable usage scenarios are still under discussion.

3) New TPRs associated with new capabilities, such as sensing-related capability, AI-related capability and sustainability, their definition are still under discussion, including whether these TPRs will be defined as qualitative metric or quantitative metric.

Figure 2 ITU-R WP5D timeline

In the following Table 1, we summarize the discussion status for each TPR.

Table 1  TPR discussion status in ITU-R WP5D

|  | TPR | Definition | Requirement values | Requirements recommendation example in M.2160 | Usage scenarios |
| --- | --- | --- | --- | --- | --- |
| 1 | Peak data rate | Clear | TBD | 50, 100, 200 Gbit/s | Immersive communication |
| 2 | User experienced data rate | Clear | TBD | 300 Mbit/s, 500 Mbit/s | TBD [Immersive communication and/or Hyper-Reliable and Low Latency Communication and/or Ubiquitous Connectivity] |
| 3.1 | Peak Spectrum Efficiency | Clear | TBD | TBD | Immersive communication |
| 3.2 | Average spectrum efficiency | Clear | TBD | 1.5 and 3 times greater than that of IMT-2020 | TBD [Immersive communication and/or Hyper-Reliable and Low Latency Communication and/or Ubiquitous Connectivity] |
| 3.3 | 5th percentile user spectrum efficiency | Clear | TBD | TBD | TBD [Immersive communication and/or Hyper-Reliable and Low Latency Communication and/or Ubiquitous Connectivity] |
| 4 | Area traffic capacity | Clear | TBD | 30 Mbit/s/m2, 50 Mbit/s/m2 | Immersive communication |
| 5 | Connection Density | Clear | TBD | 106 – 108 devices/km2 | TBD [Massive communication and/or Immersive communication] |
| 6 | Mobility | TBD | TBD | 500 – 1 000 km/h | TBD [Immersive communication and/or Ubiquitous Connectivity] |
| 6.1 | Mobility interruption time | Clear | TBD | TBD | TBD |
| 7.1 | User plane latency | TBD | TBD | 0.1 – 1 ms | Immersive communication and Hyper-Reliable and Low Latency Communication |
| 7.2 | Control plane latency | TBD | TBD | TBD | Immersive communication and Hyper-Reliable and Low Latency Communication |
| 8 | Reliability | TBD [Joint requirement on data rate, latency and reliability] | TBD | 1-10−5 to 1-10−7 | TBD [Hyper-Reliable and Low Latency Communication and/or Immersive Communication] |
| 9 | Coverage | TBD | TBD | TBD | TBD |
| 10 | Positioning | TBD | TBD | 1 – 10 cm | TBD |
| 11 | Bandwidth | Clear | TBD | TBD | TBD |
| 12 | Sensing-related capabilities | TBD | TBD | TBD | Integrated Sensing and Communication |
| 13 | AI-related capabilities | TBD | TBD | TBD | Artificial Intelligence and Communication |
| 14 | Sustainability | TBD | TBD | TBD | TBD |
| 15 | Security and resilience | TBD | TBD | TBD | TBD |
| 16 | Interoperability | TBD | TBD | TBD | TBD |

Observation 1: For some traditional TPRs, the definitions and applicable usage scenarios are clear, and requirements value(s) needs further study.

Observation 2: For some other traditional TPRs or new TPRs, the definition and applicable usage scenarios as well as requirements value(s) need further study.

RAN should study ITU IMT-2030 technical performance requirements (TPR), including the TPR definitions, applicable usage scenarios and requirements value(s).

Initial views on IMT-2030 TPRs

In the following sections, our initial views on each IMT-2030 TPR are given.

Peak data rate

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Preliminary requirements value(s): the minimum requirements for peak data rate is 5 times of IMT-2020, i.e., 100Gbps for downlink and 50Gbps for uplink.

User experienced data rate

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Preliminary requirements value(s): the minimum requirements for user experienced data rate is 5 times of IMT-2020, i.e.，500Mbit/s for downlink and 250 Mbit/s for uplink.

Spectrum efficiency

Peak spectrum efficiency

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Preliminary requirements value(s): the minimum requirements for peak spectrum efficiency is 1.5 times of IMT-2020, i.e., 45 bit/s/Hz for downlink and 22.5 bit/s/Hz for uplink.

Average spectrum efficiency

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Preliminary requirements value(s): the minimum requirements for average spectrum efficiency is 2.0 times of IMT-2020.

5th percentile user spectrum efficiency

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Preliminary requirements value(s): the minimum requirements for 5th percentile user spectrum efficiency is 3.0 times of IMT-2020.

Area traffic capacity

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Preliminary requirements value(s): the minimum requirement for area traffic capacity is 5.0 times of IMT-2020, i.e., 50 Mbit/s/m2.

Connection Density

TPR definition: Same as in IMT-2020.

Applicable usage scenario:

In IMT-2020, connection density is mainly considered for mMTC usage scenario and the target requirement is developed for the IoT traffic with low data rate and larger latency. For IMT-2030 immersive communication usage scenario, the typical use cases include immersive XR. For XR use cases, system capacity is an important metric for successful commercial deployments. One way to quantify system capacity for XR use case is to define connection density requirement, e.g., number of supported XR users per km2.

As an example, the minimum requirement for connection density is 106~107 devices per km2 for Massive Communication usage scenario and 103 ~104 devices for Immersive Communication usage scenario.

The connection density should be applicable to both Massive Communication and Immersive Communication usage scenarios.

Mobility

In IMT-2020, mobility TPR is defined as the supported maximum mobile station speed (in km/h), which are related to extreme high speed scenarios such as high speed trains, aircrafts. However, in practical deployments, UE received signal may change dramatically in short period although the device moving speed is not high (e.g., entry/exit of building, shadow of driveway bridge, underground parking, elevator, etc., also called as pain point scenarios), resulting significant degradation of user experience. This dramatic service quality change in short period in low to medium moving speed can hardly be identified by traditional mobility evaluation methodology, therefore some new evaluation methodology and performance metrics should be introduced to quantify the user experience in the pain point scenarios.

For mobility, study new evaluation methodology and performance metrics to quantify the user experience in the pain point scenarios with low to medium UE moving speed.

Latency

User plane latency

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication, Hyper Reliable and Low-Latency Communication.

Preliminary requirements value(s): the minimum requirement for user plane latency is 1ms for Immersive Communication and 0.1ms for Hyper Reliable and Low-Latency Communication.

Control plane latency

In IMT-2020, the definition of control plane latency refers to the time for a device to switch from a battery efficient state (e.g., IDLE/INACTIVE) to start of continuous data transfer (e.g., CONNECTED), including the state transition time from battery efficient state to continuous data transfer state. In 5G NR, the small data packets can already be transmitted in battery efficient state (INACTIVE), which means a device may not always have to switch from INACTIVE to CONNECTED for transmission or reception of small packets. The SDT-like feature with high energy efficiency and low signaling demand for state transition can be a promising technology for IMT-2030 as well. However, the control plane latency in such scenarios would need to be clarified/reconsidered.

For example, the control plane latency can be defined as the latency from a “battery efficient” state to the start of data transfer, including the case with the device state transition (e.g., INACTIVE to CONNECTED) and the case without state transition (e.g., small data transmission during INACTIVE).

Study control plane latency definition to include the cases with and without device state transition.

Applicable usage scenario: Immersive Communication, Hyper Reliable and Low-Latency Communication.

Preliminary requirements value(s): the minimum requirement for control plane latency is 10ms with device state transition and 4ms without device state transition.

Reliability

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Hyper Reliable and Low-Latency Communication.

Preliminary requirements value(s): the minimum requirement for reliability is 1-10-5.

Bandwidth

Preliminary requirements value(s): the maximum bandwidth for single radio frequency (RF) carrier can be 400Mhz, and the maximum aggregated system bandwidth is TBD, which is relevant to the requirement of peak data rate.

Sensing-related TPRs

In the outcome of SA1 study item in TR 22.837, sensing service KPIs for different use cases are captured, such as accuracy of positioning, accuracy of velocity, and so on. These KPIs can be used as starting point for discussion on the IMT-2030 sensing-related TPRs. It should be noted that there are significant differences in the performance requirements among different sensing use cases.

In the past two meetings of ITU-R WP5D #45 and #46, both qualitative metrics and quantitative metrics for sensing-related capabilities are proposed and discussed. Regarding the qualitative metrics, one contribution proposed to  evaluate sensing-related capabilities by using questionnaires through a Characteristics Template instead of using specific technical performance requirements with their associated target values to answer whether IMT-2030 structure/function supports integrated sensing and communication, whether IMT-2030 structure/function accommodates different sensing configurations. Regarding the quantitative metrics, more than ten sensing-related TPRs are proposed by the submitted contributions, including sensing range, detected object velocity, horizontal/vertical sensing accuracy and so on.

In our opinion, considering a limited number of quantitative sensing TPRs, the basic sensing measurement metrics, such as range, doppler/velocity and angle estimation accuracy which can characterize sensing performance can be considered as the sensing-related TPRs.

Hence, we propose the following sensing-related TPR definitions:

- Range accuracy: the difference between the measured range of and the true range of the target object.

- Angle accuracy: the difference between the measured angle and the true angle of the target object, including azimuth angle and zenith angle.

- Velocity accuracy: the difference between the measured velocity and the true velocity of the target object

  - Velocity not only refers to the velocity in linear motion, but also refers to the velocity of periodic reciprocating motion (such as breathing, heartbeat, repetitive movements and so on).

- Detection/Recognition accuracy:

- Detection accuracy: the probability of correctly detecting a target.

- Recognition accuracy: the probability of correctly identifying the state or category of the target.

Study sensing-related TPRs including range accuracy, angle accuracy, velocity accuracy and detection/recognition accuracy.

AI-related TPR

This TPR is related to Artificial Intelligence and Communication usage scenario. AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 network to support AI services.

The following qualitative requirements from ITU-R M.2160 “Framework and overall objectives of the future development of IMT for 2030 and beyond” can be considered as a starting point for AI-related TPR:

“Besides communication aspects, this usage scenario is expected to include a set of new capabilities related to the integration of AI and compute functionalities into IMT-2030, including data acquisition, preparation and processing from different sources, distributed AI model training, model sharing and distributed inference across IMT systems, and computing resource orchestration and chaining.”

Sustainability

Energy efficiency is a quantifiable metric of sustainability and is important for the success of IMT-2030 technology. The energy efficiency TPR should be defined to ensure that IMT-2030 is designed in an energy efficient way.

In IMT-2020, energy efficiency was evaluated through sleep ratio and sleep duration under unloaded case and demonstrated through high spectrum efficiency under loaded case. However, through the study of multiple energy saving projects in 3GPP release 16/17/18, it can be seen that the power consumptions of device in different power states are different, such as in micro sleep state, the relative power consumption is 45 unit, in PDCCH-only state, the relative power consumption is 100 unit, and in PDCCH+PDSCH state, the relative power consumption is 300 unit.

Under the unloaded case, even with the same configuration of discontinuous reception (DRX), due to the different power consumption in different sleep states and transition power from different sleep states to non-sleep states, the average energy consumption of the device is also different. Therefore, using the sleep ratio and sleep duration cannot fully characterize the energy consumption of the device. In addition, in unloaded case, the latency for paging/control signaling delivery may also be important for some services and device types. So, in unloaded case, the energy efficiency could be defined taking both the impact on paging/control signaling latency and energy consumption into account.

Under the loaded case, due to the traffic characteristics, it can be divided into sleep duration and data transmission duration. Due to the power consumptions of device in different power states are different, technics for decreasing the power consumption can be studied meanwhile minimizing the impact on the latency of data transmission. Therefore, in loaded case, the energy efficiency could be defined by taking both the data rate and power consumption into account, such as EE=VP  or EE=PV ,  where P is the power consumption, v is the user perceived throughput (UPT), which is widely used in 3GPP evaluations in all SI/WIs and reflects the user's experience data rate. P=EC/T, where EC is the energy consumption during an evaluation time T, EC= Pon ×Ton + Psleep ×Tsleep+ Ptransition ×Ttransition .

Although in the above discussion, device side energy efficiency is assumed, the network side energy efficiency is also very important. The definition of network side energy efficiency shall also be carefully studied, common definition between device and network side energy efficiency is preferable if possible.

Observation 3: For energy efficiency, the sleep ratio and sleep duration used in IMT-2020 evaluation cannot accurately characterize the energy efficiency of device or network.

The definition of energy efficiency should jointly consider both power/energy consumption and communication related performance metric, such as user experienced data rate, and latency for control signaling/paging or data.

For loaded case, the energy efficiency can be the ratio of data rate and power consumption.

- -  EE=VP  or EE=PV , where P is the power consumption, v is the user perceived throughput (UPT).

Conclusions

In summary, we summarize the status and progress of technical performance requirements (TPR) discussion in ITU-R WP5D and express our views on the TPR with following observations and proposals:

Observation 1: For some traditional TPRs, the definitions and applicable usage scenarios are clear, and requirements value(s) needs further study.

Observation 2: For some other traditional TPRs or new TPRs, the definition and applicable usage scenarios as well as requirements value(s) need further study.

Observation 3: For energy efficiency, the sleep ratio and sleep duration used in IMT-2020 evaluation cannot accurately characterize the energy efficiency of device or network.

- RAN should study ITU IMT-2030 technical performance requirements (TPR), including the TPR definitions, applicable usage scenarios and requirements value(s).

- The connection density should be applicable to both Massive Communication and Immersive Communication usage scenarios.

- For mobility, study new evaluation methodology and performance metrics to quantify the user experience in the pain point scenarios with low to medium UE moving speed.

- Study control plane latency definition to include the cases with and without device state transition.

- Study sensing-related TPRs including range accuracy, angle accuracy, velocity accuracy and detection/recognition accuracy.

- The definition of energy efficiency should jointly consider both power/energy consumption and communication related performance metric, such as user experienced data rate, and latency for control signaling/paging or data.

- For loaded case, the energy efficiency can be the ratio of data rate and power consumption.

- -  EE=VP  or EE=PV , where P is the power consumption, v is the user perceived throughput (UPT).

References

- RP-240823 Additional Considerations for 6G Timeline.

- R23-WP5D-C-0242!H5-N5.08!MSW-E.