3GPP TSG RAN Meeting #105								RP-241836

Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.1.4

Source:	OPPO

Title:	Discussion on check point of R19 LP-WUS WI

Document for:	Discussion & Decision

Introduction

The R19 LP-WUS WID had identified 2 directions of applying wake-up signal triggering main receivers. One is in IDLE/INACTIVE modes, another is in CONNECTED mode. The direction for CONNECTED mode may have different level of impact on RAN2 workload, depending on the exact solution and procedure.

Thus, in the WID, it had state that “Check in RAN#105 for potential TU adjustment in RAN2”. Since the RAN1 and RAN2 already had done procedure discussion, the updating based on the status should be performance in this plenary meeting.

In this paper, we provide our views on the proceeding the topic.

Discussion

As defined in the scope, RAN 1 and RAN2 has discussed all solutions for the 2 different directions.

For the direction of IDLE/INACTIVE modes, RAN1 had decided earlier paging indication will be supported, which is similar to the Rel-17 PEI. The progress is going well. RAN2 also fully discussed the direction.

In the last meeting, RAN 1 has agree the working assumption for the direction for CONNECTED mode, which has concluded 2 options to support.

Option 1-1 (Figure 1): LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

Figure 1. Indication DRX ON by LP-WUS

Option 1-2 (Figure 2): LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. The option can achieve better power saving and short scheduling latency, with longer inactive time configured. In the RAN1 #118 meeting, it is also clarified the Option may introduce a new timer to tigger PDCCH monitoring, instead of “drx-onDurationTimer”. This will be the only new functionality introduced, compared with Option 1-1.

Figure 1. PDCCH triggered by LP-WUS at least before legacy active time

Some further detail for the timer is left RAN2 to decide. From our point of view, the RAN2 work can be based on the working assumption. It is also concluded that: UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected , namely drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL. We expect the RAN2 workload will not be significantly increased due to the timer behavior discussion.

Observation 1: RAN1 has clarified the 2 options for CONNECTED mode. The option 1-2 may introduce a new timer for LP-WUS. RAN2 will not be significantly increased with work load.

In the current time unit budget, RAN 2 has already allocate 1 TU per meeting in the coming quarters. Considering the timer discussion would be clearly identified by RAN2 based on the RAN1 discussion, we consider the RAN2 will not require additional TUs.

Proposal 1: The current RAN2 TU allocation is sufficient for supporting both Options for LP-WUS procedures under RRC CONNECTED mode, no TU updating is required for RAN2.

- Remove the WID bullet of check point for CONNECTED mode.

Considering the definition of option 1-1 and 1-2, we would prefer to have some high-level description in to the WID, instead of replicated the whole agreements by RAN1.  We are also fine for not have such sub-bullets in the WID and further work could be mainly based on WGs’ decisions.

Conclusion

In this document, we discussed the progress of LP-WUS procedures in different RRC modes. Based on our analysis, we provide our observations and proposal for the procedure of CONNECTED mode:

Observation 1: RAN1 has clarified the 2 options for CONNECTED mode. The option 1-2 may introduce a new timer for LP-WUS. RAN2 will not be significantly increased with work load.

Proposal 1: The current RAN2 TU allocation is sufficient for supporting both Options for LP-WUS procedures under RRC CONNECTED mode, no TU updating is required for RAN2.

- Remove the WID bullet of check point for CONNECTED mode.