3GPP TSG RAN Meeting #105								RP-241853

Melbourne, Australia, September 9-12, 2024

Agenda Item:		9.3.1.4

Source:			Nokia

Title:			On Rel-19 work on Low-Power Wake-up Signal (LP-WUS) Document for:		Discussion and Decision

# 1	Introduction

In RAN#102 the work item on Low-Power Wake-up Signal (LP-WUS)  was approved [1], which included a checkpoint for decision in RAN#105 regarding the study on-demand SIB1 (OD-SIB1) for UEs in RRC idle/inactive mode. In this contribution we present our views on the conclusions from the study and the proposed way forward.

# 2	On UE MR PDCCH monitoring triggered by LP-WUS

The objective for PDCCH monitoring by main radio (MR) triggered by LP-WUS for UEs in CONNECTED mode is defined as follows [1]:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

The work has progressed well in RAN1 and RAN2, with agreements and working assumptions regarding standardization directions. For example, RAN1 has made the following working assumption and agreement [2]:

Working Assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

- Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

- Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

- FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

Agreement

For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

- LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

- UE is configured with legacy C-DRX configurations as Rel-18

- UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)

  - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration

- LP-WUS triggers the start of a timer during which UE monitors PDCCH

  - FFS the timer is existing timer and/or new timer

- UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected

  - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL

- No impact on RRM/RLM/BFD measurement requirements is assumed

- For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1)

  - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

  - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

- UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

RAN2 has made the following working assumption and agreements [3]:

For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

Hence, given the progress so far, which should allay the concerns raised during the approval of the WID itself, we propose the following:

Proposal: Revise the WID by removing the note on checkpoint for RAN#105, i.e. confirm that work should continue on LP-WUS with existing TU allocation.

# Conclusion

In this contribution we have discussed RAN1’s conclusion and propose the following:

Proposal: Revise the WID by removing the note on checkpoint for RAN#105, i.e. confirm that work should continue on LP-WUS with existing TU allocation.

# References

[1] RP-240801, “Low-power wake-up signal and receiver for NR (LP-WUS/WUR),” Mar. 2024.

[2] “Chair notes RAN1#118”, Aug. 2024.

[3] R2-2407572, “Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD”, Aug. 2024.