3GPP TSG RAN Meeting #105					RP-241867

Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.1.5

Source:	Ericsson

Title:	On checkpoint for normative work on on-demand SIB1

Document for:	Discussion

# 1	Introduction

The Rel-19 WID on network energy savings has following checkpoint in RAN#105 (Sept ’24):

This document proposes WID objective for moving to normative phase for on-demand SIB1 based on the study outcome.

# 2	Discussion

The RAN WGs 1,2,3 studied the procedures and signalling methods for on-demand SIB1, including evaluation assumptions and associated evaluations in RAN1 to assess potential network energy saving (NES) gains for the feature. The study is considered complete in the WGs.

The RAN1 recommendation (from RAN1#118) is as follows:

Agreement

RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19.

- Note: RAN1 strive to minimize impact to legacy UE.

- Note: RAN1 specification impact to support this feature should be minimized.

The RAN2 recommendation (from RAN2#127) is as follows:

Agreement on OD-SIB1

OD-SIB1 SI conclusion:

8.	RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

The relevant RAN3 agreements (from RAN3#124 and RAN3#125) are as follows:

RAN3#124:

Focus on case2 in RAN3 first.

RAN3#125:

Case 2 can be supported with the below details:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

Case 2 in above agreements from RAN1, 2 and 3 is the case where WUS configuration for the NES cell is provided by a neighbouring cell (Cell A), but the UE sends the WUS to and receives SIB1 from the NES cell itself (see Annex A for detailed RAN1 agreements regarding Case 2 and relevant RAN2 agreement).

To reflect the recommendations from RAN1 and RAN2 and the agreement in RAN3, we propose to update the WID to add normative work for on-demand SIB1 for only Case 2 (Option 1+B+X) with RAN2 as the leading WG. The revised objective is in section 3 and the revised WID is in [5].

# 3 Conclusion

For the normative work on on-demand SIB1 for Rel-19 NES, the following WID update (no track changes) is proposed i.e. to specify on-demand SIB1 for only Case 2. Revised WID is in RP-241868 [5].

- Specify support of on-demand SIB1 for UEs in idle/inactive mode. [RAN2/1/3]

  - Specify the necessary procedures and signaling method(s) considering only the following scenario. [RAN2/1/3]

    - UE transmits UL WUS to NES Cell

      - Triggering method by UL-WUS using PRACH

    - UE obtains the UL WUS configuration from Cell A

    - UE receives SIB1 of NES cell only from NES Cell

    - UE is notified about UL-WUS configuration updates in the NES cell

  - Specify inter NG-RAN node signalling at least for the configuration of wake-up signal. [RAN3]

  - Note 1:

    - UL WUS: uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 2: No modification of SSB will be discussed under this objective

  - Note 3: Strive to minimize impact to legacy UE.

  - Note 4: RAN1 specification impact to support this feature should be minimized.

# References

RP-241650, “Revised WID: Enhancements of network energy savings for NR”, 3GPP TSG RAN Meeting #104, Shanghai, CN, June 17-20, 2024

RP-240169, Status report for WI: Enhancements of network energy savings for NR	Ericsson, 3GPP TSG RAN Meeting #103, Maastricht, NL, March 18-21, 2024

RP-241182, Status report for WI: Enhancements of network energy savings for NR	Ericsson, Apple, 3GPP TSG RAN Meeting #104, Shanghai, CN, June 17-20, 2024

RP-241866, Status report for WI: Enhancements of network energy savings for NR	Ericsson, Apple, 3GPP TSG RAN Meeting #105, Melbourne, AUS, September 9-12, 2024

RP-241868, Revised WID: Enhancements of network energy savings for NR	Ericsson, 3GPP TSG RAN Meeting #105, Melbourne, AUS, September 9-12, 2024

# Annex A (Relevant WG agreements)

Case 2 in the agreements from RAN1, 2, and 3 is defined in the following agreements from RAN1#116 and RAN1#116bis:

Agreement

For discussion purpose, the following assumption will be used in RAN1

- Cell A: A cell that is periodically transmitting at least its own SIB1

- NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases:

- Case 1: Option 1+A+X

- Case 2: Option 1+B+X

- Case 3: Option 2+B+Y

Where the options 1/2/A/B/X/Y are defined below:

- On target cell of UL WUS transmission:

  - Option 1: UE transmits UL WUS to NES Cell

  - Option 2: UE transmits UL WUS to Cell A

- On configuration provision for UL WUS transmission

  - Option A: UE obtains the UL WUS configuration from NES Cell

  - Option B: UE obtains the UL WUS configuration from Cell A

- On receiving of SIB1

  - Option X: UE receives on-demand SIB1 from NES Cell

  - Option Y: UE receives on-demand SIB1 from Cell A

Agreement from RAN2#127 regarding UL WUS configuration updates.

- WUS configuration of NES cell from NES cell:

  - Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.