3GPP TSG RAN Meeting #105						                                       RP-241884

Melbourne, Australia, Sept 9-12, 2024

## Status Report to TSG

Agenda item:			9.4.1.4

| WI / SI Name | # Introduction of another NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_SBand |  |  |  |  |
| Unique ID | 1040129 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241661 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 6/2025 | Performance part: 6/2025 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 20% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Laehteensuo, Toni |
|  | Company | Qualcomm Incorporated |
|  | Email | tlaehtee@qti.qualcomm.com |

# 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

# 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

### 2.1.1	Agreements

### 2.1.2	Remaining Open issues

## 2.2	RAN2

### 2.2.1	Agreements

### 2.2.2	Remaining Open issues

## 2.3	RAN3

### 2.3.1	Agreements

### 2.3.2	Remaining Open issues

## 2.4	RAN4

### 2.4.1	Agreements

#### 2.4.1.1	RAN4 #112

All agreements were captured in R4-2414287 “WF on Rel-19 FR1 NTN bands” and are reproduced below

Band Numbering

Agreement: Adopt n252 as the band number for the new NR NTN S-band.

Band Plan

Agreement: Proposed band plan as follows

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n252] | 2000 MHz - 2020 MHz | 2180 MHz - 2200MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

Intended Regions and Countries

Agreement: Clarify that the new NTN S-band is only targeting north America.

Regulatory Background

Agreement:

Capture all the applicable regulations in a new section of TR 38.863

ATC is not applicable, hence out of scope for this work.

Applicable TN Bands for Coexistence

Agreement:

Focus on coexistence of S-band UL with B2/n2 and B25/n25 DL.

Capture the clarifications on the co-existence issues with B70/n70 and B66/n66 and that there is no 3GPP solution for them in the TR

TN-NTN UE-to-UE Coexistence

Agreement:

Further study the UE-to-UE coexistence between NTN S-band UL and B2/n2, B25/n25 DL, considering:

- Realistic usage scenarios

- consider UE antenna gain for smartphone of -5.5dBi

- UE-to-UE separations distance

- probability of the UE-to-UE coexistence scenarios occurring

Companies are encouraged to examine the previous work in TR 38.863 for UE-to-UE coexistence noting that the solution might need to be re-evaluated for this work.

Companies are encouraged to investigate the background scenario for applicability of the -50dBm/MHz requirement to assess its applicability to TN-NTN coexistence (e.g. reference R4-080710)

Companies are also encouraged to investigate other prior work in UE-to-UE coexistence where different values from -50 dBm/MHz have been adopted

UE Channel Bandwidths

Agreement:

For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the channel bandwidth and SCS should be defined as follows:

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| [n252] | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |

Channel Raster

Agreement:

Support both 100 kHz and 10 kHz channel raster.

Agree to mark Enhanced Channel Raster support as Mandatory as agreed in the WID Objectives.

Sync Raster

Agreement:

Specifying at least Case A for the new NTN S-band GSCN.

TX-RX Separation

Agreement:

Follow the conclusion of flexible Tx-Rx separation under the maintenance agenda.

General views on Impact to UE Requirements

Agreement:

Use the following merged table with common views on identified specification impact as a starting point.

Companies to further assess focusing on the missing or TBC sections.

| Section | Requirement | COMMON VIEWS |
| --- | --- | --- |
| Tx requirements |  |  |  |
| 6.2.1 | UE maximum output power | Some impact |  |
| 6.2.2 | UE maximum output power reduction | No impact |  |
| 6.2.3 | UE additional maximum output power reduction | Some impact |  |
| 6.2.4 | Configured transmitted power | No impact |  |
| 6.3.1 | Minimum output power | No impact |  |
| 6.3.2 | Transmit OFF power | No impact |  |
| 6.3.3 | Transmit ON/OFF time mask | No impact |  |
| 6.3.4 | Power control | No impact |  |
| 6.4.1 | Frequency error | No impact |  |
| 6.4.2 | Transmit modulation quality | No impact |  |
| 6.5.1 | Occupied bandwidth | No impact |  |
| 6.5.2 | Out of band emission (ACLR) | No impact |  |
| 6.5.2.2 | Spectrum emission mask | No impact |  |
| 6.5.2.3 | Additional Spectrum emission mask | Some impact |  |
| 6.5.2.4.1 | NR ACLR | No impact |  |
| 6.5.2.4.2 | UTRA ACLR | No impact |  |
| 6.5.3.1 | General Spurious emissions | No impact |  |
| 6.5.3.2 | Spurious emissions for UE co-existence | Some impact |  |
| 6.5.3.3 | Additional spurious emissions | Some impact |  |
| 6.5.4 | Transmit intermodulation | No impact |  |
| Rx requirements |  |  |  |
| 7.3.2 | Reference sensitivity | Some impact |  |
| 7.4 | Maximum input level | No impact |  |
| 7.5 | Adjacent channel selectivity | No impact |  |
| 7.6.1 | General | No impact |  |
| 7.6.2 | In-band blocking | TBC |  |
| 7.6.3 | Out-of-band blocking | Some impact |  |
| 7.6.4 | Narrow band blocking | Some impact |  |
| 7.7 | Spurious response | No impact |  |
| 7.8 | Intermodulation characteristics | No impact |  |
| 7.9 | Spurious emissions | No impact |  |

Other Sub-topics/Issues

Agreement:

Other topics and issues are postponed to future meetings

Work Split/Interest

Agreement:

No formal CR/Spec Work Split agreed in this meeting

Companies to report areas/topics of interest for contribution or future CR work split

Following table shall be used

| TS/TR (if applicable) | Clause/Section (if applicable | Topic/Requirement | Interested contributing companies |
| --- | --- | --- | --- |

### 2.4.2	Remaining Open issues

- System parameters including flexible Tx-RX spacing, channel raster, sync raster

- UE RF requirements including A-MPR, UE-coexistence requirements, reference sensitivity

- Detailed specification updates

## 2.5	RAN5

### 2.5.1	Agreements

### 2.5.2	Remaining Open issues

### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

### 2.6.1	Agreements

### 2.6.2	Remaining Open issues

# 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 3.1	SAx/CTs

### 3.1.1	Agreements with cross-TSG impacts

### 3.1.2	Remaining Open issues with cross-TSG impacts

# 4.	References

## RAN4#112

- R4-2411303	NTN MSS S-band band plan, regulations, and deployment scenarios 	EchoStar, Dish Network, TerreStar, Thales, Gatehouse, Novamint

- R4-2413146	Workplan for new NR NTN S-band	Qualcomm Incorporated

- R4-2411059	Discussion on UE RF requirements for the NTN new S-band	CATT

- R4-2411060	DraftCR for TS 38.101-5, Introduction on system parameters for UE supporting new S Band	CATT

- R4-2411196	New NTN S-band - System parameters and UE RF requirements	Ericsson

- R4-2411842	Discussion on system parameters and UE RF requirements for NR-NTN S band	ZTE Corporation, Sanechips

- R4-2412959	Discussion on the NTN S band definition	Huawei, HiSilicon

- R4-2413147	Co-existence considerations for the new S-band	Qualcomm Incorporated

- R4-2413305	Coexistence between S-Band NTN and Terrestrial Networks	T-Mobile USA

- R4-2411058	Discussion on SAN RF requirements for the NTN new S-band	CATT

- R4-2411061	DraftCR for TS 38.108, Introduction on system parameters for SAN supporting new S Band	CATT

- R4-2411195	New NTN S-band - SAN RF requirements	Ericsson

- R4-2411843	Discussion on SAN RF requirements for NR-NTN S band	ZTE Corporation, Sanechips

- R4-2411844	draftCR to TS38.108 Introduction of NR-NTN S band	ZTE Corporation, Sanechips