3GPP TSG RAN Meeting #105 	RP-241885

Melbourne, Australia, September 9-12, 2024	(revision of RP-241661)

Source:	Qualcomm Incorporated

Title:	Revised WID on Introduction of another NR NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

Document for:	Approval

Agenda Item:	9.4.1.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Introduction of another NR NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

Acronym: NR_NTN_SBand

{Propose an acronym. Final acronym to be confirmed at the plenary. The sign "-" is a level separator between (Feature)-(Building Block)-(Work Task). The sign "_" can be freely used. Studies have to start by "FS_". Each acronym level has to be simple and short, 7 characters max recommended}

Unique identifier:	1040129

{A number to be provided by MCC at the plenary}

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

Potential target Release:	Rel-19

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

{Tick one or more box(es). The full structure of all existing Work Items is shown in the 3GPP Work Plan in https://ftp.3gpp.org/Information/WORK_PLAN}

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |  |
| NR_NTN_solutions | 860046 | Solutions for NR to support non-terrestrial networks (NTN) | Rel-17 work item defining NR-NTN band n256, n255 |  |
| NR_NTN_LSband | 981043 | Introduction of the satellite L-/S-band | Rel-18 work item defining NR-NTN band n254 |  |

# 3	Justification

One of the objectives of Rel-17 NR_NTN_solutions WI was to study and identify relevant NTN bands. The outcome of this work is a definition of two NTN bands – part of ITU-R L band and S band - defined as band n256 and n255 in the 3GPP specifications [1, 2]. However, there is some deviation in how MSS S-band is allocated in region 2 versus in all other regions, see Figure 1. Per ITU, the Region 2 MSS S-Band had been allocated to UL 1980-2025 MHZ and DL 2160-2200 MHz. In the case of North America region (US, Canada, Mexico) the MSS band is allocated to UL 2000 MHz -2020 MHz and DL 2180 MHz -2200 MHz which is not exactly aligned to n256 definition, i.e., UL 1980 MHz – 2010 MHz, DL 2170 MHz – 2200 MHz. For consistency across North America and Region 2 as a whole, it is beneficial to have a common range of UL 2000 MHz -2020 MHz and DL 2180 MHz -2200 MHz as new 3GPP NR-NTN band.

There has been very strong interest from global satellite operators to ensure satellite service using 3GPP NTN to be more aligned with spectrum conditions, regulatory and TN-NTN ecosystem. Therefore, due to the above reason, it is necessary to define the range of UL 2000 MHz -2020 MHz and DL 2180 MHz -2200 MHz as a separate NR-NTN band.

It should be noted that this range is the same as LTE Band 23, although LTE Band 23 has a note that says, “NOTE 1: Band 6, 23 is not applicable” in the current version of 36.101 due to the use of 2000-2020 MHz for the Band 70 downlink and 2180-2200 MHz for the extended Band 66 downlink by terrestrial networks in the US. It may be beneficial to leverage the coexistence study and work done on Band 23 when defining this range for NR-NTN band.

The aim of this spectrum WID is to add support for a new NR-NTN FDD band with 2000-2020 MHz UL and 2180-2200 MHz DL.

Figure 1: Table of Frequency Allocations of ITU, United States and Canada

Reference:

[1] 	TR 38.863, “Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects” (Rel-17)

[2]	TS 38.101-5, “User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”

[3]	FCC ONLINE TABLE OF FREQUENCY ALLOCATIONS, 47 C.F.R. § 2.106, Revised on July 1, 2022 (https://transition.fcc.gov/oet/spectrum/table/fcctable.pdf)

[4]	Canadian Table of Frequency Allocations (2022), 2022 Edition 
(https://ised-isde.canada.ca/site/spectrum-management-telecommunications/en/learn-more/key-documents/consultations/canadian-table-frequency-allocations-sf10759)

# 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objective of the core part is to:

-	Specify a new NR-NTN FDD band with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz;

- Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope;

- Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario

-	Support channel bandwidths and SCS as presented in Table 4.1-1 below;

-	Mandatory support of channel raster points at step of 10 kHz;

-	Support UE Power class 3 (+23dBm);

-	Introduce the corresponding SAN and UE RF core requirements;

-	Introduce the corresponding RRM requirements.

Table 4.1-1: Channel bandwidth and SCS system parameters.

| SCS (kHz) | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 | 20 |
| 30 |  | 10 | 15 | 20 |
| 60 |  | 10 | 15 | 20 |

The specification work of this WI shall leverage the studies and requirements for NR NTN n256, n255, n254, where applicable.

All UE requirements specified as part of this WI shall be Release-independent from Rel-17.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

The objective of the performance part is to:

-	Define conformance requirements for SAN.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TR 38.863 | Support for NR NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) | RAN#107 (March 2025) | Core |
| TS 38.101-5 | Support for NR NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) | RAN#107 (March 2025) | Core |
| TS 38.108 | Support for NR NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) | RAN#107 (March 2025) | Core |
| TS 38.133 | Support for NR NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) | RAN#107 (March 2025) | Core |
| TS 38.101-5 | Support for NR NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) | RAN#108 (June 2025) | Perf |
| TS 38.108 | Support for NR NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) | RAN#108 (June 2025) | Perf |
| TS 38.181 | Support for NR NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) | RAN#108 (June 2025) | Perf |

# 6	Work item Rapporteur(s)

Laehteensuo, Toni, Qualcomm, tlaehtee@qti.qualcomm.com

# 7	Work item leadership

RAN4

# 8	Aspects that involve other WGs

None identified yet.

# 9	Supporting Individual Members

Supporting IM name

DISH Network

EchoStar

TerreStar

Omnispace

Telus

Apple

Qualcomm

Samsung

Fujitsu

ESA

Sateliot

Skylo

Gatehouse

Fraunhofer IIS

Fraunhofer HHI

Thales

TTP Plc

Inmarsat

Viasat

Airbus

Novamint

MediaTek