3GPP TSG RAN WG1 #105		                                	      RP-241906
Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.2.1

Source:	InterDigital, Inc.

Title:	On the scope of Mobility Enhancements Ph4

Document for:	Discussion/Decision

# Introduction

The Rel-19 WID on NR mobility enhancements Phase 4 contains the following objectives, currently:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we discuss necessary WID updates based on progress in the WGs.

# Discussion

## Objective on event triggered L1 measurement reporting

For the objective on event triggered L1 measurement reporting, RAN2 have progressed LTM independently from RAN1 for the MIMO WI. Specifically, due to different use-cases and requirements for mobility vs. beam management, different reporting procedures have been agreed.

While for beam management, RAN1 have agreed to 2 different and new UCI based reporting procedures, RAN2 agreed to define a new MAC CE and re-use the existing reporting procedures associated with MAC CE transmission. Additionally, RAN2 agreed that the measurement event evaluation is performed in the MAC layer. As such, there is no overlap to manage or avoid since MAC is fully in scope of RAN2, while UCI is fully in scope of RAN1.

There may still need to be some technical discussion on whether and how these features co-exist (E.g. is there any impact if both features are configured simultaneously). This can happen as part of “business as usual” towards the end of the release. For now, the features can continue to be developed independently – for LTM, RAN2 can continue defining a MAC based approach, and for beam management, RAN1 can continue defining UCI based reporting. No further narrowing or clarification of the objective is therefore necessary.

Proposal 1: Remove the sub-bullet in the WID, related to the objective on event triggered L1 measurement reporting, “RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work”

## Objective on support for CSI-RS measurements

The objective on support for CSI-RS measurements includes enabling CSI-RS based beam management and/or other necessary physical layer operations on candidate cells before LTM.

In RAN1#118, initial discussions took place on this objective. The feature lead summary [6] includes a list of items proposed in submitted contributions that can potentially be classified as “necessary physical layer operations”. It was recommended to update the WID to clarify explicitly the necessary operations to be supported.

- Item 1: CSI acquisition for candidate cell be before cell switch - Item 2: Dynamic update of measurement RS or candidate cells to perform L1-measurement - Item 3: Enhancement on TRS in candidate TCI states to enable faster tracking - Item 4: UL-based measurement - Item 5: Early DL beam management - Item 6: Retention of activated candidate TCI states after cell switch - Item 7: TA acquisition based on CSI-RS - Item 8: Autonomous TCI state activation by event triggered report

From the above, many companies identified Item 1 as necessary to prevent throughput degradation just after cell switch due to temporary unavailability of CSI report in target cell. This item is a good candidate to be explicity mentioned in the WID objective.

Item 2 is proposed based on an observation that the number of CSI-RS resources for candidate cells to configure for an LTM area can be very high (i.e. much larger than the number of SSB resources in R18 LTM). This means that the feature could require very high UE measurement capability or possibly letting UE implementation prioritize CSI-RS resources to measure on. In the first case, the cost of implementing the feature could be increased while in the second case, the network cannot be sure that the prioritization is done properly. For example, a UE that would rely on quality of SSB configured as QCL source to the CSI-RS does not know the relative power between SSB and CSI-RS. Activation/de-activation of CSI-RS resources for candidate cells solves these problems by allowing the network to explicitly indicate a subset of resources to measure. The network can decide on the subset based on earlier LTM report (e.g. based on SSB). As such, Item 2 is not a separate “other” physical layer operation but rather a potentially essential enabler for the main objective on CSI-RS measurements. Discussions and analysis on the need for this functionality should thus be allowed in RAN1 without explicit mention in the objective.

On other items:

- Item 3 is a physical layer operation on candidate cell before LTM. However, the necessity is unclear.

- Items 4, 6, 7, 8 seem not directly related to enabling CSI-RS measurements. Item 8 could potentially be discussed as part of event-triggered L1 reporting or conditional LTM.

- Item 5 may already be captured in current objective wording.

The following is thus proposed:

Proposal 2: RAN1 can discuss and specify (if found necessary) “Dynamic update of measurement RS or candidate cells to perform L1-measurement” without additional WID clarification.

Proposal 3: Replace WID objective “Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]” with “Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and CSI acquisition for candidate cell(s), before LTM [RAN1]”.

## Objective on support of conditional LTM

Currently, a checkpoint is stated in the WID for the objective on conditional LTM “checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint”.

RAN2 have progressed work on L1 based measurement event evaluation and reporting, to a point where it is relatively clear how this will be performed. For conditional LTM, it still needs to be discussed, for example, whether to re-use existing (L3) events or the new (MAC based) events, and whether beam level and/or cell level measurements should be used.

It additionally needs to be discussed what aspects of LTM can be triggered based on a condition. For example, in case of LTM measurement events it was agreed to consider event triggered reporting for support of the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

Similarly, it could be expected that a conditional event may be used for supporting both of the above purposes.

However, as all of these points require technical discussions that will need to take place in RAN2, it is suggested that the objective can simply be updated to remove the checkpoint, without narrowing or clarifying of the scope at this point in time.

Proposal 4: Remove the bullet in the WID related to the objective on conditional LTM “Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint”

# Summary

In this contribution, the following proposals are made:

Proposal 1: Remove the sub-bullet in the WID, related to the objective on event triggered L1 measurement reporting, “RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work”

Proposal 2: RAN1 can discuss and specify (if found necessary) “Dynamic update of measurement RS or candidate cells to perform L1-measurement” without additional WID clarification.

Proposal 3: Replace WID objective “Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]” with “Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and CSI acquisition for candidate cell(s), before LTM [RAN1]”.

Proposal 4: Remove the bullet in the WID, related to the objective on conditional LTM, “Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint”

The following changes would be needed to the WID

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and CSI acquisition for candidate cell(s), before LTM [RAN1] - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Specify RRM requirements related to the above objectives as necessary [RAN4]

# References

RP-241515, Revised Work Item: NR mobility enhancements Phase 4, June 2024

Chair notes, RAN1#118, Aug 2024

Chair notes, RAN2#127, Aug 2024

Chair notes, RAN2#126, May 2024

Chair notes, RAN2#125bis, Apr 2024

R1-2407319, “FL summary 2 of Measurements related enhancements for LTM”, Moderator (Fujitsu).

# Appendix: Agreeements

RAN1#118, Aug 2024 [2] Agreement - Support L1-RSRP measurement based on CSI-RS - FFS: Support L1-SINR measurement based on CSI-RS Agreement - Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported Agreement - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting Agreement - SSB based L1-RSRP measurements is supported for event triggered reporting - CSI-RS based L1-RSRP measurements is supported for event triggered reporting - FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting Agreement For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - FFS: CSI-RS for mobility Agreement - For the identification of the serving cell RS for event evaluation, - At least the following options are further studied in RAN1, where different options could apply to different LTM event - Option. 1: Derived from QCL (type-D) RS(s) of the indicated joint/DL TCI state for the serving cell - Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS or SSB is configured by the network - Option. 3: Measurement RS(s) is/are explicitly configured - Option. 4: Derived from QCL RSs of activated TCI states with the best quality, or SSB which is QCLed with the QCL RSs of activated TCI states with the best quality. - Option 6: Derived from QCL RSs of activated TCI states, or SSB which is QCLed with the QCL RSs of activated TCI states - The RSs of the candidate cell(s) for event evaluation are explicitly configure - Note: Companies are encouraged to take into account the RAN2 agreement (i.e current beam rather than best beam) for their further study. RAN2#127, Aug 2024 [3] Agreements on inter-CU LTM Reference configuration: 1. Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported. CSI resource and report configuration: 2. The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration. 3. The source CU is responsible to generate the common CSI resource configuration. Early DL sync: 4. For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused. Inter-CU LTM switch: 5. For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell. RLC and PDCP re-establishment in inter-CU LTM: 6. If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it. Handling of candidate configuration after inter-CU LTM cell switch: 7. As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network. DRB/PDU session mismatch in subsequent inter-CU LTM: 8. RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421. LTM and L3 HO: 9. L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM): - When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network. - The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations. - For the execution order between CHO and LTM, Rel-18 principle is applied. Inter-CU SCG LTM: 10. Inter-CU SCG LTM preparation can be initiated by source SN. 11. The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. 12. For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. 13. RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. 14. Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. Inter-CU MCG LTM: 15. SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). 16. Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). 17. For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure. Agreements on L1 MR event evaluation 1. Event LTM1 is not defined. 2. Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. 3. Any beam in candidate RS configuration can be used for LTM event evaluation. 4. Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. 5. R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. 6. MAC layer handles the event evaluation and measurement report triggering. Agreements on L1 measurement reporting - Event-triggered L1-measurements are reported by the UE to the network via MAC CE. RAN2#126, May 2024 [4] Agreements on inter-CU LTM: Clarification of DC 1. An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells. Xn-based and N2-based inter-CU LTM 2. Xn-based inter-CU LTM is prioritized in Rel-19. Stage-2 signaling flows and procedures 3. The preparation of inter-CU LTM configuration is initiated by the source gNB-CU. 4. For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM. 5. The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells. 6. In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed). Preparation: RRC Configuration/structure 7. The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM. LTM Candidate ID 8. For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs. Max number of LTM candidate IDs 9. The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations. Early sync: RAR based option 10. RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM. Execution: LTM Cell Switch Command 11. R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM. LTM Cell switch completion 12. Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM. 13. The LTM completion defined for intra-CU LTM is followed for R19 LTM. Agreements on measurement enhancements for LTM: 1. Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. 2. For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. 3. Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. 4. Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. 5. RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. 6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met. RAN2#125bis, Apr 2024 [5] Agreements on scenarios: 1. RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios. 2. Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch. 3. UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells. Agreements on latency analysis: - Mobility latency analysis of rel-18 intra-CU LTM is reused for Rel-19 inter-CU LTM. Agreements on early sync phase: 1. Early DL and UL sync is also supported for inter-CU LTM. Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval. 2. PDCCH ordered early RACH is supported for inter-CU LTM. 3. For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option. Agreements on LTM cell switch execution phase: 1. Upon inter-CU LTM execution, UE performs - MAC reset - RLC re-establishment - PDCP re-establishment - Security key update FFS if there is an inter-CU LTM w/o security key change. Agreements on measurements: 1. L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.