3GPP TSG RAN TSG Meeting #105 	RP-241909

Melbourne, Australia, September 9-12th, 2024

Title:	Analysis of DL synchronization with loT-NTN under Half Duplex mode at network level

Source: 	Thales, Iridium

Type:	Discussion

Document for:	Discussion

Agenda Item:	10.1.2

# Introduction

This paper provides a preliminary assessment of the DL synchronisation performance when the IoT-NTN (NB-IoT) based Satellite Access Node operates in a half-duplex mode.

# Discussion

## Synchronisation assessment

Recall of hypothesis

- Reference scenario: LEO set-1 (Satellite Tx power)

- Half Duplex operation mode at network level: Respectively 1 Tx and Rx opportunity within 9 radio frames

- Power class 3 user equipment (23 dBm Tx power, 0 dBi antenna gain)

We consider a stand-alone deployment.

The performance of NPSS detection is assessed by Monte-Carlo simulations. The DL transmission is configured with successive 90 ms periods. Each 90 ms period is made up of one active radio frame and 80 ms with no signal.

AWGN is added on top of the DL transmission.

The Doppler effect is also simulated. Because simulations are performed in baseband, the Doppler impact is applied both in the frequency domain to represent the carrier frequency offset and in the time domain to represent the timing drift effect.

A set of simulations is performed without Doppler effect. Another set of simulations is performed to simulate the initial search scenario with a carrier offset of 68 kHz, a Timing drift of -34ppm (24 ppm due to Doppler and 10 ppm due to UE XO). Those values are consistent for a 2 GHz carrier.

The NPSS is detected using a two-step method: an auto-correlation at a sampling rate of 240 kHz, followed by a cross-correlation at a sampling rate of 1.92 MHz, similarly to what is proposed in [1].

Two types of NPSS detection are simulated:

- Single shot detection. The auto-correlation peak over a single 90 ms period is selected. Its timing is then compared to the actual NPSS timing.

- Detection by recombining 4 NPSS subframes. After combining auto-correlation metrics of the latest 4 * 90 ms period, the auto-correlation peak is selected. Its timing is then compared to the actual NPSS timing.

The coarse synchronization at 240 kHz is considered successful if the NPSS is detected within +/- 5 samples, i.e., +/- 21 µs, of the actual NPSS timing.

Figure 1: Coarse NPSS timing detection by auto-correlation with a 240 kHz sampling rate

For a 1% miss detection rate of the NPSS, the min required SNR is respectively

- -1.3 dB for single NPSS received

- -5.4 dB for 4 NPSS/NSSS received during a 4x9 radio frames period

When the success criterion is met, i.e., timing error within +/- 5 samples at 240 kHz, the frequency estimation error is low enough to enable NPSS detection confirmation by cross-correlation at 1.92 MHz. For instance, the standard deviation of the frequency estimation error is lower than 150 Hz at a min SNR of -7 dB in the case of the detection by recombining 4 NPSS subframes. NPSS detection confirmation and NPSS fine timing detection are performed by cross-correlation at 1.92 MHz using a few frequency hypotheses to resolve the 14 kHz ambiguity of the auto-correlation method proposed in [1].

## Link Budget analysis

For the link budget analysis, the following parameters are considered:

- Payload characteristics for DL and UL transmissions as given in section 6.2 of [2]

- The same UE characteristics adopted in 3GPP TR 36.763 [2]: UE PC3 with antenna gain of 0dBi and NF of 7dB

- A channel Bandwidth of 180kHz in DL

- The following losses as per the TR 36.763 [2]:

| Scintillation losses | 2,2 |
| --- | --- |
| Atmospheric losses | 0,1 |
| Polarization loss | 3 |
| Shadow margin | 3 |

The following satellite parameters sets are used for link budget analysis:

| Satellite orbit | LEO-600 | LEO-600 | LEO-600 | LEO-600 | LEO-1200 | LEO-1200 | LEO-1200 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite parameter set | Set 1 | Set 2 | Set 3 | Set 4 | Set 1 | Set 2 | Set 3 |
| Central beam center elevation (deg) | 30 | 30 | 43.78 | 90 | 30 | 30 | 46.05 |
| Central Beam edge elevation (deg) | 27 | 23.8 | 27 | 30 | 26.3 | 22.2 | 30 |
| Frequency Band | S-band | S-band | S-band | S-band | S-band | S-band | S-band |

By considering the above parameters, an effective link budget analysis is performed to assess the overall performance of NPSS detection and feasibility of operating a SAN payload with a Half-Duplex at network level.

The summary of evaluation results and coverage margin analysis are provided within the following tables:

Table 1 Link budget for different satellite parameters sets

| Satellite parameter set | LEO600-Set1 | LEO600-Set2 | LEO600-Set3 | LEO600-Set4 | LEO1200-Set1 | LEO1200-Set2 | LEO1200-Set3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL | DL | DL | DL | DL | DL | DL |
| Frequency [GHz] | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 |
| TX: DL EIRP [dBm/MHz] | 64.00 | 58.00 | 58.30 | 51.45 | 70.00 | 64.00 | 63.70 |
| RX: G/T [dB/K] | -31.62 | -31.62 | -31.62 | -31.62 | -31.62 | -31.62 | -31.62 |
| Free space path loss [dB] | 159.69 | 160.40 | 159.08 | 159.08 | 165.09 | 165.83 | 164.47 |
| Atmospheric loss [dB] | 0.10 | 0.10 | 0.10 | 0.10 | 0.10 | 0.10 | 0.10 |
| Shadow fading margin [dB] | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Scintillation Loss [dB] | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 |
| Polarization loss [dB] | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Additional losses [dB] | 0.00 | 0.00 | 0.00 | 3.00 | 0.00 | 0.00 | 0.00 |
| CNR [dB]-180 kHz | 2.99 | -3.72 | -2.10 | -11.95 | 3.59 | -3.15 | -2.09 |

Table 2 Coverage margin for NPSS (Single detection)

|  | NPSS Coverage margin (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite parameter set | LEO600-Set1 | LEO600-Set2 | LEO600-Set3 | LEO600-Set4 | LEO1200-Set1 | LEO1200-Set2 | LEO1200-Set3 |
| Coverage margin [dB] | 4.29 | -2.42 | -0.8 | -10.65 | 4.89 | -1.85 | -0.79 |

Table 3 Coverage margin for NPSS (4 NPSS/NSSS received during a 4x9 radio frames period)

|  | NPSS Coverage margin (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite parameter set | LEO600-Set1 | LEO600-Set2 | LEO600-Set3 | LEO600-Set4 | LEO1200-Set1 | LEO1200-Set2 | LEO1200-Set3 |
| Coverage margin [dB] | 8.39 | 1.68 | 3.3 | -6.55 | 8.99 | 2.25 | 3.31 |

In the light of the above, we made the following observation:

Observation 1: When operating in Half Duplex mode at network level, successful 99% NPSS detection can be achieved with an adequate coverage margin for at least satellite parameter set1 and the LEO600 and LEO1200 orbits considered in 3GPP TR 36.763.

# Conclusion

In this contribution, we have examined the performance of NPSS detection in NTN NB-IoT with Half-Duplex mode of operation at network level. Our observation is listed below:

Observation 1: When operating in Half Duplex mode at network level, successful 99% NPSS detection can be achieved with an adequate coverage margin for at least satellite parameter set1 and the LEO600 and LEO1200 orbits considered in 3GPP TR 36.763.

# References

[1]	R1-161981, “NB-PSS and NB-SSS Design (Revised)”, Qualcomm Incorporated, 3GPP TSG RAN WG1 NB-IoT Ad-Hoc Meeting, Sophia Antipolis, France, March 22-24, 2016.

[2] 	3GPP TR 36.763, Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN) (Release 17)