3GPP TSG-RAN Meeting #105	RP-241911

# Melbourne, Australia,  September 9-12, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.843 | CR | 001 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TR 38.843 capturing agreements from study objectives |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm, Ericsson, CAICT |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_AIML_Air |  |  |  |  |  | Date: |  |  | 2024-09-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Capturing in TR 38.843 the endorsed RAN1 and RAN2 agreements for the study objectives in NR_AIML_Air WI which are relevant for the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN1 endorsed agreements for CSI prediction and Model identification. RAN2 endorsed agreements for UE data collection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not up to date Technical Report incorporating latest WG endorsed agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 4.2.3, 4.3, 6.2.1, 6.2.2, 6.2.2.6A, 6.2.2.7A, 6.2.2.8A, 7.1.2, 7.2.1.3, 7.2.1.3.2, 8, Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Three additional spreadsheets are attached with tables containing more CSI prediction evaluations. These are new spreadsheets which need to be added to the zip file with the other CSI evaluations results, i.e., “CSI_Evaluations_spreadsheets.zip". |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.2.1	LCM Flavours

The LCM procedure is studied for the case that an AI/ML model has a model ID with associated information and/or for the case that a given functionality is provided by some AI/ML operations. Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

From RAN1 perspective, an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation. When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model.

For UE-side models and UE-part of two-sided models:

-	For AI/ML functionality identification

-	Legacy 3GPP framework of feature is taken as a starting point.

-	UE indicates supported functionalities/functionality for a given sub-use-case.

-	UE capability reporting is taken as starting point.

-	For AI/ML model identification

-	Models are identified by model ID at the Network. UE indicates supported AI/ML models.

RAN1 studies the model identification type A with more details related to use cases. RAN1 studies the following options as starting point for model identification type B with more details related to all use cases.

- MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s)

- MI-Option 2: Model identification with dataset transfer

- MI-Option 3: Model identification in model transfer from NW to UE

- FFS: The boundary of the options

- Note: the names (MI-Opton1, MI-Option 2, MI-Option 3) are used only for discussion purpose

Additionally, the following options are proposed for model identification type B by some companies:

- MI-Option 4: Model identification via standardization of reference models (for CSI compression) . MI-Option 4 refers to the inter-vendor collaboration Option 1 of AI-based CSI compression

- MI-Option 5: Model identification via model monitoring

Regarding MI-Option 1 (Model identification with data collection related configuration(s) and/or indication(s)) of model identification type B, RAN1 further study the following aspects:

- Relationship between model ID and data collection related configuration(s) and/or indication(s)

- Information transmitted from NW to UE (if any)

- Information transmitted from UE to NW (if any)

- The associated procedure

- Usage/Applicable use case(s) of MI-Option 1

- Note: whether MI-Option 1 is needed or not is a separate discussion.

From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity)

- A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s)

  - Associated IDs for each sub use case in relation with NW-sided additional conditions

- B: UE(s) collects the data corresponding to the associated ID(s)

- C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s).

- D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model

  - relationship between model ID(s) and the associated ID(s)

  - How model ID(s) is determined/assigned, e.g.,

    - Alt.1: NW assigns Model ID

    - Alt.2: UE assigns/reports Model ID

    - Alt.3: Associated ID(s) is assumed as model ID(s)

      - “Model ID is determined/assigned for each AI/ML model” in D is not needed

    - Alt.4: Model ID is determined by pre-defined rule(s) in the specification

  - FFS: how to report

  - Note: D is to facilitate AI/ML model inference

- Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity)

- AI-Example2-1

  - A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling.

    - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope.

  - B: UE part of two-sided model(s) is(are) developed based on at least the above dataset.

  - C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW.

  - FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID)

  - Note: Some step(s) may not be needed for MI-Option2

- Note: The above example is based on the assumption of NW-first training. It is separate discussion for the assumption of UE-first training.

- Note: The study should consider the impact on inter-vendor collaboration, at least including complexity, performance, interoperability in RAN4/testing related aspects and feasibility.

- FFS: whether/how to consider UE-side additional condition(s) for the dataset

In functionality-based LCM, network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI). Models 	may not be identified at the Network, and UE may perform model-level LCM. Whether and how much awareness/interaction NW should have about model-level LCM requires further study. For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature, whereby AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

After functionality identification, necessity, mechanisms, for UE to report updates on applicable functionality(es) among functionality(es) are studied, where the applicable functionalities may be a subset of all functionalities. Applicable functionalities can be reported by the UE.

In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

After model identification, necessity, mechanisms, for UE to report updates on applicable UE part/UE-side model(s), are studied, where the applicable models may be a subset of all identified models. Applicable models can be reported by the UE.

How to handle the impact of UE’s internal conditions such as memory, battery, and other hardware limitations on functionality/model operations and AI/ML-enabled Feature is to be studied.  Note: it does not preclude any existing solutions.

For functionality/model-ID based LCM, once functionalities/models are identified, the same or similar procedures may be used for their activation, deactivation, switching, fallback, and monitoring.

Model ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations.

### 4.2.3	Additional conditions

For an AI/ML-enabled feature/FG, additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. It does not imply that additional conditions are necessarily specified. Additional conditions can be divided into two categories: NW-side additional conditions and UE-side additional conditions. Note: whether specification impact is needed is a separate discussion.

For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary):

-	Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side

-	Model training at NW and transfer to UE, where the model has been trained under the additional condition

-	Information and/or indication on NW-side additional conditions is provided to UE

-	Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality)

-	Other approaches are not precluded

-	Note: 	the possibility that different approaches can achieve the same function is not denied

Regarding the associated ID [defined in Section 4.2.1] for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell.

- FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study).

## 4.3	Collaboration levels

In this clause, various levels of collaboration between UE and Network are identified as found pertinent to the selected use cases, e.g.,

-	No collaboration: implementation-based only AI/ML algorithms without information exchange for comparison purposes

-	Various levels of UE/Network collaboration targeting at separate or joint ML operation

The following Network-UE collaboration levels are considered as one aspect for defining collaboration levels

1.	Level x: No collaboration.

2.	Level y: Signalling-based collaboration without model transfer. Note: this level includes cases without model delivery.

3.	Level z: Signalling-based collaboration with model transfer.

Level x/y boundary is understood such as Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

Level y/z boundary is defined based on whether model delivery over the air interface is done in a non-transparent manner to 3GPP signalling. Note: procedures other than model transfer/delivery are decoupled with collaboration Level y-z.

Table 4.3-1 introduces different options for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models:

Table 4.3-1: Model delivery/transfer cases

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |
| Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. |  |  |  |

When a model of a known structure at UE (e.g., Case z4) is transferred from the Network, the new model being identified (e.g., via Type B2) has the same structure as a previously identified model at the Network and UE.

For model delivery/transfer to UE (for UE-side models and UE-part of two-sided models):

-	Model delivery/transfer to UE, if feasible, may be beneficial to handle scenario/configuration specific (including site-specific configuration/channel conditions) models (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites), to reduce the device storage requirement.

-	Model delivery/transfer to UE after offline compiling and/or testing may be friendlier from UE’s implementation point of view compared to the case without offline compiling and/or testing. On the other hand, the case without offline compiling and/or testing (that can update parameter with known model structure), may have benefit at least in terms of shorter model parameter update timescale.

-	Model transfer/delivery of an unknown structure at UE has more challenges related to feasibility (e.g. UE implementation feasibility) compared to delivery/transfer of a known structure at UE.

-	For model trained at network side, Case y (w/ NW-side training) and Case z2 may incur the burden of offline cross-vendor collaboration such as sending a model to the UE-side and/or compiling a model.

-	For model trained at UE side/neutral site, Case z1 and Case z3 may incur the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the network, compared to Case y (w/ UE-side training) which does not have such burden.

-	Model storage at the 3GPP network, compared to storing the model outside the 3GPP network, may come with 3GPP network side burden on model maintenance/storage.

-	Proprietary design disclosure concern may arise from model training and/or model storage at the network side compared to other cases (such as case y with UE side training) which does not have such issue.

From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits):

- Alt. A

  - Step A-1: UE reports the supported known model structure(s) to network

  - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1

  - FFS: whether some additional step(s), and/or whether other information is needed

- Alt. B

  - Step B-0: UE reports to NW its support of model transfer/delivery case z4

  - Note: Step B-0 may be before or after Step B-1, or not necessary

  - Step B-1: NW indicates to UE the candidate known model structure(s)

  - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported

  - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2

  - FFS: whether some additional step(s), and/or whether other information is needed

- Note: Other alternative(s) is not precluded

- Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion.

From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects

- Model type/backbone (e.g., Transformer, CNN and so on)

- In case model type is a neural network

  - Number of layers

  - Layer types/structure (e.g., full connected, activation layer and so on)

  - Layer size (e.g., the number of parameters of a layer)

  - Connection between different layers

- model input/output related information

## 6.2	CSI feedback enhancement

### 6.2.1	Evaluation assumptions, methodology and KPIs

For the performance evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline. Link level simulations are optionally adopted.

For calibration purposes on the dataset and/or AI/ML model across companies, companies were encouraged to align the parameters (e.g., for scenarios/channels) for generating the dataset in the simulation as a starting point.

Performing intermediate evaluations on AI/ML model performance can be considered to derive the intermediate KPI(s) (e.g., accuracy of AI/ML output CSI) for the purpose of AI/ML solution comparison. If realistic DL channel estimation is considered, CSI accuracy is calculated using the target CSI from ideal channel and the output CSI from the realistic channel estimation. The target CSI from ideal channel equally applies to AI/ML based CSI feedback enhancement, and the baseline codebook.

KPIs and Evaluation metrics:

-	Capability/complexity: Floating point operations (FLOPs), AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters reported by companies who may select either or both

-	Reported separately for the CSI generation part and the CSI reconstruction part (for CSI compression sub-use case)

-	When reporting the computational complexity including the pre-processing and post-processing, the complexity metric of FLOPs may be reported separately for the AI/ML model and the pre/post processing. While reporting the FLOPs of pre-processing and post-processing the following boundaries are considered:

-	Estimated raw channel matrix per each frequency unit as an input for pre-processing of the CSI generation part.

-	Precoding vectors per each frequency unit as an output of post-processing of the CSI reconstruction part.

-	Additionally, reported for non-AI/ML based CSI prediction including additional complexity if applicable, e.g., update of filter, assuming whole bandwidth and one prediction sample (for CSI prediction sub-use case)

-	CSI compression: Intermediate KPIs: SGCS and/or NMSE to evaluate the accuracy of the AI/ML output CSI

-	For rank>1 cases, SGCS calculation/extension methods are to be reported:

-	SGCS separately calculated for each layer (e.g., for K layers, K SGCS values are derived respectively, and comparison is performed per layer). Companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors. Note: Eventual KPI can still be used to compare the performance.

-	The granularity of the frequency unit for averaging operation is assumed to be:

-	For 15kHz SCS: For 10MHz bandwidth: 4 RBs; for 20MHz bandwidth: 8 RBs

-	For 30kHz SCS: For 10MHz bandwidth: 2 RBs; for 20MHz bandwidth: 4 RBs

-	Other frequency unit granularities not precluded.

-	CSI compression: Intermediate KPI: model monitoring mechanism is considered as:

-	Step 1: Generate test dataset including K test samples.

-	Step 2: For each of the K test samples, a bias factor of monitored intermediate KPI (KPIDiff) is calculated as a function of KPIDiff = f ( KPIActual , KPIGenie ), where KPIActual is the actual intermediate KPI, and KPIGenie is the genie-aided intermediate KPI.

-	KPIDiff is considered for:

-	Case 1: NW side monitoring of intermediate KPI, where the monitoring accuracy is evaluated for a given ground-truth CSI format (e.g., quantized ground-truth CSI with 8 bits scalar, R16 eType II-like method, etc.) or SRS measurements, where

-	KPIActual is calculated with the output CSI at the NW side and the given ground-truth CSI format or SRS measurements.

-	KPIGenie is calculated with output CSI (as for KPIActual) and the ground-truth CSI of Float32

-	Note: if Float32 is used for KPIActual, the monitoring accuracy is 100% if KPIActual and KPIGenie are based on the same CSI sample.

-	Case 2: UE side monitoring of intermediate KPI with a proxy model, where the monitoring accuracy is evaluated for the output of the proxy model at UE:

-	Case 2-1: the proxy model is a proxy CSI reconstruction part, and KPIActual is calculated based on the inference output of the proxy CSI reconstruction part at UE and the ground-truth CSI. Note: if the proxy CSI reconstruction model is the same as the actual CSI reconstruction model at the NW, the monitoring accuracy is 100%.

-	Case 2-2: the proxy model directly outputs intermediate KPI (KPIActual)

-	KPIGenie is calculated with the output CSI at the NW side and the same ground-truth CSI.

-	KPIDiff = f ( KPIActual , KPIGenie ) can take the following forms:

-	Option 1 (baseline for calibration): Gap between KPIActual and KPIGenie, i.e. KPIDiff = (KPIActual - KPIGenie); Monitoring accuracy is the percentage of samples for which | KPIDiff| < KPIth 1, where KPIth 1 is a threshold of the intermediate KPI gap which can take the following values: 0.02, 0.05 and 0.1.

-	Option 2 (optional and up to companies to report): Binary state where KPIActual and KPIGenie, have different relationships to their threshold(s), i.e., KPIDiff = (KPIActual > KPIth 2, KPIGenie < KPIth 3) OR (KPIActual < KPIth 2, KPIGenie > KPIth 3), where KPIth 2 is considered to be the same as KPIth 3. Monitoring accuracy is the percentage of samples for which KPIDiff = 0.

-	Step 3: Calculate the statistical result of the KPIDiff over K test samples which represents the monitoring accuracy performance.

-	Note: KPIGenie is introduced for the evaluation and comparison purpose; it may not be available in the real network.

-	Note: the complexity, overhead and latency of the monitoring scheme are to be reported.

-	CSI prediction: Intermediate KPIs: calculated for each predicted instance if AI/ML model outputs multiple predicted instances

-	If collaboration level x is reported as the benchmark, the EVM to distinguish level x and level y/z based AI/ML CSI prediction is considered from the generalization aspect, e.g., collaboration level y/z based CSI prediction is modelled as the fine-tuning case or generalization Case 1, while collaboration level x based CSI prediction is modelled as generalization Case 2 or Case 3.

-	Throughput including: average UPT, 5%-ile UE throughput, and CDF of UPT

Model generalization:

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations:

-	Case 1: The AI/ML model is trained based on training dataset from one Scenario#B/Configuration#B, and then the AI/ML model performs inference/test on a dataset from the same Scenario#B/Configuration#B

-	Case 2: The AI/ML model is trained based on training dataset from a different data set than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on Scenario#B/Configuration#B.

-	Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#B/Configuration#B and a different dataset than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from Scenario#B/Configuration#B.

-	Note: Companies to report the ratio for dataset mixing

-	Note: number of the multiple scenarios/configurations can be larger than two

To verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects:

-	Various deployment scenarios (e.g., UMa, UMi, InH)

-	Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10)

-	Various carrier frequencies (e.g., 2GHz, 3.5GHz)

-	Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc.

-	Companies to report the selected scenarios for generalization verification

To verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects:

-	Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband)

-	Various sizes of CSI feedback payloads

-	Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

-	Various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h, etc.) for CSI prediction sub use case

-	Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc.

-	The selected configurations for generalization verification

-	The method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc

For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input/output dimensions, companies to report which case(s) are evaluated from the following list:

-	Case 0 (benchmark for comparison): One CSI generation part with fixed input and output dimensions to 1 CSI reconstruction part with fixed input and output dimensions for each of the different input and/or output dimensions.

-	Case 1: One CSI generation part with scalable input and/or output dimensions to N>1 separate CSI reconstruction parts each with fixed and different output and/or input dimensions

-	Case 2: M>1 separate CSI generation parts each with fixed and different input and/or output dimensions to one CSI reconstruction part with scalable output and/or input dimensions

-	Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions

For CSI compression, to achieve the scalability over different input dimensions of CSI generation part (e.g., different bandwidths/frequency granularities, or different antenna ports), the generalization cases are elaborated as follows:

-	Case 1: The AI/ML model is trained based on training dataset from a fixed dimension X1 (e.g., a fixed bandwidth/frequency granularity, and/or number of antenna ports), and then the AI/ML model performs inference/test on a dataset from the same dimension X1.

-	Case 2: The AI/ML model is trained based on training dataset from a single dimension X1, and then the AI/ML model performs inference/test on a dataset from a different dimension X2.

-	Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of X1, X2,..., Xn, and then the AI/ML model performs inference/test on a single dataset subject to the dimension of X1, or X2,…, or Xn.

-	Note: For Case 2/3, the solutions to achieve the scalability between Xi and Xj, are reported by companies, including, e.g., pre-processing to angle-delay domain, padding, additional adaptation layer in AI/ML model, etc.

For CSI compression, to achieve the scalability over different output dimensions of CSI generation part (e.g., different generated CSI feedback dimensions), the generalization cases of are elaborated as follows

-	Case 1: The AI/ML model is trained based on training dataset from a fixed output dimension Y1 (e.g., a fixed CSI feedback dimension), and then the AI/ML model performs inference/test on a dataset from the same output dimension Y1.

-	Case 2: The AI/ML model is trained based on training dataset from a single output dimension Y1, and then the AI/ML model performs inference/test on a dataset from a different output dimension Y2.

-	Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of Y1, Y2,..., Yn, and then the AI/ML model performs inference/test on a single dataset of Y1, or Y2,…, or Yn.

-	Notes: For Case 1/2/3, companies to report whether the output of the CSI generation part is before quantization or after quantization. For Case 2/3, the solutions to achieve the scalability between Yi and Yj, are reported by companies, including, e.g., truncation, additional adaptation layer in AI/ML model, etc.

Model Fine-tuning:

For the evaluation of the potential performance benefits of model fine-tuning of CSI feedback enhancement, which is optionally assessed, the following case is considered:

-	The AI/ML model is trained based on training dataset from a different dataset than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#A/Configuration#B, Scenario#B/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset Scenario#B/Configuration#B. After that, the AI/ML model is tested on Scenario#B/Configuration#B.

-	In this case, the fine-tuning dataset setting (e.g., size of dataset) is to be reported along with the improvement of performance.

Localized model:

For the evaluation of AI/ML-based CSI feedback enhancement using localized models, consider the following options to model the spatial correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

    - e.g., Dropped in a specific cell or within a specific boundary.

- Option 2: By using a scenario/configuration specific to the local region.

    - e.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

Note: While modelling the spatial correlation, strive to ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel. To report methods to generate training and testing dataset.

For the evaluation of AI/ML-based CSI feedback enhancement using localized models, regarding training,

- The k-th local model is trained on region #B_k (the k-th local region), 1<=k<=N.

- The generalized model is trained on Region #A that may be constructed via any of the following methods that is appropriate for the given generalized/local region modeling approach.

  - Region #A is the same as the union of regions #B_1, …, #B_N.

  - Region #A is a proper superset of the union of regions #B_1, …, #B_N.

  - Region #A is generated separately from regions #B_1, …, #B_N.

  - Note: companies to report which method was used.

For the evaluation of AI/ML-based CSI feedback enhancement using localized models, regarding testing,

- The trained generalized model, local model, and the non-AI/ML benchmark are tested on the regions #B_1, …, #B_N.

- In case N>1, when reporting the results, companies may report the performance of the generalized model, the local models, and the non-AI/ML benchmark, by averaging the performance over the regions #B_1,…,B_N. Companies to report the value of N.

Further details on evaluations including training collaboration types

For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), following procedure is considered as an example:

-	For each FP/BP loop,

-	Step 1: UE side generates the FP results (i.e., CSI feedback) based on the data sample(s), and sends the FP results to NW side

-	Step 2: NW side reconstructs the CSI based on FP results, trains the CSI reconstruction part, and generates the BP information (e.g., gradients), which are then sent to UE side

-	Step 3: UE side trains the CSI generation part based on the BP information from NW side

-	Note: the dataset between UE side and NW side is aligned.

-	Other Type 2 training approaches are not precluded and reported by companies

For the evaluations of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), the following evaluation cases are considered for multi-vendors,

-	Case 1 (baseline): Type 2 training between one NW part model to one UE part model

-	Case 2: Type 2 training between one NW part model and M>1 separate UE part models.

-	Companies to report the AI/ML structures for the UE part model and the NW part model

-	Case 3: Type 2 training between one UE part model and N>1 separate NW part models.

-	Companies to report the AI/ML structures for the UE part model and the NW part model

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training (NW-first training):

-	Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly

-	Step2: After NW side training is finished, NW side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part

-	Companies to report Dataset construction, e.g., the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only, or other information if applicable. Also report the Quantization behaviour, e.g., whether the shared output of the Network side CSI generation part is before or after quantization.

-	Step3: UE side trains the UE side CSI generation part based on the received set of information

-	Other Type 3 NW-first training approaches are not precluded

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with UE side training (UE-first training):

-	Step1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly

-	Step2: After UE side training is finished, UE side shares NW side with a set of information (e.g., dataset) that is used by the NW side to be able to train the CSI reconstruction part

-	Companies to report Dataset construction, e.g., the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only, or other information if applicable. Also, report the Quantization behaviour, e.g., whether the shared input of the UE side CSI reconstruction part is before or after quantization.

-	Step3: NW side trains the NW side CSI reconstruction part based on the received set of information

-	Other Type 3 UE-first training approaches are not precluded

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following evaluation cases for sequential training are considered for multi-vendors:

-	Case 1 (baseline): Type 3 training between one NW part model and one UE part model

-	Note 1: Case 1 can be naturally applied to the NW-first training case where 1 NW part model to M>1 separate UE part models

-	Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training UE part model is the same or a subset of the dataset for training NW part model

-	Note 2: Case 1 can be naturally applied to the UE-first training case where 1 UE part model to N>1 separate NW part models

-	Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training NW part model is the same or a subset of the dataset for training UE part model

-	Companies to report the AI/ML structures for the combination(s) of UE part model and NW part model, which can be the same or different

-	Case 2: For UE-first training, Type 3 training between one NW part model and M>1 separate UE part models

-	Note: Case 2 can be also applied to the M>1 UE part models to N>1 NW part models

-	Companies to report the AI/ML structures for the M>1 UE part models and the NW part model

-	Companies to report the dataset used at UE part models, e.g., same or different dataset(s) among M UE part models

-	Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models

-	Note: Case 3 can be also applied to the N>1 NW part models to M>1 UE part models

-	Companies to report the AI/ML structures for the UE part model and the N>1 NW part models

-	Companies to report the dataset used at NW part models, e.g., same or different dataset(s) among N NW part models

-	Case 4: 1-on-1 training with joint training: benchmark/upper bound for performance comparison.

For the evaluation of Type 3 (Separate training at NW side and UE side), the following cases are considered for evaluations:

-	Case 1 (baseline): Aligned AI/ML model structure between NW side and UE side

-	Case 2: Not aligned AI/ML model structures between NW side and UE side

-	Companies to report the AI/ML structures for the UE part model and the NW part model, e.g., different backbone (e.g., CNN, Transformer, etc.), or same backbone but different structure (e.g., number of layers)

-	For the evaluation of training Type 3 under CSI compression, for the benchmark case (1-on-1 joint training) for performance comparison, the structures for the pair of NW part model/UE part model for the new case are the same with the Type 3 case to be compared, e.g., if the Type 3 is Transformer#1 for NW part model and CNN#1 for UE part model, then the benchmark case for performance comparison is also Transformer#1 for NW part model and CNN#1 for UE part model with joint training.

Evaluation assumptions:

Table 6.2.1-1 presents the baseline system level simulation assumptions for AI/ML based CSI feedback enhancement evaluations.

Table 6.2.1-1: Baseline System Level Simulation assumptions for AI/ML based CSI feedback enhancement evaluations

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g., UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 2GHz as baseline, optional for 4GHz (if R16 as baseline) FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz (if R17 as baseline) |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz (if R16 as baseline) 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz (if R17 as baseline) |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g., 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | At least, FTP model 1 with packet size 0.5 Mbytes is assumed. Other options are not precluded |
| Traffic load (Resource utilization) |  | 20/50/70%. Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) CSI prediction: 100% outdoor (10, 20, 30, 60, 120 km/h) including outdoor-to-indoor car penetration loss per TR 38.901 if the simulation assumes UEs inside vehicles. No explicit trajectory modeling considered for evaluations. |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline. Up to companies to choose the error modelling method for realistic channel estimation. Error modelling in TR36.897 Table A.1-2 as a baseline if channel estimation error is modelled. Ideal DL channel estimation is optionally taken into the baseline of evaluation methodology for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Up to companies to report whether/how ideal channel is used in the dataset construction and performance evaluation/inference. Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation. |
| Phase discontinuity |  | A uniform distribution between -ϕmax,ϕmax within a time window of Twindow, where ϕmax=40 degrees and Twindow=20ms can be a baseline. - A fixed phase for all CSI-RS observations within the time window, and another fixed phase for the next time window. The phases are according to uniform distribution. |
| CSI-RS configuration |  | Periodic: 5 ms periodicity (baseline), 20 ms periodicity (encouraged) Aperiodic: Optional, CSI-RS burst with K resources and time interval m slots (based on R18 MIMO eType-II) |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. - For AI/ML based solutions: The above-mentioned "CSI feedback overhead" is calculated as max allowed bits at the given rank. - For legacy Type II CB: Option 2b is mandatorily reported by companies, while Option 2a can be optionally reported up to companies if partial NZC report is assumed for the legacy Type II CB - Option 2a: The above-mentioned "CSI feedback overhead" is calculated as each CSI reported payload with a given rank - Option 2b: The above-mentioned "CSI feedback overhead" is calculated as max allowed bits at the given rank Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | For CSI compression: Companies need to report which option is used between: - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. Additional assumptions from R17 TypeII EVM: Same consideration with respect to utilizing angle-delay reciprocity should be considered for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline. Optionally, Type I Codebook (if it outperforms Type II Codebook) can be considered for comparing AI/ML schemes. For CSI-prediction: Both of the following are taken as baseline: - The nearest historical CSI without prediction - Non-AI/ML or AI/ML with collaboration Level x based CSI prediction for which corresponding details would need to be reported Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example. Note: R18 eType II doppler codebook is assumed for CSI report for both AI/ML and Non AI/ML prediction. Companies can report the assumption for N4: 1,4 (baseline), 2,8 (optional), and the assumption for paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1). For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on CSI without compression (e.g., eigenvector from measured channel), which is taken as an upper bound for performance comparison. |
| Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations. |  |  |

Table 6.2.1-2 presents the baseline link level simulation assumptions for AI/ML based CSI feedback enhancement evaluations.

Table 6.2.1-2: Baseline Link Level Simulation assumptions for AI/ML based CSI feedback enhancement evaluations

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g., LS or MMSE) as a baseline. Ideal DL channel estimation is optionally taken into the baseline of evaluation methodology for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Up to companies to report whether/how ideal channel is used in the dataset construction and performance evaluation/inference. Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |
| Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations. |  |

CSI compression sub use case specific aspects:

For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information. At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

Figure 6.2.1-1 provides an example for the inference procedure for CSI compression. For generating the input of CSI generation model, it may need some further pre-processing on the measured channel; for the output of the CSI reconstruction model, some further post-processing may also be applied. Besides CSI feedback of quantization output, there may also be other CSI/PMI related information transmitted. There may be other examples of merging quantization/dequantization into the inference for CSI generation/reconstruction, CSI generation model/CSI reconstruction model, respectively.

Figure 6.2.1-1: An example of the CSI compression inference procedure

For the evaluation of the AI/ML based CSI compression sub use case, the following details of models are reported:

-	The structure of the AI/ML model, e.g., type (CNN, RNN, Transformer, Inception, …), the number of layers, branches, real valued or complex valued parameters, etc.

-	AI/ML model input (for CSI generation part)/output (for CSI reconstruction part) types for evaluations

-	Data pre-processing/post-processing

-	Loss function

-	Specific quantization/dequantization method, e.g., vector quantization, scalar quantization, etc,

For the evaluation of the AI/ML based CSI compression sub use cases, at least the following types of AI/ML model input (for CSI generation part)/output (for CSI reconstruction part) are considered for evaluations:

-	Raw channel matrix, e.g., channel matrix with the dimensions of Tx, Rx, and frequency unit. Companies to report the raw channel is in frequency domain or delay domain.

-	Precoding matrix. Companies to report the precoding matrix is a group of eigenvector(s) or an eType II-like reporting (i.e., eigenvectors with angular-delay domain representation).

For the evaluation of quantization aware/non-aware training, the following cases are considered and reported by companies:

-	Case 1: Quantization non-aware training, where the float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training

o	Fixed/pre-configured quantization method/parameters is applied for the inference phase. Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc

-	Case 2: Quantization-aware training, where quantization/dequantization is involved in the training process

o	Case 2-1: Fixed/pre-configured quantization method/parameters are applied during the training phase; the same quantization codebook is applied for the inference phase. Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc.

o	Case 2-2: The quantization method/parameters are updated in together with the AI/ML models during the training; when training is finished, the final quantization codebook is applied for the inference phase. Companies to report how to update the quantization method/parameters during the training

-	Quantization methods including uniform vs non-uniform quantization, scalar versus vector quantization, and associated parameters, e.g., quantization resolution, etc.

-	How to use the quantization methods are reported by companies

Considering performance impact of ground-truth quantization in the CSI compression, study high resolution quantization methods for ground-truth CSI, including at least the following options:

-	High resolution scalar quantization

-	High resolution codebook quantization, e.g., Rel-16 TypeII-like method with new parameters, in which case companies are to report the R16 Type II parameters with specified or new/larger values to achieve higher resolution of the ground-truth CSI labels, e.g., L, pv, β, reference amplitude, differential amplitude, phase, etc

-	Float32 adopted as the baseline/upper-bound for performance comparisons

-	Consider legacy values of PC6 & PC8 for performance comparison

For CSI compression sub use case with rank ≥ 1, AI/ML model setting to adapt to ranks/layers to be reported amongst the following options:

-	Option 1-1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference, any specific model operates on multi-layers jointly.

-	Option 1-2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference, the model operates on multi-layers jointly.

-	Option 2 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference.

o	Note: input/output type is Precoding matrix

o	Companies to report the setting is

	Option 2-1: layer specific and rank common (different models applied for different layers; for a specific layer, the same model is applied for all rank values), or

	Option 2-2: layer specific and rank specific (different models applied for different layers; for a specific layer, different models are applied for different rank values)

-	Option 3 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference.

o	Note: input/output type is Precoding matrix

o	Companies to report whether the setting is

	Option 3-1: layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference), or

	Option 3-2: layer common and rank specific (different models applied for different rank values; for a specific rank, the same model is applied for all layers)

For CSI compression sub use case with rank >1, for a given configured Max rank=K, the complexity of FLOPs is reported as the maximum FLOPs over all ranks each includes the summation of FLOPs for inference per layer if applicable, e.g.,

-	Option 1-1 (rank specific): Max FLOPs over K rank specific models.

-	Option 1-2 (rank common): FLOPs of the rank common model.

-	Option 2-1 (layer specific and rank common): Sum of the FLOPs of K models (for the rank=K).

-	Option 2-2 (layer specific and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with a sum of k models.

-	Option 3-1 (layer common and rank common): K * FLOPs of the common model.

-	Option 3-2 (layer common and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with k * FLOPs of the layer common model.

For CSI compression sub use case with rank >1, the storage of memory storage/number of parameters is reported as the summation of memory storage/number of parameters over all models potentially used for any layer/rank, e.g.,

-	Option 1-1 (rank specific)/Option 3-2 (layer common and rank specific): Sum of memory storage/number of parameters over all rank specific models.

-	Option 1-2 (rank common): A single memory storage/number of parameters for the rank common model.

-	Option 2-1 (layer specific and rank common): Sum of memory storage/number of parameters over all layer specific models.

-	Option 2-2 (layer specific and rank specific): Sum of memory storage/number of parameters for the specific models over all ranks and all layers in per rank.

-	Option 3-1 (layer common and rank common): A single memory storage/number of parameters for the common model

For the evaluation of CSI compression, the specific CQI determination method(s) for AI/ML can be reported by introducing an additional field in the template, e.g.,

-	Option 1a: CQI is calculated based on the target CSI from the realistic channel estimation.

-	Option 1b: CQI is calculated based on the target CSI from the realistic channel estimation and potential adjustment.

-	Option 1c: CQI is calculated based on traditional codebook.

-	Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment.

o	Option 2a-1: The CSI reconstruction part for CQI calculation at the UE same as the actual CSI reconstruction part at the NW.

o	Option 2a-2: The CSI reconstruction part for CQI calculation at the UE is a proxy model, which is different from the actual CSI reconstruction part at the NW.

-	Option 2b: CQI is calculated using two stage approach, UE derives CQI using precoded CSI-RS transmitted with a reconstructed precoder.

CSI prediction sub use case specific aspects:

Figure 6.2.1-2 provides an example for the inference procedure for CSI prediction. For generating the input of CSI prediction model, it may need some further pre-processing on the measured channel; for the output of the CSI prediction model, some further post-processing may also be applied.

Figure 6.2.1-2: An example of the CSI prediction inference procedure

For the evaluation of the AI/ML based CSI prediction sub use case, the following details of models are reported:

-	The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc.

-	The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc.

-	Including assumptions on the observation window, i.e., number/time distance of historic CSI/channel measurements

-	The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc.

-	Including assumptions on the prediction window, i.e., number/time distance of predicted CSI/channel

-	Data pre-processing/post-processing

-	Loss function

For the input CSI type, both of the following types are considered for evaluations:

-	Raw channel matrices

-	Eigenvector(s)

For SLS, spatial consistency Procedure A with 50m decorrelation distance from TR 38.901 is used (if not used, assumptions used need to be reported). UE velocity vector is assumed as fixed over time in Procedure A modelling.

### 6.2.2	Performance results

CSI_Table 1 through CSI_Table 7 in attached Spreadsheets for CSI feedback enhancement evaluations present the performance results for:

-	CSI_Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability

-	CSI_Table 2. Evaluation results for CSI compression with model generalization

-	CSI_Table 3. Evaluation results for CSI compression with model scalability

-	CSI_Table 4. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability

-	CSI_Table 5. Evaluation results for CSI compression of separate training without model generalization/scalability

-	CSI_Table 6. Evaluation results for CSI prediction without model generalization/scalability

- CSI_Table 6A. Evaluation results for CSI prediction without model generalization/scalability

-	CSI_Table 7. Evaluation results for CSI prediction with model generalization

- CSI_Table 7A. Evaluation results for CSI prediction with model generalization

- CSI_Table 8. Evaluation results for CSI prediction with localized model

For the evaluation of CSI compression of 1-on-1 joint training without model generalization/scalability, the following baselines are recommended to facilitate calibration of results:

-	Benchmark: R16 eType II CB;

-	Others can be additionally submitted, e.g., Type I CB.

-	Input/Output type: Eigenvectors of the current CSI

-	Other can be additionally submitted, e.g., eigenvectors with additional past CSI, eType II-like input, raw channel matrix, etc.

-	Ground-truth CSI quantization method: Float32, i.e., without quantization (baseline/upper-bound for performance comparison)

-	Other high resolution CSI quantization methods can be additionally submitted for comparison, e.g., R16 eType II-like method with new parameters, scalar quantization, etc.

-	Rank/layer adaptation settings for rank>1: Option 3-1, i.e., layer common and rank common.

-	Other rank>1 options can be additionally submitted for comparison, e.g., Option 1-1/1-2/2-1/2-2/3-2.

-	Quantization method: quantization-aware training (Case 2-1 or Case 2-2)

-	Quantization non-aware training can be additionally submitted for comparison

-	SQ and/or VQ is up to companies; companies are encouraged to provide results of various cases for comparison.

-	Performance metric for intermediate KPI: SGCS

-	NMSE can be additionally submitted

The CSI feedback reduction is provided for three CSI feedback overhead ranges (CSI feedback overhead A, CSI feedback overhead B, CSI feedback overhead C), where for each CSI feedback overhead range of the benchmark, it is calculated as the gap between the CSI feedback overhead of benchmark and the CSI feedback overhead of AI/ML corresponding to the same mean UPT. The various CSI feedback overhead ranges are defined as:

CSI feedback overhead A such that A ≤ β ˖80 bits

CSI feedback overhead B such that β ˖100 bits ≤ B ≤ β ˖140 bits

CSI feedback overhead C ≥ β ˖230 bits

where, β = 1 for rank = 1 and β = 1.5 for rank > 1

Note:	companies report the exact CSI feedback overhead considered.

Note:	the CSI feedback overhead reduction and gain for mean/5%tile UPT are determined at the same payload size for benchmark scheme.

Note:	"Benchmark" means the type of Legacy CB used for comparison. "Quantization/dequantization method" includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantization (SQ/VQ), etc. "Input type" means the input of the CSI generation part. "Output type" means the output of the CSI reconstruction part.

For the evaluation of CSI prediction without model generalization/scalability verification, the following baselines are recommended to facilitate calibration of results:

-	UE speed: 10km/h, 30km/h, 60km/h;

-	Others can be additionally submitted, e.g., 120km/h.

-	Input/Output type: Raw channel matrix

-	Other can be additionally submitted, e.g., eigenvectors.

-	Observation window (number/distance): 5/5ms, 10/5ms

-	Other observation window configurations can be additionally submitted for comparison, e.g., 3/5ms, 4/5ms, 8/2.5ms, 10/4ms, etc.

-	Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms

-	Other prediction window configurations can be additionally submitted for comparison, e.g., 3/5ms/5ms, 5/5ms/5ms, 4/2.5ms/2.5ms, 5/4ms/4ms, etc.

-	Performance metric for intermediate KPI: SGCS

-	NMSE can be additionally submitted.

-	Spatial consistency configuration (optional): procedure A with 50m decorrelation distance and channel updating periodicity of 1 ms.

For the evaluation of CSI prediction with model generalization/scalability verification, the following baselines are recommended to facilitate calibration of results:

-	Performance metric for intermediate KPI: SGCS

-	NMSE can be additionally submitted.

#### 6.2.2.6A	Basic performance for CSI prediction

The complexity values in terms of FLOPs and number of parameters of AI/ML models adopted in the evaluations of CSI prediction are summarized in Figure 6.2.2.6A-1.

- Results refer to Table 2 of clause 7.3, R1-2407341.

Figure 6.2.2.6A-1: Complexity of AI/ML models from evaluation results in terms of FLOPs 
and number of parameters for CSI prediction.

The complexity values in terms of FLOPs for AI/ML model and benchmark 2 of an auto-regression/Kalman filter adopted in the evaluations of CSI prediction are summarized in Figure 6.2.2.6A-2 and Figure 6.2.2.6A-3

- Results refer to Table 2-9, and Table 2-10 in R1-2407341

Figure 6.2.2.6A-2: Complexity of AI/ML models and benchmark 2 of an auto-regression/Kalman filter from evaluation results in terms of FLOPs for CSI prediction.

From a perspective of AI/ML complexity, 19 sources adopt the model subject to the computational complexity in units of FLOPs from 0.05M to 3000M. The actual model complexity may differ from the model complexity in the evaluation with respect to platform-dependent optimization on model implementations.

From a perspective of complexity of non-AI/ML benchmark, 16 sources adopt the algorithm (e.g., Kalman filter, Auto-regression, Wiener filter) subject to the computational complexity in units of FLOPs from 0.14M to 107M. For non-AI/ML benchmark, main computation complexity is dominated by filter updates, which may not need to be updated per inference at the expense of performance loss. For example, 7 sources adopt the algorithm subject to the computational complexity of filter updates and inference in units of FLOPs from 0.47M to 106M and 0.067M to 3M, respectively.

SGCS performance over benchmark 1 of the nearest historical CSI

For the CSI prediction using UE-sided model, compared to the Benchmark 1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective,

  - If spatial consistency is not adopted, and if N4=1

    - For 10km/h UE speed, 1 source observes 6% gain

    - For 30km/h UE speed, 9 sources observe 5.9%~20.6% gain and 4 sources observe 23.2%~35.4% gain. 2 sources observe 54%~106% gain

    - For 60km/h UE speed, 4 sources observe 10.5%~26.53% gain

  - If spatial consistency is adopted, and if N4=1

    - For 30km/h UE speed, 2 sources observe 22.93%~23% gain, 1 source observe 68.5% gain

    - For 60km/h UE speed, 3 sources observe 16.2%~24.3% gain

  - If spatial consistency is not adopted, and if N4=4

      - For 10km/h UE speed, 1 source observes 9.1%~29.7% gain

      - For 30km/h UE speed, 1 source observes 4.3%~12.3% gain, and 2 sources observe 17.6%~35.4%gain

  - If spatial consistency is adopted, and if N4=4

      - For 10km/h UE speed, 1 source observes -1.61%~62.9% gain

      - For 30km/h UE speed, 1 source observes 23%~34% gain, 1 source observe 20.9%~76.4% gain

      - For 60km/h UE speed, 2 sources observe 5.96%~-22% gain,

Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 20ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - 7 sources consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 1 source considers eigenvector as model input, and other sources considers Raw channel matrix as model input.

  - 1 source considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - 2 sources consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, 1 source considers per layer raw channel matrix after pre-processing, and other sources do not consider pre/post processing.

  - The performance metric is SGCS in linear value for layer 1.

Note: N4 refers to the number of predicted CSI instances

Note: Results refer to Table 2-1 of R1-2407338

SGCS performance over benchmark 2 of the nearest historical CSI

For the CSI prediction using UE-sided model, compared to the Benchmark 2 of non-AI based CSI prediction, in terms of SGCS, from channel estimation perspective

- If ideal channel estimation is adopted

  - For N4=1

    - 9 sources observe -1.2%~6.3% gain

    - 1 source observes -2% ~17% gain depending on filter complexity and filter update

  - For N4=4

    - 2 sources observe -1.6%~5.56%

    - 1 source observes -4.5%~6.93%

    - 1 source observes 3.1%~40.5%

- If realistic channel estimation is adopted,

  - For N4=1

    - 5 sources observe 0.43%~7.26% gain

    - 2 sources observe 15.2%~19.5 gain

    - 1 source observes 68% gain

    - 1 source observes 5.8%~16.4% gain depending on traffic load

  - For N4=4

    - 2 sources observe 0.1%~1.4% gain

    - 1 source observes 5%~29% gain

Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms.

- A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

- Raw channel matrix as model input and UE speed of 30km/h is assumed.

- 3 sources consider spatial consistency, and other sources do not consider spatial consistency.

- 2 sources consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

- 1 source considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

- The performance metric is SGCS in linear value for layer 1.

Note: Results refer to Table 2-3 of R1-2407338

Note: N4 refers to the number of predicted CSI instances

Mean UPT performance over benchmark 1 of the nearest historical CSI

For the CSI prediction using UE-sided model, in terms of mean UPT, gains are observed compared to Benchmark 1 of the nearest historical CSI:

- For FTP traffic with low RU (RU<=39%)

  - For 30km/h UE speed and N4=1

    - 2 sources observe -0.2%~1% gain

    - 4 sources observe 4.4%~9% gain.

  - For 60km/h UE speed, and N4=1

    - 2 sources observe 1.2%~5% gain;

  - For 30km/h UE speed and N4=4

    - 1 source observes 0.3% gain.

    - 1 source observes 7% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source observes 5% gain.

- For FTP traffic with mid RU (40<=RU<=69%)

  - For 30km/h UE speed and N4=1

    - 2 sources observe 0.9%~2.3% gain

    - 1 source observes 8.5% gain

    - 2 sources observe 22.3%~37% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source observes 1.8%~3.5% gain;

    - 1 source observes 21% gain.

  - For 30km/h UE speed and N4=4

    - 1 source observes 2.3% gain.

    - 1 source observes 29% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source observes 21% gain.

- For FTP traffic with high RU (RU>=70%)

  - For 30km/h UE speed and N4=1

    - 4 sources observe 2%~3.6% gain

    - 2 sources observe 8.5%~31.7% gain.

  - For 30km/h UE speed and N4=4

    - 1 source observes 7.2% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source observes 4.2% gain;

- For full buffer traffic:

  - For 30km/h UE speed and N4=1

    - 3 sources observe 27%~51% gain.

    - 2 sources observe 1.2%~8.7% gain.

  - For 60km/h UE speed and N4=1

    - 1 source observes 12.1% gain.

  - For 30km/h UE speed and N4=4

    - 1 source observes 11.6% gain.

Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms.

- A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

- Raw channel matrix is considered as model input

- 2 sources consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

- 3 sources consider spatial consistency, and other sources do not consider spatial consistency.

- 1 source considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

- 5 sources consider realistic channel estimation, and other sources consider ideal channel estimation.

- 1 source are modelled phase discontinuity, and other sources do not consider phase discontinuity

Note: N4 refers to the number of predicted CSI instances

Note: Results refer to Table 2-5/2-7 of R1-2407338

5% UE UPT performance over benchmark 1 of the nearest historical CSI

For the CSI prediction using UE-sided model, in terms of 5% UE UPT, gains are observed compared to Benchmark 1 of the nearest historical CSI:

- For FTP traffic with low RU (RU<=39%)

  - For 30km/h UE speed and N4=1

    - 1 source observes 1.5% gain.

    - 4 sources observe 6.7%~15.7% gain.

    - 1 source observes 27% gain.

  - For 60km/h UE speed, and N4=1

    - 2 sources observe 9%~18.3% gain;

  - For 30km/h UE speed and N4=4

    - 1 source observes 3.6% gain.

    - 1 source observes 14% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source observes 4% gain.

- For FTP traffic with mid RU (40<=RU<=69%)

  - For 30km/h UE speed and N4=1

    - 1 source observes 4.1% gain.

    - 2 sources observe 14.3%~38.9% gain.

    - 1 source observes 100% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source observes 11.3% gain;

    - 1 source observes 45% gain.

  - For 30km/h UE speed and N4=4

    - 1 source observes 10% gain.

    - 1 source observes 77% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source observes 26% gain.

- For FTP traffic with high RU (RU>=70%)

  - For 30km/h UE speed, and N4=1

    - 1 source observes 51.7% gain.

    - 3 sources observe 7.8%~15.8% gain

  - For 60km/h UE speed, and N4=1

    - 1 source observes 17.8% gain;

  - For 30km/h UE speed, and N4=4

    - 1 source observes 6.2% gain

- For full buffer traffic:

  - For 30km/h UE speed and N4=1

    - 4 sources observe 7.7%~10.5% gain.

    - 1 source observes 39.7% gain.

  - For 60km/h UE speed and N4=1

    - 1 source observes 20.6% gain.

  - For 30km/h UE speed and N4=4

    - 1 source observes 2.1% gain.

Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms.

- A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

- Raw channel matrix is considered as model input

- 2 sources consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

- 3 sources consider spatial consistency, and other sources do not consider spatial consistency.

- 1 source considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

- 5 sources consider realistic channel estimation, and other sources consider ideal channel estimation.

- 1 source are modelled phase discontinuity, and other sources do not consider phase discontinuity

Note: N4 refers to the number of predicted CSI instances

Note: Results refer to Table 2-5/2-7 of R1-2407338

Mean UPT performance over benchmark 2 of non-AI based CSI prediction

For the CSI prediction using UE-sided model, in terms of mean UPT, gains are observed compared to Benchmark 2 of a non-AI/ML based CSI prediction approach:

- For FTP traffic, with low RU (RU<=39%)

  - For 30km/h UE speed and N4=1,

    - 2 sources observe 7.6%~9% gain.

    - 4 sources observe -2.41%~1.1% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source observes -3.4% gain

    - 2 sources observe 0.9%~1.2% gain;

    - 1 source observes 11% gain.

  - For 30km/h UE speed and N4=4,

    - 1 source observes 13% gain.

    - 2 sources observe 0%~0.3% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source observes 13%

    - 1 source observes 0.14% gain

- For FTP traffic, with mid RU (40<=RU<=69%)

  - For 30km/h UE speed and N4=1,

    - 1 source observes 24% gain.

    - 3 sources observe 0.2%~5.1% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source observes -29.4% gain;

    - 1 source observes 3.1% gain;

    - 1 source observes 31% gain.

  - For 30km/h UE speed and N4=4,

    - 1 source observes 35% gain.

    - 2 sources observe -0.25%~2% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source observes -16.2% gain.

    - 1 source observes 0.25% gain.

    - 1 source observes 32% gain.

- For FTP traffic, with high RU (RU>=70%)

  - For 30km/h UE speed, and N4=1

    - 4 sources observe -0.2%~9.2% gain;

  - For 60km/h UE speed, and N4=1

    - 1 source observes 2.5% gain;

  - For 30km/h UE speed and N4=4,

    - 2 sources observe -0.8%~0.11% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source observes -19.1% gain.

    - 1 source observes 0.92% gain.

- For full buffer traffic:

  - For 30km/h UE speed and N4=1

    - 2 sources observe 10.6%~24% gain.

    - 2 sources observe 7.8%~9.7% gain.

    - 3 sources observe -0.6%~1.2% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source observes 0.2% gain

    - 1 source observes 8.4% gain

  - For 30km/h UE speed and N4= 4

    - 1 source observes 7% gain.

    - 1 source observes 6.8% gain

  - For 60km/h UE speed and N4=4

    - 1 source observes 11.6% gain

Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms.

- A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

- Raw channel matrix is considered as model input

- 2 sources consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

- 3 sources consider spatial consistency, and other sources do not consider spatial consistency.

- 1 source considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

- 5 sources consider realistic channel estimation, and other sources consider ideal channel estimation.

- 1 source are modelled phase discontinuity, and other sources do not consider phase discontinuity

Note: N4 refers to the number of predicted CSI instances

Note: Results refer to Table 2-6/2-8 of R1-247339

Note: Figure 6.2.2.6A-3 shows the Mean UPT median, 0.75 quantile, 0.25 quantile, outliers, and min/max values excluding outliers, for low RU, Mid RU, High RU and full buffer with UE speed of 30km/h.

Figure 6.2.2.6A-3: Mean UPT median, 0.75 quantile, 0.25 quantile, outliers, and min/max values excluding outliers, for low RU, Mid RU, High RU and full buffer with UE speed of 30km/h.

5% UE UPT performance over benchmark 2 of non-AI based CSI prediction

For the CSI prediction using UE-sided model, in terms of 5% UE UPT, gains are observed compared to Benchmark 2 of a non-AI/ML based CSI prediction approach:

- For FTP traffic, with low RU (RU<=39%)

  - For 30km/h UE speed and N4=1,

    - 2 sources observe 18%~26% gain.

    - 4 sources observe -1.1%~7.6% gain.

  - For 60km/h UE speed, and N4=1

    - 3 sources observe 1.9%~5.2% gain;

    - 1 source observes 17% gain.

  - For 30km/h UE speed and N4=4,

    - 1 source observes 23% gain.

    - 1 source observes -3.7% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source observes 19% gain.

    - 1 source observes 4% gain.

- For FTP traffic, with mid RU (40<=RU<=69%)

  - For 30km/h UE speed and N4=1,

    - 1 source observes 46% gain.

    - 1 source observes 18.7% gain.

    - 2 sources observe 4%~6.6% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source observes 8.6% gain;

    - 2 sources observe 23%~66% gain.

  - For 30km/h UE speed and N4=4,

    - 1 source observes 73% gain.

    - 1 source observes -9% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source observes 56% gain.

    - 1 source observes 11.5% gain.

- For FTP traffic, with high RU (RU>=70%)

  - For 30km/h UE speed, and N4=1

    - 1 source observes 1.9% gain;

    - 2 sources observe 20.7%~26.3% gain;

  - For 60km/h UE speed, and N4=1

    - 1 source observes 14.8%~16% gain;

  - For 30km/h UE speed and N4=4,

    - 1 source observes 0.9% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source observes 22.8% gain.

- For full buffer traffic:

  - For 30km/h UE speed and N4=1

    - 2 sources observe 15.7%~16.1% gain.

    - 4 sources observe 0.2%~6% gain.

    - 1 source observes -2% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source observes 0.4% gain

    - 1 source observes 11.6% gain

  - For 30km/h UE speed and N4= 4

    - 1 source observes 6.3% gain.

    - 1 source observes 21% gain

  - For 60km/h UE speed and N4=4

  - 1 source observes 26.7% gain

Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms.

- A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

- Raw channel matrix is considered as model input

- 2 sources consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

- 3 sources consider spatial consistency, and other sources do not consider spatial consistency.

- 1 source considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

- 5 sources consider realistic channel estimation, and other sources consider ideal channel estimation.

- 1 source are modelled phase discontinuity, and other sources do not consider phase discontinuity

Note: N4 refers to the number of predicted CSI instances

Note: Results refer to Table 2-6/2-8 of R1-2407339

Note: Figure 6.2.2.6A-4 shows the 5% UPT median, 0.75 quantile, 0.25 quantile, outliers, and min/max values excluding outliers, for low RU, Mid RU, High RU and full buffer with UE speed of 30km/h.

Figure 6.2.2.6A-4: 5% UPT median, 0.75 quantile, 0.25 quantile, outliers, and min/max values excluding outliers, for low RU, Mid RU, High RU and full buffer with UE speed of 30km/h.

SGCS performance over benchmark 2 of non-AI based CSI prediction, impact of channel estimation error

For the CSI prediction using UE-sided model, compared to the Benchmark 2 of non-AI based CSI prediction, in terms of SGCS, from channel estimation perspective

- If ideal channel estimation is adopted

  - For N4=1

    - 9 sources observe -1.2%~6.3% gain

    - 1 source observes -2% ~17% gain depending on filter complexity and filter update

  - For N4=4

    - 2 sources observe -1.6%~5.56%

    - 1 source observes -4.5%~6.93%

    - 1 source observes 3.1%~40.5%

- If realistic channel estimation is adopted,

  - For N4=1

    - 5 sources observe 0.43%~7.26% gain

    - 2 sources observe 15.2%~19.5 gain

    - 1 source observes 68% gain

    - 1 source observes 5.8%~16.4% gain depending on traffic load

  - For N4=4

    - 2 sources observe 0.1%~1.4% gain

    - 1 source observes 5%~29% gain

Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms.

- A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

- Raw channel matrix as model input and UE speed of 30km/h is assumed.

- 3 sources consider spatial consistency, and other sources do not consider spatial consistency.

- 2 sources consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

- 1 source considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

- The performance metric is SGCS in linear value for layer 1.

Note: Results refer to Table 2-3 of R1-2407338

Note: N4 refers to the number of predicted CSI instances

Mean UPT performance over benchmark 2 of non-AI based CSI prediction, impact of channel estimation error

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, in terms of mean UPT, gains are observed compared to Benchmark#2 of a non-AI/ML based CSI prediction, from channel estimation perspective:

- For FTP traffic, with low RU (RU<=39%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources observe -2.41%~1.8% gain.

    - For 60km/h UE speed, and N4=1

      - 2 sources observe -3.4%~0.9% gain

    - For 30km/h UE speed and N4=4,

      - 1 source observes 0.3% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources observe 7.6%~9% gain.

      - 3 sources observe 0%~1.1% gain.

    - For 60km/h UE speed, and N4=1

      - 2 sources observe -3.4%~1.2% gain

      - 1 source observes 11% gain.

    - For 30km/h UE speed and N4=4,

      - 1 source observes 13% gain.

      - 1 source observes 0% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source observes 13%

      - 1 source observes 0.14% gain

- For FTP traffic, with mid RU (40<=RU<=69%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source observes -4.5% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source observes -7.1% gain

      - 1 source observes 3.1% gain

    - For 30km/h UE speed and N4=4,

      - 1 source observes -2% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources observe 24% gain.

      - 3 sources observe 0.2%~5.1% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source observes 31% gain

      - 1 source observes -29.4% gain

    - For 30km/h UE speed and N4=4,

      - 1 source observes 35% gain.

      - 2 sources observe -0.25%~1.1% gain

    - For 60km/h UE speed, and N4=4

      - 1 source observes 32%

      - 1 source observes 0.25% gain

- For FTP traffic, with high RU (RU>=70%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source observes -4.8% gain

    - For 60km/h UE speed, and N4=1

      - 1 source observes -9% gain

      - 1 source observes 2.5% gain

    - For 30km/h UE speed and N4=4,

      - 1 source observes -0.8% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 3 sources observe 0%~0.8%

      - 1 source observes 9.2% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source observes -9% gain

    - For 30km/h UE speed and N4=4,

      - 2 sources observe 0%~0.1% gain

    - For 60km/h UE speed, and N4=4

      - 1 source observes 0.92% gain

- For full buffer model,

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source observes 24%

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1

      - 3 sources observe 7.8%~10.6% gain.

      - 3 sources observe -0.6%~1.2% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source observes 0.2% gain

      - 1 source observes 8.4% gain

    - For 30km/h UE speed and N4= 4

      - 1 source observes 7% gain.

      - 1 source observes 6.8% gain

    - For 60km/h UE speed and N4=4

      - 1 source observes 11.6% gain

Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - 2 sources consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources consider spatial consistency, and other sources do not consider spatial consistency.

  - 1 source considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

Note: N4 refers to the number of predicted CSI instances

Note: Results refer to Table 2-6/2-8 of R1-2407340

5% UE UPT performance over benchmark 2 of non-AI based CSI prediction, impact of channel estimation error

For the CSI prediction using UE-sided model, in terms of 5% UE UPT, gains are observed compared to Benchmark 2 of a non-AI/ML based CSI prediction, from channel estimation perspective:

- For FTP traffic, with low RU (RU<=39%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source observes -5.5% gain.

    - For 60km/h UE speed, and N4=1

      - 2 sources observe 4%~4.3% gain

    - For 30km/h UE speed and N4=4,

      - 1 source observes -3.7% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources observe 17% gain.

      - 2 sources observe 0%~4% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source observes 1.9% gain

      - 1 source observes 17% gain.

    - For 30km/h UE speed and N4=4,

      - 1 source observes 23% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source observes 19%

- For FTP traffic, with mid RU (40<=RU<=69%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source observes -12.9% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source observes 2.6% gain

      - 1 source observes 8.6% gain

    - For 30km/h UE speed and N4=4,

      - 1 source observes -9% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources observe 4%~6.6% gain.

      - 1 source observes 18.7% gain.

      - 1 source observes 46% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source observes 66% gain

      - 1 source observes 2.6% gain

    - For 30km/h UE speed and N4=4,

      - 1 source observes 73% gain.

      - 1 source observes 18.7% gain

    - For 60km/h UE speed, and N4=4

      - 1 source observes 56%

- For FTP traffic, with high RU (RU>=70%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source observes 3.6% gain

    - For 60km/h UE speed, and N4=1

      - 1 source observes -10.7% gain

      - 1 source observes 14.8% gain

    - For 30km/h UE speed and N4=4,

      - 1 source observes 0.9% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources observe 20.7%~26.3%

      - 1 source observes 1.9% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source observes 3.6% gain

    - For 30km/h UE speed and N4=4,

      - 1 source observes 0.9% gain

- For full buffer model,

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source observes 0.2%

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1

      - 2 sources observe 15.7%~16.1% gain.

      - 3 sources observe 2.6%~7.7% gain.

      - 1 source observes -2% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source observes 0.4% gain

      - 1 source observes 11.6% gain

    - For 30km/h UE speed and N4= 4

      - 1 source observes 6.3% gain.

      - 1 source observes 21% gain

    - For 60km/h UE speed and N4=4

      - 1 source observes 26.7% gain

Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - 2 sources consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources consider spatial consistency, and other sources do not consider spatial consistency.

  - 1 source considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

Note: N4 refers to the number of predicted CSI instances

Note: Results refer to Table 2-6/2-8 of R1-2407340

SGCS performance over benchmark 2 of non-AI based CSI prediction, impact of phase discontinuity

For the CSI prediction using UE-sided model, compared to the Benchmark 2 of non-AI based CSI prediction, in terms of SGCS, from perspective of phase discontinuity modelling

- If phase discontinuity modelling is not adopted

  - For 30km/h UE speed,

    - 2 sources observe 4.24%~4.3% gain with N4=1

    - 1 source observes 3.1% ~ 40.5% with N4=4

    - 1 source observes -1.2%~6.3% with N4=1 depending on filter updates between 10 and 70 learning window size

  - For 60km/h UE speed,

    - 1 source observes 13.5% gain with N4=1, and 1 source observes 8.2%~34.6% gain with N4=4

    - 1 source observes 0%~8% with N4=1 depending on filter updates between 10 and 70 learning window size

- If phase discontinuity modelling is adopted,

  - For 30km/h UE speed,

    - Without phase adjustment,

      - 2 sources observe 7.26%~7.8% gain with N4=1

      - 1 source observes 6.5%~23.5% gain with N4=4

      - 1 source observes -0.8%~8% with N4=1 depending on filter updates between 10 and 70 learning window size

    - With phase adjustment,

      - 1 source observes 1.2%~7.7% gain with N4=1 depending on filter updates between 10 and 70 learning window size

      - 1 source observes 25.6% gain

  - For 60km/h UE speed,

    - Without phase adjustment,

      - 1 source observes 6.6% gain with N4=1 and 6.4%~55% with N4=4

      - 1 source observes 5.7%~17.5% gain with N4=1 depending on filter updates between 10 and 70 learning window size

    - With phase adjustment, 1 source observes 4.2%~11.9% gain with N4=1 depending on filter updates between 10 and 70 learning window size

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix as model input is assumed.

  - The performance metric is SGCS in linear value for layer 1.

- Note: Results refer to Table 2-12 of R1-2407338

- Note: N4 refers to the number of predicted CSI instances

UPT performance with 20ms CSI-RS periodicity

For the CSI prediction using CSI-RS with 20ms periodicity, till the RAN1#118 meeting, in terms of mean and 5% UE UPT, gains are observed compared to Benchmark #1 of the nearest historical CSI and Benchmark #2 of non-AI/ML based CSI prediction,

- Compared to Benchmark 1

  - For N4=1 and UE speed of 3km/h, 1 source observes -18%~-7% and -26%~-8% degradation in terms of mean and 5% UE UPT, respectively.

  - For N4=4 and UE speed of 3km/h, 1 source observes 1%~6% and 6%~17% gain in terms of mean and 5% UE UPT, respectively.

  - For N4=4 and UE speed of 30km/h, 1 source observes 2%~17% and 13%~19% gain in terms of mean and 5% UE UPT, respectively, by using one 20ms periodic CSI-RS and three aperiodic CSI-RS.

- Compared to Benchmark 2

  - For N4=1 and UE speed of 3km/h,

    - 1 source observes 10%~19% and 23%~31% gain in terms of mean and 5% UE UPT, respectively.

    - 1 source observes -2.7%~-1% and -10.4%~-4.9% degradation in terms of mean and 5% UE UPT, respectively.

  - For N4=1 and UE speed of 10km/h, 1 source observes 0.6%~1% gain and -14.2%~-8% gain in terms of mean and 5% UE UPT, respectively.

  - For N4=4 and UE speed of 3km/h, 1 source observes 14%~30% and 19%~56% gain in terms of mean and 5% UE UPT, respectively.

  - For N4=4 and UE speed of 30km/h,

    - 1 source observes 17.3% and 16.1 gain in terms of mean and 5% UE UPT, receptively.

    - 1 source observes 5%~19% and 13%~40% gain in terms of mean and 5% UE UPT, respectively, by using one 20ms periodic CSI-RS and three aperiodic CSI-RS.

Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

- 1 source considers full buffer model, other sources consider FTP traffic model

- Raw channel matrix is used as the model input.

- 2 sources consider realistic channel estimation, and other sources consider ideal channel estimation.

- The performance metric is SGCS in linear value for layer 1/2/3/4 and mean UPT, 5% UPT.

- 1 source consider spatial consistency. Other sources do not consider spatial consistency.

- Note: Results refer to Table 2-13, 2-14, 2-15, 2-16 of R1-2407339

#### 6.2.2.7A	Generalization evaluations for CSI prediction

Generalization over UE speeds

For the generalization verification of CSI prediction using UE sided model over various UE speeds, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of UE speed#A and UE speed#B but not for others:

  - If UE speed#B is 10 km/h and

    - UE speed#A is 30 km/h or 120km/h, 2 sources observe a generalized performance of less than -5.5% degradation, and 1 source observes -16.2% degradation.

    - UE speed#A is 60 km/h, 2 sources observe -26.79%~-13.3% degradation.

  - If UE speed#B is 30 km/h and

    - UE speed#A is 60km/h, 4 sources observe a generalized performance of -11.4%~-2.7% degradation and 4 sources observe a generalized performance of -33.6%~-19% degradation.

    - UE speed#A is 10km/h or 120km/h, 3 sources observe a generalized performance of -72.37%~-51.5% degradation.

  - If UE speed#B is 60 km/h and

    - UE speed#A is 10km/h, 1 source observes a generalized performance of -16.4 degradation

    - UE speed#A is 30km/h, 1 source observes a generalized performance of -5.4% degradation and 6 sources observe a generalized performance of -56.3%~-13.8% degradation

  - If UE speed#B is 120 km/h and

    - UE speed#A is 30km/h, 1 source observes a generalized performance of -56.3% degradation

- For generalization Case 3, generalized performance of the AI/ML model can be achieved in general (0%~-3.8% loss) for UE speed#B subject to any of 10 km/h, 30 km/h, 60 km/h and 120 km/h, if the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, as observed by 8 sources

  - If UE speed#B is 10 km/h

    - 2 sources observe a generalized performance of less than -0.2% degradation, 1 source observes -5.1% degradation, and 1 source observes -16.87% degradation.

  - If UE speed#B is 30 km/h

    - 9 sources observe a generalized performance of less than -3.8% degradation, 1 source observes a generalized performance of -15.44% degradation.

  - If UE speed#B is 60 km/h and

    - 8 sources observe a generalized performance of less than -3.63% degradation, 2 sources observe a generalized performance of -13.5%~-6.77% degradation.

  - If UE speed#B is 120 km/h and

    - 1 source observes a generalized performance of -43.6% degradation

  - Note: For generalization Case 3, 2 sources observe performance degradations (-43.6%~-13.5% loss) for UE speed#B subject to 10 km/h, 30 km/h, 60 km/h, 120 km/h, but compared with generalization Case 2, in general the performance are still improved.

Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

- Raw channel matrix is used as the model input.

- The performance metric is SGCS in linear value for layer 1/2/3/4.

- 4 sources consider spatial consistency. Other sources do not consider spatial consistency.

- 2 sources consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

Note: Results refer to Table 3-1 of R1-2407338

Generalization over deployment scenarios

For the generalization verification of CSI prediction using UE sided model over various deployment scenarios, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain deployment scenario#B applied for inference with a same deployment scenario#B

- For generalization Case 2, generalized performance may be achieved for some certain combinations of deployment scenario#A and deployment scenario#B but not for others:

  - For deployment scenario#B is UMa

    - 2 sources observe -1.88%~0% degradation

    - 1 source observes -6.8% degradation

  - For deployment scenario#B is UMi

    - 1 source observes 0% degradation

    - 3 sources observe -4.85%~-3.03% degradation

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (-1.95%~0% loss) for deployment scenario#B subject to any of UMa and UMi, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#B as observed by 3 sources.

  - Minor loss (0%~-1.95%) are observed by 3 sources

  - Note: Moderate degradations of -5.2% are observed by 1 source for deployment scenario#B subject to Uma

Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

- Raw channel matrix is used as the model input.

- The performance metric is SGCS in linear value for layer 1/2/3/4.

- 3 sources consider spatial consistency. Other sources do not consider spatial consistency.

- 1 source considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

Note: Results refer to Table 3-2 of R1-2407339

Generalization over carrier frequency

For the generalization verification of CSI prediction using UE sided model over various carrier frequency, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain carrier frequency#B applied for inference with a same carrier frequency#B

- For generalization Case 2, significant degradations are suffered in general from the perspective of the layouts of antenna ports, as observed by 3 sources:

  - For carrier frequency#B is 2GHz

    - 1 source observes -11.4% degradation

    - 1 source observes -80.53% degradation

  - For carrier frequency#B is 3GHz or 4GHz

    - 2 sources observe -34.23%~-80.53% degradation

    - 1 source observes -14.21% degradation

- For generalization Case 3, generalized performance may be achieved for some certain combinations of carrier frequency#A and carrier frequency#B but not for others, if the training dataset is constructed with data samples subject to multiple carrier frequencies including carrier frequency#B

  - For carrier frequency#B is 2GHz

    - 1 source observes -0.5% degradation

    - 1 source observes -9.27% degradation

  - For carrier frequency#B is 3GHz or 4GHz

    - 2 sources observe -1.93%~-5.1% degradation

    - 1 source observes -14.94% degradation

Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

- Raw channel matrix is used as the model input.

- The performance metric is SGCS in linear value for layer 1/2/3/4.

- 2 sources consider spatial consistency. Other sources do not consider spatial consistency.

Note: Results refer to Table 3-3 of R1-2407339

Generalization over multiple aspects

For the generalization verification of CSI prediction using UE sided model over multiple aspects, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to certain aspects #B applied for inference with the same aspects #B,

- For generalization Case 2,

    - 1 source observes -9.8% ~ -1.5% degradation when the aspects #A is (2 GHz carrier frequency, 100% outdoor UE) and the aspects #B is (4GHz carrier frequency, 20% outdoor UE+80% indoor UE)

    - 1 source observes -10.7%~-1.8% when the aspects#A is (UMa, 2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects#B is (UMi, 4 GHz carrier frequency, 100 outdoor UE)

    - 1 source observes -21.2%~-2.4% when the aspects#A is (UMa, 2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects#B is (UMi, 4 GHz carrier frequency, 20% outdoor UE+80% indoor UE distribution)

    - 1 source observes -3% degradation when aspects#A is (30km/h UE speed, 100% UE in a car, 2GHz carrier frequency) and the aspects#B is (3km/h UE speed, 20% outdoor UE, 4GHz carrier frequency)

- For generalization Case 3,

    - 1 source observes -1.1%~0% when the aspects#A is (2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects #B is (4GHz carrier frequency, 20% outdoor UE+80% indoor UE distribution)

    - 1 source observes -2.4%~-0.8% when the aspects#A is (UMa, 2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects#B is (UMi, 4 GHz carrier frequency, 100 outdoor UE)

    - 1 source observes -3.4%~-0.3% when the aspects#A is (UMa, 2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects#B is (UMi, 4 GHz carrier frequency, 20% outdoor UE+80% indoor UE distribution)

Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

- The performance metric is SGCS in linear value for layer 1/2/3/4.

- 1 source considers eigenvector as model input, and 1 source considers Raw channel matrix as model input.

Note: Results refer to Table 3-4 of R1-2407339

#### 6.2.2.8A	Summary of Performance Results for CSI feedback enhancement

The following aspects have been studied for CSI prediction using UE-sided model in Rel-19:

- From the perspective of basic performance gain over non-AI/ML benchmark (without considering generalization),

  - It has been studied with corresponding observations on:

    - the metrics of SGCS, mean UPT, 5% UPT;

    - the benchmarks of nearest historical CSI and non-AI/ML based CSI prediction.

    - The impact of channel estimation error, phase discontinuity, spatial consistency, UE Speed, observation window, prediction window, CSI-RS periodicity.

- It has been studied with corresponding observations on complexity for both AI/ML based CSI prediction and non-AI/ML based CSI prediction.

- It has been studied on localized model including evaluation methodology, but is lack of observations.

- From the perspective of generalization over various scenarios,

  - It has been studied with corresponding observations on (with the metric of SGCS):

    - the scenario including various UE speeds, deployment scenarios, carrier frequency

- From the perspective of performance monitoring, it has been studied on boundary between Type 1 and 3 performance monitoring, and potential specification impact for each performance monitoring type 1,2 and 3.

Based on the evaluation for CSI prediction in Rel-19, the following high-level observations are provided:

- From the perspective of basic performance gain over benchmark of non-AI/ML based CSI prediction, under the same UE speed for training and inference

  - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 0%~7.8% gain depending on traffic model, in terms of mean UPT, is observed by 7 sources

  - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3.8%~20.7% gain depending on traffic model, in terms of 5% UE UPT, is observed by 7 sources.

- From a perspective of AI/ML complexity, 19 sources adopt the model subject to the computational complexity in units of FLOPs from 0.05M to 3000M. For complexity of non-AI/ML benchmark, 16 sources adopt the algorithm (e.g., Kalman filter, Auto-regression, Wiener filter) subject to the computational complexity in units of FLOPs from 0.14M to 107M. The ratio of FLOPs (AI/ML over benchmark 2) ranges from 1 to 35, which is used by majority sources. For non-AI/ML benchmark, main computation complexity is dominated by filter updates, which may not be need to be updated per inference at the expense of performance loss.

- From the perspective of performance impact on channel estimation error and phase discontinuity, compared to non-AI/ML CSI prediction, higher gain is observed by 10 sources for AI/ML based CSI prediction in the presence of channel estimation error or phase discontinuity.

- From the perspective of generalization over various scenarios/configurations (e.g., various UE speed, deployment scenario, carrier frequency) that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain scenario#B/configuration#B and applied for inference with a same scenario#B/configuration#B

  - For generalization Case 2 where the AI/ML model is trained with dataset from a different scenario#A/configuration#A, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others.

  - For generalization Case 3 where the training dataset is constructed with data samples subject to multiple scenarios/configurations including scenario#B/configuration#B, generalized performance of the AI/ML model can be achieved.

### 7.1.2	CSI feedback enhancement

Items considered for studying the necessity, feasibility, potential specification impact:

In CSI compression using two-sided model use case:

Performance monitoring:

-	Model performance monitoring related assistance signalling and procedure.

-	Metrics/methods including:

-	Intermediate KPIs (e.g., SGCS)

-	Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK).

-	Legacy CSI based monitoring: schemes using additional legacy CSI reporting

-	Other monitoring solutions, at least including the following option:

-	Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection

-	NW-side performance monitoring:  NW monitors the performance and make decisions of model/functionality activation/ deactivation/updating/switching. Impact to enable performance monitoring using an existing CSI feedback scheme as the reference, including the association between AI/ML scheme and existing CSI feedback scheme for monitoring, are considered. Note: The metric for monitoring and comparison includes intermediate KPI and eventual KPI.

-	UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model/functionality activation/deactivation/updating/switching. Impact on triggering and means for reporting the monitoring metrics, including periodic/semi-persistent and aperiodic reporting, and other reporting initiated from UE, are not precluded.

Intermediate KPI based model monitoring:

The following intermediate KPI-based model monitoring options were proposed by companies:

-	NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side.

-	UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side.

-	Network may configure a threshold criterion to facilitate UE to perform model monitoring.

-	UE-side monitoring based on the output of the CSI reconstruction model at the UE-side

-	Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. Network may configure a threshold criterion to facilitate UE to perform model monitoring.

Fallback mode:

-	Potential specification impact for supporting co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode

NW/UE alignment:

-	Alignment of the quantization/dequantization method and the feedback message size between Network and UE, including the following:

-	For vector quantization scheme, the format and size of the VQ codebook, and the size and segmentation method of the CSI generation model output

-	For scalar quantization scheme, uniform and non-uniform quantization with format, e.g., quantization granularity, consisting of distribution of bits assigned to each float.

-	Quantization alignment for CSI feedback between CSI generation part at the UE and CSI reconstruction part at the NW is needed, e.g.,

-	through model pairing process,

-	alignment based on standardized quantization scheme.

-	Additional methods not precluded.

Model input/output:

-	Output-CSI-UE and input-CSI-NW at least for Precoding matrix

-	Option 1a: The precoding matrix in spatial-frequency domain

-	Option 1b: The precoding matrix represented using angular-delay domain projection

-	whether Option 2: Explicit channel matrix (i.e., full Tx * Rx MIMO channel) is also studied depends on the performance evaluations:

-	Option 2a: raw channel is in spatial-frequency domain

-	Option 2b: raw channel is in angular-delay domain

-	CSI part 1 includes at least CQI for first codeword, RI, and information representing the part 2 size. CSI part 2 includes at least the content of CSI generation part output. Other CSI report formats are not precluded.

UE side data collection:

-	Enhancement of CSI-RS configuration to enable higher accuracy measurement.

-	Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc.

-	The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side.

-	Signalling for triggering the data collection

NW side data collection:

-	Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement.

-	Contents of the ground-truth CSI including:

-	Data sample type, e.g., precoding matrix, channel matrix etc.

-	Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like).

-	Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator)

-	Latency requirement for data collection

-	Signalling for triggering the data collection

-	Ground-truth CSI report for NW side data collection for model performance monitoring, including:

-	Scalar quantization for ground-truth CSI

-	Codebook-based quantization for ground-truth CSI

-	RRC signalling and/or L1 signalling procedure to enable fast identification of AI/ML model performance

Aperiodic/semi-persistent or periodic ground-truth CSI report

-	Ground-truth CSI format for model training, including scalar or codebook-based quantization for ground-truth CSI. The number of layers for which the ground-truth data is collected, and whether UE or NW determine the number of layers for ground-truth CSI data collection, are considered.

In CSI compression using two-sided model use case with training collaboration Type 3, for sequential training, at least the following aspects have been identified for dataset delivery from RAN1 perspective, including:

-	Dataset and/or other information delivery from UE side to NW side, which can be used at least for CSI reconstruction model training

-	Dataset and/or other information delivery from NW side to UE side, which can be used at least for CSI generation model training

-	Potential dataset delivery methods including offline delivery, and over the air delivery

-	Data sample format/type

-	Quantization/de-quantization related information

CSI configuration and report:

-	NW configuration to determine CSI payload size, e.g., possible CSI payload size, possible rank restriction and/or other related configuration.

-	How UE determines/reports the actual CSI payload size and/or other CSI related information within constraints configured by the network.

-	Relevant UCI format considering the legacy CSI reporting principle with CSI Part 1 and Part 2 as a starting point, where Part 1 has a network configured fixed size and Part 2 size is dynamic, determined by information in Part 1.

For CQI determination in CSI report, if CQI in CSI report is configured.

-	Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including

-	Option 1a: CQI is calculated based on target CSI with realistic channel measurement

-	Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment

-	Option 1c: CQI is calculated based on legacy codebook

-	Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including

-	Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment

-	Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW.

-	Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder.

-	Notes: feasibility of different options should be evaluated. Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated. Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signalling overhead.

Feasibility and methods to support the legacy CSI reporting principles:

-	The priority rule regarding CSI collision handling and CSI omission

-	Codebook subset restriction

-	Input-CSI-NW/output-CSI-UE considered in angular-delay domain, beam restriction can be based on legacy SD basis vector-based input CSI in angular domain.

-	CSI processing Unit

Potential specification enhancement on:

-	CSI-RS configurations (not including CSI-RS pattern design enhancements)

-	CSI configuration

-	For network to indicate CSI reporting related information, e.g., gNB indication to the UE of one or more of following:

-	Information indicating CSI payload size

-	Information indicating quantization method/granularity

-	Rank restriction

-	Other payload related aspects

-	CSI reporting configurations

-	For UE determination/reporting of the actual CSI payload size, UE reports related information as configured by the NW

-	CSI report UCI mapping/priority/omission

-	CSI processing procedures

In CSI compression using two-sided model use case, feasibility and procedure to align the information that enables the UE to select a CSI generation model(s) compatible with the CSI reconstruction model(s) used by the gNB is studied. At least the following options have been proposed by companies to define the pairing information used to enable the UE to select a CSI generation model(s) that is compatible with the CSI reconstruction model(s) used by the gNB:

-	Option 1: The pairing information is in the forms of the CSI reconstruction model ID that NW will use.

-	Option 2: The pairing information is in the forms of the CSI generation model ID that the UE will use.

-	Option 3: The pairing information is in the forms of the paired CSI generation model and CSI reconstruction model ID.

-	Option 4: The pairing information is in the forms of by the dataset ID during type 3 sequential training.

-	Option 5: The pairing information is in the forms of a training session ID to a prior training session (e.g., API) between NW and UE.

-	Option 6: The pairing information is up to UE/NW offline co-engineering alignment, transparent to 3GPP specification.

-	Note: the disclosure of the vendor information during the model pairing procedure and model identification procedure should be considered.

-	Note: If each UE side model is compatible with all NW side model, the information is not needed for the UE.

-	Note: Above does not imply there is a need for a central entity for defining/storing/maintaining the IDs.

In CSI prediction using UE-sided model use case:

Data collection:

In CSI prediction using UE sided model use case, at least the following aspects have been proposed by companies on data collection, including:

-	Signalling and procedures for the data collection

-	Data collection indicated by NW

-	Requested from UE for data collection

-	CSI-RS configuration

-	Assistance information for categorizing the data, if needed

-	The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side.

Performance monitoring:

For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM:

-	Type 1:

-	UE calculates the performance metric(s)

-	UE reports performance monitoring output that facilitates functionality fallback decision at the network

-	Performance monitoring output details can be further defined

-	NW may configure threshold criterion to facilitate UE side performance monitoring (if needed).

-	NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

-	Type 2:

-	UE reports predicted CSI and/or the corresponding ground-truth

-	NW calculates the performance metrics.

-	NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

-	Type 3:

-	UE calculates the performance metric(s)

-	UE reports performance metric(s) to the NW

-	NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

-	Functionality selection/activation/deactivation/switching as defined for other UE side use cases can be reused, if applicable.

-	Configuration and procedure for performance monitoring

-	CSI-RS configuration for performance monitoring

-	Performance metric including at least intermediate KPI (e.g., NMSE or SGCS)

-	UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report

-	Note: down selection is not precluded.

-	Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW.

For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively.

- The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured.

For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843,

- Type 1

  - Definition/configuration of performance metric

  - Definition of threshold criterion, if configured

  - Definition/configuration and report of monitoring output, and corresponding report mechanism

- Type 2

  - Definition/configuration and report of ground truth CSI, and corresponding report mechanism.

- Type 3

  - Definition/configuration and report of performance metric, and corresponding report mechanism.

- For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action

#### 7.2.1.3	Data collection

Data collection plays a crucial role in enabling the different use cases. Therefore, it is important to define the best approaches for collecting data to support UE-side and network-side model inference, monitoring, and training.

Table 7.2.1.3-1 lists existing data collection mechanisms available in current RAN specifications for the UE to report measurements to another entity acting as termination point for this data. As highlighted in clause 4.2, the analysis/selection of the data collection frameworks should focus on the RRC CONNECTED state for both data generation and reporting. As such, the Table can provide useful insights into existing methods with respect to various categories identified as relevant for data collection method selection.

Table 7.2.1.3-1. Existing data collection methods identified.

| Involved network entity (termination point) | RRC state to generate data | Max payload size per reporting* | Contents to be collected | 1) End-to-End report latency** | Report type | Security and Privacy |
| --- | --- | --- | --- | --- | --- | --- |
| Method: Logged MDT |  |  |  |  |  |  |
| TCE/OAM (Data can be utilized by gNB) | IDLE / INACTIVE | <9kbyte | - L3 cell/beam measurements - location information - sensor information - timing information | 1) Procedure latency***: - Latency to enter CONNECTED state - Latency to receive gNB request signalling (~20ms) 2) Air interface signalling latency****: - ~20ms (RRC) 3) Other latency: - Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message Privacy via user consent |
| Method: Immediate MDT |  |  |  |  |  |  |
| TCE/OAM (Data can be utilized by gNB) | CONNECTED | <9kbyte | - L3 cell/beam measurements - location information - sensor information | 1) Procedure latency: - Report interval:  120ms~30min for periodic report  TTT for event triggered report 2) Air interface signalling latency: - ~20ms (RRC) 3) Other latency: - Forwarding latency between gNB and TCE | - Event triggered - Periodic reporting | AS security via RRC message Privacy via user consent |
| Method: L3 measurements |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | L3 cell/beam measurements | 1) Procedure latency: - Report interval:  l20ms~30min for periodic report  TTT for event triggered report 2) Air interface signalling latency: - 20ms (RRC) | - Event triggered report - Periodic reporting | AS security via RRC message |
| Method: L1 measurement (CSI reporting) |  |  |  |  |  |  |
| gNB | CONNECTED | <1706bit in PUCCH <3840bit in PUSCH | L1 CSI measurement | 1) Procedure latency: - Report interval:  4-320 slot for periodic and semi-persistent report  0-32 slot after reception of DCI for aperiodic report 2) Air interface signalling latency: - 1 TTI (PUCCH) | - Aperiodic report - Semi-persistent report - Periodic report | No AS security |
| Method: UE Assistance Information (UAI) |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | Assistance information to show UE preference | 1) Procedure latency: - Upon generation of UE's preference 2) Air interface signalling latency: - ~20ms (RRC) | Up to UE implementation when to report | AS security via RRC message |
| Method: Early measurements |  |  |  |  |  |  |
| gNB | IDLE / INACTIVE | <9kbyte | L3 cell/beam measurements | 1) Procedure latency: - Latency to enter CONNECTED state - Latency to receive gNB request signalling (~20ms) 2) Air interface signalling latency: - ~20ms (RRC) | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message |
| Method: LPP |  |  |  |  |  |  |
| LMF | CONNECTED | <9kbyte | Location information | 1) Procedure latency: - Latency to get upper layer trigger (for UE triggered) - Or latency to receive network request message (~20ms) 2) Air interface signalling latency: - ~20ms (RRC) 3) Other latency: - Forwarding latency between gNB and LMF | - UE-triggered - Network-triggered | AS security via RRC message |
| *: The payload size doesn't consider signalling overhead. **: The End-to-End report latency is the latency from availability of the measurement report at the UE side to the availability of the measurement report at the terminated network entity. The time to generate data or perform measurements depends on RAN1/RAN4 specification. ***: Procedure latency is the latency caused by procedures, including procedure to ready for reporting (e.g., entering CONNECTED state, report interval). ****: Air interface signalling latency is the latency to transmit one report, e.g., RRC signalling latency or PUCCH signalling latency. |  |  |  |  |  |  |

##### 7.2.1.3.1	Considerations for network-side data collection

A set of general data collection principles is expected to be considered for network-side model training. These include:

-	UE to support data logging,

-	UE to report the collected data periodically, event-based, and on-demand,

-	The UE memory, processing power, energy consumption, signalling overhead should be considered.

Note:	The above principles can be revised depending on RAN1 requirements.

Furthermore, and regarding the use cases in this study, the following is considered.

For CSI and beam management use cases, the training of network-side models can consider both gNB and OAM-centric data collection mechanisms. The gNB-centric data collection implies that the gNB can configure the UE to initiate/terminate the data collection procedure. The potential impact of L3 signalling for the reporting of collected data should be assessed.

On the other hand, OAM-centric data collection implies that the OAM provides the configuration (via the gNB) needed for the UE to initiate/terminate the data collection procedure. MDT framework can be considered to achieve this. The potential impact on MDT for RRC_CONNECTED state should be assessed.

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

Note:	For gNB- and OAM-centric data collection, there may be a need to consult with RAN3 and SA5 whether/how OAM is to be involved.

Note:	For possible impacts due to positioning use cases, there may be a need to consult with RAN3 whether/how NRPPa is to be involved.

##### 7.2.1.3.2	Data collection for UE-side model training

The following options were discussed in RAN2:

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

3.	UE collects training data and transfers it to OAM. OAM transfers the trainingdata to the server for data collection for UE-side model training/OTT server.

The options listed above were analysed to understand potential specification impact. The analysis included aspects such as termination entities, data transfer path, whether control plane (CP) or user plane (UP) should be used to transfer the data, protocol layers involved, MNO controllability and visibility over the collected data, etc. The result of this analysis can be found below in Table 7.2.1.3.2-1.

It is worth noting that different options for the data content visibility were discussed. The different levels of data content visibility are captured in the Note 3 in the Table 7.2.1.3.2-1.

Note:	RAN2 discussed that, except for the case of standardized data content, the data content visibility and any level of controllability could be achieved via SLA (Service Level Agreement). However, SLA is out of RAN2 scope.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | FFS: level of controllability (Note 5) (Note 1) | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure, FFS other procedures | Via RRC procedure |
| Possible Options for data content visibility in MNO (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. UE proprietary information can only be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of possible MNO controllability and visibility via Option 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 cannot reach consensus on the feasibility of UP tunnel in Options 2 and 3 |  |  |  |  |

Related to privacy, it has been stressed in RAN2 the importance that any potential mechanism to collect UE side data for model training purposes (including the options 1a, 1b, 2, 3 listed above) must comply with privacy protection regulations, requirements, laws and/or policies. An informative Annex is included at the end of this document capturing examples of privacy concerns for different stakeholders participating in the discussion.

# 8	Conclusions

The following aspects have been studied for the general framework of AI/ML over air interface for one-sided models and two-sided models:

-	Various Network-UE Collaboration Levels

-	Functionality-based LCM and model-ID-based LCM

-	Functionality/model selection, activation, deactivation, switching, fallback

-	Functionality identification and model identification

-	Data collection

-	Performance monitoring

-	Various model identification Types and their use cases

-	Reporting of applicable functionalities/models

-	Method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

-	Model delivery/transfer and analysis of various model delivery/transfer Cases

The above studied aspects for General Framework can be considered for developing/specifying AI/ML use cases and common framework (if needed for some aspects) across AI/ML use cases.

Rel-19 continues to study different options of model identification and the associated details, and achieve the following conclusions:

- From RAN1 perspective, model identification is at least applicable to some of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported).

- The model identification procedure dedicated to MI-Option2 for one-sided model is not pursued for Rel-19 normative work.

- The model identification procedure dedicated to MI-Option5 is not pursued for Rel-19 normative work.

Rel-19 continues to study the details of model delivery/transfer cases and analyses their pros and cons, and achieves the following conclusions

- From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported).

- From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons:

  - Risk of proprietary design disclosure

  - Burden of offline cross-vendor collaboration

- From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y):

  - No much benefit compared to Case y

  - Risk of proprietary design disclosure

  - Large burden of offline cross-vendor collaboration

  - Additional burden on model storage within in 3GPP network

- From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19.

RAN1 is recommending extending the study of the model identification, and model transfer/model delivery based on RAN1 understanding the study is not completed.

CSI feedback enhancement:

CSI compression sub use case:

The performance benefit and potential specification impact were studied for AI/ML based CSI compression sub use case.

Evaluation has been performed to assess AI/ML based CSI compression from various aspects, including performance gain over non-AI/ML benchmark, model input/output type, CSI feedback quantization methods, ground-truth CSI format, monitoring, generalization, training collaboration types, etc. Some aspects were studied but not fully investigated, including the options of CQI/RI calculation, the options of rank>1 solution.

Performance gain over baseline and computational complexity in FLOPs are summarized in clause 6.2.2.8.

Potential specification impact on NW side/UE side data collection, dataset delivery, quantization alignment between CSI generation part at the UE and CSI reconstruction part at the NW, CSI report configuration, CSI report format, pairing information/procedure and monitoring approach were investigated but not all aspects were identified.

The pros and cons are analysed for each training collaboration types, and each training collaboration type has its own benefits and limitations in different aspects. The study has investigated the feasibility of the studied training collaboration types and necessity of corresponding potential RAN1 specification impact. However, not all aspects have been concluded.

Both NW side and UE side performance monitoring were studied, some but not all aspects were concluded.

From RAN1 perspective, there is no consensus on the recommendation of CSI compression for normative work.

At least the following aspects are the reasons for the lack of RAN1 consensus on the recommendation of CSI compression for normative work:

-	Trade-off between performance and complexity/overhead.

-	Issues related to inter-vendor training collaboration.

Other aspects that require further study/conclusion are captured in the summary above.

CSI prediction sub use case:

The performance and potential specification impact were studied for AI/ML based UE side CSI prediction sub use case.

Evaluations have been performed to assess AI/ML based CSI prediction from various aspects, including performance compared to baseline, model input/output type, generalization over UE speed, etc. Some aspects are studied but lack observations, including scalability over various configurations and generalization over other scenarios and approach of fine tuning. Performance monitoring accuracy has not been evaluated.

Performance compared with baseline is summarized in clause 6.2.2.8.

Potential specification impact on data collection and performance monitoring are discussed in clause 7.2.2. Limited specification aspects were considered.

From RAN1 perspective, there is no consensus on the recommendation of CSI prediction for normative work.

The reason for the lack of RAN1 consensus on the recommendation of CSI prediction for normative work:

-	Lack of results on the performance gain over non-AI/ML based approach and associated complexity.

Other aspects that require further study/conclusion are captured in the summary above.

CSI prediction sub use case in Rel-19:

From RAN1 perspective, study of CSI prediction has been completed and performance improvement is observed with increased complexity.

Beam management:

This study focuses on evaluation of potential benefits of AI/ML-based beam management and analysis of potential enhancements to enable AI/ML for beam management.

During the study, BM-Case1 (Spatial-domain downlink beam prediction) and BM-Case2 (Temporal-domain downlink beam prediction), as described in clause 5.2, are selected as the representative sub use cases.

Evaluation scenarios and KPIs are described in clause 6.3.1, and the detailed evaluation results from different sources and the key observations are captured in clause 6.3.2.  Evaluation results have shown that it is beneficial to enable AI/ML for beam management in the considered evaluation scenarios.

The necessity, feasibility, benefit and potential specification impacts of potential enhancements to enable AI/ML for beam management were studied from different aspects, and the outputs are captured in clause 7.

For AI-based beam management, from RAN1 perspective, at least the following are recommended for normative work:

-	Both BM-Case1 and BM-Case2:

-	BM-Case1: Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams

-	BM-Case2: Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams

-	DL Tx beam prediction for both UE-sided model and NW-sided model

-	Necessary signalling/mechanism(s) to facilitate data collection, model inference, and performance monitoring for both UE-sided model and NW-sided model

-	Signalling/mechanism(s) to facilitate necessary LCM operations via 3GPP signalling for UE-sided model

Positioning accuracy enhancements:

This study focused on the analysis of potential enhancements necessary to enable AI/ML for positioning accuracy enhancements with NR RAT-dependent positioning methods.

Evaluation scenarios and KPIs were identified for system level analysis of AI/ML enabled RAT-dependent positioning techniques as described in clause 6.4.

Direct AI/ML positioning and AI/ML assisted positioning were identified and selected as the representative sub-use cases. Evaluation results have shown that in considered evaluation scenarios (i.e., InF-DH, and other InF scenarios), both direct AI/ML positioning and AI/ML assisted can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods. Various aspects of AI/ML for positioning accuracy enhancement were investigated and evaluated as described in clause 6.4 that provides summary of evaluation results from different sources.

Based on the conducted analysis, it is recommended to proceed with normative work for AI/ML based positioning.

The necessity, feasibility and potential enhancements to facilitate the support of AI/ML for positioning accuracy enhancements with NR RAT-dependent positioning methods were studied and the outcome are outlined in clause 7.

It is recommended to specify necessary measurement, signalling and procedure to facilitate training, inference, monitoring and/or other LCM operations for both direct AI/ML positioning and AI/ML assisted positioning, specifically:

-	specify necessary signalling of data collection; investigate the necessity of other information for supporting data collection, and if needed, specify during normative work

-	investigate on the necessity and signalling details of measurement enhancements, and if needed, specify during normative work

-	investigate on the necessity and signalling details of monitoring method(s), and if needed, specify during normative work

A variety of enhancements for measurements (e.g., based on extensions to current positioning measurements or with new measurements) were also identified as potentially beneficial (e.g., trade-off positioning accuracy requirement and signalling overhead) and are recommended to be investigated further and if needed, specified during normative work.

Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-05 | RAN1#109e |  |  |  |  | TR skeleton | 0.0.0 |
| 2023-05 | RAN1#113 | R1-2306235 |  |  |  | RAN1 agreement up to and including RAN1#112bis-e | 0.1.0 |
| 2023-08 | RAN1#114 | R1-2308681 |  |  |  | RAN1 agreements from RAN1#113 and RAN1#114 | 0.2.0 |
| 2023-09 | RAN#101 | RP-231766 |  |  |  | TR presented for information at RAN#101 [same as R1-2308681] | 1.0.0 |
| 2023-10 | RAN1#114b | R1-2310163 |  |  |  | Added clause numbering for CSI and BM performance results. | 1.1.0 |
| 2023-11 | RAN1#115 | R1-2312055 |  |  |  | RAN1 agreements from RAN1#114b | 1.2.0 |
| 2023-11 | RAN1#115 | R1-2312764 |  |  |  | Agreements from RAN1#115, RAN2#124 and RAN4#109 | 1.3.0 |
| 2023-12 | RAN#102 | RP-233133 |  |  |  | TR presented for approval at RAN#102 [same as R1-2312764] | 2.0.0 |
| 2023-12 | RAN#102 | RP-233946 |  |  |  | Editorial corrections for RAN approval, removal of square brackets in Scope clause. | 2.0.1 |
| 2023-12 | RAN#102 |  |  |  |  | Rel-18 TR under change control further to RAN approval | 18.0.0 |

Annex B:
Informative Annex: Privacy concerns

This Annex compiles some examples of privacy concerns on data collection for UE-side model training raised by stakeholders during RAN2’s discussion.

MNO:

- Network Information Disclosure: MNOs may inadvertently disclose sensitive network information, such as deployment strategies, network configurations, and performance metrics, to servers outside their network.

- Data Transfer Risks: MNOs are obliged to protect private and/or undisclosed information about the network of users/customers when transferring data to external servers. This information may be disclosed/leaked without authorization. This could include subscriber identities, locations, website visited, phone calls, etc.

- Regulatory Restrictions: Operators are bound by regulations which mandate the protection of customer data. Thus, any lack of control over data may lead to unwanted exposure of personal information. Non-compliance with regulatory guidelines due to improper data handling could result in significant fines or restrictions for the operators.

Network Vendor:

- Sensitive Information Leakage: Network vendors may possess proprietary algorithms, system designs, and other intellectual property that are integral to their competitive edge. Unintentional disclosure of such information to third parties could undermine their market position and lead to potential legal issues.

- Implementation Details Exposure: The specific details of how network equipment is implemented, including software and hardware design, are crucial for maintaining the security and integrity of the network. If such information is disclosed, it could be exploited for malicious purposes or used by competitors to gain insights into the vendor's technology.

- Radio Topology and Settings Disclosure: Disclosing details such as radio topology and specific radio configurations should be prevented, because such information is sensitive and could affect operational security.

- Violation of user privacy regulation: The equipment of the network vendor may be used for collecting user’s data without getting approval/consent from the user in advance, and this behaviour may violate the local regulations and risks the sales of the equipment.

Chipset Vendor:

- Proprietary Technology Exposure: Chipset vendors develop specialized hardware and software that may contain trade secrets or patented technologies. There is a risk that the sensitive data could be exposed to a second vendor without the original chipset vendor's knowledge, which could compromise their competitive advantage and innovation.

- Respect for Implementation Secrecy: There is a universal understanding within the industry that chipset vendors often add proprietary layers on top of standardized specifications, and these unique implementations are critical for maintaining a diverse and successful ecosystem. The non-disclosure of such proprietary information is seen as essential for the continued success of industry standards.

OEM:

- User Information Disclosure: OEMs handle a vast amount of user data, including personal information, usage habits, and location data. There is a significant privacy concern if this information is disclosed to external entities without user consent, potentially violating privacy laws and damaging the OEM's reputation. OEMs are adamant that user data should not be shared with third-party entities without explicit and informed user consent, since a UE vendor might be legally bound by a data protection agreement with the end-user.

- Proprietary Technology Exposure: OEM vendors develop specialized hardware and software that may contain trade secrets or patented technologies. There is a risk that this information could be unintentionally disclosed to unauthorized parties, leading to privacy breaches. Another risk is that some sensitive data of an OEM vendor may be exposed to third parties without the knowledge of the OEM vendor.