3GPP TSG RAN Meeting #105																	RP-241972

Melbourne, Australia, September 9-12, 2024

Agenda item:	9.3.4.3

Source:	Apple

Title:	On scenario for CSSF optimization in NR RRM Phase 5

Document for:	Discussion

# Introduction

In previous RAN4 meetings, the scenario for enhancing CSSF outside gap in CA/DC scenarios has been discussed but unfortunately different companies had different interpretation on the WID scope description. It was hard to align the understanding inside RAN4 regarding the exact scope for CSSF enhancement, and this paper proposes to revise the WID for a clear scenario definition for future work in RAN4.

The current scenario definition for CSSF enhancement in WID [1] was highlighted below:

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

# Discussion

In current WID description, CSSF can be enhanced for UE not in multiple-Rx simultaneous reception mode, and in RAN4 discussion following interpretations are proposed [2]:

### Issue 2-1: Clarification on the bullets in WID for this CSSF optimization [Way forward]: FFS: - Option a (ZTE, CMCC, Intel, CATT, MTK): - Rel-19 CSSF optimization applies for the both cases: (1)UE is not capable of Rel-18 multi-Rx simultaneous reception, (2)UE is capable of Rel-18 multi-Rx simultaneous reception but not work in multiple-Rx reception mode currently. - Note: if option a is agreeable, then work on wording polishing based on option 1a/1b. - Option b (HW, MTK, Samsung, E///): - Rel-19 CSSF optimization applies for case that UE is not capable of Rel-18 multi-Rx simultaneous reception. - Option c (Apple, Nokia, QC): - Rel-19 CSSF optimization applies for CA/DC scenarios with at least two FR2 serving cells, independently of the UE support of multi-Rx capabilities. - Option d (newly added): - Same Rel-19 CSSF optimization applies for the both cases: (1) UE is not capable of Rel-18 and Rel-19 multi-Rx simultaneous reception, (2) UE is capable of Rel-18 or Rel-19 multi-Rx simultaneous reception but not work in multiple-Rx reception mode currently.

UE not in the multiple-Rx simultaneous reception can be two possibilities: (1)UE is not capable multi-Rx simultaneous reception, (2)UE is capable of multi-Rx simultaneous reception but not work in multiple-Rx reception mode currently. However, the multi-Rx simultaneous reception might be Rel-18 multi-Rx simultaneous reception or Rel-19 multi-Rx simultaneous reception in this WI. Since the technical part of “not work in multiple-Rx reception mode currently” is still under discussing in RAN4, companies cannot directly agree on option a or option d.

The different interpretation of the scenario of WI may result into different discussion directions and will cause problem to converge the solutions from companies. Thus, we propose to clarify the scenario to apply CSSF enhancement to facilitate the WI discussion, i.e., consider the CSSF enhancement if UE is not performing multiple-Rx based simultaneous L3 measurement.

In order to make the discussion more efficient, we propose to clarify the unclear scenario of “UE is capable of multi-Rx simultaneous reception but not work in multiple-Rx reception mode currently”. The description in the WID can be revised to more explicit that only consider the CSSF enhancement if UE is not performing multiple-Rx based simultaneous L3 measurement.

Proposal 1: Revise the WID to only consider the CSSF enhancement for the case when UE is not performing multiple-Rx based simultaneous L3 measurement.

The proposed revision is below:

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not performing multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

# Conclusion

This paper proposes to revise the WID for a clear scenario definition for future work in RAN4.

Proposal 1: Revise the WID to only consider the CSSF enhancement for the case when UE is not performing multiple-Rx based simultaneous L3 measurement.

The proposed revision is below:

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not performing multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

# References

[1] RP-240830, New WID WI resulting from discussion on RRM enhancements, RANP#103

[2] R4-2414026, WF for NR_RRM_Ph5_Part1, Apple, RAN4#112