3GPP TSG RAN Meeting #105	RP-241975

Melbourne, Australia, September 9-12, 2024

Agenda item:	9.3.1.3

Source:	SK Telecom

Title:	On the checkpoint of Rel-19 AI/ML based CSI prediction

Document for:	Discussion and Decision

1	Introduction

At the RAN #102, Rel-19 WI on AI/ML for NR Air Interface is approved and it includes several study objectives on the CSI (Channel State Information) feedback enhancement with corresponding checkpoint at RAN #105 (Sept. ’24) [1]. According to these study objectives, further study of CSI prediction sub use case in Rel-19 is started at RAN1 #116 (Feb. ’24) and it has been completed from RAN1 perspective at RAN1 #118 (Aug. ’24) with following evaluation summary and conclusion [2].

Observation Based on the evaluation for CSI prediction, the following high-level observations are provided: - From the perspective of basic performance gain over benchmark of non-AI/ML based CSI prediction, under the same UE speed for training and inference - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 0%~7.8% gain depending on traffic model, in terms of mean UPT, is observed by 7 sources - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3.8%~20.7% gain depending on traffic model, in terms of 5% UE UPT, is observed by 7 sources. - From a perspective of AI/ML complexity, 19 sources adopt the model subject to the computational complexity in units of FLOPs from 0.05M to 3000M. For complexity of non-AI/ML benchmark, 16 sources adopt the algorithm (e.g., Kalman filter, Auto-regression, Wiener filter) subject to the computational complexity in units of FLOPs from 0.14M to 107M. The ratio of FLOPs (AI/ML over benchmark 2) ranges from 1 to 35, which is used by majority sources. For non-AI/ML benchmark, main computation complexity is dominated by filter updates, which may not be need to be updated per inference at the expense of performance loss. - From the perspective of performance impact on channel estimation error and phase discontinuity, compared to non-AI/ML CSI prediction, higher gain is observed by 10 sources for AI/ML based CSI prediction in the presence of channel estimation error or phase discontinuity. - From the perspective of generalization over various scenarios/configurations (e.g., various UE speed, deployment scenario, carrier frequency) that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain scenario#B/configuration#B and applied for inference with a same scenario#B/configuration#B - For generalization Case 2 where the AI/ML model is trained with dataset from a different scenario#A/configuration#A, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others. - For generalization Case 3 where the training dataset is constructed with data samples subject to multiple scenarios/configurations including scenario#B/configuration#B, generalized performance of the AI/ML model can be achieved. Agreement From RAN1 perspective, study of CSI prediction has been completed and performance improvement is observed with increased complexity.

Although RAN1 concluded that study of CSI prediction has been completed and the performance improvement is observed with increased complexity, RAN1 fail to draw any recommendation for normative work in Rel-19. In this contribution, we discussed our views on the scope for Rel-19 AI/ML based CSI Prediction.

2	Discussion

2.1	Key use case of AI CSI prediction at UE

Globally, mid-band TDD (Time Division Duplex) is major 5G spectrum. Nearly all FDD (Frequency Division Duplex) spectrums in low-band are already occupied by previous generation of cellular network technologies such as 3G, 4G. Also, low-band spectrum inherently has smaller bandwidth (e.g., 10MHz pair) compared to that of mid-band (e.g., 100MHz unpair) and thus it is impossible to provide sufficient throughput performance boost that 5G customers are normally expecting to their operators.

In mid-band TDD, at cell center, uplink SRS (Sounding Reference Signal) estimation results are normally used for downlink beamforming weight design (i.e., channel reciprocity based beamforming) due to its huge performance gain compared to other method if there are available SRS antenna switching resource for UEs (User Equipments). However, at cell edge, only CSI report from UE is available for downlink beamforming weight design (i.e., codebook based beamforming). Each SRS is uplink 1Tx signal and the purpose of receiving SRS at the base station is to obtain channel state information between each Tx antenna port of UE and each Rx antenna port of base station and thus it is impossible to do any Rx combining for enhancing signal quality at the gNB receiver (e.g., maximum ratio combining among 64/32 Rx ports in base station for uplink data channel) as discussed in [3].

In other words, key use case of ‘Rel-19 AI/ML based CSI prediction’ is cell edge performance enhancement as shown in Figure 1 below. At the same time, from Rel-19 evaluation summary, AI/ML based CSI prediction shows 3.8%~20.7% gain over non-AI/ML based CSI prediction in terms of 5% UPT which represents cell edge performance gain. It is much higher than 0~7.8% mean UPT gain which represents overall performance gain including cell center.

[Figure 1. Key use cases of AI CSI prediction at network and UE]

- In mid-band TDD, key use case of Rel-19 AI/ML based CSI prediction at UE is cell edge performance enhancement (At cell center, SRS based beamforming is used).

- From Rel-19 evaluation summary, AI/ML based CSI prediction shows much higher gain in terms of 5% UPT which represents cell edge performance gain compared to mean UPT which represents overall performance gain including cell center.

2.2	Complexity of AI/ML workload

Traditional RAN physical layer processing has some limitation on hardware acceleration based on parallel processing due to its sequential characteristic (e.g., cyclic redundancy check & segmentation → forward error correction encoding & rate matching → modulation, layer and resource element mapping → precoding & beamforming → fronthaul transmission to radio unit). However, in contrast to this, new AI/ML workload (e.g., inference of AI/ML based CSI prediction) is suitable to hardware acceleration based on the latest computer architecture designed for this purpose (i.e., parallel processing of AI workload) such as GPU (graphics processing unit) and NPU (neural processing unit).

Also, more importantly, cost-to-performance of hardware accelerators for AI inferencing continue to advance at a rapid pace including on-device chipsets as the global demands for AI processing skyrocketed. Therefore, we believe that the increased complexity of AI/ML based CSI prediction over non-AI/ML benchmark is not critical bottleneck for future commercialization of AI/ML based CSI prediction feature.

- AI/ML workload (e.g., inference of AI/ML based CSI prediction) is suitable to hardware acceleration based on the latest computer architecture designed for this purpose (i.e., parallel processing of AI workload) such as GPU and NPU.

- Considering rapid improvement of AI hardware accelerator, increased complexity of AI/ML based CSI prediction over non-AI/ML benchmark is not critical bottleneck.

3	Conclusion

In this contribution, we discussed our views on the scope for Rel-19 AI/ML based CSI Prediction.

- In mid-band TDD, key use case of Rel-19 AI/ML based CSI prediction at UE is cell edge performance enhancement (At cell center, SRS based beamforming is used).

- From Rel-19 evaluation summary, AI/ML based CSI prediction shows much higher gain in terms of 5% UPT which represents cell edge performance gain compared to mean UPT which represents overall performance gain including cell center.

- AI/ML workload (e.g., inference of AI/ML based CSI prediction) is suitable to hardware acceleration based on the latest computer architecture designed for this purpose (i.e., parallel processing of AI workload) such as GPU and NPU.

- Considering rapid improvement of AI hardware accelerator, increased complexity of AI/ML based CSI prediction over non-AI/ML benchmark is not critical bottleneck.

Based on the discussion in the previous sections we propose the following:

- Specify AI/ML based CSI prediction using UE-side model in the scope of Rel-19 AI/ML for NR Air Interface WI including at least following:

- Necessary signaling/mechanism(s) to facilitate LCM (life cycle management) operations specific to the CSI prediction use case, if any

References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm (Moderator)

- R1-2407341, Summary #4 of CSI prediction, Moderator (LG Electronics)

- RP-232011, On CSI-RS extension beyond 32 ports in R19 MIMO evolution, SK Telecom, Deutsche Telekom