3GPP TSG RAN meeting #105										RP-241984

Melbourne, Australia, September 9-12, 2024

Status Report to TSG

Agenda item:			9.3.4.9

| WI / SI Name | Enhanced requirements and conductive test methodology for NR NTN and IoT NTN |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_IoT_NTN_req_test_enh |  |  |  |  |
| Unique ID | 1030086 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241281 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 16% | Performance Part: 10% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Runsen Tang |
|  | Company | Samsung |
|  | Email | runsen.tang@samsung.com |
| Rapporteur | Name | Juan Zhang |
|  | Company | Xiaomi |
|  | Email | zhangjuan8@xiaomi.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.4	RAN4

2.4.1	Agreements

In RAN4#112, the following agreements were made.

General aspects and work plan

Agreement:

- The work plan in R4-2412554 is approved.

UE RF requirements for NTN HPUE

General starting point for co-ex assumptions and scenarios

Agreement:

- Agree to use TR 38.863 and WF R4-2217473 for NR-NTN and IoT-NTN HPUE coex study assumptions and scenarios as starting point.

  - The detailed modifications to these references will be discussed and agreed in case-by-case manner.

Issue 2-2-1: NTN scenarios for co-ex study

Agreement:

- For the scenario, consider the scenarios in TR 38.863 as a baseline

  - Prioritize GEO and LEO1200 for co-existence evaluation

    - LEO600 is not precluded for the requirements and the conclusion for LEO1200 can be applied to LEO600

Issue 2-2-2: TN scenarios for co-ex study

Agreement:

- Prioritize Urban Macro for Scenario 4 and Rural Macro for Scenario 5

Issue 2-2-3: Co-ex scenario # to be studied

Agreement:

- Consider both Scenario 4 and 5 for co-existence study

  - Prioritize scenario 4 since it is the worst case

- Do not consider Scenario 1, 2, 3 and 6

Issue 2-3-1 NTN and TN network isolation distance

Agreement:

- Consider isolation distance in scenario 4 and 5 for both NR-NTN HPUE coex and IoT-NTN HPUE coex.

- Use the isolation distance 1.5km as the starting point

- Other values for isolation distance are not precluded

Issue 2-3-2 TN network topology

Agreement:

- Reuse existing method from TR 38.863 as baseline, and encourage companies to further evaluate any performance impact with and w/o wrap-around functionality.

Issue 2-3-3 NTN UE dropping methods

Agreement:

- Random dropping on circle, and to use uniform distribution for random.

Issue 2-3-4 Observed TN cells for Scenario 4 evaluation in Table 6.2.1.1-1 in TR 38.863

Agreement:

- Option 1 as baseline assumption, other options not precluded.

Issue 2-3-5 NTN HPUE ratio

Agreement

- Assuming all NR-NTN/IoT-NTN UEs are HPUE, i.e. 100% HPUE ratio as baseline.

- FFS other ratio values

Issue 2-3-6: NR and NR-NTN channel bandwidth

Agreement

- Consider 20MHz system bandwidth for NR-NTN and NR in co-existence study.

Issue 2-4-1: NR-NTN SAN parameter set

Agreement:

- To use Set-1 as NTN SAN parameters.

Issue 2-4-2: IoT-NTN UL UE number and SCS

Agreement

- Consider 9 UEs for 15kHz, 18 and 36UEs for 3.75kHz single tone as starting point

- FFS whether to down scope.

Issue 2-4-3: NR-NTN UE parameter for PC2

Agreement

- Baseline: Use NF as 9dB, and Tx power 26/29/31 dBm for PC2/1.5/1 NTN HPUE

- FFS option of 7dB NF for PC2 and LEO600.

Issue 2-4-4: NTN HPUE uplink power control

Agreement

- To update the existing uplink power control parameters considering different power classes and bandwidth

  - Update Pmax to 26/29/31 for PC2/1.5/1;

  - Update Rmin to -66/-69/-71 assuming Pmin as -40;

  - Update BW to corresponding transmission BW.

- Agree on the following detailed parameters

where:

- Pmax = 26dBm for PC2, 29 dBm for PC1.5, 31 dBm for PC1.

-Rmin = -66 dB for PC2, -69 dB for PC1.5, -71 dB, assuming -40dBm minimum output power,

-CLx-ile and γ are set as following:

-CLx-ile = 10*log10(Pmax) – (SNRtarget + 10*log10(kTB) + NF ),

Where:

SNRtarget and BW

For NR-NTN HPUE:

Option 1: 3 dB SNR target and 5MHz UL BW (One company proposed for LEO only)

Option 2: 15 dB SNR target and 2RB UL BW as baseline assumption

NF is the SAN noise figure, i.e. 4.3dB.

10log10(kT) = -174dBm/Hz.

γ = 1 For uplink scenario.

Issue 2-5-1: Power classes for co-ex study

Agreement

- All power classes listed in WID will be considered for co-existence study.

Issue 2-5-2: Handheld and non-handheld type

Agreement:

- To use TR 38.863 UE characteristics as starting point for NTN HPUE co-ex studies.

Issue 2-6-1: Preliminary results and observations

Agreement

- Please take submitted results and analysis for information

- Encourage companies to follow work plan and submit co-ex results starting from next meeting.

Issue 2-6-2: Running document to capture assumptions

Agreement

- To revise the R4-2412557 in this meeting to capture agreements for information.

Issue 3-1-1: General considerations

Agreement

- The study of NTN HPUE Tx requirements generally would have to consider co-existence results, UE implementation feasibility, and existing HPUE requirements.

Issue 3-1-2: Consideration of band(s)

Agreement:

- Use 2GHz for the co-existence study

- WI can be completed when the band specific requirements for one pair of {n256,256}and {n255, 255} are completed in this WI

  - The contributions on the other existing Rel-18 FR1-NTN bands for specifying common requirements are allowed.

- Specify the band specific requirements for other potential NTN bands in the separate spectrum WI

Issue 3-1-3: Consideration of power classes

Agreement

- All power classes in WID, including Power class 1/1.5/2 for NR-NTN and Power class 1/2 for IoT-NTN, will be further discussed

Issue 3-1-4: Consideration of regulation(s)

Agreement

- Applicable regulations should be taken into account for requirements discussions.

- If any identified corrections to be made for PC3, those should be considered in another work item, it is not in the current scope of this WI.

Issue 3-1-5: Consideration of HPUE Architecture

Agreement:

- For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements.

- For PC2 IoT NTN, at least 1Tx shall be considered

- For other PCs (both IoT NTN and NR NTN), FFS

Issue 3-2-1: NR-NTN HPUE feasibility for different power classes

Issue 3-2-2: NB-IoT based IoT-NTN HPUE feasibility for different power classes

Issue 3-2-3: eMTC based IoT-NTN HPUE feasibility for different power classes

Agreement of Issue 3-2-1 to Issue 3-2-3:

- For PC2, it is feasible to support both handheld and non-handheld devices.

- FFS other power classes for applicable device type.

Issue 3-3-1: Maximum output power

Agreement

- Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance can be considered as starting point for PC2/1.5/1 for NTN HPUE including NR-NTN and IoT-NTN.

Issue 3-3-2: MPR

Agreement:

- FFS the MPR requirements. Following potential factors to be further discussed:

  - ACLR from co-existence study

  - Current TN MPR requirements

  - MPR simulation assumptions

  - Other factors not precluded if identified.

Issue 3-3-3: A-MPR

Agreement

- FFS A-MPR requirements. Following potential factors to be further discussed:

  - Simulation reduction by not considering large RB allocations, CP-OFDM, higher-order modulations

  - A-MPR simulation assumption including ACLR from co-existence study

  - Applicable bands

  - Regional requirements for A-MPR

  - Other factors

Issue 3-3-4: SAR for handheld

Agreement:

- Using P-MPR as the starting point

- The other solutions are not precluded

  - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling

Issue 3-3-5: SEM

Agreement:

- For NR-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs

- For IoT-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs

Issue 3-3-6: ACLR

Agreement:

- The ACLR requirements will depend on co-existence study outcomes.

Issue 4-1: General considerations

Agreement

- Rx RF requirements including Max input level, ACS, blocking characteristics, spurious response, intermodulation characteristics, Rx spurious emissions in existing spec can be applied to other power class.

Issue 4-2: RSD for NR-NTN

Agreement

- Reference sensitivity degradation (RSD) requirements is needed for NTN HPUE.

  - Further discuss the following aspects:

    - Use similar way in TN spec by introducing a new table of RSD compared to the PC3

    - Whether default Tx-Rx frequency separation is used for test point

Issue 4-3: RSD for IoT-NTN

Agreement:

- For NB-Iot based Iot-NTN, no RSD needed for HPUE

- For eMTC based Iot-NTN with half duplex, no RSD needed for HPUE

- For eMTC based Iot-NTN with full duplex, further discuss RSD impact

Less than 5MHz for NTN (BS RF part)

BS requirements:

Agreements:

| NR SAN RF conducted Tx/Rx requirement | Proposal for 3MHz channel bandwidth in FR1-NTN bands |
| --- | --- |
| 6.2 Satellite Access Node output power | No specification impact. |
| 6.3.2 RE power control dynamic range | No specification impact. |
| 6.3.3 Total power dynamic range | Minimum requirement calculated according to transmission bandwidth configuration in Table 6.3.3.2-1. |
| 6.4 Transmit ON/OFF power | No specification impact. |
| 6.5 Transmitted signal quality | No specification impact. |
| 6.6.2 Occupied bandwidth | No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 6.6.3.2-1 and Table 6.6.3.2-2. |
| 6.6.4 Out-of-band emissions | No specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact |
| 6.7 Transmitter intermodulation | No specification impact. |
| 7.2 Reference sensitivity level | Add minimum requirements for 3 MHz in Table 7.2.2-1 and Table 7.2.2-2. Scaling from 5 MHz can be used. Use FRC G-FR1-A1-7, same as in TN. |
| 7.3 Dynamic range | Add minimum requirements for 3 MHz in Table 7.3.2-1. Scaling from 5 MHz can be used. Reuse TN FRC G-FR1-A2-15 (to be introduced to A.2). |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Define ACS requirement similar to TN for 3MHz in Table 7.4.1.2-1, and assume same interferer frequency offset values as TN 3 MHz in Table 7.4.1.2-2. |
| 7.4.2 In-band blocking | No specification impact. |
| 7.5 Out-of-band blocking | No specification impact. |
| 7.6 Receiver spurious emissions | No specification impact. |
| 7.7 Receiver intermodulation | No specification impact. |
| 7.8 In-channel selectivity | Add minimum requirements for 3 MHz in Table 7.8.2-1 and Table 7.8.2-2. Reuse TN FRC G-FR1-A1-20 (to be introduced to A.1). |
| A.1 Fixed Reference Channels for RF Rx requirements (QPSK, R=1/3) | FRC for 3MHz in-channel selectivity need to be added in Table A.1-1. Reuse TN FRC G-FR1-A1-20. Rename the FRC to indicate “NTN” can be discussed as Rel-18 maintenance. |
| A.2 Fixed Reference Channels for dynamic range (16QAM, R=2/3) | FRC for 3MHz needs to be added in Table A.2-1. Reuse TN FRC G-FR1-A2-15. Rename the FRC to indicate “NTN” can be discussed as Rel-18 maintenance. |

Less than 5MHz for NTN (UE RF part)

Issue 1-1-1: Channel bandwidth

Agreement

- Only introduce 3MHz channel bandwidth with 15kHz SCS

Issue 1-1-2: Operating bands for 3MHz channel bandwidth

Agreement

- Applicable NTN bands: band n256, n255 and n254

Issue 1-1-3: Maximum transmission bandwidth configuration(NRB)

Agreement

- Reuse 15 RBs for 3MHz channel bandwidth with 15kHz SCS for NR NTN.

Issue 1-1-4: Minimum guardband

Agreement

- Reuse 142.5kHz minimum guardband for 3MHz channel bandwidth with 15kHz SCS for NR NTN.

Issue 1-1-5: Asymmetric channel bandwidth

Agreement

- Introduce 3 MHz channel bandwidth for UL in n254 in Table 5.3.6-1 as 3MHz UL + X MHz DL

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n254 | 3 | 10,15 | 1 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [11], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

Issue 1-2-1: Channel spacing

Agreement

- Reuse the existing channel spacing formula in TS 38.101-1

Issue 1-2-2: Channel raster

Agreement

- Adopt 100kHz and 10kHz channel raster for 3MHz channel bandwidth.

Issue 1-2-3: Synchronization raster

Agreement

- Reuse TN sync raster for 3MHz channel bandwidth and extend to 3000MHz for NTN FR1 3MHz channel bandwidth as below table:

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:4999, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 41636 |

- Include note1:

  - NOTE 1: Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.

Issue 1-2-4: Additional sync raster

Agreement:

- No additional sync raster point is needed for n254, n255 and n256

Issue 1-2-5: SS raster entry

Agreement:

- Option 1: Applicable SS raster entries can be defined as below for 3 MHz channel bandwidth in bands n256, n256, n254

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 37492 – <1> – 37629 |
| n255 | 15 kHz | Case A | 34267 – <1> – 34424 |
| n254 | 15 kHz | Case A | 39060 – <1> – 39129 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Less than 5MHz for NTN (RRM part)

Issue 1-1-1: Clarification on 15KHz SC

Agreement:

- To confirm only 15KHz SC needs to be considered.

Issue 1-1-2: CA

Agreement:

- Not define any CA related RRM requirements for NTN less than 5MHz.

Issue 1-1-3: CSI-based measurements

Agreement

- Prioritize SSB based RLM and L1 measurement for NTN less than 5MHz

Issue 1-1-4: Other enhanced aspects beyond Rel18

Agreement:

For less than 5MHz NTN in this WI:

- Not define requirements for positioning, CGI, mTRP, CSI-RS based L3 measurement.

- Not define requirements for Redcap.

Issue 1-1-5: Applicability requirement clarificaiton

Agreement

- An applicability requirement is needed to clarify which R17/R18 NTN requirement can be applied for less than 5MHz band without any change in R19.

Issue 1-2-1: Baseline requirement which can be taken as the start point for NTN less than 5MHz requirements

Agreements:

- RRM requirement for TN less than 5MHz shall be used as baseline to design NTN less than 5MHz requirement

Issue 1-2-2-1: Channel bandwidth assumption

Agreement:

- To align with the agreement in BDaT session.

Issue 1-2-2-2: SSB/PBCH assumption

Agreement

- To align with the agreement in BDaT session.

Issue 1-2-2-3: CORRSET

Agreement

- To align with the agreement in BDaT session.

Issue 1-3-1: whether IDLE/INACTIVE RRM requirement for NTN in current TS38.133 will be impacted becasue of less than 5MHz bandwidth

Agreement

- No core part impact.

Issue 1-3-2: Handover

Agreement:

- Define HO delay requirement for NTN less than 5MHz, and take extension of delay for TN less than 5MHz as baseline.

- For NTN specific HO requirement:

  - Time/location-based CHO delay requirement will be defined for NTN less than 5MHz.

  - Satellite switch time requirement will be defined for NTN less than 5MHz.

  - Further discuss the details of the requirements.

NTN testing for NGSO

Issue 1-1-1: Work plan on channel model and requirements

Agreement

- Focus on TE-emulated channel model in initial stage for NGSO testing

    - Once sufficient progress made for channel model introduction, RAN4 can start the discussion on corresponding requirements and tests according to WI objective.

Issue 1-1-2: Relation-ship between TE-emulated channel model for satellite motion and fading channel model

Agreement:

- Decouple TE-emulated time-varying Doppler (frequency) and delay shifts due to satellite mobility from traditional fading channel

Issue 1-2-1 Methodology for Time varying Doppler and Delay shifts modelling

Agreement

- RAN4 aims to introduce TE-emulated time varying doppler shift and delay shift model for satellite motion with following candidate options:

  - Option 1: Eckstein-Hechler model

  - Option 2: Keplerian model

  - Option 3: Existing model from TR 38.811 5.3.4.3

  - Option 4: Proposal 2 from R4-2412552

dht=modvNGSO⋅t, 2dh,max-dh,max Tt=dtc=dh(t)2+hNGSO2c Ft=-fddhtd(t)=-fddh(t)dht2+hNGSO2 where - Tt: Propagation delay of the signal from the satellite at time t, - F(t): Doppler (frequency) shift of the signal from the satellite at time t. - dh(t): the horizontal distance between the satellite and UE at time t, - d(t): the distance between the satellite and UE at time t, - dh,max: the maximum horizontal distance between the satellite and UE, i.e., hNGSO/tan⁡(E), - c: the speed of light, - fd: Maximum Doppler shift (Hz)

  - Option 5: Proposal 2 from R4-2412783

- The instant satellite speed v is given as: v=μR+h - The initial angle β of vector geocentric to UE relative to the x axis is given as: β=π-α-arcsin⁡R∙cosαR+h - The satellite position s is given as: s=j∙R+h - The UE position u is given as: u=R∙ej∙β+v∙tR+h - The Doppler fd is given as: fd=vc∙fc∙realu-su-s - The delay T is given as: T=¦u-s¦c where μ=398600.4418×109m3s2 is standard gravitational parameter, R=6371008.7714m is earth average radius, h=600000m is satellite altitude for LEO-600, α is initial angle of vector UE to satellite relative to vector UE to clockwise side horizon, t is the time, c=299792458ms is the speed of light, fc=2×109Hz is the carrier frequency.

  - Other options not precluded

Note: Companies are encouraged to bring more information for option 1 and option2.

- Predefined files which generated by GMAT can be considered as reference for the comparison purpose of proposed channel model

  - FFS on the initial ephemeris information and parameters

Issue 1-2-2 Parameters for TE-emulated channel model

Agreement

- RAN4 further discuss the necessary parameters with associated channel model, following parameters list for information purpose:

  - Satellite altitude:

    - Option 1: LEO -600km only

    - Option 2: Both LEO-600km and LEO-1200km Elevation angle range

  - UE position

  - Whether to consider a non-zero time-varying feeder link delay

- Other parameters not precluded

Issue 1-2-3 Test procedure/configuration

Agreement

- During test, ephemeris information shall be updated periodically and aligned with TE-emulated channel model which match with satellite motion trajectory

Issue 1-3-2 demodulation requirements

Agreement

- RAN4 further discuss how to apply the varying Doppler and delay shifts to the UE performance requirements and necessary simulation assumption on UE side pending on channel model discussion progress.

  - UE assumption on propagator model for satellite motion for RAN4 simulation purpose

    - Option 1: Keplerian model

    - other options not precluded

- RAN4 further discuss test applicability if new PDSCH demodulation requirements introduced pending on channel model discussion progress.

2.4.2	Remaining Open issues

  - NTN HPUE co-existence study results collection

  - Remaining details on NTN HPUE RF requirements

    - IoT-NTN HPUE RF requirements

    - NR-NTN HPUE RF requirements

  - Remaining details on BS RF requirements for NTN less than 5MHz

  - Remaining details on UE RF requirements for NTN less than 5MHz

  - Remaining details on RRM requirements for NTN less than 5MHz

  - Remaining details on NTN testing for NGSO

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#112

R4-2411062	Discussion on SAN RF requirements for NTN support spectrum less than 5 MHz

R4-2411063	Discussion on UE RF requirements for NTN support spectrum less than 5 MHz

R4-2411064	DraftCR for TS 38.101-5, Introduction on system parameters for UE supporting less than 5 MHz channel bandwidth

R4-2411065	DraftCR for TS 38.108, Introduction on system parameters for SAN supporting less than 5 MHz channel bandwidth

R4-2411066	Discussion on coexistence study for NTN support HPUE

R4-2411067	Discussion on UE Tx requirement for NTN support HPUE

R4-2411144	On restricted set of A-MPR simulations for the satellite band UE Tx requirements

R4-2411145	Initial A-MPR results for PC2 on NTN L-/S-band

R4-2411352	Discussion on less than 5MHz for NTN RRM requirement

R4-2411453	On less than 5MHz for NTN RRM

R4-2411467	Discussion on Enhanced RRM requirements and conductive test methodology for NTN

R4-2411499	Discussion on NTN HPUE TX RF requirements

R4-2411500	Discussion on NTN HPUE RX RSD requirements

R4-2411505	Discussion on RF requirements in NR_IoT_NTN_req_test_enh WI for NTN HPUE

R4-2411540	emission limit for HPUE in IoT NTN

R4-2411541	Supporting 3MHz BW for Redcap/eRedcap NTN

R4-2411549	Initial views on HPUE for NTN in bands n253, n255 and n256

R4-2411602	Discussion on co-existence of NTN HPUE

R4-2411603	Discussion onTx requirements of NTN HPUE

R4-2411604	Discussion on system parameters of less than 5MHz for NTN

R4-2411605	Discussion on UE RF requirements of less than 5MHz for NTN

R4-2411620	Discussion on RRM requirements for less than 5MHz NTN

R4-2411659	Discussion on HPUE TX requirements for NR NTN

R4-2411710	Discussion on NTN testing for NGSO

R4-2411722	Discussion on NTN testing for NGSO

R4-2411771	coexistence study of NTN HPUE

R4-2411811	Topic summary for [112][216] NR_IoT_NTN_req_test_enh

R4-2411855	Discussion on system parameters for NTN less than 5MHz

R4-2411856	Discussion on UE RF requirements for NTN less than 5MHz

R4-2411857	Discussion on SAN RF requirements for NTN less than 5MHz

R4-2411949	Discussion on less than 5 MHz NTN system parameter aspects

R4-2412071	Discussion on the co-existence simulation assumptions for NTN HPUE

R4-2412077	Discussion on system parameters for less than 5MHz in FR1-NTN bands

R4-2412078	Discussion on UE RF requirements for less than 5MHz in FR1-NTN bands

R4-2412099	Feasibility study and Initial analysis for NTN HPUE RF Tx Requirements

R4-2412100	Initial analysis for NTN HPUE RF Rx Requirements

R4-2412111	Discussion on RRM impacts on Rel-19 NR-NTN in less than 5MHz

R4-2412125	On Co-existence considerations for NR and IoT NTN related to High Power UE (HPUE) for NTN

R4-2412235	Discussion on RRM requirements for enhanced requirements and test methodology for NR and IoT NTN

R4-2412357	Discussion on Tx requirements for NTN HPUE

R4-2412381	Discussion on system parameters for NTN support spectrum less than 5 MHz

R4-2412435	NTN UE RF requirements for less than 5MHz CBW

R4-2412437	NTN System parameters for less than 5MHz CBW

R4-2412463	Simulation for NTN co-existence study

R4-2412528	SAN RF requirements for Less than 5MHz supporting in NTN FR1 bands

R4-2412529	System parameters for Less than 5MHz supporting in NTN FR1 bands

R4-2412530	UE RF requirements for Less than 5MHz supporting in NTN FR1 bands

R4-2412532	NTN testing for NGSO

R4-2412552	TE-emulated channel model for NTN UE demodulation requirements

R4-2412554	Workplan for enhanced requirements and conductive test methodology for NR-NTN and IoT-NTN

R4-2412555	Discussion on coexistence study scenarios and assumptions

R4-2412556	Preliminary HPUE NR-NTN and IoT-NTN co-ex results for information

R4-2412557	Running documents for co-ex assumptions

R4-2412558	Discussion on NTN HPUE Tx requirements

R4-2412559	Discussion on NTN HPUE Rx requirements

R4-2412668	Initial discussion on RRM requirements for less than 5MHz for NTN

R4-2412718	Discussion on coexistence study for NTN HPUE

R4-2412719	Discussion on Tx requirements for NTN UE

R4-2412720	Discussion on Rx requirements for NTN UE

R4-2412725	Initial discussion on HPUE NTN RX requirement

R4-2412726	Initial discussion on HPUE NTN TX requirement

R4-2412783	Discussion on NTN testing for NGSO for demodulation performance requirements

R4-2412829	Topic summary for [112][127] NR_IoT_NTN_HPUE

R4-2412838	Discussion on NTN HPUE co-existence study

R4-2412839	Discussion on NTN HPUE TX requirements

R4-2412840	Discussion on NTN HPUE RX requirements

R4-2412866	Satellite Trajectory Motion and Channel Model for NTN test cases

R4-2412867	On RRM scope of work for less than 5 MHz

R4-2412922	Coexistence study for NTN HPUE

R4-2412963	General discussion on coexistence study for NR NTN HPUE

R4-2412964	General discussion on Tx requirements for NR NTN HPUE

R4-2412965	General discussion on Rx requirements for NR NTN HPUE

R4-2413043	Discussion on RRM requirements for less than 5M in NTN

R4-2413144	Support for NTN high power UE

R4-2413145	System parameters and synchronization raster for NTN less than 5 MHz

R4-2413189	(NR_IoT_NTN_req_test_enh-Core) Support of less than 5MHz for NR NTN

R4-2413232	Views on NR NTN Less than 5 MHz within wider channel BW

R4-2413333	Discussion on dynamic NTN testing for NGSO

R4-2413352	Discussion on FR1-NTN High Power UE (HPUE)

R4-2413361	On system parameters for less than 5 MHz for NTN

R4-2413362	On UE RF requirements for less than 5 MHz for NTN

R4-2413363	On SAN RF core requirements for less than 5 MHz for NTN

R4-2413365	On UE RF Tx requirements for NTN HPUE

R4-2413366	On UE RF Rx requirements for NTN HPUE

R4-2413412	Topic summary for [112][312] NR_IoT_NTN_less_than_5MHz_UERF

R4-2413413	Topic summary for [112][313] NR_IoT_NTN_less_than_5MHz_BSRF

R4-2413429	Topic summary for [112][329] NTN_testing_NGSO_channel_model

R4-2413516	Way Forward for [112][329] NTN_testing_NGSO_channel_model

R4-2413518	Way Forward for [112][313] NR_IoT_NTN_less_than_5MHz_BSRF

R4-2413528	Way Forward for [112][312] NR_IoT_NTN_less_than_5MHz_UERF

R4-2413974	WF for RRM of NTN with channel BW less than 5MHz

R4-2414275	WF on UE RF requirements for NTN HPUE

R4-2414276	Simulation assumption for co-existence study

R4-2414452	WF on UE RF requirements for NTN HPUE

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template