3GPP TSG RAN Meeting #105								RP-242011

Melbourne, Australia, Sep 9-12, 2024

Status Report to TSG

Agenda item:			9.3.4.2

| WI / SI Name | # NR base station (BS) RF requirement evolution for FR1/FR2 and testing |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_BS_RF_req_evo |  |  |  |  |
| Unique ID | 1030078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241284 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 09/2025 | Performance part: 03/2026 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 20% | Performance Part: 5% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Xue, fei |
|  | Company | ZTE Corporation |
|  | Email | Xue.fei25@zte.com.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4 #112:

- Agreement:

- Without the extension of EERIP definition for generic use case;

- Expected EIRP (EEIRP) is defined as the average value of the EIRP, with the averaging being performed over  supported weighted beamforming directions over time within the BS horizontal and vertical steering range and the averaging being performed over horizontal angles from −180° to +180° and the specified elevation angle range θHL ≤ θ < θHH in Table 9.9.2-1.

- FFS on the definition of beam steering range and further clarify the relationship between beam steering range and beam peak direction sets.

- Keep the above definition in the clause 9.9

- θHL ≤ θ < θHH and capture this in the  abbreviation section.

Issue 2-2  CAR related issue

- Agreement:

  - To declare the supported mechanical tilt for CAR/beam steering direction sets for BS conformance testing

Issue 2-3  The applicability of multi-user beamforming

- Agreement:

  - To focus on the single user beamforming for EEIRP measurement.

Issue 4-1:

- Agreement:

  - Proposal 1: EEIRP calculation in discrete form in the conformance test specification.

EIRPθn,φm=k=1KwkEIRPkθn,φm,αk,βk

, where wk is a beam weighting and EIRPkθn,φm,αk,βk is the EIRP pattern per measured test beam.

- EEIRPi=θHHi−θHLisinθHHi−sinθHLiMN∙n=nLinHim=1MEIRPθn,θmcosθn

- Where

- M is the number of the equally divided intervals over the horizontal (azimuth) range of −180° to +180°;

- N is the number of the equally divided intervals over the specified vertical (elevation) range of θL to θH.

- nL	is the lowest elevation sampling angle within the θL , θH bounding range

- nH	is the highest elevation sampling angle within the θL , θH bounding range

- θn is the vertical angle measured within the n-th interval out of the vertical range of θL to θH, where the interval is from θL+n−1NθH−θL to θL+nNθH−θL,

- φm is the horizontal angle measured within the m-th interval out of the horizontal range of −180° to +180°, where the interval is from −π+2π∙m−1M to −π+2π∙mM.

- i means different bin index.

Issue 4-5: Test environment

- Agreement:

  - Only normal condition is considered.

Issue 4-8: Test tolerance requirements

- Agreement:

  - To agree the test tolerance as 0dB;

2.4.2	Remaining Open issues

- Core part:  Open issues for co-location reference antenna

  - Investigate whether the BS/IAB OTA co-location reference antenna definition need be improved for FR1, and if feasible, update the definition.

- Per part:  open issues for U6GHz EIRP mask

Issue 4-2: Test vectors for EEIRP mask

- Agreement:

  - Companies are encourage to provide the inputs for test beams  for EERIP measurement

Issue 4-3: Measurement grid over elevation angle and azimuth angle

- Agreement:

  - Companies are encourage to provide the inputs for measurement step size  for EERIP measurement;

Issue 4-4: RF channels for the conformance testing

- Agreement:

  - Companies are encourage to provide the inputs for RF channels for conformance testing

Issue 4-6: Test procedures

- Agreement:

  - Companies are encourage to provide the inputs for test procedures for EEIRP measurement.

Issue 4-7: Measurement uncertainty and CI

- Agreement:

  - Companies are encourage to provide the inputs for MU and CI values based on the assumption of test beams and measurement grid size.

- The performance part: Open issues for the reduction of testing scope and simplification BS TRP test methods for FR1 and FR2

  - Identify BS excessive OTA test scope cases and reduce it with the aim to shorten the test duration time for selected RF requirements, e.g., TX IM, RX out of band blocking, for NR FR1.

  - Investigate and if possible, simplify BS TRP test methods for FR1 and FR2 by improving the applicability and not removing any of the existing TRP test methods.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

RAN4#112

- R4-2411020	Step Wise Procedure for the Validation of EIRP	Spark NZ Ltd

- R4-2411078	Discussion on conformance test for EIRP mask for U6GHz	CATT

- R4-2411516	Views on EIRP mask considerations for upper 6GHz	Qualcomm Germany

- R4-2411641	Discussion on OTA spatial emission requirement	Samsung

- R4-2411642	Discussion on OTA co-location reference antenna enhancement	Samsung

- R4-2411643	Discussion on OTA spatial emission conformance testing	Samsung

- R4-2411872	On general aspects related to BS RF evolution WI	Ericsson

- R4-2412704	TR 38.908 Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz	ZTE Corporation, Sanechips

- R4-2412705	TR 38.908 Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz	ZTE Corporation, Sanechips

- R4-2412706	Draft CR for introduction of U6GHz EIRP mask	ZTE Corporation, Sanechips

- R4-2412707	TP to TR 38.908 Background of U6GHz EEIRP mask requirement	ZTE Corporation, Sanechips

- R4-2412708	Further discussion on OTA test enhancement	ZTE Corporation, Sanechips

- R4-2412709	Further discussion on Expected EIRP mask for upper 6GHz	ZTE Corporation, Sanechips

- R4-2412898	On Expected EIRP mask core requirement	Nokia

- R4-2412899	Expected EIRP mask testing	Nokia

- R4-2412912	On the topic of BS RF evolution related to co-location requirements	Ericsson

- R4-2413126	TR 38.908 Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz	ZTE Corporation, Sanechips

- R4-2413219	Introduction of OTA spatial emission above the horizon requirement for BS operating in band n104	Ericsson

- R4-2413220	Conformance test aspects for the requirement on OTA spatial emission above the horizon for BS operating in band n104	Ericsson

- R4-2413221	Draft CR to TR 38.xyz: Technical background information for FSS UL protection requirement for band n104	Ericsson

- R4-2413222	Draft CR to TS 38.104: Addition of spatial emission requirement to protect FSS UL within band n104 in subclause 9.9	Ericsson

- R4-2413234	OTA test enhancements - CLTA	Nokia

- R4-2413274	Draft CR to TS 38.104 Expected EIRP requirement inroduction	Huawei, HiSilicon

- R4-2413275	EEIRP Beam direction definitions	Huawei, HiSilicon

- R4-2413276	Expected EIRP conformance – Test Vectors, MU budget and Conformance test method	Huawei, HiSilicon

- R4-2413277	TX IMD requirements for high frequency band in FR1	Huawei, HiSilicon

- R4-2413405	Topic summary for [112][305] NR_BS_RF	Moderator (ZTE)

- R4-2413508	Ad-hoc meeting minutes for [112][305] NR_BS_RF	ZTE

- R4-2413509	Way Forward for [112][305] NR_BS_RF	ZTE

- R4-2413574	Draft CR for introduction of U6GHz EIRP mask	ZTE Corporation, Sanechips

- R4-2413575     TP to TR 38.908 Background of U6GHz EEIRP mask requirement

- R4-2413576      TP to TR 38.908 U6GHz EEIRP mask requirement