3GPP TSG RAN Meeting #105	RP-242014

Melbourne, Australia, September 9-12, 2024

Agenda Item:	15

Source: 	CEPRI

Title: 	Discussion on Symbol-level Precoding for MU-MIMO System

Document for: 	Discussion and Decision

Introduction

Traditionally, interference is usually viewed as a performance limiting factor in wireless communication systems which should be mitigated. In a typical multi-user transmission, the existence of interference is based on the observation that the transmit signals for different users are superimposed in wireless communication channels. Conventional multiuser precoding schemes are designed as interference mitigation or even cancellation[1]-[5]. These methods can be obtained via the instantaneous channel state information (CSI) in each block, which can be interpreted as block-level precoding (BLP).

Recent studies have shown that symbol-level precoding (SLP) is able to manipulate both the power and the direction of the interfering signals on the received complex plane on a symbol level such that the interference can act as an useful source of the desired signal power. This leads to introducing the notion of constructive interference (CI). It was firstly introduced in [6], where it is shown that the instantaneous interference can be categorized into constructive and destructive. Accordingly, in addition to CSI, the instantaneous data information of all users are used to design the precoder. Thus, SLP optimization problem then needs to be solved for each symbol period. Moreover, one can also formulate the SLP problem to directly find the optimal transmit vector, as proposed in [7], instead of finding the precoding matrix. It shows that instantaneous signal to-noise radio (SNR) of the received symbol could increase which therefore contribute to the symbol detection and further improves the system performance [8][9], due to the effective use of interference.

Discussion

With the availability of instantaneous data information of all users in MIMO systems, Maha Alodeh et al. pioneered the field of SLP attempting to search different views of exploiting interference. The task of SLP soon drew great attention from researchers, with a variety of solutions being developed. According to different optimization goals, existing SLP methods could be classified into several broad categories: Power Minimization SLP [10]-[15], SINR Minimization SLP [29], Symbol Error Probability (SEP) Optimization SLP[16][17], etc.

Observation 1: Power Minimization

SLP can exploit interference, thereby reducing transmit power at the transmitter. Therefore, many studies take power minimization as the objective function. J. Krivochiza et al.[13] designed some computationally efficient SLP methods for sum power minimization subject to the receiving signal being within the CI area, which can be applied for the M-th order phase-shift keying (M-PSK) symbol modulation. SLP schemes have been proposed to formulate the optimization as a non-negative least-squares(NNLS) problem [13], [14], which can be efficiently solved by the fast NNLS algorithm [15]. SLP scheme for QPSK modulation separated the real and imaginary part of the optimization problem then transforms them to the first quadrant of the constellation for reducing the complexity of NNLS [13].

Alireza et al. first discussed the optimal and relaxed CIRs for a generic constellation, and then showed that the SLP problem can always be formulated as a convex optimization problem defined on CI regions [10]. A new SLP technique [11] based on a non-negative convex quadratic optimization problem was proposed which can be solved by a closed-form algorithm with a fixed number of iterations.

Proposal 1: Study how to how to utilize interference to reduce transmit power in the system by integrating SLP.

Observation 2: Symbol Error Probability (SEP) Optimization

In [16], the authors first derived the average symbol error probability (SEP) for the CI precoder with PSK signaling. In addition, power allocation based on minimizing the sum symbol error probabilities (Min-Sum) and the power allocation based on minimizing the maximum SEP (Min-Max) were investigated. A modified feasible direction algorithm was developed [17] to handle the complex expression of SER, where an extra projection step was proposed to enhance the efficiency of the feasible direction with the proof that the optimal precoded signal for SER minimization shall be the linear combination of the channel vectors.

Proposal 2: Study how to explore the optimization of system architecture using SLP algorithms to reduce the SEP, thereby enhancing system stability and reliability.

Conclusion

In this contribution, the Symbol-level precoding methods for MU-MIMO system were discussed with giving corresponding proposals:

Proposal 1: Study how to how to utilize interference to reduce transmit power in the system by integrating SLP.

Proposal 2: Study how to explore the optimization of system architecture using SLP algorithms to reduce the SEP, thereby enhancing system stability and reliability.

Reference

[1] M. Joham, W. Utschick, and J. A. Nossek, “Linear transmit processing in MIMO communication systems,” IEEE Trans. Signal Process., vol. 53, no. 8, pp. 2700–2712, Aug. 2005.

[2] T. Haustein, C. von Helmolt, E. A. Jorswieck, V. Jungnickel, and V. Pohl, “Performance of MIMO systems with channel inversion,” in Proc. IEEE 55th Veh. Technol. Conf. (VTC Spring), vol. 1. Birmingham, AL, USA, 2002, pp. 35–39.

[3] C. B. Peel, B. M. Hochwald, and A. L. Swindlehurst, “A vector-perturbation technique for near-capacity multiantenna multiuser communication—Part I: Channel inversion and regularization,” IEEE Trans. Commun., vol. 53, no. 1, pp. 195–202, Jan. 2005.

[4] M. Bengtsson and B. E. Ottersten, “Optimal and suboptimal transmit beamforming,” in Handbook of Antennas in Wireless Communications. Waltham, MA, USA: Elsevier, Jan. 2001.

[5] M. Schubert and H. Boche, “Solution of the multiuser downlink beamforming problem with individual SINR constraints,” IEEE Trans. Veh. Technol., vol. 53, no. 1, pp. 18–28, Jan. 2004.

[6] C. Masouros and E. Alsusa, “Soft linear precoding for the downlink of DS/CDMA communication systems,” IEEE Trans. Veh. Technol., vol. 59, no. 1, pp. 203–215, Jan. 2010.

[7] M. Alodeh, S. Chatzinotas, and B. Ottersten, “Constructive multiuser interference in symbol level precoding for the MISO downlink channel,” IEEE Trans. Signal Process., vol. 63, no. 9, pp. 2239–2252, May 2015.

[8] C. Masouros, T. Ratnarajah, M. Sellathurai, C. B. Papadias, and A. K. Shukla, “Known interference in the cellular downlink: A performance limiting factor or a source of green signal power?” IEEE Commun. Mag., vol. 51, no. 10, pp. 162–171, Oct. 2013.

[9] G. Zheng, I. Krikidis, C. Masouros, S. Timotheou, D.-A. Toumpakaris, and Z. Ding, “Rethinking the role of interference in wireless networks,” IEEE Commun. Mag., vol. 52, no. 11, pp. 152–158, Nov. 2014

[10] A. R. Haqiqatnejad, F. Kayhan, and B. E. Ottersten, “Constructive interference for generic constellations,” IEEE Signal Process. Lett., vol. 25, no. 4, pp. 586–590, Apr. 2018.

[11] Krivochiza J, Merlano-Duncan J C, Chatzinotas S, et al. “M-QAM modulation symbol-level precoding for power minimization: Closed-form solution,” in Proc. IEEE Int. Symp. Wireless Commun. Syst. (ISWCS), 2019, pp. 395-399.

[12] A. Haqiqatnejad, F. Kayhan, and B. E. Ottersten. (2018). Robust Design of Power Minimizing Symbol-Level Precoder Under Channel Uncertainty. [Online]. Available: https://arxiv.org/abs/1805.02395

[13] J. Krivochiza, A. Kalantari, S. Chatzinotas, and B. Ottersten, “Low complexity symbol-level design for linear precoding systems,” arXiv:1711.09062, 2017.

[14] J. Krivochiza, J. Merlano-Duncan, S. Andrenacci, S. Chatzinotas, and B. Ottersten, “Closed-form solution for computationally efficient symbol-level precoding,” in Proc. IEEE Global Commun Conf (GLOBECOM), Dec. 2018, pp. 1–6.

[15] R. Bro and S. De Jong, “A fast non-negativity-constrained least squares algorithm,” J. Chemometrics, vol. 11, no. 5, pp 393–401, 1997.

[16] A. Salem and C. Masouros. (2019). Error Probability Analysis and Power Allocation for Interference Exploitation Over Rayleigh Fading Channels. [Online]. Available: https://arxiv.org/abs/1910.03102

[17] Zhang, Ling, et al. "Symbol Error Rate Minimization Based Constructive Interference Precoding for Multi-User Systems." IEEE Access, vol.9, no.9, pp: 42543-42555, Jul 2021.