3GPP TSG RAN meeting #105 									RP-242015

Melbourne, Australia, September 9-12, 2024

Status Report to TSG

Agenda item:			9.3.3.1

| WI / SI Name | Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_ENDC_SON_MDT_Ph4-Core |  |  |  |  |
| Unique ID | 1021092 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234038 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 30% | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

It’s noted that RAN2 has used more time in the meetings than allocated to this topic and still the progress is behind schedule, and it should be noted that two of the WI-objectives have not even been started to be treated in RAN2 yet. If no more TUs are assigned, there will be a risk to not be able to finish the WI on time for RAN2 parts. Requested action: To address the problem, RAN plenary should assign 0.5 =>1TU  from RAN2#127bis to RAN2#131 for this WI.

Source:

| Leading WG |  | RAN WG3 |
| --- | --- | --- |
| Rapporteur | Name | Xufei Dong |
|  | Company | China Unicom |
|  | Email | dongxf37@chinaunicom.cn |
|  | Name | Miaoqi Zhang |
|  | Company | CMCC |
|  | Email | zhangmiaoqi@chinamobile.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

The WI rapporteur suggest to increase the RAN2 TU by 0.5 TU per meeting, starting from 09/2024 till the completion of the core part, i.e. 09/2025. The reason for change is due to TU under-estimation given the amount of RAN2 impact according to the existing work item objectives.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#127 (Aug 2024)

- MRO enhancements for Rel-18 mobility features

  - MRO for LTM

-	Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC.

-	introduce a new field in RLF report to indicate the LTM recovery cell id.

-	RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch.

-	We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

-	RA-based access will not be a new triggering condition for the SHR report.

-	HO interruption time will not be a new triggering condition for the SHR report

-	Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios.

-	We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases.

-	We will not log interruption time in SON reports (e.g. SHR) for LTM.

-	We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not.

-	Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec.

  - MRO for CHO with candidate SCG(s)

-	UE includes following information in RLF report:

Time information regarding condition fulfilment for CHO with candidate SCGs. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

Measurement results of PCells and PSCells.

  - MRO for S-CPAC

None.

- SON MDT for Slicing

None.

- SON MDT for NTN

None.

- Leftovers from Rel-18

-	It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state.

-	Send an LS [32] to RAN3 to ask if a NW solution is good enough to achieve the agreement above, or if a UE based solution is needed.

-	To support MRO for SCG failure in EN-DC, enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId, timeSCGFailure.

-	Do not add logging of sdt-RSRP-Threshold, since already agreed by RAN3 to not support it.

-	UE logs and reports the failure cause for SDT to the network.

2.2.2	Remaining Open issues

-	When SDT failure happens, details on whether other information (e.g. data volume) can also be included in the same report are TBD.

-	FFS on explicit or implicit access type indicator in the RLF report.

-	TBD if explicit/implicit indicator to indicate whether a neighbour cell is an LTM candidate cell or not.

-	Details on time information regarding condition fulfilment for CHO with candidate SCGs are FFS.

-	Details on logging and reporting the failure cause for SDT to the network are FFS, e.g. if we down select some of the failure causes.

2.3	RAN3

2.3.1	Agreements

RAN3#125(Aug 2024)

- MRO enhancements

  - MRO for LTM

- Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover.

- Take the existing detection mechanism descriptions as baseline.

- For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

- CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure.

  - MRO for CHO with candidate SCG(s)

- MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.

- MRO for case 4/5/6 is in the scope.

- The RLF report includes: The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

  - MRO for S-CPAC

- Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.

- SON/MDT for Intra-NTN Mobility

- MRO for CHO with a time-based trigger condition:

  - Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

- Distinguish if the RLF happens before or after T1

  - Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

- SON/MDT for Network Slicing

- For the immediate MDT, it is not needed to add measurements related to the slice unavailability and the lack of resources.

- R18 Leftovers

- Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for EN-DC and NGEN-DC SCG failure, and necessary updates can be added on top of it if needed.

- Introduce new SCG FAILURE INFORMATION REPORT message and SCG FAILURE TRANSFER message over X2 interface.

- The information that stored and reported in the RA report for SDT: the time from the start of RA-SDT to the reporting of RA report

- LS Out

- LS to SA5 on SON for Network Slicing and enhanced area scope [59]

2.3.2	Remaining Open issues

- FFS to enhance failure scenario.

- CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed.

- FFS whether there is any specification impact for case 4/5/6.

- Case 1c, 2c, 2c, 3c, 7c, 8c are FFS.

- The RLF report includes:

- Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

- FFS whether to optimize the association of the PCell and PSCell.

- For the CPAC failures which occurred during an S-CPAC procedure, CPC Execution above refers to the initial CPC execution or the subsequent CPC execution.

- Editor’s note: FFS whether the differentiation between initial and following CPC is needed.

- FFS on whether it matters if RLF happens within the duration or after the duration period.

- The RLF report includes for single time-based trigger condition:

-      Absolute time (measured UTC time) when RLF occurred

-      Measured UTC time when RLF CHO is executed

- Whether to consider the fallback issue from RACH-less HO to RACH-based HO for NTN?

- Whether to introduce any enhancement for the Area Scope of logged MDT for NTN over NG interface?

- The slice ID will be added as a filter to the area scope (for the both, management- and signalling-based immediate MDT related to slice). The intention is to limit the number of reports sent to the OAM, thus the UE selection will not be amended.

- FFS on other SDT related information.

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#127

- R2-2406527	Discussion on random access report for LTM	ASUSTeK	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2406883	Discussion on MRO for R18 mobility	Lenovo	discussion	Rel-19

- R2-2406959	Discussion on MRO enhancements for R18 mobility features	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407005	Discussion on MRO Enhancements for Mobility	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407029	MRO for Rel-18 mobility	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407052	MRO enhancements for Rel-18 mobility features	Samsung	discussion

- R2-2407064	Discussion on MRO enhancement for LTM	China Unicom	discussion	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407065	Discussion on MRO enhancement for CHO with candidate SCGs	China Unicom	discussion	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407094	MRO enhancements for LTM	NEC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407095	MRO for CHO with candidate SCG(s)	NEC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407099	MRO enhancement for SON and MDT	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407105	MRO for R18 Mobility	LG Electronics	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407119	MRO for LTM	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407120	MRO for CHO with candidate SCG	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407122	Configuring UE based TA acquisition for LTM	Rakuten Mobile, Inc	discussion	Rel-19

- R2-2407191	SON/MDT reports for LTM 	Kyocera	discussion

- R2-2407218	SON support for MRO	Ericsson	discussion	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407333	Discussion on MRO enhancements for Rel-18 mobility features	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407362	Discussion on MRO enhancement for R18 mobility features	Sharp	discussion

- R2-2407386	MRO for Rel-18 mobility features	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407106	Logging for Unchanged PCI Mobility in NTN	LG Electronics	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2406884	Discussion on MRO for MR-DC SCG failure	Lenovo	discussion	Rel-19

- R2-2406986	MHI Enhancement for SCG Deactivation/Activation	CMCC, CATT, Ericsson, ZTE, Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407006	Consideration on leftovers from Rel-18 SONMDT	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407030	Rel-18 leftovers for SON MDT	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407037	SON/MDT enhancements for leftover topics from R18	Samsung	discussion

- R2-2407100	SON and MDT Rel-18 leftover issues	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407249	On Rel.18 leftovers	Ericsson	discussion	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407334	Discussion on support of the Rel-18 leftovers	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407364	Discussion on R18 leftovers for SON MDT	Sharp	discussion

- R2-2407387	RACH optimization for SDT	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2407664	LS on MHI enhancement solution for SCG deactivation and activation RAN2(CMCC)

RAN3#125

- R3-244065	[TP for TS 37.340 BL CR, SON] Failure handling and optimal use of Subsequent CPAC	Nokia

- R3-244066	[TP to TS 38.423 BL CR, SON] Solution to avoid restarting MRO in NTN deployments	Nokia

- R3-244067	[TP to TS 38.300 BL CR, SON] MDT solution for slice support and slice-related mobility enhancements	Nokia

- R3-244160	MRO for LTM	NEC

- R3-244207	Discussion on MRO for LTM	Samsung

- R3-244223	Failure scenarios on MRO for CHO with candidate SCGs	Samsung, Cybercore, ZTE

- R3-244224	Discussion on MRO for CHO with candidate SCGs and S-CPAC	Samsung

- R3-244225	Discussion on the support of the leftovers in Rel-18 SON/MDT	Samsung

- R3-244256	Discussion on MRO enhancements for LTM	China Telecom

- R3-244257	Discussion on SON enhancements for network slicing	China Telecom

- R3-244260	Discussion on SON enhancements for NTN	Samsung

- R3-244264	Discussion on MRO for mobility	CATT

- R3-244265	Discussion on intra-NTN mobility for SONMDT	CATT

- R3-244266	Discussion on network slicing for SONMDT	CATT

- R3-244267	Discussion on SON enhancement for MR-DC SCG failure	CATT

- R3-244278	(TP for SON BLCR for 38.300, 38.401, 38.473, 37.340) MRO enhancement for R18 mobility mechanisms	Huawei

- R3-244279	SON for Network Slicing	Huawei

- R3-244280	(TP for SON BLCR for 36.300, 36.423) Rel-18 SON and MDT leftovers	Huawei

- R3-244306	Discussion on MRO for LTM	Lenovo

- R3-244307	(TP for SON BLCR for 37.340) Discussion on MRO for subsequent CPAC and CHO with candidate SCG(s)	Lenovo

- R3-244308	Discussion on MRO for intra-NTN mobility	Lenovo

- R3-244309	(TP for SON BLCR for 36.300) Discussion on MRO for MR-DC SCG failure	Lenovo

- R3-244335	MRO Enhancements for LTM and CHO with Candidate SCG(s)	Nokia

- R3-244430	MRO enhancements for R18 mobility mechanisms	Qualcomm Incorporated

- R3-244431	SON MDT for NTN	Qualcomm Incorporated

- R3-244432	SON MDT for network slicing	Qualcomm Incorporated

- R3-244433	MRO enhancements for LTM and CHO with candidate SCGs	Ericsson

- R3-244434	SON/MDT enhancements for NTN	Ericsson

- R3-244435	On slice specific MDT measurements and SON enhancements	Ericsson

- R3-244436	(TP for SON BL CR for TS 38.413)  MDT Area Scope for Network Slicing	Ericsson, InterDigital, Deutsche Telekom, Telecom Italia, T-Mobile USA

- R3-244437	Further discussion on RACH optimization for SDT	Ericsson

- R3-244460	(TP to BL CR for TS 38.300) Further discussion on SONMDT enhancements for NTN	ZTE Corporation

- R3-244461	(TP to BL CR for TS 36.300) Rel-18 leftovers	ZTE Corporation

- R3-244481	SON for Intra-NTN mobility	Huawei

- R3-244532	Workplan for Rel-19 SON_MDT Enhancement	China Unicom, CMCC

- R3-244533	Discussion on MRO Enhancements	China Unicom

- R3-244534	Discussion on SONMDT enhancements for NTN mobility	China Unicom

- R3-244535	Discussion on SONMDT enhancements for network slicing	China Unicom

- R3-244536	Discussion on MRO issues related to LTM	LG Electronics Inc.

- R3-244573	Discussion on SONMDT for network slicing	CMCC

- R3-244575	(TP to 38.300) Discussion on SONMDT enhancement for NTN	CMCC

- R3-244604	Discussion on MRO enhancements for LTM	CMCC

- R3-244615	(TP for TS 38.300 TS37.340)Discussion on MRO	ZTE Corporation

- R3-244616	Further consideration on SON/MDT for Slicing	ZTE Corporation

- R3-244727	SoD of MRO for LTM	CMCC

- R3-244728	SoD of MRO for others	Samsung

- R3-244729	SoD of SONMDT NTN related	ZTE Corporation

- R3-244730	SoD of SONMDT slicing related	Nokia

- R3-244731	SoD of SONMDT leftovers	Lenovo

- R3-244764	(TP for TS37.340) on MRO for S-CPAC	Samsung

- R3-244766	(TP for TS36.423) on MRO for SCG failure	Samsung

- R3-244777	(TP to BL CR for TS 36.300) MRO for EN-DC SCG Failure	ZTE Corporation, Nokia, Samsung, Lenovo, Ericsson, Huawei

- R3-244782	(TP for 38.300) MRO enhancement for LTM	ZTE Corporation

- R3-244815	(TP for SON BL CR for TS 38.423) MDT Area Scope for Network Slicing	ZTE corporation

- R3-244816	(TP for SON BL CR for TS 38.413)  MDT Area Scope for Network Slicing	Ericsson, InterDigital, Deutsche Telekom, Telecom Italia, T-Mobile USA

- R3-244819	[TP to TS 38.300 BL CR, SON] MDT solution for slice support and slice-related mobility enhancements	Nokia

- R3-244820	LS on SON for Network Slicing and enhanced area scope	RAN3(Nokia)

- R3-244822	SoD of SONMDT slicing related	Nokia

- R3-244823	LS on SON for Network Slicing and enhanced area scope	RAN3(Nokia)

- R3-244825	(TP for TS36.423) on MRO for SCG failure	Samsung, Lenovo, Huawei, Ericsson, CATT, ZTE, CMCC, Nokia

- R3-244826	(TP for SON BL CR for TS 38.300) MRO enhancement for LTM	ZTE Corporation, Samsung, Nokia, Lenovo

- R3-244827	(TP for TS37.340) on MRO for S-CPAC	Samsung, Nokia

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template