3GPP TSG RAN Meeting #105						RP-242022

Melbourne, Australia, September 9-12, 2024

Agenda item:	15

Source:	Ericsson, Nokia, Interdigital, Huawei, Samsung, ZTE, Intel Corporation

Title:	Status of work on IMT-2030 in ITU-R WP5D

Document for:	Information

# Background

ITU-R WP5D has been working on 6G since 2021 under the ITU designation IMT-2030 and produced several deliverables in the area during the 2019-2023 study period, including:

- Recommendation ITU-R M.2160 [1]: “Framework” recommendation for IMT-2030.

- Report ITU-R M.2516 [2]: Study of Future technology trends.

The Framework recommendation [1] contains several important elements that define and set the scope for IMT-2030:

- Trends of IMT-2030, including motivation and societal considerations, user and application trends, technology trends, envisaged frequency bands and spectrum harmonization, and feasibility in bands above 100 GHz.

- Usage scenarios of IMT-2030, expanding on the usage scenarios from IMT-2020 and new usage scenarios:

  - Immersive Communication (expanding eMBB)

  - Hyper Reliable and Low-Latency Communication (expanding URLLC)

  - Massive Communication (expanding mMTC)

  - Ubiquitous Connectivity

  - Artificial Intelligence and Communication

  - Integrated Sensing and Communication

- Capabilities of IMT-2030 comprises a total of 15 capabilities, where many are enhanced capabilities from IMT-2020, while others are new capabilities to support the expanded and new Usage scenarios.

The Usage scenarios and related Capabilities form a basis for the continuing work in the 2023-2027 study period for defining Technical Performance Requirements (TPRs) for IMT-2030 and the related Evaluation methodologies and Test environments. This paper gives an overview of the ongoing WP5D work and status of the planned deliverables. Further information on the IMT-2030 process and deliverables can be found at the ITU-R WP 5D’s IMT-2030 website.

# Technical performance Requirements

The new TPRs report for IMT-2030 corresponds to Report ITU-R M.2410 [3] for IMT-2020 and will define a set of minimum technical performance requirements for IMT-2030 candidate radio interface technologies. Work on the report has been ongoing for two meetings during 2024 and substantial technical input was made by many administrations and other members of ITU-R WP5D.

All inputs received are merged into a Working document [6], which forms a basis for further discussions and agreements. It should be noted that the WP5D process is different from 3GPP, in that input contributions are not treated separately in terms of decisions and way forward. This is instead done through a merged working document, where each contributed part is discussed, cleaned up, often merged, and sometimes removed. Only the final text is agreed, while intermediate text in the working document is just working text based on inputs, with Editor’s notes explaining the status.

The present working document [6] contains the following candidate TPRs, based on inputs received until the WP 5D#46 meeting in June 2024. As such, they are subject to further addition/deletion/modification in WP 5D October 2024 and subsequent meetings. This concerns in particular any new TPR for the new IMT-2030 usage scenarios.

4.1	Peak data rate

4.2	User experienced data rate

4.3	Spectrum Efficiency

4.3.1	Peak Spectrum Efficiency

4.3.2 	Average spectrum efficiency

4.3.3	5th percentile user spectrum efficiency

4.4	Area traffic capacity

4.5	Connection Density

4.6	Mobility

4.6.1	Mobility interruption time

4.7	Latency

4.7.1	User plane latency

4.7.2	Control plane latency

4.8	Reliability

4.X	Joint requirement on data rate, latency and reliability

4.9	Coverage

4.10	Positioning

4.11	Bandwidth

4.12	Sensing-related capabilities

4.13	AI-related capabilities

4.14	Sustainability

4.14.1	[Energy consumption]

4.14.2	Energy efficiency

4.15	Security and resilience

4.16	Interoperability

Most of the TPRs listed have a definition in the working document, often based on the legacy IMT-2020 definition from [3] or new text from the Framework recommendation [1]. Any proposals or discussion of target values for the TPRs are not kept in the body text but are instead moved to an Annex.

There are three Annexes to the Working document:

- ANNEX 1: Mapping between capabilities (TPRs) and usage scenarios

- ANNEX 2: Additional Text from contributions to WP 5D #45

- ANNEX 3: Inputs on minimum requirements for technical performance

The WP5D work plan calls for completion of the report by February 2026. Important milestones (marked yellow) for the continued work are identified below (from [7]):

- October 2024	(WP5D#47)	Continue to discuss candidate TPRs; Send external LS (if needed).

- February 2025	(WP5D#48)	Preliminarily agree on the list of TPRs; Send external LS (if needed).

- June 2025	(WP5D#49)	Review and update the TPR list; Start discussion of target values.

- October 2025	(WP5D#50)	Agree on the list of TPRs; Preliminarily agree on target values.

- February 2026	(WP5D#51)	Finalize document as Draft new report.

The work plan thus calls for a quite stable set of TPRs by early 2025 and to agree on target values in the second half of 2025.

# Evaluation and Test environments

The work on Evaluation and test environments has not formally started, but a first discussion was held at the recent WP5D#46 meeting in June. A report will be developed for IMT-2030 that corresponds to Report ITU-R M.2412 [4] for IMT-2020 and will set guidelines for the procedure, the methodology and the criteria to be used in evaluating the candidate IMT-2030 radio interface technologies for a number of test environments. For each TPR, evaluation can either be by analysis, simulation or inspection.

While no formal agreements were made at WP5D#46, some informal consensus was identified:

- The definition of Test environments remains from IMT-2020 as being a combination of geographic environment and usage scenario.

- The five test environments from IMT-2020 remain as baseline but would be based on the expanded usage scenarios (Immersive Communication, Hyper Reliable and Low-Latency Communication, and Massive Communication).

For the three new usage scenarios (Ubiquitous connectivity, Artificial Intelligence and Communication, and Integrated Sensing and Communication), there were several candidate test environments proposed and discussed.

Formal discussions will start in October 2024 (WP5D#47). The overall IMT-2030 time plan calls for the Evaluation report to be completed by June 2026 (WP5D#52).

# Submission templates

The final document that ITU-R WP5D will draft for the IMT-2030 requirement and evaluation is the report on “Requirements, evaluation criteria and submission templates”, corresponding to Report ITU-R M.2411 [5] for IMT-2020.

The plan is to start in June 2025 (WP5D#49) and to complete work on the new report in October 2026 (WP5D#53)

# Conclusion

This document presented an overview of the ongoing work on IMT-2030 requirements and evaluation in ITU-R WP5D, for consideration by TSG RAN. According to the time plan depicted in above section 2, WP 5D plans to preliminarily agree on the list of TPRs at its #48 meeting in February 2025. This is suggested to be considered when arranging the objectives for 3GPP RAN Plenary meeting in December 2024.

# References

- Recommendation ITU-R M.2160-0 (11/2023), “Framework and overall objectives of the future development of IMT for 2030 and beyond”.

- Report ITU-R M.2516-0 (11/2022), “Future technology trends of terrestrial IMT systems towards 2030 and beyond”.

- Report ITU-R M.2410-0 (11/2017), “Minimum requirements related to technical performance for IMT-2020radio interface(s)”.

- Report ITU-R M.2412-0 (10/2017), “Guidelines for evaluation of radio interface technologies for IMT-2020”.

- Report ITU-R M.2411-0 (11/2017), “Requirements, evaluation criteria and submission templates for the development of IMT-2020”.

- “Working Document towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]: Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, Annex 5.8 to Working Party 5D Chairman’s Report.

- “Detailed workplan for the development of Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]”, Annex 2.20.12 to Working Party 5D Chair’s Report.