3GPP TSG RAN Meeting #105							RP-242027

Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.2.3

Source:	Ericsson

Title:	Checkpoint for ISAC

Document for:	Discussion, Decision

# 1	Introduction and proposal

The Rel-19 ISAC SID [1] has checkpoints as follows in RAN#105 (Sept ’24):

“It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.”

Some progress has been made in both sensing scenarios and channel modelling although RAN1 is behind the workplan [2] in that calibration of ISAC channel model has not started yet. The work plan [2] mentions the importance of calibration for potential scope extension as follows.

To prepare for any potential expanded scope, the calibration on the validated ISAC channel model for at least one scenario for one or more sensing modes needs to start from RAN1 #118 meeting (August 2024).

Note that calibration is different from validation, since calibration is a task across companies that takes place when a channel model has been completed. Calibration should also be based on the TR, since correct interpretation and removal of ambiguities is part of the calibration effort.

Calibration is thus impossible at this stage and due to the large workload in this study item, it would be detrimental to the ongoing work to add additional study objectives. Hence our proposal is the following (with more detailed motivations in Section 2).

Don’t include additional study beyond ongoing channel modelling in Rel-19 ISAC SI.

# 2	Consideration of the scope of ISAC SI

## 2.1 Considerations related to workplan

Both running CR and calibration were planned to start in RAN1#118 [2]. The intention of running CR is to capture agreements and it is the first part of the process to align companies’ understanding of the channel model. The second part is calibration, which has the purpose of making sure that companies have implemented the model in the same way. Calibration cannot be completed until the agreements have been converted into TR text via the running CR, however preparations for calibration can start earlier, such as agreeing on which simulation settings should be used and which metrics that should be compared.

Both capturing the agreements in a running CR and performing calibration are prerequisites of evaluation study. ISAC channel model is not complete for any sensing scenario and any sensing mode, which prevents the discussions on running CR and calibration.

In RAN1#118, some evaluation parameters for UAV sensing scenario were agreed with some others FFS. There was no discussion on parameters of other sensing scenarios. In addition, scope extension doesn’t necessitate an increase of time unit. Consequently, less time would be spent on channel modelling, which may cause de-prioritization of some sensing scenarios or some sensing modes.

A complete TR and channel calibration are prerequisites of additional study. However, ISAC channel model is not complete for any sensing scenario and any sensing mode, which prevents the discussions on running CR and calibration.

Scope extension without an increase of time unit may cause de-prioritization of some sensing scenarios or some sensing modes.

## 2.2 The availability of channel model

All six sensing modes are included in SID, meaning a sensing mode involves BS – target link and/or UE – target link. Five types of sensing targets are considered in the SID. A common channel modelling framework and target modelling applicable to all sensing scenarios and sensing modes should be strived for. On the other hand, different sensing modes require modelling of different background channels, including UE-BS channel, UE mono/bi-static channel and BS mono/bi-static channel, and different RCS modelling, mono-static or bi-static. Both the commonalities and differences across sensing scenarios and sensing modes need to progress in parallel.

A common channel modelling framework and target modelling applicable to all sensing scenarios and sensing modes should be strived for with the consideration of different background channel and mono-static or bi-static RCS modelling.

ISAC channel consists of target channel and background channel. Based on the latest agreements, ISAC channel modelling study mainly includes the following aspects.

- Modelling Tx - target link and target - Rx link, including LOS ray and NLOS rays caused by stochastic clutter and/or environment object

- Concatenation of the two links by taking into account the impact of target, e.g., radar cross section (RCS), cross-polarization matrix (XPR), and Doppler

- Background channel modelling

Legacy UE-BS communication channel models can be reused to a large extent to model the BS - target link, for those cases where targets are in similar locations as UEs. However, UE - BS communication channel models may not be directly reused to model the UE – target link, because targets, like vehicles, indoor and outdoor humans, are parameterized with different height, mobility, and distribution from those of BS. To what degree the existing UE - BS channel can be reused to model UE – target link and what new values of large-scale and small-scale parameters are needed should be studied in RAN1.

UE - BS communication channel models cannot be directly reused to model the UE – target link, because targets, e.g., vehicles, indoor and outdoor humans, are parameterized with different height, mobility, distribution from those of BS.

Once the two links are modelled, impacts of targets on the concatenation should be considered. There are multiple options in agreements or proposals regarding radar cross section (RCS) and cross-polarization matrix (XPR) are angle-dependent or not. Further measurements and down selection are needed. Further study on complexity reduction for concatenation-based approach, Doppler, Type-2 EO modelling, power normalization, etc., were just agreed in RAN1#118. RAN1 has not started discussions on some crucial and challenging issues yet, including spatial consistency, forward scattering, blocking, micro-Doppler, etc.

Further measurements and down selection are needed for radar cross section (RCS) and cross-polarization matrix. Some issues were just agreed to be studied, while some other crucial issues have not been discussed in RAN1 yet.

Background channel can be UE-BS channel, UE mono/bi-static channel, BS mono/bi-static channel. The first one is supported 38.901. Several options of UE/BS mono/bi-static channel modelling were agreed in RAN1#118 for further study. The study of background channel is important for sensing because it models the direct interference which is often several tens of dB stronger than the received sensing signal based on measurements [3]. A precise background channel modelling is crucial to sensing performance. In the context of ISAC, sensing will need to coexist with co-channel communication. The coexistence could be between neighboring bands, neighboring cells, or even within a cell, depending on the methods that will be studied for ISAC. Therefore, the relation between the target channel, used for sensing, and the background channel, affecting communication, self-interference, and cross-interference, is absolutely crucial to capture in an ISAC channel model. Considering the importance of the background channel for sensing itself and ISAC, this is a topic that deserves attention and fact-based decisions rather than hastily devised modeling approaches. Validation using measurements is one important step towards creating a model that could reliably be used in ISAC evaluations.

Modelling of UE mono/bi-static channel, BS mono/bi-static channel is crucial to sensing performance, since it provides the direct interference which is often several tens of dB higher than the received sensing signal based on measurements. It is important for ISAC as well with regard to coexistence between neighboring bands, neighboring cells, or even within a cell.

Validation using measurements is one important step towards creating a model that could reliably be used in ISAC evaluations.

A uniform channel modelling across sensing scenarios and sensing modes is beneficial for future evaluation study, which can avoid the incomparable evaluation results based on different methods or assumptions which are up to companies to report. An extension of scope would increase the risk of not finishing the open issues discussed above and leaving time for running CR and calibration before Rel-19 concludes.

An extension of scope would increase the risk of not finishing the open issues discussed above and the risk of not leaving sufficient time for running CR and calibration before Rel-19 concludes.

It would be unfortunate if the channel model study needs to continue for one more release to reach a model that is calibrated and ready to be used for ISAC evaluations as it would prohibit such studies in the 6G study item.

# 3 Conclusion

In this contribution, we made the following observations:

Observation 1	A complete TR and channel calibration are prerequisites of additional study. However, ISAC channel model is not complete for any sensing scenario and any sensing mode, which prevents the discussions on running CR and calibration.

Observation 2	Scope extension without an increase of time unit may cause de-prioritization of some sensing scenarios or some sensing modes.

Observation 3	A common channel modelling framework and target modelling applicable to all sensing scenarios and sensing modes should be strived for with the consideration of different background channel and mono-static or bi-static RCS modelling.

Observation 4	UE - BS communication channel models cannot be directly reused to model the UE – target link, because targets, e.g., vehicles, indoor and outdoor humans, are parameterized with different height, mobility, distribution from those of BS.

Observation 5	Further measurements and down selection are needed for radar cross section (RCS) and cross-polarization matrix. Some issues were just agreed to be studied, while some other crucial issues have not been discussed in RAN1 yet.

Observation 6	Modelling of UE mono/bi-static channel, BS mono/bi-static channel is crucial to sensing performance, since it provides the direct interference which is often several tens of dB higher than the received sensing signal based on measurements. It is important for ISAC as well with regard to coexistence between neighboring bands, neighboring cells, or even within a cell.

Observation 7	Validation using measurements is one important step towards creating a model that could reliably be used in ISAC evaluations.

Observation 8	An extension of scope would increase the risk of not finishing the open issues discussed above and the risk of not leaving sufficient time for running CR and calibration before Rel-19 concludes.

Given the observations, we propose the following:

Proposal 1	Don’t include additional study beyond ongoing channel modelling in Rel-19 ISAC SI.

# References

R1-240799 “Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Maastricht, RAN#103, March 2024.

R1-2404630, “Updated work plan on channel modelling for ISAC”, Fukuoka, RAN1#117, May 2024

R1-2406906, “Discussion on ISAC Deployment Scenarios”, Ericsson, RAN1#118, August 2024