3GPP TSG RAN Meeting #105	RP-242033

# Melbourne, Australia, September 9-12, 2024

# Source:	China Telecom

# Title:	New WID: Coverage enhancements for NR Phase 3

# Document for:	Approval

# Agenda Item:	9.1.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Coverage enhancements for NR Phase 3

Acronym: NR_Cov_Enh_Ph3

Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 860036 | Study on NR coverage enhancements | Rel-17 Study item on NR coverage enhancements |
| 900061 | NR coverage enhancements | Rel-17 Work item on NR coverage enhancements |
| 940095 | Further NR coverage enhancements | Rel-18 Work item on NR coverage enhancements |

3	Justification

Coverage is one of the key factors that an operator considers when commercializing cellular communication networks as its direct impact on service quality, CAPEX and OPEX. In real deployment scenarios, UL performance often becomes the bottleneck, especially with emerging vertical use cases such as heavy UL traffic for activities like video uploading. In Rel-17 work item 900061 “NR Coverage Enhancements”, NR coverage has been extended for some of the bottleneck channels identified in the Rel-17 study item 860036 “Study on NR coverage enhancements”, in particular for PUSCH, PUCCH and Msg3. Additionally, in Rel-18 work item 940095, multiple PRACH transmissions with same Tx beam was introduced to enhance PRACH coverage performance. While significant progress has been made to improve UL coverage thus far, there remains a need for further enhancement due to the following reasons:

- For multiple PRACH transmissions, a UE which fulfils the beam correspondence requirements without Tx beam sweeping, may identify the best UL Tx beam based on the DL Rx beam. This leads to higher antenna gain and PRACH success probability increases. However, a UE which fulfils the beam correspondence requirement with Tx beam sweeping, may not identify the best UL Tx beam based on the DL Rx beam and may need to perform UL beam sweeping to this end. Enabling multiple PRACH transmissions with different Tx beams allows these UEs to quickly conduct UL beam sweeping and access the network. When enabled alongside the baseline multiple PRACH transmissions with same Tx beam, this would yield a more homogeneous UL coverage in the cell during initial access, irrespective of UE capabilities. When multiple PRACH transmissions with different Tx beams are enabled, beam indication for subsequent UL transmissions can also be realized, e.g., beam indication for Msg3 transmission, thereby enhancing the coverage of subsequent UL transmissions.

- Msg5 PUSCH could also be the coverage bottleneck during the initial access. This is primarily due to the large payload size of Msg5, e.g., exceeding 100 Bytes, which is 1 to 2 orders of magnitudes larger than the typical Msg3 payload size. Given that Msg5 PUSCH is scheduled by DCI format 0_0 with CRC scrambled by C-RNTI and does not currently support repetition transmission, it would be advantageous to specify enhancements to support PUSCH repetition scheduled by DCI 0_0 with C-RNTI.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The detailed objectives for coverage enhancement of this work item include:

- Specify following PRACH coverage enhancements [RAN1, RAN2]

  - Multiple PRACH transmissions with different Tx beams for 4-step RACH procedure.

  - UL beam indication based on multiple PRACH transmissions with different Tx beam for Msg3.

  - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable.

  - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.

  - Note 3: The multiple PRACH are transmitted over ROs associated with the same SSB.

- Specify enhancements to support PUSCH repetition scheduled by DCI 0_0 with C-RNTI [RAN1, RAN2]

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Specify the following performance requirements [RAN4]

- Base station demodulation performance requirements

- Base station conformance testing

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.212 | NR; Multiplexing and channel coding | RAN#108 | Core Part |
| 38.213 | NR; Physical layer procedures for control | RAN#108 | Core Part |
| 38.214 | NR; Physical layer procedures for data | RAN#108 | Core Part |
| 38.300 | NR; NR and NG-RAN Overall description; Stage-2 | RAN#109 | Core Part |
| 38.306 | NR; User Equipment (UE) radio access capabilities | RAN#109 | Core Part |
| 38.321 | NR; Medium Access Control (MAC) protocol specification | RAN#109 | Core Part |
| 38.331 | NR; Radio Resource Control (RRC); Protocol specification | RAN#109 | Core Part |
| 38.104 | NR; Base Station (BS) radio transmission and reception | RAN#111 | Perf. Part |
| 38.141-1 | NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing | RAN#111 | Perf. part |
| 38.141-2 | NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing | RAN#111 | Perf. part |

6	Work item Rapporteur(s)

7	Work item leadership

Primary: RAN WG1

Secondary: RAN WG2, RAN WG4

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name