3GPP TSG RAN Meeting #105							   	RP-242057

Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.1.3

Source:			Mediatek Inc.

Title:		Rel-19 AI/ML Radio: Way Forward on Data Collection for UE-side model training

Document for:	Decision

# Introduction

Following options were discussed in RAN2 for data collection for UE-side model training [1][2]. A comprehensive analysis of these options has been conducted, focusing on various aspects such as data transfer paths, protocol layers, MNO visibility of data content, and MNO control over the data collection process. Despite the detailed analysis, RAN2 has not yet reached a consensus on a preferred option. Key areas of debate remain, notably concerning the data visibility level and the control extent associated to Option 1b.

In this contribution, we share our views on the way forward on data collection for UE-side model training, considering the ongoing discussions within RAN2.

1a. UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected. 1b. UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. 3. UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

# Discussion

Option 1a

Option 1a involves UEs collecting and directly transferring training data to Over-The-Top (OTT) servers, bypassing the MNO's infrastructure. This approach has no expected impact on 3GPP specifications, as it operates independently of the MNO's control. It represents a non-standardized approach, operating independently of 3GPP specifications, thus, it falls outside the purview of 3GPP standardization efforts. Given that 3GPP lacks insight into the implementation details of such a solution, it should not be adopted as a baseline for standardization. Instead, we should focus on options that align with 3GPP's standardization efforts and provide MNOs with the necessary control and visibility.

Proposal 1: Given that Option 1a operates outside the purview of 3GPP and lacks standardization specifics, it should not be adopted as a baseline solution within the 3GPP framework.

Option 1b, 2/3

According to current RAN2 discussion [1][2], both option 2 and option 3 provide full controllability for MNOs, allowing them to manage data transfer without the need for a SLA. They offer the possibilities for full, partial, or no standardized visibility of data content. The situation for Option 1b is less clear-cut. The level of MNO controllability and visibility is marked as "FFS", indicating that RAN2 cannot reach a consensus without input from SA groups.

Options 2 and 3 are favored from a RAN perspective for several reasons:

- While option 1b, exemplified by EVEX, offers a approach with a coarse level of controllability through API endpoints for session management and event exposure, option 2 and 3 allows for more precise, dynamic, and adaptive network control, directly influenced by the RAN node's real-time assessment of the radio environment. These options ensure that MNOs have full and granular control over the data.

- Option 2 and option 3 provides the possibility of full visibility of the data content at the RAN node, which can guarantee the integrity and security of the data collection process at RAN node when necessary.

- Option 2 and option 3 mitigate privacy concerns more effectively than Option 1b, as the data is stored within the MNO's domain, reducing the risk of unauthorized access or misuse.

- Data transfer is intrinsically linked to the configuration of data collection, which is to be done via control plane.

- Options 2 and 3 align with existing procedures of data collection for network-side model training and transfer, promoting consistency and efficiency.

- The RRC signaling impact is similar for both options, with the gNB receiving IEs containing the data and executing the forwarding process.

Proposal 2: Options 2 and 3 should be endorsed for data collection for UE-side model training as they offer the possibilities of full controllability and full visibility for MNOs and pose fewer privacy concerns.

# Conclusion

Proposal 1: Given that Option 1a operates outside the purview of 3GPP and lacks standardization specifics, it should not be adopted as a baseline solution within the 3GPP framework.

Proposal 2: Options 2 and 3 should be endorsed for data collection for UE-side model training as they offer the possibilities of full controllability and full visibility for MNOs and pose fewer privacy concerns.

# Reference

[1] R2-2407785, Data collection table modified online, RAN2 Chair (InterDigital)

[2] R2-2407807, CR on RAN2 inputs to TR 38.843, Ericsson