3GPP TSG RAN meeting #105								                      RP-242084

Melbourne, Australia, September 9-12, 2024

Status Report to TSG

Agenda item:			9.3.4.1

| WI / SI Name | UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_ENDC_RF_Ph4 |  |  |  |  |
| Unique ID | 1030077 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241656 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Sep. 2025 | Performance part: Mar. 2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 35% | Performance Part: 15% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Primary: Ye Liu Secondary: Borsato Ronald |
|  | Company | Huawei AT&T |
|  | Email | leo.liuye@huawei.com ronald.borsato@att.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#112

UE RF Core part:

- For HPUE CA/DC, WF was approved in R4-2414277.

- PC1.5 for intra-band contiguous and non-contiguous UL CA

  - MPR evaluation methodology and assumption

    - MPR and CANS_04 A-MPR studies for PC1.5 contiguous intra-band ULCA focusses on TxD architecture and may account for PSD imbalance with up to 6dB.

      - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB

    - PC1.5 contiguous intra-band ULCA based on dualPA architecture with two LOs is not specified in R19

    - MPR studies for PC1.5 non-contiguous intra-band ULCA focusses on dualPA architecture with two LOs and may account for PSD imbalance with up to 6dB

      - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB

    - PC1.5 non-contiguous intra-band ULCA based on TxD architecture is not specified in R19

  - PCMAX,C

    - For intra-band UL NC CA with dualPA-architecture, the upper bound of Pcmax,c is 26dBm per CC

      - Whether or how to reflect the above agreement in spec is FFS

    - For intra-band UL contiguous CA with TxD, the upper bound of Pcmax,c is 29dBm per CC

      - Whether or how to reflect the above agreement in spec is FFS

  - PCMAX

    - For intra-band contiguous ULCA with 2Tx-TxD, the upper bound of Pcmax is 29dBm

    - For intra-band UL NC CA with dual PA architecture, the upper bound of Pcmax is 29dBm

- 2-band Inter-band UL NR-CA/EN-DC with 2Tx and/or 3Tx

  - Configuration

    - For PC1.5 inter-band UL CA/EN-DC, the work for the case PC2 FDD band as one of the constituent band would be triggered after example combo(s) is added in the WID

  - Spec organization

    - Adopt same way as in Rel-18

  - 3UL CC with 3Tx

    - Don not consider 3ULCC with 3Tx CA_nXA-nY(2A)/ DC_XA_nY(2A) scenario

- Increasing UE transmission power

  - The methodology

    - Use higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability

    - No new power class is introduced

  - The scenarios to be considered in Rel-19

    - For both 2Tx and 3Tx, higherPowerLimit-R17/higherPowerLimtMRDC-R17 is enabled for any specified inter-band band combination

  - MSD rules

    - RAN4 can discuss the MSD general rules, and strive to down-select among the following options

      - PC2/PC1.5 MSD requirements are not specified. UE shall meet the Rx requirements of default power class, with default power class transmission

      - Using some equations or look-up tables or other means to derive PC2/PC1.5 MSD from respective PC3 MSD

      - Other solutions are not precluded

      - NOTE: RAN4 continues normal procedure specifying MSD for all PC’s band combination supports until down-selection is done

- For power boosting and/or MPR reduction, WF was approved in R4-2414441.

- Power domain enhancements for single carrier

  - Approaches for scenario 1

    - Further evaluate the reduced MPR values with relaxation of ACLR/SEM to certain levels or waived

    - Other ways to reduce MPR are not precluded

  - Approaches of converting outer RB allocation to inner RB allocation

    - Focused on extended UE CBW on common issues, which could also be applied for BS CBW based approach

  - Whether extended UE CBW edge could exceed the BS CBW edge

    - The extended UE CBW should not exceed the BS CBW, FFS multi-CC/BS transmitted BW case

  - Whether and where to use IBE in the larger BS CBW

    - Focus on how to extend IBE for usage between edges of UE CBW and extended UE CBW

  - Boundary to apply SE

    - [SE applicability depends on the extended CBW]

  - Whether and how to consider the Asymmetrical extended CBW approach

    - Asymmetrical extended CBW approach can be considered, FFS the details

  - Signaling aspects

    - To discuss the signaling aspects after sufficient evaluation of power boosting and/or MPR reduction in terms of relaxed requirements

- MPR applicability for FR1 intra-band UL CA

  - The work scope for FR1 NC CA should be limited to only study MPR applicability

- MPR applicability for FR2

  - Whether new CA MPR for CABW < 400MHz should be considered in the WI

    - Hold on discussions on whether new MPR requirement is defined for CABW < 400MHz unless the WID can be updated accordingly.

    - Only MPR applicability needs to be discussed in this WID, and defining new MPR requirement is out of scope

  - Whether FR2 MPR enhancement for the single UL case is also applicable for intra-band DL non-contiguous CA with single UL

    - FR2 MPR enhancement for the single UL case is not applicable for intra-band DL non-contiguous CA with single UL unless the WID is revised

  - All FR2 power classes could be considered for the MPR enhancement

  - MPR reduction applies to both FR2-1 and FR2-2

  - Applicable MPR for FR2 UL CA with DL intra band CA

    - In general, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW)

    - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused

    - No need to specify new column for <400MHz CABW in the MPR table. FFS specific spec change to accommodate 200MHz BW granularity

    - FFS whether to take the minimum values of MPR between the intra-band CA case and the single carrier for FR2-2

- For 6Rx for handheld and FWA UE, WF was approved in R4-2414285.

- REFSENS

  - General considerations for specifying ΔRIB,6R value

    - Include n104 in the high band (n77, n78, n79) category

    - Different value for handheld UE and FWA

    - Companies encouraged to provide their views on the ZTE proposal in Proposal 5 in R4-2411883 and any further positions on verification of 6Rx receiver requirements at future meetings given that ΔRIB,6R values are now defined

- ΔRIB,6R values for handheld UE and FWA

  - Adopt the average value of ΔRIB,6R for handheld UE from company proposals (updated to include company proposals from RAN4#111) and specify it for handheld UE devices.

| Average | Band n41 | -3.4 |
| --- | --- | --- |
|  | Band n77, n78, n79, n104 | -3.0 |

  - Adopt the average value of ΔRIB,6R for FWA UE from company proposals indicating different FWA UE values from handheld UE values and specify it for FWA UE devices

| Average | Band n41 | -3.7 |
| --- | --- | --- |
|  | Band n77, n78, n79, n104 | -3.3 |

- SRS antenna switching and ΔTRxSRS

  - Whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

    - Further discuss the following options

      - Option 1: Do not consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

      - Option 2: Consider additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

  - Have additional 3dB for power class 2 when ΔPPowerClass = 0dB and not indicating Tx diversity capability

  - ∆TRxSRS values

    - RAN4 agrees to adopt the average values for Bands n41, n77, and n78. For Band n79 and n104, further analysis is needed. Take Option 10 and option 11 as the starting point, not precluding any other numbers

      - Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS for bands whose FUL_high is lower than the FUL_low of n79.

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n41, n77, n78 | 4.0 | 3.5 | 4.5 |

      - Option 10: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS without considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104.

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79, n104 | 5.4 | 4.7 | 5.9 |

      - Option 11: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104.

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79 | 5.4 | 4.7 | 5.9 |
|  | Band n104 | 5.7 | 5.0 | 6.3 |

- MIMO layer evaluation for 6Rx UE

  - Tightening BS EVM requirement

    - Do not tighten the minimum requirements of BS Tx EVM

    - Further discuss the assumption of Tx EVM for 6-layer performance evaluation

  - 6-Layer Performance Evaluation Assumptions

    - RAN4 to further discuss and align the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios

  - Performance requirements for 6Rx

    - RAN4 to defer decision after evaluation of 6 MIMO layer performance for handheld and FWA devices

- 6-layer support

  - RAN4 to further discuss the following options

    - Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios

    - Option 2: 6-layer support is feasible for FWA devices

    - Option 3: Companies to align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility using the following as a starting point

      - Feasibility criteria

        - Consider 4-layer vs 6-layer throughput performance

      - Simulation assumptions

        - Only consider link-level simulation for performance evaluations

        - Simulation assumptions provided in Annex A of R4-2414285 as a starting point. Companies to clarify the settings used in the simulation assumptions for BS correlation matrix and Tx EVM

    - Option 4: 6-layer support is not feasible for handheld UEs.

  - RAN4 to further discuss if 6-Layer support should be considered an optional feature

- SRS IL imbalance issue

  - Whether to solve SRS IL imbalance issue in Rel-19

    - RAN4 to further discuss the following options

      - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue.

      - Option 2: Continue to pursue a solution to the SRS IL imbalance issue dependent on the outcome of Issue 4-1-2 in the approved WF in R4-2410751

  - Initial Considerations for SRS IL imbalance issue

    - Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL

      - Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL.

      - Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation

  - SRS IL imbalance issue solutions

    - Companies encouraged to provide compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain

RRM Core part:

- WF on RRM requirements was approved in R4-2413972.

- RRM requirements for R19 UE RF enhancements for NR FR1/FR2 and EN-DC

  - No RRM work is expected due to the WI objective on introducing support for the higher power UE for CA

  - No RRM work is expected due to the WI objective on power domain enhancements

  - Cell identification delay, measurement delay, mobility requirements for 6Rx capable UE

    - Agree on the following and no core spec impact

      - The legacy core RRM requirements for identification delay, measurement delay, mobility requirements, scheduling restriction are reused for 6 Rx capable UEs.

      - Note: Interruption requirements at SRS antenna switching for 6Rx capable UE is discussed in a separate issue

  - Interruption requirements at SRS antenna switching for 6Rx capable UE, to be further discussed with the following options

    - Option 1: To wait for RAN1 conclusion on the 6Rx SRS antenna switching pattern to make sure that the legacy requirements can be reused.

    - Option 2: The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed

RRM Perf part:

- WF on RRM requirements was approved in R4-2413972.

- RRM performance requirements for 6Rx capable UE

  - RAN4 introduce the antenna connection for 6Rx capable UEs in TS 38.133 A.3.6, where the following aspects are specified

    - Testing principles for 6Rx capable UEs

    - RLM and BFD testing

  - SNR level for RLM and BFD testing for 6Rx capable UE, to be further discussed with the following options

    - Option 1: Reuse SNR levels specified for 4Rx capable UE

    - Option 2: RAN4 to discuss impact on RLM/BFD tests when operating with 6 Rx UEs

2.4.2	Remaining Open issues

HPUE for CA/DC in terrestrial network (TN)

- Specify the generic requirements of high power UE (HPUE) for NR uplink (UL) CA in FR1 and EN-DC with NR FR1 bands

  - Power class 1.5 (PC1.5) UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx

    - Specify the requirements for intra-band UL contiguous CA with or without UL-MIMO

      - Example band combinations:

        - CA_n41C, CA_n78C, CA_n77C, CA_n79C for intra-band uplink contiguous CA configurations

      - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution

    - Specify the requirements for intra-band UL non-contiguous CA without UL-MIMO

      - Example band combinations:

        - CA_n78(2A), CA_n77(2A) for intra-band uplink non-contiguous CA configurations

      - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution

    - NOTE: leave the other band combination specific requirements to the corresponding Rel-19 basket WIs

  - PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA, and PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA

    - Focus on the SAR solution

    - Enable power class 2 (PC2) and PC1.5 of two band inter-band uplink CA and EN-DC with 3Tx for handheld UE

      - Identify and update the requirements if necessary

    - Only PC3 is considered for LTE FDD in EN-DC

    - NOTE: leave the band combination specific requirements, e.g., MSD to the corresponding Rel-19 basket WIs

  - Investigate and if feasible, support increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band uplink CA and EN-DC HPUE with the different existing power classes which have already been specified

  - Introduce the signaling to support the above objectives, if needed.

  - Consider release independency, if needed

Power boosting and/or MPR reduction

- Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA

  - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier

    - Include the following scenarios:

      - when there is no adjacent in-band/out-of-band co-existence issue

      - when a UE uses a narrower channel bandwidth within a wider BS bandwidth

    - Include both (e)RedCap UE (only PC3) and non-RedCap UE

    - Limited to QSPK and 16QAM

  - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration

    - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1

    - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2

    - MPR requirement is not applicable until the SCell is activated

  - Necessary signaling to support the above objectives

6Rx for handheld and FWA UE  [RAN4, RAN1]

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

    - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

    - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

Performance part:

6Rx for handheld and FWA UE

- Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario

  - Investigate and if necessary, specify RLM test cases to support 6Rx

  - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx

    - Support 4 MIMO layers at least, and follow the conclusion in core part for whether to support 6 MIMO layers

  - Specify the SDR requirements to support 6Rx

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#112

- R4-2412818	Topic summary for [112][116] NR_ENDC_RF_Ph4_part1	Moderator (Huawei)

- R4-2412819	Topic summary for [112][117] NR_ENDC_RF_Ph4_part2	Moderator (Samsung)

- R4-2412820	Topic summary for [112][118] NR_ENDC_RF_Ph4_part3	Moderator (ATT)

- R4-2411808	Topic summary for [112][213] NR_ENDC_RF_Ph4	Moderator (Huawei)

- R4-2414277	WF on high power UE	Samsung

- R4-2414441	WF on power domain enhancements	Huawei, HiSilicon

- R4-2414285	WF on requirements for 6Rx	AT&T

- R4-2413972	WF on RRM requirements for R19 UE RF enhancements for NR FR1/FR2 and EN-DC

- R4-2411168	MPR for PC1.5 intra-band contiguous UL CA	Apple

- R4-2411169	Rel-19 HPUE for inter-band UL CA/EN-DC with 2Tx or 3Tx	Apple

- R4-2411170	On Rel-19 increasing UE transmission high power limit	Apple

- R4-2411302	On equal PSD vs equal power spectral regrowth	Skyworks Solutions Inc.

- R4-2411315	Views on HPUE intra-band CA	Samsung

- R4-2411316	Views on HPUE inter-band CA with 2Tx or 3Tx	Samsung

- R4-2411317	Views on Increasing higher power limit feature	Samsung

- R4-2411595	Discussion on PC1.5 TDD intra-band CA	Xiaomi

- R4-2411596	Discussion on PC1.5 UE for two band NR inter-band uplink CA	Xiaomi

- R4-2411597	Discussion on increasing high power limit for inter-band CA DC with 2Tx and or 3Tx	Xiaomi

- R4-2411646	On PC1.5 intra-band contiguous ULCA with 2Tx	Skyworks Solutions Inc.

- R4-2411648	High power UE RF requirements for intra-band CA in TN	Meta Ireland

- R4-2411649	High power inter-band CA/DC UE RF requirements including 2Tx/3Tx within 2 bands	Meta Ireland

- R4-2411672	Framework for intra-band UL CA with PC1.5	Ericsson

- R4-2411869	HPUE for inter-band UL CA and EN-DC	LG Electronics

- R4-2411870	HPUE for increasing high power limit	LG Electronics

- R4-2411880	Further discussion on R19 Increasing UE transmission power limit	ZTE Corporation, Sanechips

- R4-2411881	Further discussion on R19 2Tx/3Tx PC2/1.5 Inter-band NR CA/ENDC	ZTE Corporation, Sanechips

- R4-2411882	Further discussion on R19 PC1.5 Intra-band UL CA	ZTE Corporation, Sanechips

- R4-2412007	UE RF Enh 4: On necessity of additional MSD requirement for HPUE	Nokia

- R4-2412008	UE RF Enh 4: Increasing UE transmission power	Nokia

- R4-2412024	HPUE for intra-band UL CA	LG Electronics Finland

- R4-2412073	Further discussion on HPUE for intra-band contiguous and non-contiguous CA	vivo

- R4-2412092	Discussion on UL inter-band UL CA or DC with 2Tx or 3Tx	vivo

- R4-2412093	Discussion on increasing transmission high power limit for CA HPUE	vivo

- R4-2412264	Discussion on SAR solutions for new Rel-19 inter-band EN-DC HPUE scenarios	CHTTL

- R4-2412277	On PC1.5 intra-band non-contiguous ULCA with dualPA	Skyworks Solutions Inc.

- R4-2412349	R19 MPR for PC1.5 contiguous CA	OPPO

- R4-2412350	R19 MPR for PC1.5 NC CA	OPPO

- R4-2412432	Views on SAR Solution	China Telecom

- R4-2412619	MSD for HPUE	Qualcomm France

- R4-2412675	Discussion on MSD rules and UE types for HPUE CA	NTT DOCOMO, INC.

- R4-2413028	Discussion on PC1.5 for intra-band CA	Huawei, HiSilicon

- R4-2413029	Discussion on Increasing UE transmission high power limit	Huawei, HiSilicon

- R4-2413225	On wider applicability of higherPowerLimit-r17	Qualcomm Incorporated

- R4-2413400	RF requirements for HPUE for CA terrestrial networks	Qualcomm Technologies Int

- R4-2411049	On improved MPR for intra-band ULCA when only one CC has RBs allocated	Skyworks Solutions Inc.

- R4-2411108	Further discussion on power domain enhancements for single carrier	CATT

- R4-2411109	Further discussion on MPR applicability for FR1 intra-band UL CA	CATT

- R4-2411110	Further discussion on MPR applicability for FR2	CATT

- R4-2411153	On Rel-19 power domain enhancement	Apple

- R4-2411314	Draft Rel-19 CR on MPR applicability for intra-band contiguous CA with single CC with activated cell	Samsung

- R4-2411326	Discussion on MPR reduction for FR1 single carrier	Samsung

- R4-2411327	Discussion on MPR reduction for FR1 intra-band UL CA	Samsung

- R4-2411535	Further Views on MPR Reduction	Sony

- R4-2411601	Discussion on power domain enhancement for single carrier	Xiaomi

- R4-2411631	Power boosting and MPR reduction	Qualcomm Technologies Int

- R4-2411632	MPR applicability for FR1 intra-band UL CA	Qualcomm Technologies Int

- R4-2411674	Power domain enhancements for single carrier	Ericsson

- R4-2411675	MPR applicability for non-contiguous UL CA in fragmented spectrum	Ericsson

- R4-2411693	Discussion on MPR reduction for FR2 CA	Samsung

- R4-2411851	Discussion on power domain enhancements for single carrier	ZTE Corporation, Sanechips

- R4-2411852	Discussion on MPR applicability for FR1 intra-band UL CA	ZTE Corporation, Sanechips

- R4-2411853	Discussion on MPR applicability for FR2	ZTE Corporation, Sanechips

- R4-2412009	UE RF Enh 4: Power domain enhancements for single carrier	Nokia

- R4-2412010	R19 UE RF Enh 4 MPR applicability for FR1 intra-band UL CA	Nokia

- R4-2412085	Discussion on power domain enhancements for NR single carrier	vivo

- R4-2412086	Discussion on MPR applicability for FR1 intra-band UL CA	vivo

- R4-2412351	R19 MPR reduction for single carrier	OPPO

- R4-2412356	Discussion on FR2 CA MPR improvement	NTT DOCOMO, INC.

- R4-2412433	Views on Power domain enhancements for single carrier	China Telecom

- R4-2412553	Discussion on power domain enhancement for NR single carrier	MediaTek (Wuhan) Inc.

- R4-2412568	On power domain enhancements for single carrier	Huawei, HiSilicon

- R4-2412569	On MPR applicability for FR1 intra-band UL CA	Huawei, HiSilicon

- R4-2412570	On MPR applicability for FR2 CA	Huawei, HiSilicon

- R4-2412579	On reduced MPR when BS CBW is larger than UE CBW	Skyworks Solutions Inc.

- R4-2413226	Activation-based FR2 MPR enhancement	Qualcomm Incorporated

- R4-2413456	Discussion on MPR applicability for FR1 intra-band UL CA	LG Electronics UK

- R4-2413457	Discussion on Power domain enhancements for single carrier	LG Electronics UK

- R4-2411168	MPR for PC1.5 intra-band contiguous UL CA	Apple

- R4-2411169	Rel-19 HPUE for inter-band UL CA/EN-DC with 2Tx or 3Tx	Apple

- R4-2411170	On Rel-19 increasing UE transmission high power limit	Apple

- R4-2411302	On equal PSD vs equal power spectral regrowth	Skyworks Solutions Inc.

- R4-2411315	Views on HPUE intra-band CA	Samsung

- R4-2411316	Views on HPUE inter-band CA with 2Tx or 3Tx	Samsung

- R4-2411317	Views on Increasing higher power limit feature	Samsung

- R4-2411595	Discussion on PC1.5 TDD intra-band CA	Xiaomi

- R4-2411596	Discussion on PC1.5 UE for two band NR inter-band uplink CA	Xiaomi

- R4-2411597	Discussion on increasing high power limit for inter-band CA DC with 2Tx and or 3Tx	Xiaomi

- R4-2411646	On PC1.5 intra-band contiguous ULCA with 2Tx	Skyworks Solutions Inc.

- R4-2411648	High power UE RF requirements for intra-band CA in TN	Meta Ireland

- R4-2411649	High power inter-band CA/DC UE RF requirements including 2Tx/3Tx within 2 bands	Meta Ireland

- R4-2411672	Framework for intra-band UL CA with PC1.5	Ericsson

- R4-2411869	HPUE for inter-band UL CA and EN-DC	LG Electronics

- R4-2411870	HPUE for increasing high power limit	LG Electronics

- R4-2411880	Further discussion on R19 Increasing UE transmission power limit	ZTE Corporation, Sanechips

- R4-2411881	Further discussion on R19 2Tx/3Tx PC2/1.5 Inter-band NR CA/ENDC	ZTE Corporation, Sanechips

- R4-2411882	Further discussion on R19 PC1.5 Intra-band UL CA	ZTE Corporation, Sanechips

- R4-2412007	UE RF Enh 4: On necessity of additional MSD requirement for HPUE	Nokia

- R4-2412008	UE RF Enh 4: Increasing UE transmission power	Nokia

- R4-2412024	HPUE for intra-band UL CA	LG Electronics Finland

- R4-2412073	Further discussion on HPUE for intra-band contiguous and non-contiguous CA	vivo

- R4-2412092	Discussion on UL inter-band UL CA or DC with 2Tx or 3Tx	vivo

- R4-2412093	Discussion on increasing transmission high power limit for CA HPUE	vivo

- R4-2412264	Discussion on SAR solutions for new Rel-19 inter-band EN-DC HPUE scenarios	CHTTL

- R4-2412277	On PC1.5 intra-band non-contiguous ULCA with dualPA	Skyworks Solutions Inc.

- R4-2412349	R19 MPR for PC1.5 contiguous CA	OPPO

- R4-2412350	R19 MPR for PC1.5 NC CA	OPPO

- R4-2412432	Views on SAR Solution	China Telecom

- R4-2412619	MSD for HPUE	Qualcomm France

- R4-2412675	Discussion on MSD rules and UE types for HPUE CA	NTT DOCOMO, INC.

- R4-2413028	Discussion on PC1.5 for intra-band CA	Huawei, HiSilicon

- R4-2413029	Discussion on Increasing UE transmission high power limit	Huawei, HiSilicon

- R4-2413225	On wider applicability of higherPowerLimit-r17	Qualcomm Incorporated

- R4-2413400	RF requirements for HPUE for CA terrestrial networks	Qualcomm Technologies Int

- R4-2411568	RRM requirements for NR FR1/FR2 and EN-DC Phase 4	Nokia, Nokia Shanghai Bell

- R4-2412223	Discussion on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4	Huawei, HiSilicon

- R4-2412404	Discussions on RRM impact of Rel-19 WI on UE RF enhancements WI	Ericsson

RAN4#111

- R4-2408935	Topic summary for [111][124] NR_ENDC_RF_Ph4_part1	Moderator (Huawei)

- R4-2408936	Topic summary for [111][125] NR_ENDC_RF_Ph4_part2	Moderator (Samsung)

- R4-2408937	Topic summary for [111][126] NR_ENDC_RF_Ph4_part3	Moderator (ATT)

- R4-2410565	WF on high power UE	Samsung

- R4-2410749	WF on power domain enhancements	Huawei, HiSilicon

- R4-2410751	WF on requirements for 6Rx	AT&T

- R4-2407091	Views on intra-band contiguous and non-contiguous UL CA with PC1.5	Apple

- R4-2407092	Views on inter-band UL NR-CA/EN-DC with 2 bands and 2Tx and/or 3Tx	Apple

- R4-2407093	Views on increasing UE transmission high power limit	Apple

- R4-2407371	PC1p5 intra-band ULCA MOP for 2Tx and Dual-PA cases	Skyworks Solutions Inc.

- R4-2407396	On increased power cases and levels	Skyworks Solutions Inc.

- R4-2407550	Consideration on HPUE for CA and EN-DC	CATT

- R4-2407575	Request for PC2/PC1.5 inter-band FDD-FDD UL CA configuration	Dish Network

- R4-2407584	RF requirements for HPUE for CA terrestrial networks	Qualcomm Technologies Int

- R4-2407630	Views on HPUE for Intra-band CA in terrestrial network	Samsung

- R4-2407631	Views on HPUE for inter-band NR-CA and EN-DC in terrestrial network	Samsung

- R4-2407632	Views on Increasing UE transmission power	Samsung

- R4-2407713	Proposal for UL CA_n41C PC2 A-MPR	T-Mobile USA

- R4-2407721	General framework for intra-band UL CA with PC1.5	Ericsson

- R4-2407806	Discussion on PC1.5 TDD intra-band CA	Xiaomi

- R4-2407807	Discussion on inter-band UL NR CAEN-DC with 2 bands and 2Tx andor 3Tx	Xiaomi

- R4-2407808	Discussion on increasing high power limit for inter-band CA DC with 2Tx and or 3Tx	Xiaomi

- R4-2407892	Discussion on SAR solutions for new Rel-19 inter-band HPUE scenarios	CHTTL

- R4-2407909	Discussion on Rel-19 UE RF HPUE enhancements for 3Tx configurations	MediaTek (Shenzhen) Inc.

- R4-2407980	HPUE for intra-band UL CA	LG Electronics

- R4-2407981	HPUE for inter-band UL CA and EN-DC	LG Electronics

- R4-2407982	HPUE for increasing high power limit	LG Electronics

- R4-2408028	High power UE RF requirements for intra-band CA UE	Facebook Japan G.K.

- R4-2408030	High power UE RF requirements for inter-band CA/EN-DC UE	Facebook Japan G.K.

- R4-2408033	Criteria for specifying new MSD test cases	Qualcomm Incorporated

- R4-2408034	Feasibility of wider application of higherPowerLimit-r17	Qualcomm Incorporated

- R4-2408076	Views on SAR Solution of R19 HPUE for CA and EN-DC	China Telecom

- R4-2408121	Discussion on increasing UE transmission high power limit for CA HPUE	vivo

- R4-2408123	Further discussion on HPUE for intra-band contiguous and non-contiguous CA	vivo

- R4-2408131	Discussion on UL inter-band UL CA or DC with 2Tx or 3Tx	vivo

- R4-2408136	Further discussion on SAR solution of HPUE	vivo

- R4-2408351	On R19 PC1.5 Intra-band contiguous and non-contiguous UL CA	ZTE Corporation, Sanechips

- R4-2408352	On R19 2Tx/3Tx PC2/1.5 Inter-band NR CA/ENDC	ZTE Corporation, Sanechips

- R4-2408353	On R19 Increasing UE transmission power limit	ZTE Corporation, Sanechips

- R4-2408724	HPUE MSD related issues	Nokia

- R4-2408758	R19 3Tx inter-band enh	OPPO

- R4-2409029	Discussion on increasing UE high power limit feature	NTT DOCOMO INC.

- R4-2409643	Discussion on PC1.5 for intra-band CA	Huawei, HiSilicon

- R4-2409644	Discussion on PC1.5 for inter-band UL CA and EN-DC	Huawei, HiSilicon

- R4-2409645	Discussion on Increasing UE transmission high power limit	Huawei, HiSilicon

- R4-2407070	On MPR reduction for NR	Apple

- R4-2407227	On MPR reduction for NR UL CA	Apple

- R4-2407404	On emission requirement relaxations for power domain enhancements	Skyworks Solutions Inc.

- R4-2407409	Further Views on MPR reduction	Sony

- R4-2407551	Consideration on MPR applicability for FR1 intra-band UL CA	CATT

- R4-2407585	Power boosting and MPR reduction	Qualcomm Technologies Int

- R4-2407586	MPR applicability for FR1 intra-band UL CA	Qualcomm Technologies Int

- R4-2407590	Discussion on MPR reduction for FR1 single carrier	Samsung

- R4-2407591	Discussion on MPR reduction for FR1 intra-band UL CA	Samsung

- R4-2407629	Draft Rel-19 CR on MPR applicability for intra-band contiguous CA with single CC with activated cell	Samsung

- R4-2407722	Power domain enhancements for single carrier?	Ericsson

- R4-2407723	MPR applicability for non-contiguous UL CA in fragmented spectrum	Ericsson

- R4-2407814	Discussion on power domain enhancement for single carrier	Xiaomi

- R4-2407815	Discussion on power domain enhancement for FR1 intra-band UL CA	Xiaomi

- R4-2407816	Discussion on power domain enhancement for FR2 intra-band UL CA	Xiaomi

- R4-2407893	Discussion on MPR reduction for FR2 CA	Samsung

- R4-2407910	Discussion on power domain enhancement for NR single carrier 	MediaTek (Shenzhen) Inc.

- R4-2408035	FR2 CA MPR enhancement by changing BW basis for MPR table	Qualcomm Incorporated

- R4-2408132	Discussion on power domain enhancements for NR single carrier	vivo

- R4-2408133	Discussion on MPR applicability for FR1 intra-band UL CA	vivo

- R4-2408134	Discussion on MPR applicability for FR2	vivo

- R4-2408222	Discussion on enhanced MPR scenario	CMCC

- R4-2408755	Discussion on FR2 MPR improvement for single carrier and intra band CA	NTT DOCOMO INC..

- R4-2408763	R19 power enhance for single carrier	OPPO

- R4-2408764	R19 MPR applicability for FR1 intra-band UL CA	OPPO

- R4-2408794	Discussion on power domain enhancements for single carrier	ZTE Corporation, Sanechips

- R4-2408795	Discussion on MPR applicability for FR1 intra-band UL CA	ZTE Corporation, Sanechips

- R4-2408796	Discussion on MPR applicability for FR2	ZTE Corporation, Sanechips

- R4-2409169	On power domain enhancements for single carrier	Huawei, HiSilicon

- R4-2409170	On MPR applicability for FR1 intra-band UL CA	Huawei, HiSilicon

- R4-2409171	On MPR applicability for FR2 CA	Huawei, HiSilicon

- R4-2409194	Power domain enhancements for single carrier	Nokia

- R4-2409628	Discussion on power domain enhancements for single carrier	LG Electronics UK

- R4-2407071	Views on REFSENS and SRS IL Imbalance for 6Rx	Apple

- R4-2407272	Views on Maximum Number of MIMO Layers for 6Rx UEs	Apple

- R4-2407313	Discussion on NR 6RX UE RF REFSENS requirements	Mediatek India Technology Pvt.

- R4-2407552	Consideration on 6Rx requirements	CATT

- R4-2407698	Feasibility evaluation of MIMO layer for 6Rx UE	MediaTek inc.

- R4-2407699	UE SRS IL imbalance for 6Rx UE	MediaTek inc.

- R4-2407803	Discussion on reference sensitivity requirements for 6Rx UE	Xiaomi

- R4-2407804	Discussion on SRS antenna switching requirements for 6Rx UE	Xiaomi

- R4-2407988	Discussion for 6 Rx REFSENS	LG Electronics France

- R4-2407989	Discussion for SRS antenna switching and ?TRxSRS	LG Electronics France

- R4-2407990	Discussion for SRS IL imbalance issue	LG Electronics France

- R4-2407992	Discussion on reference sensitivity requirements for 6RX UE	Spreadtrum Communications

- R4-2407993	Discussion on SRS antenna switching requirements for 6RX UE	Spreadtrum Communications

- R4-2407994	Discussion on SRS IL imbalance for 6RX UE	Spreadtrum Communications

- R4-2408031	Discussion on 6Rx REFSENS and other RF requirements for single carrier 	Facebook Japan G.K.

- R4-2408069	On SRS IL imbalance	China Telecom

- R4-2408122	Discussion on SRS IL imbalance reporting	vivo

- R4-2408124	Further discussion on 6Rx REFSENS requirements for FWA and Handheld UE	vivo

- R4-2408125	Simulation results of MIMO layer evaluation for 6Rx UE	vivo

- R4-2408126	Further discussion on SRS antenna switching requirements	vivo

- R4-2408354	On 6Rx Reference sensitivity requirements	ZTE Corporation, Sanechips

- R4-2408355	On 6Rx SRS antenna switching requirements	ZTE Corporation, Sanechips

- R4-2408356	On 6Rx SRS antenna IL	ZTE Corporation, Sanechips

- R4-2408614	Views on SRS insertion loss compensation and reporting enhancements	Intel Corporation

- R4-2408725	6Rx for handheld and FWA UE Reference sensitivity requirements	Nokia

- R4-2408726	6Rx for handheld and FWA SRS IL balance	Nokia

- R4-2408759	R19 6Rx REFSENS	OPPO

- R4-2408760	R19 6Layer for handheld UE	OPPO

- R4-2408761	R19 6Rx SRS IL	OPPO

- R4-2408762	R19 6Rx SRS IL imbalance	OPPO

- R4-2408840	6Rx REFSENS	Qualcomm France

- R4-2408841	6Rx SRS antenna switching requirements	Qualcomm France

- R4-2408842	6Rx SRS IL Imbalance issue	Qualcomm France

- R4-2408843	Number of MIMO layers for 6Rx	Qualcomm France

- R4-2409049	Discussion on REFSENS requirement for 6Rx UE	Google Inc.

- R4-2409050	Discussion on MIMO layer for 6Rx UE	Google Inc.

- R4-2409052	Discussion on SRS antenna switching requirements for 6Rx UE	Google Inc.

- R4-2409053	Discussion on SRS IL imbalance for 6Rx UE	Google Inc.

- R4-2409091	Discussion on MIMO layer evaluation for 6Rx UE	ZTE Corporation, Sanechips

- R4-2409172	On 6Rx reference sensitivity requirements for FR1 UE	Huawei, HiSilicon

- R4-2409173	On MIMO layer evaluation for 6Rx UE	Huawei, HiSilicon

- R4-2409174	On 6Rx SRS antenna switching requirements	Huawei, HiSilicon

- R4-2409175	On SRS IL imbalance issue	Huawei, HiSilicon

- R4-2409459	Views on MIMO layer for 6Rx UE	Samsung

- R4-2409460	Views on SRS IL imbalance	Samsung

- R4-2409666	Discussion on reference sensitivity for 6Rx	Ericsson

- R4-2409667	Discussion on the support of MIMO layers for 6Rx UE	Ericsson

- R4-2409736	SRS Power Imbalance Determination	Lenovo

- R4-2409759	On need for SRS insertion loss imbalance reporting	Ericsson India Private Limited

- R4-2409760	On DeltaT_RxSRS requirement for 6Rx UEs	Ericsson India Private Limited

RAN4#110-bis

- R4-2405282	Topic summary for [110bis][129] NR_ENDC_RF_Ph4	Moderator(Huawei)

- R4-2406583	WF on HPUE UE for UL CA and EN-DC	Samsung

- R4-2406584	WF on power domain enhancement	Huawei

- R4-2406585	WF on requirements for 6Rx	AT&T

- R4-2406718	LS on 3T6R and 4T6R antenna switching SRS	Huawei, HiSilicon

- R4-2404178	Views on 6Rx for handheld UEs	Apple

- R4-2404184	On Rel-19 HPUE for CA and DC	Apple

- R4-2404194	On power domain enhancement for NR	Apple

- R4-2404208	High power UE for CA in terrestrial networks	Qualcomm Technologies Int

- R4-2404209	Power boosting and MPR reduction	Qualcomm Technologies Int

- R4-2404270	Views on MPR reduction	Sony

- R4-2404455	Initial consideration on HPUE for CA in terrestrial network (TN)	CATT

- R4-2404456	Initial consideration on power domain enhancement for single carrier operation	CATT

- R4-2404485	On SAR Solution of R19 HPUE for CA	E-surfing Digital

- R4-2404539	Discussion on 6Rx initial consideration points for single carrier	Meta Ireland

- R4-2404541	High power UE for intra/inter-band CA/DC including 2Tx/3Tx in TN	Meta Ireland

- R4-2404544	Discussion on NR 6Rx UE	Xiaomi

- R4-2404546	Discussion on power domain enhancement	Xiaomi

- R4-2404551	Discussion on PC1.5 TDD intra-band CA	Xiaomi

- R4-2404552	Discussion on PC1.5 UE for two band NR inter-band uplink CA	Xiaomi

- R4-2404553	Discussion on increasing high power limit for inter-band CA DC with 2Tx and or 3Tx	Xiaomi

- R4-2404587	Discussion on 6RX for handheld UE and FWA	Spreadtrum Communications

- R4-2404588	Discussion on MPR reduction for FR1 and FR2	Samsung

- R4-2404605	Discussion on HPUE for CA and EN-DC	LG Electronics

- R4-2404616	Initiatory discussion on power domain enhancement	E-surfing Digital

- R4-2404623	PC 1.5 for band combinations and the general framework for high power classes	Ericsson

- R4-2404624	MPR reductions for UE-specific bandwidths and for UL intra-band CA	Ericsson

- R4-2404642	Discussion on MPR reduction for NR single carrier and NR intra-band UL CA	vivo

- R4-2404665	Discussion on high power UE for CA in terrestrial network	vivo

- R4-2404666	Discussion on 6Rx for handheld and FWA UE	vivo

- R4-2404745	Discussions on TDD intra-band CA HPUE	China Unicom

- R4-2404793	Discussion for 6 Rx	LG Electronics France

- R4-2404885	Initial consideration on R19 UE RF enh for HPUE in TN	Huawei, HiSilicon

- R4-2404901	On architecture aspects for PC1p5 intra-band ULCA support	Skyworks Solutions Inc.

- R4-2404930	Views on SRS insertion loss compensation and reporting enhancements	Intel Corporation

- R4-2404935	Views on 6Rx for handheld and FWA UE	Samsung

- R4-2404936	FR2 MPR requirements improvement for single carrier or intra band CA	NTT DOCOMO, INC.

- R4-2404986	Views on HPUE	Samsung

- R4-2405059	Initial discussion on Rel-19 High power UE (HPUE) support for EN-DC	CHTTL

- R4-2405064	Discussion on power boosting and/or MPR reduction	ZTE Corporation

- R4-2405083	UE SRS IL imbalance for 6Rx UE	Shanghai Chen Si Electronics

- R4-2405182	R19 3Tx inter-band enh	OPPO

- R4-2405183	R19 6Rx delta RIB	OPPO

- R4-2405226	Discussion on R19 FR1 TN HPUE requirements	ZTE Corporation

- R4-2405227	Discussion on NR 6Rx requirement	ZTE Corporation

- R4-2405306	Discussion on enhanced MPR scenarios	CMCC

- R4-2405409	Discussion on PC 1.5 EN-DC	Nokia

- R4-2405486	Work plan for Rel-19 UE RF enhancements	Huawei, HiSilicon, AT&T

- R4-2405497	Discussion and overview on Rel-19 UE RF HPUE enhancements	MediaTek Inc.

- R4-2405606	Discussion on MPR reduction for power domain enhancement	LG Electronics UK

- R4-2405614	On power domain enhancement for NR single carrier and NR intra-band UL CA for PC2 and PC3	Huawei, HiSilicon

- R4-2405615	On RF requirements for 6Rx smartphone	Huawei, HiSilicon

- R4-2405619	Discussion on power domain enhancement for NR single carrier	MediaTek (Wuhan) Inc.

- R4-2405727	Initial considerations on 6RX	Qualcomm France

- R4-2405933	Initial discussion on SRS insertion loss imbalance reporting and 6Rx requirements	Ericsson India Private Limited

- R4-2405964	Initial views on UL CA and EN-DC with 3Tx for handheld UE	NTT DOCOMO INC.

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template