3GPP TSG RAN Meeting #105	   RP-242098

Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.2.2

Source:	Lenovo

Title:	Clarification on scope of Rel-19 Ambient IoT SI

Document for:	Discussion

# 1	Introduction

RAN#103 approved a revised Rel-19 SID on Ambient IoT solutions [1] and moderator summaries in RAN#103 and RAN#104 [2] [3] discussed on the way forward for several key issues.

This contribution discusses two aspects where we see a potential need for clarification of the SI scope.

# 2	Discussion

The contribution is related to RAN1 discussion on two important topics which requires further guidance from RAN plenary: The first aspect is related to the inventory completion time related to latency definition of the inventory use cases considering multiple devices and the second aspect is related to the differentiated design for device-2b.

## 2.1	On definition of Inventory completion time for multiple devices

The Rel-19 SID on the RAN design target says that the definition of latency can be refined in the RAN WGs depending on the use case. The RAN-plenary level Rel-18 SI which is documented in TR 38.848 [3] says in section 5.6 that the ‘RAN design targets’ on latency has two values : one is for the longer latency target of 10 seconds and the other is the shorter latency target of 1 second and the definition of latency can refined by RAN WGs.

The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices

The Rel-19 SID prioritized rUC1 (indoor inventory) and rUC4 (indoor command) for study and tasked RAN WGs to refine the definition of latency suitable for the use case. The RAN#103, discussed that the latency mentioned in section 5.6 of TR 38.848 is for a single device but did not specify the use case.

- Confirm that study of design of energy harvesting signal/waveform is out of SI scope in Rel-19 - The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1] - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds - Note: this value can be revisited in future RAN plenary meetings, if necessary - TR 38.848 clause 5.6 statement on latency remains the case with respect to a single device, i.e.: “NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only.” No SID revision is necessary

From the RAN#104 discussions, it was agreed to define inventory completion time for multiple devices as part of the study.

Conclusion RAN1 to define “successfully received” for the purpose of evaluations of the latency for a single A-IoT device. It is in scope of the study to define inventory completion time for multiple devices.

Furthermore, RAN1#118 defined inventory completion time for evaluation purpose only and RAN1 will not define a target for the inventory completion time.

Agreement The following performance metric is considered for evaluation purpose only, - Inventory completion time for multiple A-IoT devices o For inventory-only use case, the ‘Inventory completion time for multiple A-IoT devices’ is defined as the time a reader successfully completed the inventory process for at least 99% of all A-IoT devices within the coverage of the reader, assuming device density of 1.5 devices per m2. Note: RAN1 will not define a target for the inventory completion time

Agreement Companies should report their assumptions for evaluation (if any) of the inventory completion time for multiple devices. Companies can refer to the table in section 2.1 of R1-2407327 for information.

The commercial RFID readers available in the market provides the inventory speed [5] [6] in their data sheet, which is the number of tags inventoried per second. Similarly, it would be good to evaluate and define inventory completion time from multiple devices from 3GPP based reader.

For inventory use case, the multiple device latency can vary depending on the chosen multiplexing scheme i.e., TDMA; FDMA, CDMA. Individual single device latency can be meaningless as it does not provide much information on the completion time of an inventory. In such a case, a worst-case single device latency can be taken as a reference for the inventory completion time as the worst-case single device latency should not exceed the inventory completion time from a reader system perspective. However, even the worst-case single device latency cannot provide statistics about the number of inventoried devices within an inventory round.

Our proposal is to the define a target for the inventory completion time for multiple devices similar to the inventory speed to read RFIDs and the intention of the proposal is to make observation and conclusion from companies results eventually selecting a target performance metric to compare against commercially available RFID reader’s inventory speed with baseline multiplexing scheme. Hence, RAN should allow RAN1 to select a target based on the company’s evaluation results.

Observation 1: Commercial RFID reader defines inventory speed which is number of RFID tags inventoried per second in their data sheet.

Observation 2: RAN did not define a target for the inventory completion time for multiple devices to compare against commercially available RFID readers.

Observation 3: RAN1#118 agreed on the definition of the inventory completion time for multiple devices and evaluation assumptions are captured in the FL summary for reporting values.

Proposal 1: RAN should provide guidance to RAN1 to select a target value for the inventory completion time for multiple devices based on companies evaluation results.

## 2.2	Differentiated design for Device type 2b

The Ambient IoT SID insists to study a harmonized air interface design for different device categories but also states to study differences wherever necessary.

General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm,DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

Some companies thinks that the harmonized design as developing unified solution for all device types without considering the constraints and capabilities of various device types. Proponents of the unified solution thinks that the unified solution should be optimally designed for device type-1 operation which affects the operation and performance of the device type-2. The harmonized design should therefore be interpreted instead as a baseline design for Ambient IoT devices and some features can be removed, added depending on the device capabilities. The SID allows the differences in solution to be studied to optimally operate each device type. The following table compares some example design features for device type-1 and device type-2.

| D2R Features | Device type-1 | Device type-2b | Reason for differentiated design for device type-2 |
| --- | --- | --- | --- |
| Waveform | ON-OFF keying (OOK), BPSK | OOK, BPSK, MSK | MSK provides continuous signal benefitting the active device PA. |
| Channel coding | No FEC | FEC i.e., convolutional codes | Convolutional codes for device type-2 considering longer range |
| Transport block sizes from higher layer | < 150 bits Considering payload, CRC etc., | < 400 bits | Device SFO compensation allows higher TB size to be transmitted. |
| Segmentation | No | Yes | Higher payload of upto 1000 bits can be transmitted using higher layer segmentation |
| Device SFO compensation | No | Yes | Device compensates SFO and residual SFO compensated by reader thereby reducing reader receiver complexity. |
| Device power states | ON, OFF | ON, Sleep/memory retention state, OFF | Saves discharging for device type 2 |
| Duty cycle-based operation / device monitoring configuration | No | Yes | Device type 2 capable entering sleep/memory retention states |
| Wake up signal/trigger | No | Yes | Device type 2 wake up from sleep/memory retention state |
| Slot based access - TDMA | Single access slot i.e., reader sends R2D command with slot counter in every slot | Multi-access slot i.e., reader sends R2D command for N associated slots | Prevents device type 2 from monitoring every slot for R2D command. |
| CDMA | No | Yes | Atleast random access resources can be CDMed enabling reader to detect multiple devices to achieve faster access and less discharging time |

Table 1: Example D2R feature differences between device type 1 and device type 2b

Observation 4: Some companies thinks that the harmonized design means unified solution for all device types and hence the unified solution optimally designed considering only device type-1.

Observation 5: SID allows differences to be studied for different device types.

Proposal 2: Harmonized design should be interpreted as a baseline design feature for Ambient IoT devices while some features can be removed, added depending on the device capabilities.

Proposal 3: RAN should provide guidance to RAN1/RAN2 to allow studying differentiated D2R  solutions considering device type 2b in the remainder of study as the present thinking of harmonized design as unified solution for all device types favors device type-1 oepration while it is not optimal for device type 2b operation.

# 3	Conclusion

This contribution discusses two aspects where we see a potential need for clarification of the SI scope for the RAN1-led Rel-19 SI on Ambient IoT solutions. Based on the discussions in the previous sections, we make the following proposals:

Observation 1: Commercial RFID reader defines inventory speed which is number of RFID tags inventoried per second in their data sheet.

Observation 2: RAN did not define a target for the inventory completion time for multiple devices to compare against commercially available RFID readers.

Observation 3: RAN1#118 agreed on the definition of the inventory completion time for multiple devices and evaluation assumptions are captured in the FL summary for reporting values.

Proposal 1: RAN should provide guidance to RAN1 to select a target value for the inventory completion time for multiple devices based on companies evaluation results.

Observation 4: Some companies thinks that the harmonized design means unified solution for all device types and hence the unified solution optimally designed considering only device type-1.

Observation 5: SID allows differences to be studied for different device types.

Proposal 2: Harmonized design should be interpreted as a baseline design feature for Ambient IoT devices while some features can be removed, added depending on the device capabilities.

Proposal 3: RAN should provide guidance to RAN1/RAN2 to allow studying differentiated D2R  solutions considering device type 2b in the remainder of study as the present thinking of harmonized design as unified solution for all device types favors device type-1 operation while it is not optimal for device type 2b operation.

# References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, Huawei (moderator, RAN1 Vice-Chair), March 2024.

R1-240827, “Moderator's summary on R19 Ambient IoT”, Huawei, March 2024.

TR 38.848 V18.0.0, “Study on Ambient IoT (Internet of Things) in RAN”, September 2023.

3GPP TS 22.369 V19.1.0, " Service requirements for ambient power-enabled IoT", March 2024

https://www.uhfrfidantenna.com/sale-28586987d-impinj-r2000-built-in-uhf-rfid-fixed-reader-peak-inventory-speed-700-tags-sec.html

https://www.focus-rfid.com/oem-customized-long-distance-warehouse-inventory-management-uhf-rfid-fixed-reader-product/