3GPP TSG RAN Meeting #105	RP-242101

Melbourne, Australia, September 9-12, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Rel-19 NR MIMO Ph5

Agenda Item:	9.3.1.1

Document for:	Discussion and Decision

- Introduction

In Rel-19 NR MIMO Ph5 [1], the whole online/offline are well organized and then it is considerably ahead of schedule. (in RAN1#105, overall completion level: ~55% in core part [2]). To be more specific, we have the following observation:

- UE initiated beam management (UEIBM), as the heaviest topic in Rel-19 NR MIMO Ph5, has progressed well and is on track: the framework of the trigger event type (Event 1, 2, and 7) & trigger event detection, report format and report container/procedure has been well designed, and then the left-over issue(s) become controllable.

- Then, for other topics (up-to-128-port/HBF CSI, CJT-calibration, UL 3Tx operation, and asymmetric DL sTRP/UL mTRP deployment), major RAN1 (~90% or even more) workload is completed.

Therefore, in RAN1#105, we believe that it is an opportune moment to assess the workload and consider any potential scope extension. Meanwhile, in RAN#104, there was a brief offline/online discussion on potential scope extension for Rel-19 MIMO evolution, and then we have the following guidance from RAN Chair [3].

Conclusion Postpone discussions on the following topics until RAN#105. These topics will not be discussed in RAN1 before the decision in RAN#105. - Support of two TAs as part of enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios - SRS port grouping as part of CSI enhancement - Partial-coherent UL codebook for 3Tx multi-panel - Open loop based UL precoder cycling

In this contribution, we elaborate our views on the two-TA extension for asymmetric DL sTRP/UL mTRP deployment in Rel-19 NR-MIMO Ph5, supported by technical analysis and comprehensive evaluation.  In our view, this topic has the highest priority in Rel-19 NR-MIMO scope. After that, we provide our detailed thoughts on other topics, including SRS port grouping, partial-coherent UL codebook for 3Tx and open loop based UL precoding cycling.

- Discussion on scope extension for Rel-19 MIMO evolution

  - Two-TA extension for asymmetric DL sTRP/UL mTRP deployment

Heterogeneous Network can be deployed to improve UL throughput. Given different power rating between the macro gNB and micro nodes, a UE may transmit UL to either the macro gNB or non-co-located micro nodes in order to maximize UL throughput while receiving DL transmission from the macro gNB. As an option to further reduce energy consumption and hardware cost, the micro nodes can reduce or even turn off DL transmissions (e.g., UL-configured TRP proposed by infra-vendor/operators in RAN#101). Moreover, as an alternative, UL-only TRP [4] can be assumed as another candidate for deployment in such case.

In addition to enhancements on UL transmission power control specified in the MIMO Ph5 WID, it is crucial to assess TA related enhancement, mainly due to the reason that it is becoming the bottleneck of UL performance and also the commercial network deployment in terms of asymmetric UL sTRP/ DL mTRP scenarios.

Given that downlink is received from a single TRP as stated in WID, single DCI based mTRP/sTRP for uplink transmission is expected to be the basic scheduling scheme in this scenario, in which, as legacy, only one single TAG can be used for uplink transmissions. As a critical uplink transmission requirement, uplink propagation delay difference between macro and micro nodes should be strictly limited by timing error limit Te as specified in TS 38.133 [6]. For reference, timing error limit Te of uplink transmission in FR1 and FR2 is provided in the following Table 1.

Table 1	Timing error limit Te of uplink signals

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc (390.9ns) |
|  | 30 | 30 | 8*64*Tc (260.6ns) |
|  |  | 60 | 7*64*Tc (228.0ns) |
| 2 | 120 | 60 | 3.5*64*Tc (114.0 ns) |
|  |  | 120 | 3.5*64*Tc (114.0 ns) |
| Note 1: Tc is the basic timing unit defined in TS 38.211 Note 2: Other cases of timing error limit Te are provided in Table 7.1.2-1 in TS 38.133 |  |  |  |

Correspondingly, system-level simulation is performed to evaluate the uplink propagation delay difference between macro and micro nodes in asymmetric UL sTRP/ DL mTRP scenarios, assuming that asymmetric UL sTRP/DL mTRP scenarios with ideal synchronization. In this simulation, the distribution of macro and micro nodes is depicted in Figure 2. Wherein, micro nodes are allocated at the cell-edge of macro node with ideal synchronization assumption, and ISD of macro nodes is set to 500m@FR1 and 200m@FR2, respectively. Besides, the UE can transmit uplink transmission towards to micro nodes depending on network scheduling, and uplink propagation delay difference is calculated based on the clusters with peak power in both macro and micro nodes. The detailed evaluation assumption can be found in Appendix.

Figure 2	Distribution of macro and micro nodes in SLS of asymmetric DL sTRP/UL mTRP scenarios

To investigate the impact of uplink propagation delay difference between macro and micro nodes in both FR1 and FR2, the following two cases are evaluated under the above asymmetric DL sTRP/ UL mTRP scenario:

- Case 1: Uplink propagation delay difference between macro and micro nodes in FR1@SCS = 30kHz, ISD = 500m, Te = 260.6ns.

- Case 2: Uplink propagation delay difference between macro and micro nodes in FR2@SCS = 120kHz, ISD = 200m, Te = 114.0ns.

As the simulation results provided in Figure 3, it can be observed that the uplink propagation delay difference of ~70% UE cannot meet the timing error limit Te in both Case-1@FR1 and Case-2@FR2, in where only one single TAG is used in asymmetric DL sTRP/ UL mTRP scenario. In this sense, the performance of the vast majority of uplink transmissions cannot be guaranteed, or, as a result, the network may have to densely deploy more macro/micro nodes to compensate the performance impact  Consequently, maintaining single TA UL operation drastically impairs the practical deployment of asymmetric DL sTRP/ UL mTRP.

Figure 3	SLS results of uplink propagation delay difference between macro and micro nodes in asymmetric DL sTRP/ UL mTRP scenario

Observation 1: In asymmetric UL sTRP/ DL mTRP scenarios applied with one single TAG, approximately 70% of UEs experience an uplink propagation delay difference between macro and micro nodes that fails to meet the timing error limit Te in both FR1 and FR2. Consequently, this shortcoming adversely impacts the performance of most uplink transmission.

In light of the above assessment, more than one TAG (e.g., up to two TAGs) need to be supported for asymmetric DL sTRP/ UL mTRP scenarios. Furthermore, it is needed for the case of single DCI based UL mTRP operation and also single DCI based STRP (i.e., DPS between different UL TRPs). Note that two TAs for MDCI MTRP has been specified in Rel-18, it can be well utilized as a starting point to fulfill the work of specifications. More precisely, we just use what we had in the current specifications, i.e., one UL TCI state is associated with one out of two TAGs, and remove the extra condition that “coresetPoolIndex needs to be configured” in asymmetric DL sTRP/UL mTRP scenarios.

Observation 2: The workload of normative work on supporting two TAs for single DCI based UL MTRP/STRP in asymmetric DL sTRP/UL mTRP scenarios can be quite minimal, i.e., through releasing the condition “coresetPoolIndex needs to be configured” of two TAs operation that introduced for Rel-18 M-DCI MTRP.

Besides, for any other issues raised by previous offline discussion, we have the following brief analysis:

- Regarding DL reference timing determination, in our views, there is no need for any extra specification impacts/efforts due to the fact that it was already agreed that DL reference timing determination for each TA is still based on the DL RS defined in the current spec for asymmetric DL sTRP/UL mTRP.

- After that, regarding physical overlapping in time domain caused by different TAGs in case of single DCI based MTRP PUSCH TDM repetition, especially for back-to-back UL transmissions, further optimization on the top of recently specified UE behavior(s) in current spec may NOT be needed as our initial assessment:

  - If UE supports FG 40-2-9 (partial dropping of overlapping UL transmission reduction), the physical overlapping caused by two TAGs can be handled by the UE capability itself.

  - If UE does not support FG 40-2-9, the physical overlapping caused by two TAGs can be handled by gNB scheduling in implementation, e.g., unscheduled symbol(s) of the former transmission in the overlapping duration for PUSCH repetition Type A (inter-slot based PUSCH repetition), appreciated invalid symbol configuration (i.e., by the higher layer parameter invalidSymbolPattern) for PUSCH repetition Type B (intra-slot based PUSCH repetition).

  - Even if the above scheduling cannot be operated in a corner case, gNB still can manage the situation by either not enabling two TAs or dynamically switching to STRP PUSCH TDM repetition to handle this without any extra UE complexity.

Proposal 1: To fully enable asymmetric DL sTRP/UL mTRP deployment scenarios, support two TAs for single DCI based multi-TRP/panel and single TRP.

- Reuse Rel-18 rule of two TAs for multi-DCI based multi-TRP/panel and remove the restriction that coresetPoolIndex needs to be configured.

Accordingly, we have the following suggestion for objective update of Rel-19 NR-MIMO Ph5 in red.

5. Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Extending Rel-18 M-DCI-based 2TA toward UL TRP(s) and DL sTRP without using CoresetPoolIndex, while assuming legacy PRACH resources

  - Other potential scope extension

In this subsection, we elaborate our views on other potential scope extension besides for two-TA extension as mentioned above,

SRS port grouping as part of CSI enhancement

The increase of UE DL RX(s) has become another hot topic, but can have two separate schemes: coherent joint-reception and non-coherent joint reception. For the former, it is a normal design with ideal performance but with high UE complexity; then, for the latter, it can have lower complexity even with some performance loss (it should be noticed that, attention should be paid on how to handle antenna switching in such case).  Therefore, we can consider 6Rx/8Rx UE enhancements with lower complexity as another candidate in Rel-19 MIMO evolution, assuming that there is no increase of TU allocation and only SRS port grouping is focused as greater than 4 layer DL transmission.

Partial coherent UL codebook for 3Tx

Considering that non-coherent codebook transmission for 3Tx has been specified as stated in NR MIMO Ph5 WID, we may need to further justify the motivation/necessity of further introducing asymmetric UE antenna pattern for 3Tx. Meanwhile, the scope of this potential enhancement should be clear and fully specified, and, from our perspective, the following restriction should be considered as a starting point:

- Clarification-1: Only one coherent port grouping scheme is considered, e.g., PUSCH port 1000 and 1002. Then, the UL codebook corresponding to coherent port group in such case is to fully reuse legacy UL coherent codebook for 2 Tx (i.e., TPMI 2~5 in Table 2 for 3Tx RANK 1~2, and TPMI 1~2 in Table 3 for 3-Tx RANK 3), instead of overloading RAN1 MIMO workload by introducing fancy/over-optimization design.

- Clarification-2: As clarified in the existing objective of 3Tx UL operation, UL full power transmission mode 1 and 2 are not supported.

Table 2	Precoding matrix  for single-layer transmission using two antenna ports (c.f. Table 6.3.1.5-1 in TS 38.211)

| TPMI index | (ordered from left to right in increasing order of TPMI index) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 5 |  |  |  |  |  |  | - | - |

Table 3	Precoding matrix  for two-layer transmission using two antenna ports with transform precoding disabled (c.f., Table 6.3.1.5-4 in TS 38.211)

| TPMI index | (ordered from left to right in increasing order of TPMI index) |  |  |  |
| --- | --- | --- | --- | --- |
| 0 – 2 |  |  |  |  |

Open loop based UL precoder cycling

Similar to the previous topic on 3Tx partial coherent UL Tx, a study phase is needed to evaluate the UL precoder cycling scheme to understand its performance benefit and specification impact, before agreeing on specifying it. In order to prevent from the huge workload, we should restrict the corresponding scope well, and then the following can be considered as a starting point, from our perspective:

- Clarification-1: Open-loop based UL-precoder cycling is only applied to UL RANK1 transmission.

- Clarification-2: Dynamic switching between open-loop based UL-precoder cycling and normal UL transmission scheme (TPMI-based) should be supported for falling back to normal transmission.

- Clarification-3: Rather than fully reusing DL PRG granularity, a large size of UL PRG (e.g., >12 PRBs) should be supported for facilitating UL channel estimation and MU-MIMO.

Proposal 2: Assuming no TU re-allocation in NR MIMO Ph 5, we can consider the following aspects with limited-workload objectives.

- SRS port grouping without spec impacts on less than or equal to RANK-4 DL-CSI acquisition

- Partial coherent UL codebook for 3Tx, assuming that legacy UL coherent codebook for 2 Tx is fully used for the only one coherent port group, and then UL full power transmission mode 1 and 2 are not supported.

- Open-loop based UL precoding cycling with the following clarification on the corresponding WID objective:

  - Only UL RANK1 transmission is supported for UL 2-Tx operation;

  - Dynamic switching between open-loop based scheme and normal UL codebook-based transmission is supported as a basic feature;

  - The candidate size(s) of UL PRG should be greater than or equal to a threshold, e.g., >12 PRBs.

- Conclusion

In this contribution, we elaborate our views on the scope extension of Rel-19 NR-MIMO Ph5 evolution. Our observations and proposals are listed as follows.

Two-TA extension for asymmetric DL sTRP/UL mTRP deployment

Observation 1: In asymmetric UL sTRP/ DL mTRP scenarios applied with one single TAG, approximately 70% of UEs experience an uplink propagation delay difference between macro and micro nodes that fails to meet the timing error limit Te in both FR1 and FR2. Consequently, this shortcoming adversely impacts the performance of most uplink transmission.

Observation 2: The normative workload on supporting two TAs for single DCI based UL MTRP/STRP in asymmetric DL sTRP/UL mTRP scenarios can be quite minimal, i.e., through releasing the condition “coresetPoolIndex needs to be configured” of two TAs operation that introduced for Rel-18 M-DCI MTRP.

Proposal 1: To fully enable asymmetric DL sTRP/UL mTRP deployment scenarios, support two TAs for single DCI based multi-TRP/panel and single TRP.

- Reuse Rel-18 rule of two TAs for multi-DCI based multi-TRP/panel and remove the restriction that coresetPoolIndex needs to be configured.

Other potential scope extension

Proposal 2: Assuming no TU re-allocation in NR MIMO Ph 5, we can consider the following aspects with limited-workload objectives.

- SRS port grouping without spec impacts on less than or equal to RANK-4 DL-CSI acquisition

- Partial coherent UL codebook for 3Tx, assuming that legacy UL coherent codebook for 2 Tx is fully used for the only one coherent port group, and then UL full power transmission mode 1 and 2 are not supported.

- Open-loop based UL precoding cycling with the following clarification on the corresponding WID objective:

  - Only UL RANK1 transmission is supported for UL 2-Tx operation;

  - Dynamic switching between open-loop based scheme and normal UL codebook-based transmission is supported as a basic feature;

  - The candidate size(s) of UL PRG should be greater than or equal to a threshold, e.g., >12 PRBs.

- References

- RP-234007, New WID: NR MIMO Phase 5, Samsung (Moderator)

- RP-241775, SR for RAN 105 on NR MIMO Phase 5, Samsung

- RAN#105 Meeting Report, RAN Chair

- RP-232073, Views on Rel-19 MIMO/UL enhancements, NTT DOCOMO, INC.

# Appendix

Table A-1	System-level simulation parameters for asymmetric DL sTRP/UL mTRP scenario in FR1

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 4GHz |
| Bandwidth | 20MHz |
| SCS | 30kHz |
| Network Layout | Outdoor FWA (TR 38.901): UMa |
| Macro BS antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1; 4,4). (dH,dV) = (0.5, 0.8)λ |
| Micro BS antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (2,2,2,1,1; 2,2). (dH,dV) = (0.5, 0.8)λ |
| UE antenna configuration | 8Tx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2). (dH,dV) = (0.5, 0.5)λ |
| Scheduling | Subband PF |
| Traffic model | fullbuffer |
| ISD | 500 m |
| UE layout | 100% Outdoor |
| UE speed | 3km/h |
| Number of average UE per macro cell | 20 |
| Max UE power | 23 dBm |
| UL power control | Open loop TPC |
| Power control parameter | P0 = -60, Alpha = 0.8 |
| Modulation | Up to 64QAM |
| Layer | adaptive from 1 to 4 |

Table A-2	System-level simulation parameters for asymmetric DL sTRP/ UL mTRP scenario in FR2

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 30GHz |
| Bandwidth | 50MHz |
| SCS | 120kHz |
| Network Layout | Outdoor FWA (TR 38.901): UMa |
| BS antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,1,2; 4,4). (dH,dV) = (0.5, 0.8)λ |
| UE antenna configuration | 8Tx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2; 2,2). (dH,dV) = (0.5, 0.5)λ |
| Scheduling | Subband PF |
| Traffic model | fullbuffer |
| ISD | 200 |
| UE layout | 100% Outdoor |
| UE speed | 3km/h |
| Number of average UE per macro cell | 20 |
| Max UE power | 23 dBm |
| UL power control | Open loop TPC |
| power control parameter | P0 = -60, Alpha = 0.8 |
| Modulation | Up to 64QAM |
| Layer mapping | adaptive from 1 to 4 |