3GPP TSG RAN meeting #105	                                   	RP-242102

Melbourne, Australia, September 9-12, 2024

Title: 	Discussion on Rel-19 Network energy saving

Source: 	ZTE Corporation, Sanechips

Agenda item:	9.3.1.5

Document for:	Discussion and decision

Introduction

The WI for WID for Enhancements of network energy savings includes the following objective for on-demand SIB1:

Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

In this paper, we analyze the progress in each WG and give suggestion on the next step for this objective.

On-demand SIB1

The following cases have been identified in RAN1 [1][2][3]:

- Case 1: Option 1+A+X

- Case 2: Option 1+B+X

- Case 3: Option 2+B+Y

Where the options 1/2/A/B/X/Y are defined below:

- On target cell of UL WUS transmission:

  - Option 1: UE transmits UL WUS to NES Cell

  - Option 2: UE transmits UL WUS to Cell A

- On configuration provision for UL WUS transmission

  - Option A: UE obtains the UL WUS configuration from NES Cell

  - Option B: UE obtains the UL WUS configuration from Cell A

- On receiving of SIB1

  - Option X: UE receives on-demand SIB1 from NES Cell

  - Option Y: UE receives on-demand SIB1 from Cell A

In the past four RAN1 meetings, there has been a lot of progress on UL WUS configuration and on demand SIB1 acquisition, especially for case 2.

In RAN1#117 meeting, case 1 has been de-prioritized. Furthermore, in RAN1#118 meeting, RAN1 has recommended to only specify Case 2 in Rel-19 [4].

Agreement in RAN1#117 For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3 - Case 2 (Option 1+B+X) is feasible from RAN1 perspective. - Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

Agreements in RAN1#118 Conclusion For on-demand SIB1 in idle/inactive mode, RAN1 was not able to achieve consensus to support Case 1 (Option 1+A+X) in Rel-19, while Case 1 is technically possible from RAN1’s perspective. Agreement RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19. - Note: RAN1 strive to minimize impact to legacy UE. - Note: RAN1 specification impact to support this feature should be minimized. Agreement For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective. - RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for Case 3

In RAN2, the discussion has been focusing on case 2 since the first meeting, i.e. the scenario 1a in the following RAN2#125bis agreement, with a lot of progress made on the WUS configuration, triggering method, procedure, SIB1 validity and impact to UE[5][6][7]:

Agreements at RAN2#125bis on OD-SIB1: 1. At least RAN2 starts scenario 1a (Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell). Other scenarios are not excluded.

In the latest RAN2 meeting, there has been discussion on case 3 with no consensus reached and it is also recommended to specify case 2 in normative phase:

Agreements at RAN2#127on OD-SIB1 Scenarios: No consensus for RAN1 case 3 in RAN2. OD-SIB1 SI conclusion: RAN2 conclude that on-demand SIB1 is feasible from RAN2

In RAN3, the discussion also focuses on case 2 [8][9][10]:

[RAN3#125 Agreement]

Case 2 can be supported with the below details:

-	UL WUS configuration is decided by NES Cell DU.

-	UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

-	Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

[RAN3#124 Agreement] Focus on case2 in RAN3 first.

[RAN3#123bis Agreement] Scenario1: UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell.

- UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed

Observation 1: For the case1, case 2 and case 3 of on demand SIB1 listed in RAN1,

- RAN1 and RAN2 recommend only case 2 for normative phase.

- RAN3 focus on details of supporting case 2.

- RAN1, RAN2 and RAN3 have considerate progress on case 2.

For case 1, the only difference is that UE obtain the UL WUS configuration from NES cell instead of Cell A. Some companies propose case 1 to be a complementary solution to case 2 under certain circumstance, e.g. the NES cell broadcast SIB1 for some time and provide the UL WUS configuration for UE to request SIB1 after it stops broadcasting SIB1. However, we understand that UE would leave NES cell and re-select to a Cell A after NES cell stops broadcasting SIB1 as shown in the following figure. With this, UE would still acquire the WUS configuration from Cell A and there is no need for a NES cell to provide WUS configuration for its own.

Figure 1: Expected UE and NW behavior for case 1

Observation 2: Case 2 is sufficient for on-demand SIB1 in idle and inactive mode and there is no need for NES cell to provide WUS configuration of its own.

For case 3, Cell A is expected to provide SIB1 for NES cell, which require more coordination between Cell A and NES cell so that up-to-date SIB1 can be provided. Since the required signaling to provide SIB1 of another cell, the number of NES cells supported by Cell A would be quite limited and capacity in the cell A would be impacted. Moreover, based on the evaluation results in [11], the additional NES gain from case 3 over case 2 is -0.63%~0.92% depending on traffic load, i.e., no additional NES gain can be provided by case 3. Considering the complexity in implementation, case 3 is not welcome to most companies.

Observation 3: Case 3 requires more coordination between Cell A and NES cell to provide valid SIB1 thus would bring more complexity in implementation.

With the above consideration, we suggest to specify on demand SIB1 only for Case 2 in the normative phase.

Proposal: Specify Case 2, i.e. UE obtains the UL WUS configuration from Cell A, transmits UL WUS to NES Cell and receives on-demand SIB1 from NES Cell, in the normative phase of on-demand SIB1 for UEs in idle/inactive mode.

Proposed modification on the objectives

The following updates to the objectives are proposed as a result:

----------------------------------------------------------------------------------------------

The objectives of the work item are the following:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

- Study Specify procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, for the case that UE obtains the UL WUS configuration from Cell A, transmits UL WUS to NES Cell and receives on-demand SIB1 from NES Cell, including: [RAN1/2/3]

Note: Cell A: A cell that is periodically transmitting its own SIB1 and provide UL WUS configuration of NES cell(s).

  - Triggering method by uplink wake-up-signal using PRACH resource an existing signal/channel.

  - Wake-up-signal configuration provisioning to UE

    - Note: No modification of SSB will be discussed under this objective

  - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

  - Checkpoint for normative work in RAN#105

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

- Specify the corresponding core requirements, for the above features [RAN4].

----------------------------------------------------------------------------------------------

Conclusion

In this contribution, we have the following observations and proposal:

Observation 1: For the case1, case 2 and case 3 of on demand SIB1 listed in RAN1,

- RAN1 and RAN2 recommend only case 2 for normative phase.

- RAN3 focus on details of supporting case 2.

- RAN1, RAN2 and RAN3 have considerate progress on case 2.

Observation 2: Case 2 is sufficient for on-demand SIB1 in idle and inactive mode and there is no need for NES cell to provide WUS configuration of its own.

Observation 3: Case 3 requires more coordination between Cell A and NES cell to provide valid SIB1 thus would bring more complexity in implementation.

Proposal: Specify Case 2, i.e. UE obtains the UL WUS configuration from Cell A, transmits UL WUS to NES Cell and receives on-demand SIB1 from NES Cell, in the normative phase of on-demand SIB1 for UEs in idle/inactive mode.

The following updates to the objectives are proposed as a result:

----------------------------------------------------------------------------------------------

The objectives of the work item are the following:

1. Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

2. Study Specify procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, for the case that UE obtains the UL WUS configuration from Cell A, transmits UL WUS to NES Cell and receives on-demand SIB1 from NES Cell, including: [RAN1/2/3]

Note: Cell A: A cell that is periodically transmitting its own SIB1 and provide UL WUS configuration of NES cell(s).

  - Triggering method by uplink wake-up-signal using PRACH resource an existing signal/channel.

  - Wake-up-signal configuration provisioning to UE

    - Note: No modification of SSB will be discussed under this objective

  - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

Checkpoint for normative work in RAN#105

3. Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

4. Specify the corresponding core requirements, for the above features [RAN4].

----------------------------------------------------------------------------------------------

References

- Final_Minutes_report_RAN1#116b_v100

- Final_Minutes_report_RAN1#116_v100

- Final_Minutes_report_RAN1#117_v100

- Chair notes RAN1#118 eom0

- R2-2404102	RAN2#125bis Meeting Report

- R2-2406202	RAN2#126 Meeting Report

- R2_127_R18 SL_MOB_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM

- RAN3#123-bis_draft_MeetingReport

- RAN3#124_MeetingReport_v20240605

- RAN3_125_agenda_20240823EOM

- R1-2406410. Discussion on on-demand SIB1 for NES, ZTE Corporation, Sanechips, RAN1#118.

Annex- RAN1/2/3 agreements

- RAN1 agreements

- RAN1#118

Agreement

For further work on on-demand SIB1 in idle/inactive mode, as a baseline, it is assumed that the transmit power control of UL WUS transmission based on PRACH is applied in the same manner as the legacy PRACH transmission.

- FFS: Potential optimization of the power ramp-up procedure

Agreement

For further work on on-demand SIB1 in idle/inactive mode related to dedicated PRACH resource usage, RAN1 to study the following options:

- Option 1 (shared RO): The dedicated WUS resource shares the same PRACH resource pool with PRACH resource for other usages.

- Option 2 (separated RO): The dedicated WUS resource uses an independent RACH resource pool with PRACH resource for other usages.

Agreement

RAN1 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Agreement

RAN1 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Agreement

At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options:

- Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission

- Option 2: starting time and duration are indicated in the UL WUS configuration

Agreement

At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options:

- Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission

  - FFS: Definition of RAR reception time

- Option 2: The reference time point is defined based on the UL-WUS transmission time

- Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission

Agreement

For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective.

- RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for Case 3

Agreement

RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19.

- Note: RAN1 strive to minimize impact to legacy UE.

- Note: RAN1 specification impact to support this feature should be minimized.

Agreement

For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options:

- Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell

- Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration

- Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS.

Combination of multiple options is not precluded.

- RAN1#117

Agreement

For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3

- Case 2 (Option 1+B+X) is feasible from RAN1 perspective.

- Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

Agreement

For further study of on-demand SIB1 in idle/inactive mode, it is assumed that always-on SSB is transmitted on the NES cell with on-demand SIB1.

Agreement

At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point:

- Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window

  - FFS: How the search space zero configuration is provided (e.g. from  searchSpaceZero  in  MIB  or  from a new search space that is indicated by UL-WUS configuration)

  - FFS: Details of the time window, including at least the starting time and duration

  - FFS: Whether/how to support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS

Agreement

From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it.

At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

Agreement

For further study of on-demand SIB1 in idle/inactive mode, on the spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB, and UL WUS, as UL WUS is using dedicated PRACH resource, it is assumed that spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB and UL WUS can follow legacy mechanism.

- RAN1#116bis

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases:

- Case 1: Option 1+A+X

- Case 2: Option 1+B+X

- Case 3: Option 2+B+Y

Where the options 1/2/A/B/X/Y are defined below:

- On target cell of UL WUS transmission:

  - Option 1: UE transmits UL WUS to NES Cell

  - Option 2: UE transmits UL WUS to Cell A

- On configuration provision for UL WUS transmission

  - Option A: UE obtains the UL WUS configuration from NES Cell

  - Option B: UE obtains the UL WUS configuration from Cell A

- On receiving of SIB1

  - Option X: UE receives on-demand SIB1 from NES Cell

  - Option Y: UE receives on-demand SIB1 from Cell A

Agreement

RAN1 to further study the following UE operation scenarios in the UL WUS design:

- Scenario 1: UE requests SIB1 to camp on NES cell

- Scenario 2: UE request SIB1 to perform random access procedure to make RRC connection to NES cell

Agreement

RAN1 to further study UE identification of NES cell with on-demand SIB1 based on one, both, or combination of the following options:

- Option 1: By WUS configuration

- Option 2: By PBCH payload of NES cell

Agreement

Companies to report at least the following key settings used in the evaluation/simulation of achievable NES gain with on-demand SIB1 in idle/inactive mode

- Setting A: SIB1 period (20ms/40ms/160ms)

- Setting B1: Cell load (Empty/low/medium)

- Setting B2: Traffic model

- Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

- Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

- Setting E: PRACH configurations (including PRACH configuration index in 38.211 Table 6.3.3.2-3) for WUS and initial/random access

- Setting F: Cat1/Cat2 BS

- Setting G: Number of SSB beams

- Setting H: NES gain/loss on Cell A

  - Further discuss in RAN1#116bis on the evaluation assumptions for Cell A

- Setting I: On-demand SIB1 transmission rate (how often UE requests on-demand SIB1)

Agreement

For further study of the NES gain/loss evaluation assumption on Cell A with on-demand SIB1 on NES cell for idle/inactive mode UE,

- Assume the following for network energy evaluation of Cell A in FR1:

  - Company to report among empty/low/medium cell load as defined in 38.864

  - Same Cat BS as the Non-NES cell

  - 30kHz SCS, DDDSU TDD pattern

    - Same SSB period as the Non-NES cell and company to report SIB1 period

  - Same number of SSBs in a SSB burst as the Non-NES cell with SSB pattern case C

  - 20ms PRACH configuration periodicity for WUS and/or initial access RACH and company to report RACH configuration index in 38.211 Table 6.3.3.2-3

  - Same SSB/CORESET0 multiplexing pattern and same SIB1 PDSCH time domain resource allocation as the Non-NES cell

  - Same traffic model as the Non-NES cell

  - Companies to report the assumption of WUS configuration provision or UL WUS monitoring or on-demand SIB1 transmission on Cell A if Case 2 (Option 1+B+X) or Case 3 (Option 2+B+Y) is considered

Agreement

For UL WUS design for SIB1 request, at least dedicated PRACH resource is the assumption for further study in RAN1

- FFS: Details on time, frequency, and/or PRACH preamble resources for UL WUS

- FFS: whether RACH resource for SIB1 request could be used for an initial access procedure and/or an on-demand SI procedure

Companies to consider the following for future meetings

- Option 1: SIB1 monitoring occasions within a time window

  - FFS: The starting time and duration of the time window

  - FFS: Interval between two SIB1 monitoring occasions in the time window

  - FFS: How gNB informs UE the details related to the time window

- Option 2: Periodic SIB1 monitoring occasions until gNB turns off the SIB1 transmission

  - FFS: The staring time of the SIB1 monitoring occasions

  - FFS: How gNB informs UE the SIB1 transmission is turned off

  - FFS: How gNB informs the UE the details related to periodicity

- Other options are not precluded

- FFS: Further details on SIB1 monitoring occasions

Agreement

Conditions for triggering UL WUS transmission is up to RAN2. Any related work in RAN1 to be triggered by RAN2 LS. Send an LS to RAN2.

- RAN1#116

Agreement

For discussion purpose, the following assumption will be used in RAN1

- Cell A: A cell that is periodically transmitting at least its own SIB1

- NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 studies the following options.

On target cell of UL WUS transmission:

- Option 1: UE transmits UL WUS to NES Cell

- Option 2: UE transmits UL WUS to Cell A

On configuration provision for UL WUS transmission

- Option A: UE obtains the UL WUS configuration from NES Cell

- Option B: UE obtains the UL WUS configuration from Cell A

Other options are not precluded

Agreement

For further study of achievable NES gain with on-demand SIB1 for idle/inactive mode UE,

- Assume the following for network energy evaluation of non-NES cell in FR1:

  - Empty/low/medium cell load as defined in 38.864

  - Cat 1/Cat 2 BS as defined in 38.864

  - 30kHz SCS, DDDSU TDD pattern

    - Case A: 20ms SSB period with 20ms SIB1 period;

    - Case C: 20ms SSB period with 160ms SIB1 period;

    - Case D: 20ms SSB period with 40ms SIB1 period;

Note: Other SSB/SIB1 periodicity assumptions are not precluded (up to companies to report)

  - 4 or 8 SSBs in a SSB burst with SSB pattern case C

  - 20ms or 160ms PRACH monitoring period

- Assume the following for network energy evaluation of NES cell in FR1:

  - Empty/low/medium cell load as defined in 38.864

  - Cat 1/Cat 2 BS as defined in 38.864

  - 30kHz SCS, DDDSU TDD pattern

    - Case 1: 20ms SSB period with no SIB1 transmitted;

Note: Other SSB/SIB1 assumptions are not precluded (up to companies to report)

  - 4 or 8 SSBs in a SSB burst with SSB pattern case C

  - 20ms/160ms UL WUS monitoring period

- Note: SSB/CORESET0 multiplexing pattern 1 is used

Agreement

- For study of UL WUS design, consider at least PRACH as a starting point

  - FFS: Whether there is dedicated PRACH resource for SIB1 request

- Other option(s) not precluded

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 to discuss triggering conditions for sending UL-WUS.

Agreement

For the study of on-demand SIB1 for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported including associated details.

Agreement

For the further study on UL WUS configuration among the following options:

- Option 1: Pre-defined UL WUS configuration.

- Option 2: UL WUS configuration that applies to multiple NES cell.

- Option 3: UL WUS configuration that applies to a single NES cell.

- RAN2 agreements

- RAN2#127

Agreements on OD-SIB1

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

- RAN2#126

Agreements on on-demand SIB1:

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbor NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behavior is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

- RAN2#125bis

Agreements on on-demand SIB1:

- At least RAN2 starts scenario 1a (Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell). Other scenarios are not excluded.

- RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1.

- UL WUS configuration includes at least RACH configuration.

- A UE needs to know a UL WUS configuration to request SIB1 of which cell.

- Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

Agreements on on-demand SIB1:

- The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

- RAN3 agreements

- RAN3#125

Case 2 can be supported with the below details:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

- RAN3#124

[Agreement] Focus on case2 in RAN3 first.

- RAN3#123bis

Agreement:

Scenario1: UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell.

- UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed