3GPP TSG RAN Meeting #105								RP-242118

Melbourne, Australia, September 9-12, 2024

Agenda Item:		9.3.1.3

Source:			Nokia, T-Mobile

Title:				On Checkpoint for Study of Data Collection for UE-side AI/ML Models

Document for:		Decision

### 1	Checkpoint

The WID for AI/ML for NR Air Interface [1] provided a deadline for the following checkpoint on data collection.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24):

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

Now that the RAN plenary meeting during which this deadline will be passed has arrived, we will discuss the status of this study objective on data collection in RAN2, including input from RAN1, and the next steps to take.

### 2	Status of RAN2 discussion on Data Collection

Since the Study Item on AI/ML for the air interface began, a significant amount of discussion has been had on the topic of data collection for NW-side models and for UE-side models. While the discussion on data collection for NW-side models has progressed with a collection of agreements, the discussion on data collection for UE-side models has stalled. Data collection for NW-side models is briefly discussed to show that RAN2 could work on common solutions for data collection, reducing the overall specification burden during this release.

#### 2.1	Data Collection for NW-side Models

For NW-side data collection, OAM and the Minimization of Drive Test (MDT) framework have been agreed to be used for data collection for NW-side models for beam management.

Excerpts from the chair notes for previous RAN2 meetings this year have been provided to show the progress on data collection for NW-side models. The agreements mainly pertain to the beam management use case while RAN2 is waiting for more progress in RAN1 on the positioning use case. The agreements address the logging of measurements to reduce the frequency of data collection-relation messages and to allow for proper prioritization of critical network traffic.

RAN2#125bis [2]

During RAN2#125bis, the following agreements were made to support the same data collection approach for NW-side models for gNB-centric and OAM-centric approaches to data collection. Furthermore, it was agreed that MDT could be enhanced for this purpose.

1	For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches

2	We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

3	RAN2 supports enhancements to MDT for data collection framework for training.  FSS Whether to enhance logged or immediate MDT

RAN2#126 [3]

During RAN2#126, the following agreements were made to support immediate MDT for the beam management use case. Note these agreements in the following section, 2.2, on data collection for UE-side models, as they align with Option 3 (OAM) for data collection for UE-side models.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

RAN2#127 [4]

During RAN2#127, the following agreements were made on the configuration of data collection for all use cases.

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

Observation 1: RAN2 has agreed to support immediate MDT and enhancements thereto for data collection for training NW-side beam management models.

#### 2.2	Data Collection for UE-side Models

##### 2.2.1	Options for Data Collection for UE-side Models

RAN2 has developed a table, 7.2.1.3.2-1, copied in in Annex 6.2, describing four options for data collection for UE-side models. These options are summarized as follows [5], considering changes agreed during RAN2#127 [6]:

- 1a: Non-standardized data collection with a direct connection between the UE and a third-party server, called an over-the-top (OTT) server. This option does not enable MNO control over the data collection process or visibility of the data collected and transmitted by the UE. This option is considered a user-plane approach.

- 1b: Standardized data collection with a direct, standardized, connection between the UE and a server within the MNO network. This option does not enable MNO control over the RAN aspects of the data collection process, and visibility of the data collected and transmitted by the UE is not inherent to the option. This option is considered a user-plane approach.

- 2: Standardized data collection with standardized communication between the UE and the core network. We assume that the LMF is considered to be part of the core network in relation to this option. RAN2 did not conclude on which NW entities in the CN would be involved. At least for the positioning use case, wherein the LMF could be in control of the data collection procedures, RAN-level control is possible. Visibility of data is also possible. Minimal control can be achieved at the PDU session level, but this is out of RAN scope.

- 3: Standardized data collection with standardized communication between the UE and the gNB. In this option, OAM would initiate the data collection procedures and configure the gNB to configure UEs to collect data and transmit the collected data to the gNB, which would then forward the data to OAM. This option could be realized with the minimization of drive test (MDT) framework. Because the protocol between the UE and gNB is standardized, visibility of the data is possible.

RAN2 has been unable to conclude on or reach consensus on several aspects of data collection, likely due to the impact on many more working groups than only RAN2. The lack of consensus is particularly evident from the following notes at the end of the table in Annex 6.1 [6].

- Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion.

- Note 5: RAN2 cannot reach consensus on the level of possible MNO controllability and visibility via Option 1b without input from SA groups.

- Note 6: RAN2 has not concluded on the need for partial and no visibility options.

- Note 7: RAN2 cannot reach consensus on the feasibility of UP tunnel in Options 2 and 3

Observation 2: RAN2 has been unable to reach consensus or conclude on several aspects of data collection, including the level of controllability and visibility of Option 1b, the need for partial and no visibility of data, and the feasibility of a UP tunnel for Options 2 and 3.

##### 2.2.2	Requirements for Data Collection for Model Training

RAN2 was provided an LS from RAN1 on data collection requirements for each of the three use cases [7]. We will focus on beam management and positioning in this contribution since CSI prediction and CSI compression are still under study by RAN1. The associated tables are provided in Annex 6.2, and excerpts are provided below for reference and further discussion.

Beam Management

“Based on existing L1-RSRP reporting methodology, i.e., 7 bits for the strongest beam and 4 bits for the remaining beams, for Set B = 16 as an example, the typical data size would be 67 (hence up to ~100 bits), and for Set A = 128 as an example, the typical data size would be 515 (hence up to ~500 bits) if all beams in Set A were to be collected [7].”

For beam management, the typical data size per measurement instance was determined to be around 500 bits. For reference, an RRC message can carry around 9KB, or 72,000 bits, meaning that a single message could carry up to 144 samples. Making a generous estimate that one sample can be collected per 10ms, the message volume for a UE collecting and then sending 144 data samples for training UE-side models in beam management would require less than 1 RRC message per second.

Positioning

“Size depends on number of PRS/SRS resources, measurement type (timing, power, and/or phase info) and report format: ~100 bits to 1000s bits per PRS/SRS resource [7].”

For positioning, an estimate of between 100 bits and 1000 bits per PRS/SRS resource is given. Take as an assumption a data size in the middle of the range as an estimate. Then, make the same assumption as above regarding measurement frequency, combined with the fact that an RRC message, a ULInformationTransfer, would be used to carry the LPP message. Then we still arrive at a message volume for a UE collecting data for training UE-side positioning models of less than 1 RRC message per second.

Observation 3: The data size for data collection for training UE-side models for the beam management and positioning use cases is on average around 500 bits per measurement instance.

In all cases, the latency requirement for data collection for training UE-side models is considered “relaxed”, defined by RAN1 as on the order of “minutes, hours, days, or no latency requirement”, supports the idea of logging collected data for efficiency purposes.

Observation 4: The latency requirement for data collection for training UE-side models is considered “relaxed”, which was defined by RAN1 as on the order of “minutes, hours, days, or no latency requirement.”

Because more than one UE will be collecting data, the data collection burden of a single UE should not be so significant as to require larger message sizes at this stage.

##### 2.2.3	Visibility During the Data Collection Process

The concept of MNO’s visibility of data during the data collection process was discussed, resulting in definitions for three levels of visibility captured in Note 3 of the table in Annex 6.1 [6].

Note 3: The following options are identified to realize the different levels of data content visibility to the MNO:

•	Full visibility for standardized data content.

•	Partial visibility for partially standardized data content (e.g. UE proprietary information can be included 	transparently together with the standardized data message).

•	No standardized visibility (e.g. UE proprietary information can only be included transparently).

For Options 2 and 3, there was discussion on the possibility of a transparent container to achieve partial visibility, i.e., RAN would be aware of the transmission of data and its nature, but not the exact contents. However, RAN2 has not reached a consensus on whether to support such a container for proprietary data collected by UEs.

Observation 5: RAN2 discussed the possibility of supporting a transparent container inside of a standardized message to support partial visibility of data, e.g., proprietary data collected from UEs but could not reach an agreement to support such a container.

##### 2.2.4	Control Over the Data Collection Process

Despite the lack of agreement on certain aspects of the table, several companies cosigned on the need for the network to control the data collection process [9]. The paper provides two observations and one proposal, which are copied below.

Observation 1: For the training of a UE-sided model, collecting the data from UEs without network control/awareness is highly inefficient, as a UE may not have the relevant/required information (e.g., UE not configured with the proper measurement configuration).

Observation 2: Collecting large amount of data for the training of a UE-sided model transparently from the RAN/CN may lead to traffic congestion and the operator may not be able to ensure that the required QoS is provided to the UEs in the network.

Proposal 1: The network should have full control of the data collection process for the training of a UE-sided model, including the initiation, termination and management of the data collection and data transfer.

Essentially, it is argued that NW awareness and control over the data collection process is required for efficiency and to ensure that enablement of data collection does not eliminate the gains that AI/ML features have the potential to offer. In other words, it is argued that the NW is in the best position to determine data collection configurations which would match with future configurations for AI/ML-enabled features.

Observation 6: Many companies in RAN2 argue that the NW should have full control over the data collection process for the training of a UE-side model, including the initiation, termination of the data collection and data transfer.

##### 2.2.5	Other WGs Impact

Lastly, the table in Annex 6.1 [6] includes a row, copied below, listing the working groups that would likely be impacted by each option for data collection for UE-side AI/ML Models.

|  | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |

We think that it would be beneficial for RAN to down select to Options 2 and 3 so that RAN2 can begin detailed work on the data collection mechanisms without immediately engaging with other working groups and enabling timely completion of the work item. From the perspective of RAN, we believe that if the UE can be configured by the NW to collect data and transmit it to a NW entity such as the gNB or LMF, that other WGs can pick up the data from an interface in the RAN-domain entity and determine how to configure the RAN-domain entity to initiate and configure data collection.

That is not to say that it is somehow a trivial task, but it is important that RAN2 can commence work on what RAN2 knows how to do: RRC, MAC, and LPP, and communication between UEs and gNBs and LMFs.

Observation 7: RAN2 listed other WGs which could be impacted by different options for data collection for the training of UE-side models. If RAN helps RAN2 by down-selecting to options 2 and 3, RAN2 could proceed with work on detailed solutions between the UE and gNB, and between the UE and LMF.

### 3	Next Steps

Because RAN2 has been unable to conclude on a way forward for data collection for UE-side AI/ML models, during this study phase of the WI, in our view it is important that RAN helps RAN2 in concluding the study phase and down-selecting to the option 2 and 3 and thereby allow RAN2 to move to work item phase on detailed specification work on data collection for UE-side AI/ML models.

Proposal 1: RAN plenary to provide instruction to RAN2 on which data collection option(s) to proceed with work.

Proposal 2: Select option 2 and option 3 for data collection for UE-side AI/ML models to allow RAN2 to move to work item phase and specification development.

### 4	Conclusion

In this contribution, the following observations were made.

Observation 1: RAN2 has agreed to support immediate MDT and enhancements thereto for data collection for training NW-side beam management models.

Observation 2: RAN2 has been unable to reach consensus or conclude on several aspects of data collection, including the level of controllability and visibility of Option 1b, the need for partial and no visibility of data, and the feasibility of a UP tunnel for Options 2 and 3.

Observation 3: The data size for data collection for training UE-side models for the beam management and positioning use cases is on average around 500 bits per measurement instance.

Observation 4: The latency requirement for data collection for training UE-side models is considered “relaxed”, which was defined by RAN1 as on the order of “minutes, hours, days, or no latency requirement.”

Observation 5: RAN2 discussed the possibility of supporting a transparent container inside of a standardized message to support partial visibility of data, e.g., proprietary data collected from UEs, but could not reach an agreement to support or not support such a container.

Observation 6: Many companies in RAN2 argue that the NW should have full control over the data collection process for the training of a UE-side model, including the initiation, termination of the data collection and data transfer.

Observation 7: RAN2 listed other WGs which could be impacted by different options for data collection for the training of UE-side models. If RAN helps RAN2 by down-selecting to options 2 and 3, RAN2 could proceed with work on detailed solutions between the UE and gNB, and between the UE and LMF.

And we make the following proposals.

Proposal 1: RAN plenary to provide instruction to RAN2 on which data collection option(s) to proceed with work.

Proposal 2: Select option 2 and option 3 for data collection for UE-side AI/ML Models to allow RAN2 to move to work item phase and specification development.

### 5	References

[1] RP-240774, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, March 2024

[2] R2-2404102, “RAN2#125bis Meeting Report”, MCC, April 2024, Changsha, China

[3] R2-2406202, “RAN2#126 Meeting Report”, MCC, May 2024, Fukuoka, Japan

[4] R2-240xxxx, “RAN2#127 Meeting Report”, MCC, August 2024, Maastricht, The Netherlands

[5] TR38.843, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 3GPP, December 2023

[6] R2-2407807, “CR on RAN2 inputs to TR 38.843”, Ericsson, August 2024

[7] R2-2311720, “Reply LS on Data Collection Requirements and Assumptions”, Qualcomm, November 2023

[8] R2-2407773, “Report of [AT127][021][AIML PHY] UE side data collection”, InterDigital Inc., August 2024, Maastricht, The Netherlands

[9] R2-2407413, “Data collection for the training of a UE-sided model”, InterDigital Inc., Ericsson, Nokia, T-Mobile USA Inc., BT PLC., Deutsche Telekom, Verizon, NTT Docomo, Charter Communications Inc., August 2024

### 6	Annex

#### 6.1	RAN2 Analysis of different data collection options for UE-side model training

The following table, 7.2.1.3.2-1, on data collection options for UE-side model training includes changes agreed in RAN2#127, pending RAN1 approval for addition to TR38.843.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | FFS: level of controllability (Note 5) (Note 1) | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure, FFS other procedures | Via RRC procedure |
| Possible Options for data content visibility in MNO (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. UE proprietary information can only be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of possible MNO controllability and visibility via Option 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 cannot reach consensus on the feasibility of UP tunnel in Options 2 and 3 |  |  |  |  |

#### 6.2	RAN1 Data Collection Requirements for Beam Management and Positioning

RAN1 provided the following tables for data collection requirements for UE-side models [7]. The tables have been abbreviated only to contain the details relevant to training of UE-side models.

For Beam management

| LCM purpose | UE-side/NW-side models | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | UE-side, NW-side | L1-RSRPs and/or beam-IDs | See Note 1 for L1-RSRPs | Relaxed |  |

Note 1: There is no agreement on the data size of L1-RSRPs for Set A or Set B, but the following typical data size is provided as guidance for RAN2 discussion. Based on existing L1-RSRP reporting methodology, i.e., 7 bits for the strongest beam and 4 bits for the remaining beams, for Set B = 16 as an example, the typical data size would be 67 (hence up to ~100 bits), and for Set A = 128 as an example, the typical data size would be 515 (hence up to ~500 bits) if all beams in Set A were to be collected. For BM Case 2, the data size L1-RSRPs for Set A and Set B represents the data size per predicted future time instance and per history measurement time instance, respectively. Payload size may not be fixed.

For positioning

| LCM purpose | Case | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | All Cases | Measurements (corresponding to model input): timing, power, and/or phase info See Note 2 | Size depends on number of PRS/SRS resources, measurement type (timing, power, and/or phase info) and report format: ~100 bits to 1000s bits per PRS/SRS resource See Note 3 | Relaxed |  |
|  | Direct AI/ML positioning | Label: Location coordinates as model output | 56 to 144 bits See Note 3 | Relaxed |  |
|  | AI/ML assisted positioning | Label: Intermediate positioning measurement (timing info, LOS/NLOS indicator) as model output See Note 2 | 10s bits to 100s bits per PRS/SRS resource See Note 3 | Relaxed |  |

Note 1: The necessity and feasibility of difference cases (Case1 to Case3b) needs further discussion/conclusion.

Note 2: For measurements as model input, no agreement on measurement types (i.e., time, power, and/or phase) in RAN1 for all cases (i.e., Case1 to Case3b). Measurement types (including their necessity) and sizes/dimension needs to be further discussed. Candidate measurement types discussed/evaluated for model input include CIR (contains timing, power and phase information), PDP (contains timing and power information), DP (contains timing information). For labels (i.e., model output) of AI/ML assisted positioning (Case2a, Case3a), RAN1 identified an initial listing of candidates that provide performance benefits (i.e., timing info, LOS/NLOS indicator). RSRP/RSRPP is for further discussion.

Note 3: The measurement size of one data sample = (measurement data size of one PRS/SRS resource)*(number of PRS/SRS resources needed for model input). The label size of one data sample = (label data size of one PRS/SRS resource)*(number of PRS/SRS resources needed for model output). The quantization and bit representation of time, power, and phase information (including their necessity) still need to be further discussed.  Existing specification allows reporting of up to 64 PRS/SRS resources per frequency layer for one positioning fix. For evaluations, most companies considered up to 18 TRPs. It should be noted that AI/ML positioning is not restricted to work only with maximum of 18 TRPs.

Common Notes for all sub-use-cases:

- In answering latency requirements, RAN1 used the following descriptions:

  - Relaxed (e.g., minutes, hours, days, or no latency requirement)

  - Near-real-time (e.g., several tens of msecs to a few seconds)

  - Time-critical (e.g., a few msecs)