3GPP TSG RAN Meeting #105								RP-242121

Melbourne, Australia, September 9-12, 2024

Agenda Item:		9.3.1.3

Source:			Nokia

Title:				On Checkpoint for Study of CSI feedback enhancements

Document for:		Decision

### 1	Checkpoint

The WID for AI/ML for NR Air Interface [1] includes a deadline in RAN#105 for the following checkpoints on CSI feedback enhancements.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (UE-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

In this contribution we discuss the status of CSI prediction and CSI compression studies in RAN1 and propose how to move forward in Release 19.

### 2	CSI Prediction checkpoint

RAN1 completed its study on CSI prediction with the following agreement made in RAN1#118:

Agreement From RAN1 perspective, study of CSI prediction has been completed and performance improvement is observed with increased complexity.

Considering that RAN1 completed its study we see that RAN1 could move to work item phase.

Proposal 1: We propose the following CSI prediction objectives to be added to the work item

- New objectives for core part:

  - Support for AI/ML-based UE-sided CSI prediction encompassing [RAN1/RAN2]:

    - Predicted CSI reporting using existing CSI codebooks (enhanced Type II codebook, e.g.)

    - Specify enhancements of periodic CSI-RS and aperiodic CSI-RS burst to enable a shorter prediction time with periodic CSI-RS (shorter than the period)

    - Specify necessary signaling/mechanism(s) to facilitate LCM operations specific to the CSI prediction use case, if any

    - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

- Objective updates for performance part:

  - Modify the following statement in the performance part:

    - For Beam Management, and Positioning Accuracy enhancement, and CSI Prediction use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models

### 3	CSI compression checkpoint

RAN1 was not able to complete its study on CSI compression yet and therefore recommends to continue the study as shown by the following RAN1#118 agreement:

Agreement RAN1 is recommending extending the study of the AI/ML CSI compression based on RAN1 understanding the study on CSI compression is not completed.

We support to continue the CSI compression study but we see that it is also important to recognize that it is not realistic to assume that CSI compression can move to work item phase in Release 19 as significant amount of work is also needed in RAN4 and RAN2. Also, RAN1 has still lots of normative work remaining on different areas of the AI/ML for air interface work item. Therefore, it would be important to indicate to RAN1 that it has time until the end of the Release 19 to complete the study as normative work is only expected in Release 20.

Proposal 2: Allow RAN1 to continue CSI compression study until the end of Release 19 with an expectation that normative work will only be done in Release 20.

### 4	Conclusions

In this contribution we have discussed the status of CSI prediction and CSI compression studies in RAN1. We propose that the CSI prediction can move to normative phase and CSI compression studies will continue for the rest of release 19 to avoid taking too much valuable time from other important RAN1 normative work on AI/ML for air interface.

Proposal 1: We propose the following CSI prediction objectives to be added to the work item

- New objectives for core part:

  - Support for AI/ML-based UE-sided CSI prediction encompassing [RAN1/RAN2]:

    - Predicted CSI reporting using existing CSI codebooks (enhanced Type II codebook, e.g.)

    - Specify enhancements of periodic CSI-RS and aperiodic CSI-RS burst to enable a shorter prediction time with periodic CSI-RS (shorter than the period)

    - Specify necessary signaling/mechanism(s) to facilitate LCM operations specific to the CSI prediction use case, if any

    - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

- Objective updates for performance part:

  - Modify the following statement in the performance part:

    - For Beam Management, and Positioning Accuracy enhancement, and CSI Prediction use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models

Proposal 2: Allow RAN1 to continue CSI compression study until the end of Release 19 with an expectation that normative work will only be done in Release 20.

### 5	References

[1] RP-240774, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, March 2024