3GPP TSG RAN Meeting #105	RP-242130

Melbourne, Australia, September 9-12, 2024

Title: 	Discussion on Rel-19 XR WID

Source: 	CATT

Agenda item:	9.3.2.3

Document for:	Discussion and decision

Introduction

In this paper, we discuss the two check points in XR Phase-3 WID [1]:

The Rel-19 XR Phase 3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. NOTE: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. NOTE: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Specify Core requirements related to the above objectives as necessary [RAN4] - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3] - PDU set based handling - ECN marking - Burst Arrival Time reporting, if needed - PSI Discard coordination, if needed - Note: No RAN2 impact from above items NOTE: Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.

Discussion

The two check points for multi-modality and scheduling enhancements are discussed respectively based on the progress in RAN2 till now (as cited in the Annex).

- 2.1 Multi-Modality

For multi-modality, RAN2 work has focused on the below aspects of multi-modal service support during the study phase:

- How to provide multi-modality related info to RAN;

- Joint admission control specific to multi-modal service;

- PDU Set discard across QoS flows;

- LCP enhancements w.r.t. sync threshold;

- Multiple active DRX configurations;

- Independent configuration of drx-StartOffset for the secondary DRX group.

The first three items have been agreed to be further discussed/supported (with agreement or working assumption) in potential normative phase per related RAN2 agreements (see the Annex). Since there are WG agreements to support specification work on related topics, the objective on multi-modality support can proceed into the normative phase.

The specific scope of the normative work on multi-modality should respect the RAN2 agreements till now. Therefore, the below three aspects can be further specified in the normative phase for multi-modality support:

- UL multi-modal association reporting by UAI.

- Joint admission control for multi-modal service.

- PDU Set discard across QoS flows by taking multi-modal information into account.

One thing that needs additional attention is that for above bullet c, it can be supported or not depends on whether necessary information can be provided by SA2/4 to the UE AS or gNB, as per RAN2 agreement. For UL PDU Set discard across QoS flows, the necessary information (e.g. sync info) needs to be provide by SA2/4 to the UE AS, while for DL, the necessary information (e.g. sync info) needs to be provide by SA2/4 to the gNB. This needs to be reflected in the WID as well.

Proposal 1: RAN concludes that the support of multi-modality proceeds in normative phase with the following scope and responsible WGs:

- UL multi-modal information reporting by UAI. [RAN2]

- Joint admission control for multi-modal service. [RAN2, RAN3]

- PDU Set discard across QoS flows by taking multi-modal information into account. [RAN2, RAN3]

  - NOTE: Above bullet c) is discussed/specified, only if SA2/4 confirms to provide necessary information (e.g. sync info) to the UE AS/gNB.

- 2.2 Scheduling enhancement

For scheduling enhancement, RAN2 in the study phase agreed to only further discuss LCH prioritization and DSR enhancements in the potential normative phase. Hence, we think RAN can conclude that the scheduling enhancement objective proceeds in normative phase, with also the scope respecting the RAN2 agreements made in the study phase, i.e. supporting the following two aspects:

- LCP enhancement with LCH prioritization.

- DSR enhancement by reporting multiple remaining time and buffer size for one LCG.

Proposal 2: RAN concludes that scheduling enhancement objective proceeds in normative phase with the below scope and responsible WGs:

- LCP enhancement with LCH prioritization. [RAN2]

- DSR enhancement by reporting multiple remaining time and buffer size for one LCG. [RAN2]

Conclusion

In this contribution, we discussed the two check points in XR Ph-3 and have the following proposals:

Proposal 1: RAN concludes that the support of multi-modality proceeds in normative phase with the following scope and responsible WGs:

- UL multi-modal information reporting by UAI. [RAN2]

- Joint admission control for multi-modal service. [RAN2, RAN3]

- PDU Set discard across QoS flows by taking multi-modal information into account. [RAN2, RAN3]

  - NOTE: Above bullet c) is discussed/specified, only if SA2/4 confirms to provide necessary information (e.g. sync info) to the UE AS/gNB.

Proposal 2: RAN concludes that scheduling enhancement objective proceeds in normative phase with the below scope and responsible WGs:

- LCP enhancement with LCH prioritization. [RAN2]

- DSR enhancement by reporting multiple remaining time and buffer size for one LCG. [RAN2]

References

- RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3	RAN#103

- Annex: RAN2 agreements for XR Ph-3

- RAN2#125bis

Regarding multi-modality (study phase):

-	For the purpose of study, RAN2 assumes that UE and gNB have some kind of multi-modal information;

-	FFS what information is needed/useful, e.g. just multi-modal ID, association between the flow, synchronization requirement etc.

-	RAN2 will study both UL and DL directions based on the assumption of multi-modality association knowledge at RAN/UE;

-	RAN2 will focus on analysing potential usage and benefits (e.g. in terms of capacity and power saving) of multi-modal association knowledge;

-	Areas to study include: synchronization between the flows, FFS impact on QoS insurance and other areas;

-	RAN2 assumes that traffic of different modals having different QoS requirements is mapped to different QoS flows;

-	For different XR traffic flows belonging to the same Multi-modal service and having different QoS requirements, it should be possible to provide differentiated QoS handling over the air. RAN2 should study if that is possible with current mechanism or new ones are needed;

-	Existing QoS flow to DRB mapping framework is used as a baseline, i.e. up to gNB how to map QoS flows to DRBs.

Regarding Scheduling Enhancements (study phase):

-	RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied:

-	Alternative 1: Enhance LCP restrictions/LCH selection;

-	Alternative 2: Enhance LCH prioritization;

-	RAN2 should consider potential impact on traffic from SRBs.

-	RAN2 will study enhancing existing DSR with additional information, e.g. multiple pairs of remaining time/buffer information, importance - FFS whether this only includes more information on delay-critical data or also information about non-delay critical data.

- RAN2#126

Regarding multi-modality (study phase):

-	Support Multi-Modality awareness in RAN in Rel-19 for UL and DL.

Regarding scheduling enhancements (study phase):

-	For LCP enhancements, LCP Prioritisation:

-	Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR;

-	The solution should consider impact on UE complexity (as already indicated in SI objective description);

-	For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

-	Use additional priority configured to LCHs in case of these LCHs with delay-critical data.

-	FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH.

-	For LCP enhancements, LCH Restrictions:

-	We try to avoid RAN1 impacts;

-	RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way;

-	For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial;

-	The solutions should not disallow non-delay critical data from using an UL grant.

-	For LCP enhancements, Granularity:

-	LCP prioritization within a logical channel will not be considered in RAN2 discussions;

-	FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

-	For DSR enhancements:

-	Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG;

-	FFS whether DSR triggering is impacted;

-	FFS whether PDU set importance needs to be included.

- RAN2#127

Regarding multi-modality (study phase):

- Working assumption: Regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB.

- FFS how this is used by the gNB. FFS whether this can be applied to DL.

- RAN2 considers that based on multi-modal information:

    - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose.

    - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

- For UL, RAN2 does not intend to perform LCP enhancements due to complexity vs gains concerns.

- For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN.

- RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

- RAN2 thinks in case this is feasible, it should be limited to intra-DRB case.

- Not support multiple active DRX configurations

- Not support independent configuration of drx-StartOffset for the secondary DRX group

Regarding scheduling enhancements (study phase):

- RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR.

- Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG.

- For enhanced DSR:

    - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds

    - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR

    - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)