3GPP TSG RAN Meeting #105	RP-242135

Melbourne, Australia, September 9-12, 2024

# Source:	CATT

# Title:	New WID: Enhanced NR support for UAV (Uncrewed Aerial Vehicles)

# Document for:	Information

# Agenda Item:	9.1.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Enhanced NR support for UAV (Uncrewed Aerial Vehicles)

Acronym:

Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:	Rel-19

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This description is either a …

|  | Study Item |
| --- | --- |

or a

| Normative Work Item: tick applicable boxes below |  |
| --- | --- |
|  | Stage 1 |
| X | Stage 2 |
| X | Stage 3 |
|  | Other (e.g. testing) |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 941105 | WI on NR Support for UAV | The present WID will use this WID as basis for the enhancement. |
| 1040037 | Phase 3 for UAS, UAV and UAM | The present WID will consider potential NG-RAN impact resulting from this WID. |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

Dependency on non-3GPP (draft) specification:

None.

3	Justification

In recent years, increasing interests in unscrewed aerial vehicles (UAV) based services have been witnessed globally, with the application of UAV based services being regarded a new factor facilitating the development of various industries. In some regions, there are great interests from governments and operators to build a “Low-altitude” network to support “Low-altitude” economy by UAV communications, with strong industrial radiations (e.g. logistics, transportation, agriculture, meteorology, etc.) and expectation of trillion-level industrial and market scale. Great expansion and growth of commercial drone/UAV markets are witnessed in variety of regions throughout the globe.

In Release 18, support of UAV based services was introduced and specified in NR, aiming at more effective support of UAV based services in NR RAN. However, there are still the following limitations based on Rel-18 NR UAV design with potential enhancements able to be considered:

- Mobility aspect: There has been no consideration on UAV-specific factors in those advanced mobility mechanisms in NR (e.g. CHO, LTM, etc.), making 5G NR insufficient/ineffective to support the mobility requirements of UAV communication thus far. UAV is characterized by the pre-determined flight path, and the number of detectable cells and the range of the detected cells highly depend on the height of the UAV. To this end, considering the flight path and height information in CHO/LTM mechanism would lead to more efficient mobility handling (e.g. height based CHO trigger, subsequent LTM, etc.) and bring overhead-saving gain (e.g. height based event triggered L1 measurement reporting). Also, in a practical deployment of the NW supporting UAV based service in a given area, it is typically expected that the gNBs are deployed with an inter-site distance of several kilometres (e.g. 3 ~ 5 km) but with no Xn deployment due to e.g. cost-saving consideration. However, current NR specifications only support Xn based CHO for terrestrial network, preventing the current CHO design supporting UAV mobility in practical NW deployment.

Some UAV based services may not expect continuous large volume data transmissions (e.g. atmosphere/logistics monitoring data transfer, instantaneous transfer of video data collected in hovering status, etc.). In such a scenario, the UAV does not need to be kept in CONNECTED continuously, but can usually be sent to IDLE when data transfer finishes or to INACTIVE state to perform small data transmission. In practice, a gNB aiming to provide better support of UAV based service can deploy additional antenna with up-tilted angles, in order for a better coverage to the air than normal down-tilted gNBs serving only  ground UEs (which can serve UAVs only by side lobes). Such gNBs may co-exist with the legacy gNBs only serving normal ground UEs in a typical deployment scenario. Current IDLE/INACTIVE mobility in NR does not consider such UAV specific gNB deployment. It may thus be preferable to consider UAV specific IDLE/INACTIVE mobility enhancement, ensuring the UAVs to be served by the dedicated gNB deployment.

- UAV detection and supervision: In practice, there is the need for the network to know the real-time motion status of UAV, so as to carry out effective supervision in the network (e.g. detection/control on communication of spurious UAVs). For example, the network may need to be aware of whether a UAV is in the flying status, and thus start supervision on its communication (e.g. release of connectivity and de-registration, if needed) w.r.t. its dynamics in a timely and on-demand manner. This is done by the 5G network based on the interaction with supervision platform. Such demand may apply to both pre-R18 and R18 UAVs. It is possible for NG-RAN to identify a UAV with related dynamics via implementation-specific manners (e.g. via measurements, interference detection, etc.). However, due to the lack of specification support, NG-RAN currently cannot send such information to 5GC, which thus cannot contact the supervision platform to provide necessary control to the UAV communications as intended.

In addition, there is the conclusion made in SA2 UAS Phase-3 SI that no-transmit zone (NTZ) enforcement is based on UE implementation. RAN WGs may need to study whether any UE access stratum operations prevent such implementation dependent NTZ enforcement as concluded by SA2.

Based on above justifications, normative work need be done in the above aspects to support enhanced NR support of UAV in NG-RAN.

4	Objective

The objectives of the work item are the following:

4.1	Objective of SI or Core part WI or Testing part WI

- Specify CHO enhancements, considering flight path planned and/or height related events and practical RAN deployment to support UAV communication: [RAN2, RAN3]

- Specify height-based/flight path based CHO trigger events. [RAN2]

- Specify NG-based CHO in terrestrial network for UAV. [RAN3]

- Specify UAV detection and  motion status notification from NG-RAN to 5GC: [RAN3]

- Specify the NW signalling procedure and information exchange over NG interface (e.g. UAV identification request/response, related motion status, etc.).

- Specify UAV-specific IDLE/INACTIVE mobility enhancement: [RAN2]

- Specify mechanisms prioritizing frequency(ies)/cell(s) with enhanced UAV-based service support for cell (re)selection.

- Extend the use of altitude based SSB to support IDLE/INACTIVE mobility.

[4.	Specify support of LTM enhancement for UAV, taking height and flight path of UAV into consideration, including following objectives: [RAN2, RAN3]

- Signalling reduction on L1 measurement report for undetectable candidate cells (e.g. UAV specific trigger event). [RAN2]

- Subsequent LTM with UE autonomous candidate cell activation/deactivation (based on planned flight path). [RAN2, RAN3]

- NOTE: This objective is to be carried out based on the outcome of related work in Mob Phase-4 WI. ]

[5.	Align UE AS operations with SA2 conclusion on NTZ enforcement: [RAN2]

- Study if current specification allows the implementation-specific NTZ enforcement, and introduce necessary specification change/clarifications to align with SA2 conclusion (if needed). ]

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.306 | UAV capabilities | RAN#111 |  |
| 38.331 | UAV signaling/reporting | RAN#111 |  |
| 38.300 | UAV Stage-2 description | RAN#111 |  |
| 38.413 | NG-RAN; NG Application Protocol (NGAP) | RAN#111 |  |
| 38.423 | NG-RAN; Xn Application Protocol (XnAP) | RAN#111 |  |
| 38.101-1 | Additional OOBE requirements applicable to aerial UE | RAN#111 |  |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

7	Work item leadership

RAN2

Secondary responsible Working Group(s): RAN3.

8	Aspects that involve other WGs

None so far.

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

Supporting IM name