3GPP TSG RAN Meeting #105                                                                           RP-242139

Melbourne, Australia, September 9-12, 2024

Source:	CATT

Title:	Views on normative phase of Topology Enhancement in Rel-19

Agenda Item:	9.2.7

Document for:	Discussion

1 Introduction

As RAN#105 is the check point for the study of Rel-19 Topology Enhancement to be transitioned into a normative phase, in this contribution, we provide our views on normative work of Topology Enhancement in Rel-19.

2 Discussion

2.1 On normative phase of WAB

In the last RAN3 meeting [1], RAN3 confirmed the feasibility of WAB functionality and recommended the normative phase for WAB in clause 6.1 of the draft TR 38.799 [2]. The principles for normative work and the aspects that normative work should specify have already been described in the conclusion for WAB’s study.

Clause 6.1 in the draft TR 38.799[2]: - The normative work for WAB should be based on the functionalities, terminology and requirements captured in the present TR. Addition of further details during normative phase is not precluded. - The normative work should consider the following architectural aspects for WAB according to the present TR: - Backhauling of the WAB-gNB’s NG, Xn and OAM traffic is conducted over the WAB-MT’s PDU session(s). - WAB-gNBs can establish Xn interface(s) with the WAB-MT’s serving BH RAN node and with other surrounding gNBs. - The interface between the WAB-MT and the co-located WAB-gNB is out-of-scope for the normative phase. - Split architecture of the WAB-gNB is out-of-scope for the normative phase. - Authorization procedures for the WAB-MT are out of RAN3 scope, and are expected to be handled by SA2. RAN3 should define the WAB-node behaviour in case the authorization status of the WAB-MT and/or WAB-gNB changes. - The normative phase should define the integration procedure for WAB nodes following the description in the present TR. - Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures. - During the normative phase, handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility should be defined, including the case where the WAB-MT’s BH PDU session changes. - During the normative phase, the procedure to support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB, should be defined in cooperation with SA2. - Solutions for mobility will be further analysed during normative phase. - RAN3 concludes that the two-logical-gNB solution is feasible. - It remains to be verified, based on feedback from SA2 and, potentially, RAN2 if needed, whether the single-gNB solution is feasible and whether enhancements are needed. - During the normative phase, enhancements to the UE’s ULI that reflect the WAB node’s location should be defined. - During the normative phase, the handling of the following should be discussed and captured: - PCI collision. - Reconfiguration of TAC and RANAC on WAB-gNBs. - Avoidance of multi-hop WAB topology. - Radio-resource coordination between access and backhaul links.

In our view, above description is clear enough and well reflects the conclusion for the study phase of WAB. So, it is proposed that objectives on basic WAB functionalities follow above conclusion for WAB’s study.

Proposal 1: For the Objectives to support basic functionalities of WAB, follow the corresponding conclusions in TR 38.799[2].

Furthermore, the last paragraph of conclusion for WAB’s study in clause 6.1 of TR 38.799[2] recommended the potential enhancements to WAB. It was recommended that following aspects may be included in the normative work for WAB and thus can be discussed during the normative phase:

Clause 6.1 in the draft TR 38.799[2]: The normative phase may further discuss the following: - Handling of backhaul link degradation by the backhaul network and the WAB-gNB. - Xn connection management (e.g., potential avoidance of setting up Xn between WAB-gNBs, dynamic establishment/removal of Xn connectivity with BH-RAN-node and surrounding NG-RAN-nodes). - NG connection management (e.g., procedures for NG connection removal/suspension). - QoS (e.g., PDB, PER). - Handling of RRC_INACTIVE UEs.

Note that the aforementioned bullets have not been discussed during the study phase, with various companies proposing their inclusion in subsequent normative phase. While it is uncommon to discuss techniques in the normative phase without prior discussion in the study phase, we are willing to align with the majority's opinion. Nevertheless, from our perspective, the conclusion included in the aforementioned bullets lacks sufficient clarity. If objectives of WAB’s normative work include this part, it is imperative that the scope for these bullet points is explicitly defined to avoid ambiguity or controversy during the normative phase.

Proposal 2: Given the absence of technical discussion on potential WAB enhancements during SI phase, in order to prevent any ambiguity or controversy that may arise during the normative phase, it is proposed to define the corresponding objectives explicitly and clearly.

On “Handling of backhaul link degradation”:

As the backhaul link of WAB-node is wireless, the issue of degradation of backhaul link would impact the backhauling for WAB-gNB’s traffic via WAB-MT’s BH PDU session, and would require the enhancements to WAB. Hence, this issue should be discussed and specified (if needed). It’s feasible to capture this bullet in the normative work of WAB.

Proposal 2-1: Objectives of WAB’s normative work include handling of backhaul link degradation.

On “Xn connection management” and “NG connection management”:

The scope of “Xn connection management” or “NG connection management” is too large to specify, thus they would need to be refined.

Actually, companies have identified the associated issues during the last RAN3 meeting and those issues are captured in the following “e.g.” brackets. For the issues that are clear enough as clarified in the following bracket, it is proposed to replace “Xn connection management” and “NG connection management” with the brackets following respectively.

Proposal 2-2: Objectives of WAB’s normative work include potential avoidance of setting up Xn between WAB-gNBs and dynamic establishment/removal of Xn connectivity with BH-RAN-node and surrounding NG-RAN-nodes.

Proposal 2-3: Objectives of WAB’s normative work include procedures for NG connection removal/suspension.

On “QoS”:

In general, the QoS which requires end-to-end assurance needs to be studied in SA2 firstly. For example, the PDB which includes AN PDB and CN PDB is determined by 5GC, rather than the NG-RAN. Accordingly, in WAB scenario, whether and how the PDB is impacted needs to be studied in SA2 firstly, but not in RAN.

However, it has been definitely described in SID of Rel-19 VMR (i.e., the project associated with WAB in SA2) as follows that the QoS aspects which impact existing mechanisms are not considered. Accordingly, the QoS enhancement is not in the scope of the normative phase of VMR for Rel-19 [4]. So, during Rel-19 WAB, RAN should not initiate discussion on QoS enhancements assumed to be studied by SA2 firstly.

SID of Rel-19 VRM: Note5: In Rel-19, no enhancement is aimed to the existing QoS mechanisms for the wireless backhaul.

On the other hand, the QoS mapping of BH PDU session has been discussed by SA2 during the SI phase for Rel-19 VMR, and the relative conclusion is captured in clause 8.1 in TR 23.700-06[5]. That requires no enhancement to exiting QoS mechanism, but would impact the traffic (including Xn, NG and OAM traffic) mapping handling in WAB-gNB and hence involves RAN.

Clause 8.1 in TR 23.700: - If QoS optimization of the N3 connection is required, then the MWAB-gNB is configured (e.g. by OAM) with the mapping of the 5QIs to the DSCP field for TNL used by the UPFs/gNBs in the PLMN/SNPN serving the UE via the MWAB-gNB. This is then used to establish SDFs at the BH-PDU session used for N3 connection(s) based on existing procedures.

Considering the above status in SA2, it is proposed to clearly specify the scope of the “QoS” bullet as the support of the QoS mapping of the WAB-gNB’s traffic (including Xn, NG and OAM traffic) to BH PDU session and preclude other aspects (e.g., PDB or PER) belonging to SA2 scope.

Proposal 2-4: Objectives of WAB’s normative work include handling QoS mapping for the WAB-gNB’s traffic to BH PDU session and preclude other enhancements (e.g., PDB and PER) belonging to SA2 scope.

On “Handling of RRC_INACTIVE UEs”:

There are various aspects pertaining to RRC_Inactive mode UEs, but this bullet seems not clear enough as the issue is not clarified by present text. One potential issue may arise for retrieving the context of the RRC_INACTIVE UEs due to dynamic Xn interface removal of WAB-node during the mobility of the WAB-node. In our view, that can be addressed by proper Xn removal procedure between the WAB-node and surrounding RAN nodes, and thus the corresponding objective can be assumed being contained in the objective of dynamic establishment/removal of Xn connectivity if necessary. As the issue for RRC_INACTIVE UEs handling still needs to be identified, it’s proposed to preclude the “RRC_INACTIVE UEs handling” from the objectives of WAB’s normative work.

Proposal 2-5: As the issue of RRC_INACTIVE UEs handling can be addressed by proper establishment/removal of Xn connectivity, this bullet can be removed from objectives of WAB’s normative work.

Based on above discussion, we suggest several enhancements to be included in objectives of WAB’s normative phase according to the last paragraph of conclusion for WAB’s study in TR 38.799[2].

Proposal 3: Following text is proposed to be captured by objectives for WAB in WID:

Support of enhancements to WAB, if needed, including [RAN3, RAN2]: - Handling of backhaul link degradation by the backhaul network and the WAB-gNB. - Potential avoidance of setting up Xn between WAB-gNBs - Dynamic establishment/removal of Xn connectivity with BH-RAN-node and surrounding NG-RAN-nodes. - Procedures for NG connection removal/suspension. - Handling of QoS mapping for WAB-gNB’s traffic (including Xn, NG and OAM traffic) to BH PDU session.

2.2 On normative phase of Femto

In the last RAN3 meeting [1], RAN3 has completed the study of 5G NR femto, and given the conclusion of study for Femto in clause 6.2 in the draft TR 38.799 [2] as follows.

Clause 6.2 in the draft TR 38.799[2]: ### 6.2.1 Architecture for NG interface Option 1, namely direct connection of an NR Femto node to the 5GC via the NG interface, is already possible. In order to maintain the existing infrastructure for an operator who has deployed LTE HeNBs, Option 2 with an optional NR Femto GW is recommended for a normative phase. ### 6.2.2 Architecture for Xn interface Option A, namely direct connection of an NR Femto node to other NR Femto nodes / gNBs via the Xn interface, is already possible. ### 6.2.3 Access Control For Access control, NR Femto reuses the existing CAG functionalities, no stage3 impact has been identified. ### 6.2.4 Access to Local Services In order to support access to local services, NR Femto nodes reuse LADN and edge computing functionality as specified in TS 23.501 [6] and TS 23.548 [8]. Aspects out of RAN3 scope, described in Sec. 5.4, have been identified and may require further analysis.

According to above conclusion for the study of Femto, the option 1 and option 2 of NG interface were recommended to be specified in normative phase with option A of Xn interface was selected for normative work. Furthermore, access control was recommended to be specified with reuse of existing CAG functions, and support of access to local services was recommended to leverage LADN and edge computing functionality. As the conclusion is clear and nothing controversial is observed, objectives of Femto’s normative work should follow the conclusion.

Proposal 4: Objectives of Femto’s normative work follow the conclusion of Femto’s study in TR 38.799[2].

3 Conclusion

In the previous sections we made the following proposals:

Proposal 1: For the Objectives to support basic functionalities of WAB, follow the corresponding conclusions in TR 38.799[2].

Proposal 2: Given the absence of technical discussion on potential WAB enhancements during SI phase, in order to prevent any ambiguity or controversy that may arise during the normative phase, it is proposed to define the corresponding objectives explicitly and clearly.

Proposal 2-1: Objectives of WAB’s normative work include handling of backhaul link degradation.

Proposal 2-2: Objectives of WAB’s normative work include potential avoidance of setting up Xn between WAB-gNBs and dynamic establishment/removal of Xn connectivity with BH-RAN-node and surrounding NG-RAN-nodes.

Proposal 2-3: Objectives of WAB’s normative work include procedures for NG connection removal/suspension.

Proposal 2-4: Objectives of WAB’s normative work include handling QoS mapping for the WAB-gNB’s traffic to BH PDU session and preclude other enhancements (e.g., PDB and PER) belonging to SA2 scope.

Proposal 2-5: As the issue of RRC_INACTIVE UEs handling can be addressed by proper establishment/removal of Xn connectivity, this bullet can be removed from objectives of WAB’s normative work.

Proposal 3: Following text is proposed to be captured by objectives of WAB in WID:

Support of enhancements to WAB, if needed, including [RAN3, RAN2]: - Handling of backhaul link degradation by the backhaul network and the WAB-gNB. - Potential avoidance of setting up Xn between WAB-gNBs - Dynamic establishment/removal of Xn connectivity with BH-RAN-node and surrounding NG-RAN-nodes. - Procedures for NG connection removal/suspension. - Handling of QoS mapping for WAB-gNB’s traffic (including Xn, NG and OAM traffic) to BH PDU session.

Proposal 4: Objectives of Femto’s normative work follow the conclusion of Femto’s study in TR 38.799[2].

4 Reference

- Draft Report of 3GPP TSG RAN3 meeting #125, MCC

- R3-244849, TR skeleton for additional topological enhancements for NR, RAN3#125

- SP-240493,Study on Vehicle Mounted Relays Phase 2, SA#103

- SP-240989, New WID on Vehicle Mounted Relays Phase 2, SA#104

- 3GPP TSG SA, TR 23.700-06, Study on architecture enhancements for vehicle-mounted relays - Phase 2, v19.0.0 (2024-06)