3GPP TSG-RAN Plenary Meeting #105                                                RP-242141

9 Sep- 12 Sep 2024 Melbourne, Australia

Source: 	Vodafone, Deutsche Telekom, Orange, Lenovo, Google, Fraunhofer IIS and Fraunhofer HHI, ETRI, CEWiT

Title: 		On demand SIB 1 and the need for case 1 for multi-vendor RAN

Document for: Decision

Agenda Item: 9.3.1.5

## 1	Introduction

During 3GPP TSG RAN Meeting#104 it was agreed to have a checkpoint in RAN#105 for normative work for on-demand SIB1 objective within “Enhancements of network energy savings for NR” work item (RP-241650).

RAN WG1 and RAN WG2 discussed 3 scenarios for the support of on demand SIB1 within Release 19 as seen in Figure 1 and described below:

- Case 1: Standalone, where configuration to request SIB1, SIB1 request and SIB1 are delivered within the SIB1-less NES cell

- Case 2: Anchor: configuration to request SIB1 is delivered by an Anchor cell, but SIB1 request and SIB1 are delivered within the SIB1-less NES cell

- Case 3: Anchor: configuration to request SIB1, SIB1 request and SIB1 (for the SIB1-less NES cell) are delivered by the Anchor cell

Figure 1 - On Demand SIB1 scenarios.

The following RAN1 conclusions were done in RAN1#118 [2] for the three scenarios, highlighting in particular their feasibility:

Agreement For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective. - RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for Case 3 Conclusion For on-demand SIB1 in idle/inactive mode, RAN1 was not able to achieve consensus to support Case 1 (Option 1+A+X) in Rel-19, while Case 1 is technically possible from RAN1’s perspective. Agreement RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19. - Note: RAN1 strive to minimize impact to legacy UE. - Note: RAN1 specification impact to support this feature should be minimized.

## 2	Discussion on “on demand SIB1 Operation” and

## weaknesses of case 2

This discussion addresses concerns related to the applicability of case 2 in real-world network deployments, particularly regarding its potential limitations and impact on UE performance.

Limited Applicability:

Coverage Dependency: case 2 relies heavily on the presence of a 5G "coverage layer" in all areas where NES cells are deployed. However, this may not be feasible in all situations. There is a significant risk that UEs might prioritize connecting to an LTE layer with higher priority over the 5G coverage layer, thereby missing the opportunity to receive the SIB1 configuration necessary for requesting further system information, thus reducing the usage of 5G.

Impact on UE Battery Life:

Barring Cycle: When a UE is connected to an LTE layer, it will continue to measure neighboring frequencies and may detect an NES cell transmitting SSBs. In such cases, the UE will attempt to acquire the Master Information Block (MIB) and SIB1. Since SIB1 is not transmitted in case 2, the UE will bar the cell for 300 seconds and then reattempt to measure the cell after this period. This repetitive process can significantly drain the UE's battery, particularly for stationary UEs, leading to a potentially never-ending cycle of measurement and barring.

Initial Cell Selection Issues:

Cell Selection: During initial cell selection, a UE might first discover a cell that lacks SIB1. If SIB1 is absent, the UE will bar the cell for 300 seconds. Meanwhile, the UE may find an "anchor" cell and receive a configuration to request SIB1, but the original SIB1-less cell remains barred. During this period (300 seconds), the UE could switch to another layer (e.g., LTE) and may delete the configuration, leading to the same situation as mentioned earlier.

Observation 1: Case 2 has limited applicability due to the need of an Anchor cell which also raises issues for UE Battery Life and the Initial cell selection procedure.

Observation 2: In order to provide continued support for legacy NR devices, SIB1-less operation is mainly applicable to new NR bands e.g. upper 6 GHz. This observation is valid for all of cases 1, 2, 3 above.

While e.g. upper 6 GHz might normally only be deployed on macros that have e.g., n78 deployed, there could be smaller cell deployments of upper 6 GHz too. Enabling energy saving on the smaller cells would be useful. For competitive procurement of the smaller cells, it is crucial not to be dependent on the macro vendor in delivering the WUS configuration.

Observation 3: Case 1 allows independent rollout of macro network and smaller cells and avoids any interoperability issues between different macro and small cell vendors.

## 3	Potential Solutions for Case 1

During discussions, operators noted that the information in SIB1 does not frequently change. It is also likely that SIB1 could be used on specific frequencies (e.g. in Band n104, upper 6 GHz) where legacy UEs do not operate (considering that majority of the devices are legacy pre-release 19 UEs). Thus, the configuration to request SIB1 could be kept static and minimal.

Potential solutions for a UE to obtain UL WUS configuration directly from the NES Cell (case 1) are described in e.g. R1-2407103

Observation 4: Case 1 is improving both mobile and network performance. Its inclusion in Rel 19 could enhance the adoption of the SIB1-less feature, making it more viable in various scenarios.

The co-signing companies recommend supporting Case 1 in Rel 19 in addition to case 2.

## Conclusion

Observation 1: Case 2 has limited applicability due to the need of Anchor cell which also raise issues for UE Battery Life and the Initial cell selection procedure.

Observation 2: In order to provide continued support for legacy NR devices, SIB-1-less operation is mainly applicable to new NR bands e.g. upper 6 GHz. This observation is valid for all of cases 1, 2, 3 above.

Observation 3: case 1 allows independent rollout of macro network and smaller cells and avoids any interoperability issues between different macro and small cell vendors.

Observation 4: Case 1 is improving both mobile and network performance. Its inclusion in Rel 19 could enhance the adoption of the SIB1-less feature, making it more viable in various scenarios.

Proposal 1: The co-signing companies recommend supporting Case 1 in Rel 19 in addition to case 2.

## References:

- R1-2407103, “On-demand SIB1 for NES”, Fraunhofer IIS, Fraunhofer HHI, Vodafone, Deutsche Telekom, CEWiT, 3GPP TSG RAN WG1 #118, Maastricht, Netherlands, August 19th – 23rd, 2024

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #118, Maastricht, Netherlands, August 19th – 23rd, 2024