3GPP TSG-RAN WG4 Meeting #112	R4-2411139

# Maastricht, Netherlands,  August 19th - August 23th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.741 | CR | 0002 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_LSband-Core) Correction of NS_05N in band n254 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Globalstar |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_LSband-Core |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NS_05N 5MHz channel features two A-MPR regions, A1 and A3. However, RB region A3 was incorrectly implemented in Table 6.2.1.3a-3. Thus, this CR updates the NS_05N A-MPR region definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 6.2.1.3a-3, the A-MPR region A3 for 5MHz channel from NS_05N is corrected accordingly: RB_start is changed from ‘<= 3.6’ to ‘>=3.6’ and LCRB is changed from ‘>0.36’ to ‘>0’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NS_05N 5MHz channel will have incorrect definition of the A-MPR regions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.101-5 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.2.1.3a	NS values and associated A-MPR

For the 5MHz channel, Figure 6.2.1.3a-1 below presents a graphical summary of the required power back-off values for different 5MHz channels:

-	The 5MHz channel right at the lower edge of the band requires power back-off even for small RB allocations irrespective of the fact whether it is FCC or ETSI regulations. If the channel is shifted by 1.4MHz, then at least small RB allocations will not require power back-off exceeding baseline MPR requirements. Shifting the 5MHz channel beyond the 1615.7MHz centre frequency results in no A-MPR at all.

-	As for the upper edge of the band, the 5MHz channel under the FCC regulations will not require any additional A-MPR requirements even if it is right at the edge because there are no out-of-band emission requirements for the upper frequencies. However, with the ETSI regulations we will need to provide offset of 1.6MHz, upon which we will not be exceeding baseline MPR requirements.

-	Because of the same regulations for the lower frequencies, A-MPR values can be the same for NS_03N (FCC) and NS_04N (ETSI lower range) at centre frequencies 1612.5 <= fc < 1615.7MHz. Furthermore, it is possible to split A-MPR values into two regions – 1612.5 <= fc < 1613.9MHz and 1613.9 <= fc < 1615.7MHz – whereupon the second region will not require A-MPR for small RB allocations.

-	There is no need for A-MPR for NS_03N (FCC) at centre frequencies above 1615.7MHz, but NS_05N (ETSI upper range) requires A-MPR for center frequencies at 1622.4 < fc <= 1624MHz.

Figure 6.2.1.3a-1: Summary of required power back-off regions for the 5MHz channel for FCC (top) and ETSI (bottom) regulations.

For the 10MHz channel:

-	For the channel right at the lower edge of the band, A-MPR might be needed even for small RB allocations at the channel edges because of the strict out-of-band emission requirements. Shifting the channel farther away from the lower edge basically improves A-MPR with the switching point at the centre frequency of around 1620.1MHz (i.e. 1.4MHz offset from the upper edge), after which at least for the ETSI requirements emission requirements from the upper frequencies start influencing allocations at the channel’s upper edge. Based on that A-MPR values can be considered the same for NS_03N (FCC) and NS_05N (ETSI) for the centre frequencies at 1615 <= fc < 1620.1MHz.

-	For the channel right at the upper edge of the band, A-MPR is generally not needed for small allocations under the FCC regulations, which is reflected in the corresponding A-MPR for NS_03 for the channel at 1621.5MHz. However, with NS_05N (ETSI) A-MPR is still required because of the out-of-band emission requirements for upper frequencies.

-	As can be seen from Figure 6.2.1.3a-2 below, the only major difference between the FCC and ETSI A-MPR regions is that the former can have optimised 10MHz channel placed right at the upper edge of the band.

Figure 6.2.1.3a-2: Summary of required power back-off regions for the 10MHz channel for FCC (top) and ETSI (bottom) regulations.

For the 15MHz channel, more or less same A-MPR is needed for NS_03N (FCC) and NS_05N (ETSI) irrespective of the fact where exactly the channel is configured and whether it is under the FCC or ETSI regulations.

Figure 6.2.1.3a-3: A-MPR regions for NS_03N (5, 10 and 15MHz channels).

Table 6.2.1.3a-1: A-MPR values for NS_03N (FCC)

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc < 1621.5 |  | <= 6.48 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  | fc = 1621.5 |  | >= 7.2 | A1 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 | 4.0 | 6.5 | 1.5 |
|  | QPSK | 2.5 | 4.0 | 2.5 | 6.0 | 7.0 | 2.0 |
|  | 16QAM | 3.0 | 4.5 | 3.0 | 6.5 | 7.5 | 2.5 |
|  | 64QAM | 3.5 | 5 | 3.5 | 7 | 8 | 3 |
|  | 256QAM | 4.5 | 6 | 4.5 | 8 | 9 | 4 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 256QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |

Figure 6.2.1.3a-4: A-MPR regions for NS_04N (5MHz channels).

Table 6.2.1.3a-2: A-MPR values for NS_04N (ETSI lower sub-range)

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |

|  | Modulation | A1 | A2 | A3 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 |
|  | QPSK | 2.5 | 4.0 | 2.5 |
|  | 16QAM | 3.0 | 4.5 | 3.0 |
|  | 64QAM | 3.5 | 5 | 3.5 |
|  | 256QAM | 4.5 | 6 | 4.5 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 |
|  | 256QAM | 3.5 | 6.0 | 4.0 |

Figure 6.2.1.3a-5: A-MPR regions for NS_05N (5, 10 and 15MHz channels).

Table 6.2.1.3a-3: A-MPR values for NS_05N (ETSI upper sub-range)

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1622.4 < fc <= 1624 | >= 3.6 | > 0 | A3 |
|  |  |  | >= 2.88 | A1 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc <= 1621.5 |  | <= 7.2 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  |  | > 7.2 | > 0 | A6 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 1.5 | 5.0 | 1.5 | 6.5 | 6.5 | 2.0 |
|  | QPSK | 1.5 | 5.0 | 1.5 | 6.5 | 7.0 | 2.5 |
|  | 16QAM | 2.0 | 5.5 | 2.0 | 7.0 | 7.5 | 3.0 |
|  | 64QAM | 2.5 | 6 | 2.5 | 7.5 | 8 | 3.5 |
|  | 256QAM | 3.5 | 7 | 3.5 | 8.5 | 9 | 4.5 |
| CP-OFDM | QPSK | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 16QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 64QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 256QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |