3GPP TSG-RAN WG1 Meeting #118 	R1-2407402

Maastricht, Netherlands, August 19th – 23rd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0657 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to TS 38.213 on OLPC for SL PRS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Intel Corporation), vivo, ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 16.2.3A: - Based on Rel-18 agreement, SL pathloss-based OLPC for SL PRS should share the same principle as PSSCH power control. That is, SL pathloss based OLPC can only be used in unicast and if a higher layer filtered RSRP is reported to the UE transmitting the SL PRS from the UE intended to receive the SL PRS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 16.2.3A: - Add condition for SL pathloss-based OLPC for SL PRS, ie., if a SCI format scheduling the SL PRS transmission includes a cast type indicator field indicating unicast, and if a higher layer filtered RSRP is reported to the UE transmitting the SL PRS from the UE intended to receive the SL PRS.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Description of SL pathloss-based OLPC for SL PRS is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 16.2.3A	SL PRS

A UE determines a power PSL-PRSi for a SL PRS transmission on a resource pool in SL PRS transmission occasion i on active SL BWP b of carrier f as:

PSL-PRSi=minPCMAX,PMAX,CBR,minPSL-PRS,Di, PSL-PRS,SLi

where,

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PMAX,CBR is determined by

-	if the resource pool is a shared SL PRS resource pool, a value of sl-MaxTxPower based on a priority level and a CBR range for a CBR measured in slot i-N, where N is the congestion control processing time [6, TS 38.214]; if sl-MaxTxPower is not provided, PMAX,CBR=PCMAX. The priority level is same for PSSCH and SL PRS

-	if the resource pool is a dedicated SL PRS resource pool, a value of sl-PRS-MaxTx-Power based on a priority level and a CBR range for a CBR measured in slot i-N, where N is the congestion control processing time [6, TS 38.214]; if sl-PRS-MaxTx-Power is not provided, PMAX,CBR=PCMAX. The priority level is for SL PRS

-	if a value for PO,D is provided

-	PSL-PRS,Di=PO,D+10log102μ∙MRBSL-PRSi+αD∙PLD [dBm]

-	else

-	PSL-PRS,Di=minPCMAX,PMAX,CBR [dBm]

where

-	if the resource pool is a shared SL PRS resource pool, PO,D is a value of dl-P0-PSSCH-PSCCH or dl-P0-PSSCH-PSCCH-r17; else, if the resource pool is a dedicated SL PRS resource pool, PO,D is a value of dl-P0-SL-PRS

-	if the resource pool is a shared SL PRS resource pool, αD is a value of dl-Alpha-PSSCH-PSCCH, if provided, and αD=1 if dl-Alpha-PSSCH-PSCCH is not provided; else, if the resource pool is a dedicated SL PRS resource pool, αD is provided by dl-Alpha-SL-PRS, if provided, and αD=1 if dl-Alpha-SL-PRS is not provided

-	PLD=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that

-	the RS resource qd is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c

-	the RS resource qd is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c

-	MRBSL-PRS(i) is a number of resource blocks for the SL PRS transmission occasion i and μ is a SCS configuration for the SL PRS transmission

-	if a value for PO,SL is provided, if a SCI format scheduling the SL PRS transmission includes a cast type indicator field indicating unicast, and if a higher layer filtered RSRP is reported to the UE transmitting the SL PRS from the UE intended to receive the SL PRS

-	PSL-PRS,SLi=PO,SL+10log102μ∙MRBSL-PRSi+αSL∙PLSL [dBm]

-	else

-	PSL-PRS,SLi=minPCMAX,PSL-PRS,Di [dBm]

where

-	if the resource pool is a shared SL PRS resource pool, PO,SL is a value of sl-P0-PSSCH-PSCCH or sl-P0-PSSCH-PSCCH-r17; else, if the resource pool is dedicated for SL PRS transmissions, PO,SL is a value of sl-P0-SL-PRS

-	if the resource pool is a shared SL PRS resource pool, αSL is a value of sl-Alpha-PSSCH-PSCCH, if provided and αSL=1 if sl-Alpha-PSSCH-PSCCH is not provided; else, if the resource pool is a dedicated SL PRS resource pool, αSL is provided by sl-Alpha-SL-PRS if provided, and αSL=1 if sl-Alpha-SL-PRS is not provided

-	PLSL=referenceSignalPower – higher layer filtered RSRP, where

-	referenceSignalPower is obtained

-	if the resource pool is a shared SL PRS resource pool, from a PSSCH transmit power per RE summed over the antenna ports of the UE and higher layer filtered across PSSCH transmission occasions using a filter configuration provided by sl-FilterCoefficient,

-	else, if the resource pool is a dedicated SL PRS resource pool, from a SL PRS transmit power per RE and higher layer filtered across SL PRS transmission occasions using a filter configuration provided by sl-FilterCoefficient

-	higher layer filtered RSRP is a RSRP, as defined in [7, TS 38.215], that is reported to the UE from a UE receiving the SL PRS transmission and is obtained

-	if the resource pool is a shared SL PRS resource pool, from a PSSCH DM-RS using a filter configuration provided by sl-FilterCoefficient

-	else, if the resource pool is a dedicated SL PRS resource pool, from a SL PRS using a filter configuration provided by sl-FilterCoefficient

-	MRBSL-PRS(i) is a number of resource blocks for the SL PRS transmission occasion i and μ is a SCS configuration for the SL PRS transmission

<Unchanged text omitted>